3GPP TSG-RAN WG1 Meeting #94	R1-1809532

Gothenburg, Sweden, August 20th – 24th 2018

Agenda Item:	6.2.2.1

Source:	Ericsson

Title:	Feature summary of 6.2.2.1 UE-group wake-up signal

Document for:	Discussion, Decision

# 1	Introduction

12 contributions consider UE-group wake-up signal [1]-[12]. In addition, WUS UE-grouping was treated at RAN1 #94 in an earlier session, where the following agreements were made [13]:

Agreement:

UE-group WUSs are only multiplexed in the same NB as associated PO

- FFS TDM/FDM/CDM for UE-group MWUS multiplexing

Agreement

Rel-16 UE-group MWUS sequence should consider at least

- Fallback to legacy UE behavior

- Inter-cell interference randomization

- UE group ID for different UE-group MWUS

- Reuse of Rel-15 sequences is not precluded

- Effect of sequence detection on UE complexity

# 2	Discussion

The core of the different contributions is summarized below based on topic. Based on the positions of the different companies, feature lead proposals are presented that may be possible to agree upon.

### Introduce UE-grouping

Whether it can be agreed already now to introduce UE-grouping:

Yes:			Huawei, vivo

Further study:	Ericsson, Nokia, DoCoMo, ZTE

WUS UE-grouping should be studied further before agreed to be implemented.

### UE-group configuration

How UE-group configuration should be provided:

SIB:	Intel, Qualcomm, vivo, ZTE

RRC:	MediaTek

Configuration of WUS UE-grouping is transmitted in system information.

### # UE groups

The (maximum) number of UE-groups:

Max #

2:	Intel, ZTE

3:	Nokia

4:	Nokia, Qualcomm

≤16:	(Sony)

Whether that number is static or configurable:

Static:

Configurable:	Nokia, Qualcomm, Sony

The number of UE-groups is configurable, and the maximum number of groups is FFS.

### UE-group multiplexing

How UE-groups should be multiplexed in NB-IoT:

TDM:					Qualcomm, Samsung

TDM+CDM:				Intel, LGE, Samsung

CDM:					Ericsson, vivo

FFS:					Ericsson, ZTE

For CDM, two alternatives exist:

- Multigroup detection, where UEs are required to detect multiple sequences

- Superpositioning, where multiple sequences are superimposed

CDM multigroup detection:	Ericsson, Huawei, Intel, LGE, ZTE

CDM superpositioning:		vivo

### UE-group differentiation

What to base UE-group differentiation on:

UE ID:	Ericsson, Huawei, Intel, LGE, MediaTek, Qualcomm, ZTE

Coverage:	LGE

DRX/eDRX:	Nokia

Gap size:	Nokia

Service:	LGE

Send LS to RAN2 to determine base for UE-grouping, where RAN1 sees the following alternatives:

  - UE ID

  - Coverage

  - DRX/eDRX

  - Gap configuration

  - Services

### Management of Legacy WUS

Whether Rel-16 WUS UE-grouping is compatible with Rel-15 WUS:

Further study:		Nokia, Qualcomm, ZTE

Backwards compatible:	Ericsson (inband), LGE, Sony

Co-existence between Rel-16 WUS UE-grouping and legacy WUS is FFS.

## Sequence design

How to design sequences, if needed:

Init seed and phase shifts:	Huawei

Scrambling:				LGE

Cover codes:			LGE

## LTE-MTC agreements

The following proposals are based on previous agreements from the earlier LTE-MTC session. Changes are presented with yellow highlighting.

UE-group WUSs are only multiplexed in the same NBcarrier as associated PO

- FFS TDM/FDM/CDM for UE-group NWUS multiplexing.

Rel-16 UE-group NWUS sequence should consider at least

- Fallback to legacy UE behaviour

- Inter-cell interference randomization

- UE group ID for different UE-group NWUS

- Reuse of Rel-15 sequences is not precluded

- Effect of sequence detection on UE complexity

# Conclusion

Based on the feature lead summary in the previous section, the following feature lead proposals are made:

Proposal 1	WUS UE-grouping should be studied further before agreed to be implemented.

Proposal 2	Configuration of WUS UE-grouping is transmitted in system information.

Proposal 3	The number of UE-groups is configurable, and the maximum number of groups is down selected between 2, 3, 4, 8.

Proposal 4	Send LS to RAN2 to determine base for UE-grouping, where RAN1 sees the following alternatives:

  - UE ID

  - Coverage

  - DRX/eDRX

  - Gap configuration

  - Services

Proposal 5	Co-existence between Rel-16 WUS UE-grouping and legacy WUS is FFS.

Proposal 6	UE-group WUSs are only multiplexed in the same NB as associated PO

- FFS TDM/FDM/CDM for UE-group MWUS multiplexing.

Proposal 7	PORel-16 UE-group MWUS sequence should consider at least

- Fallback to legacy UE behaviour

- Inter-cell interference randomization

- UE group ID for different UE-group MWUS

- Reuse of Rel-15 sequences is not precluded

- Effect of sequence detection on UE complexity

# References

R1-1808043, UE-group wake-up signal in NB-IoT, Ericsson, RAN1 #94, Gothenburg, Sweden, August 2018.

R1-1808107, On support for UE-group wake-up signal, Huawei, RAN1 #94, Gothenburg, Sweden, August 2018.

R1-1808216, On UE-group wake-up signal for NB-IoT, vivo, RAN1 #94, Gothenburg, Sweden, August 2018.

R1-1808439, UE group wake-up signal for NB-IoT, Nokia,

R1-1808474, Discussion on UE-grouping wake up signal in NB-IoT, LG Electronics,

R1-1808640, Discussion on Wake-up signal for NB-IoT, ZTE,

R1-1808660, UE-group wake-up signal for NB-IoT, Intel, RAN1 #94, Gothenburg, Sweden, August 2018.

R1-1808737, UE-group wake-up signal for NB-IoT, Samsung, RAN1 #94, Gothenburg, Sweden, August 2018.

R1-1808959, UE-Group WUS in NB-IoT, MediaTek, RAN1 #94, Gothenburg, Sweden, August 2018.

R1-1809031, UE-group wake-up signal, Qualcomm, RAN1 #94, Gothenburg, Sweden, August 2018.

R1-1809131, UE-group wake-up signal for Rel.16 IoT, NTT DOCOMO, RAN1 #94, Gothenburg, Sweden, August 2018.

R1-1809483, UE-group wake-up signal for NB-IoT, Sony, RAN1 #94, Gothenburg, Sweden, August 2018.

Session Notes for Agenda Item 6.2.1, Samsung, RAN1 #94, Gothenburg, Sweden, August 2018.