3GPP TSG RAN WG1 Meeting #94	R1-1809659

Gothenburg, Sweden, August 20 – 24, 2018

Agenda Item:	6.2.3

Source:	Huawei, HiSilicon

Title:	Outcome of offline discussion for additional SRS symbols

Document for:	Discussion and decision

Offline consensus:

The time location of possible R16 SRS symbols in one normal UL subframe for a cell are FFS:

- Option 1: All symbols in one slot can be used for SRS from cell perspective

  - E.g., The other slot in the subframe may be used for PUSCH transmission for sTTI-capable UE.

- Option 2: All symbols in one subframe can be used for SRS from cell perspective

- Option 3: A subset of symbols in one slot can be used for SRS from cell perspective

  - E.g., The other slot in the subframe may be used for PUSCH transmission for sTTI-capable UE.

- Note: Companies are encouraged to provide evaluations for down-selection

- Other options are not precluded

- FFS whether it has spec impacts or not for cell-specific configuration

Offline conclusion:

Companies are encouraged to submit evaluations results based on following assumptions as a starting point, which is to be checked in next meeting. Baseline:

| ​Parameter | Value |
| --- | --- |
| Carrier frequency​ | 2 GHz​ |
| Bandwidth​ | 20 MHz​ ( or 10MHz) |
| Duplex​ | TDD, LTE Configuration 2 (DL-to-UL ratio = 4, [U D D D D] )​ |
| Subcarrier spacing​ | 15 kHz​ |
| Polarization​ | eNB: X pole (+/-45° )​ UE: + pol (0/90°)​ |
| Macro eNB antenna array | (M,N,P) = (2,4,2) with vertical spacing 0.8λ and horizontal spacing 0.5λ |
| Macro eNB antenna element model​ (only applicable to CDL and other details FFS) | Elevation beamwidth = 65º​ |
|  | Azimuth beamwidth = 65º​ |
| UE Speed​ | 3km/h, 30km/h, 60km/h​ |
| UE antenna | 2 receive antennas Isotropic (0 dBi) ​ |