3GPP TSG RAN WG1 Meeting #94bis	R1-1811158

Chengdu, China, October 8th – 12th, 2018

Source:	Sharp

Title:	Evaluation Results of Area traffic capacity for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

Introduction

In RAN#81, the evaluation data of eMBB, mMTC, and URLLC were collected for initial ITU submission. The evaluation needs to continue and update for the final ITU submission. In this contribution, we provide our evaluation result of area traffic capacity.

Area traffic capacity

As defined in [1], the area traffic capacity is calculated using a TRxP density, a channel bandwidth, and a DL average spectral efficiency. The minimum requirement of area traffic capacity is defined in [2].

In InH-eMBB test environment, TRxPs are placed in simulation area of 120 m ×50 m [1]. In 12 TRxPs case, the TRxP density equals to 0.002 (TRxP/m2). The DL average spectral efficiency is obtained from system level evaluation, that is 11.029 (bit/s/Hz). The detailed evaluation results and corresponding assumptions under evaluation configuration for InH-eMBB config. B are described in our companion contribution [3]. With the evaluation results, using 2 component carriers with 400 MHz bandwidth as the maximum value for 120 kHz SCS, the area traffic capacity is calculated as in Table 1.

- Table 1. Area traffic capacity

| InH-eMBB Config. B |  |  |  |  |
| --- | --- | --- | --- | --- |
| TRxP density (TRxP/m2) | channel bandwidth (MHz) | Average spectral efficiency (bit/s/Hz) | Area traffic capacity (Mbit/s/m2) | Requirement (Mbit/s/m2) |
| 0.002 | 800 | 11.029 | 17.646 | 10 |

Observation 1: NR satisfies the area traffic capacity in indoor hotspot-eMBB configuration B when assumed channel bandwidth is 800 MHz.

Conclusion

In this contribution, we show our evaluation results on area traffic capacity. According to our results, we have the following observation:

Observation 1: NR satisfies the area traffic capacity in indoor hotspot-eMBB configuration B when assumed channel bandwidth is 800 MHz.

References

- ITU-R M2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Oct. 2017.

- ITU-R M2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- R1-1811157, “Evaluation Results of Spectral efficiency in InH eMBB for IMT-2020 self-evaluation”, sharp, 3GPP TSG RAN WG1 Meeting #94bis, Oct. 2018.