3GPP TSG-RAN WG1 Meeting #94bis	R1-1811515

Chengdu, People's Republic of China, October 8th – 12th 2018

Agenda Item:	6.1.5.3.1

Source:	Ericsson

Title:	Maintenance issues for TDD DL aspects for Rel-15 NB-IoT

Document for:	Discussion and Decision

# 1	Introduction

Currently it is captured in Section 10.2.3.4 TS36.211 that

“

For NPDSCH carrying BCCH, the yp(k-1)Msymbap, …, ypkMsymbap-1 is mapped to  subframes in sequence and then repeated until MrepNPDSCHNSF subframes have been transmitted, where

-	k=2 for mapping NPDSCH carrying SystemInformationBlockType1-NB to subframe #3 for frame structure type 2;

-	k=1 otherwise.

”.

Frame structure type 2 is used for NB-IoT TDD, and additional SIB1-NB repetition in subframe#3 only applies for FDD. Therefore, it is proposed to correct the text.

# 2	Text Proposal

----------------------------------------------------------------- Text start -----------------------------------------------------------------

#### 10.2.3.4	Mapping to resource elements

------------------------------------------------------------Unchanged text omitted ----------------------------------------------------

For NPDSCH carrying BCCH, the yp(k-1)Msymbap, …, ypkMsymbap-1 is mapped to  subframes in sequence and then repeated until MrepNPDSCHNSF subframes have been transmitted, where

-	k=2 for mapping NPDSCH carrying SystemInformationBlockType1-NB to subframe #3 for frame structure type 1;

-	k=1 otherwise.

------------------------------------------------------------Unchanged text omitted ----------------------------------------------------

------------------------------------------------------------------Text end ------------------------------------------------------------------