3GPP TSG RAN WG1 Meeting #94bis	R1-1810102

Chengdu, China, October 8th – 12th, 2018

Agenda Item:	7.1.2.5

Source:	Huawei, HiSilicon

Title:	Remaining issues on simultaneous reception or transmission over CC/BWP(s)

Document for:	Discussion and Decision

- Introduction

According to [1][2], to deal with simultaneous reception/transmission, RAN1 will lead the discussion on several cases as listed in Table 1.

Table 1 R1-1809882 Guidance on simultaneous reception of channels or RS

| Category | RAN1 | RAN4 |
| --- | --- | --- |
| SSB+PDCCH/PDSCH |  | RAN4 to clarify |
| SSB+CSIRS | Discuss in MIMO, SSB+(CSIRS TRS, CSIRS-CSI, CSI-IM) | RAN4 to clarify other combinations |
| CSIRS+PDCCH/PDSCH | Discuss in MIMO, (CSIRS TRS, CSIRS-CSI, CSI-IM) + PDCCH/PDSCH | RAN4 to clarify other combinations |
| PDCCH+PDSCH | Discuss in MIMO |  |
| CSIRS+CSIRS | Discuss in MIMO (except CSIRS-RRM/RLM/BFD/L1-RSRP + CSIRS-RRM/RLM/BFD/L1-RSRP) |  |
| PDCCH+PDCCH | Discuss in DL control |  |
| PDSCH + PDSCH | Discuss in DL control |  |
| PUSCH + PUSCH | Discuss in MIMO |  |
| SRS + (SRS, PUCCH, PUSCH) | Discuss in MIMO |  |
| PUCCH + PUCCH | Discuss in UL control |  |
| PUSCH + PUCCH | Discuss in UL control |  |
| PRACH+ (SRS, PUCCH, PUSCH) | Discuss in UL control |  |

In this paper, we provide some general considerations and our preference on the specific combinations listed above.

- Default QCL/spatial-relation for simultaneous reception/transmission

The general principle of prioritization is based on the integrity of the physical channel/signal, the importance of the physical channel/signal, and the numerology and CC index when it comes to simultaneous reception/transmission across carriers.

In NR, different numerologies can be configured for different CCs. Thereby, it is possible that, in terms of time duration, one symbol in CC x corresponds to several symbols in CC y, as shown in Figure 1. The question is: if different QCL assumptions are applied for PDCCH on CC0@60KHz and PDCCH on CC1@120KHz, which QCL should be assumed? If the CC with a larger SCS is prioritized (i.e., CC1 has priority over CC0 in Figure 1), on the first CC1 symbol @120KHz, UE shall assume beam1, on the second CC1 symbol, UE shall switch to beam0 to receive CC0 symbol since there is no signal transmitted on CC1 now or there is no simultaneous reception issue. However, this is not a reasonable solution, since UE missed the first half of the CC0 symbol. Actually, the reception of PDCCH in CC0 may even be split into several parts with different QCL assumptions.

Figure 1.  Overlapping between one signal and several signals in two CCs

Considering the integrity of received signal, it is more reasonable to guarantee that the whole single symbol can be received by one QCL assumption, thus when a priority rule by SCS is considered, default QCL assumption is determined by the reception signal in the CC with the lowest SCS, i.e., with the longest symbol duration.

When multiple CCs are configured with the same SCS, or on a single CC/BWP, the determination of default QCL assumption can be based on a predefined prioritization of the processing of one physical signal over another, considering the importance of different physical channel/signals.

In downlink, physical channels having a broadcast nature (e.g., common PDCCH/PDSCH) come first since they may contain the most important information such as paging or system information update. Then unicast PDCCH contains the scheduling information for unicast PDSCH and/or CSI-RS and it should be certainly prioritized. Data channel usually carries more useful information than reference signal and its reception should be guaranteed, except for the case that a set of CSI-RS resources is scheduled for UE Rx beam refinement, for which, UE is not expected to receive PDSCH on CSI-RS symbols if the CSI-RS resource set is configured with repetition “ON”. In addition, Since CSI-RS may have multiple functionalities in NR, detailed rules can be added for different types of CSI-RS. CSI-RS for radio link quality monitoring (e.g., for RLM/beam failure detection) is to ensure that the UE is still in good connection, CSI-RS for BM is to align the directional transmission and reception, and CSI-RS for CSI acquisition is mainly based on the results of BM. Thus, it is easy to sort the order as CSI-RS for RLM/BFR > CSI-RS for BM > CSI-RS for CSI acquisition.

In the uplink, similar logic applies, i.e., control channel > data channel > reference signal. In addition, the random access channel is the one responsible for accessing the network, which should naturally come to the top priority in uplink.

To summarize, the priority order for DL/UL can be as follows:

DL: Broadcast PDCCH/PDSCH > unicast PDCCH > unicast PDSCH > CSI-RS

Wherein, CSI-RS for RLM/BFR > CSI-RS for BM > CSI-RS for CSI acquisition

UL: PRACH > PUCCH > PUSCH > SRS

Wherein, SRS for BM > SRS for NCB > SRS for CB

For PDSCH/PUSCH transmitted on two CCs, if there is no difference in numerology, RNTI type can be used to determine the priority since different RNTIs represent different types of content carried in the data channel. For example, P-RNTI / SI-RNTI / RA-RNTI indicate that scheduled data channel contains paging information/system information/RAR respectively, which are all key information for the UE to be maintained/to access the network. Moreover, the transmission scheduled in a grant free fashion is naturally prioritized.

To summarize, the priority order for RNTI of PDSCH/PUSCH is given as follows:

PDSCH: P-RNTI / SI-RNTI / RA-RNTI > CS-RNTI (Grant free) > C-RNTI (Grant based)

PUSCH: CS-RNTI (Grant free) > C-RNTI (Grant based) > SP-CSI-RNTI

In addition to the rules mentioned above, if the signal type, SCS, and RNTI is the same for channels/RSs on different CCs, the primary CC should have the higher priority.

In summary, the priority of channels or signals can be determined based on the following rules.

Proposal 1: The default QCL assumption or spatial relation for multiple channel/signal(s) among multiple CCs is determined as follows:

Rule#1: SCS in increasing order;

Rule#2: Type of physical channel/signal;

- DL: Broadcast PDCCH/PDSCH > unicast PDCCH > unicast PDSCH > CSI-RS

  - CSI-RS for RLM/BFR > CSI-RS for BM > CSI-RS for CSI acquisition

- UL: PRACH > PUCCH > PUSCH > SRS

  - SRS for BM > SRS for NCB > SRS for CB

Rule#3: RNTI of DCI scheduling data channel;

- PDSCH: P-RNTI / SI-RNTI / RA-RNTI > CS-RNTI (Grant free) > C-RNTI (Grant based)

- PUSCH: CS-RNTI (Grant free) > C-RNTI (Grant based) > SP-CSI-RNTI

Rule#4: CC index in increasing order;

Based on the general rules in Proposal 1, on the remaining issues that will be discussed in MIMO session, we provide our preferences as follows.

Table 2 Preference on simultaneous reception of channels or RS for the cases in R1-1809882

| Category | Same CC/BWP | Different CCs/BWPs |
| --- | --- | --- |
| SSB+(CSIRS TRS, CSIRS-CSI, CSI-IM) | NW implementation | NW implementation |
| (CSIRS TRS, CSIRS-CSI, CSI-IM) + PDCCH/PDSCH | PDCCH/PDSCH is prioritized | Firstly the priority is based on the SCS in increasing order, if the SCS were the same, then follow the rule as in the left column. |
| PDCCH+PDSCH | Broadcast PDCCH/PDSCH > unicast PDCCH > unicast PDSCH | Firstly the priority is based on the SCS in increasing order, if the SCS were the same, then follow the rule as in the left column. |
| CSIRS+CSIRS (except CSIRS-RRM/RLM/BFD/L1-RSRP + CSIRS-RRM/RLM/BFD/L1-RSRP) | Aperiodic > semi-persistent > Periodic | Firstly the priority is based on the SCS in increasing order, if the SCS were the same, then follow the rule as in the left column. |
| PUSCH + PUSCH | Depends on the RNTI of DCI scheduling data channel: CS-RNTI (Grant free) > C-RNTI (Grant based) > SP-CSI-RNTI | Firstly the priority is based on the SCS in increasing order, if the SCS were the same, then follow the rule as in the left column. |
| SRS + (PUCCH, PUSCH) | PUCCH/PUSCH is prioritized | Firstly the priority is based on the SCS in increasing order, if the SCS were the same, then follow the rule as in the left column. |
| SRS + (SRS) | Aperiodic > semi-persistent > Periodic | Firstly the priority is based on the SCS in increasing order, if the SCS were the same, then follow the rule as in the left column. |

- Summary of proposals

Based on the above discussion, we have the following proposals:

Proposal 1: The default QCL assumption or spatial relation for multiple channel/signal(s) among multiple CCs is determined as follows:

Rule#1: SCS in increasing order;

Rule#2: Type of physical channel/signal;

- DL: Broadcast PDCCH/PDSCH > unicast PDCCH > unicast PDSCH > CSI-RS

  - CSI-RS for RLM/BFR > CSI-RS for BM > CSI-RS for CSI acquisition

- UL: PRACH > PUCCH > PUSCH > SRS

  - SRS for BM > SRS for NCB > SRS for CB

Rule#3: RNTI of DCI scheduling data channel;

- PDSCH: P-RNTI / SI-RNTI / RA-RNTI > CS-RNTI (Grant free) > C-RNTI (Grant based)

- PUSCH: CS-RNTI (Grant free) > C-RNTI (Grant based) > SP-CSI-RNTI

Rule#4: CC index in increasing order;

- References

3GPP, “RAN1 Chairman’s Notes”, Gothenburg, Sweden, August 20th – 24th, 2018

Intel, R1-1809882, “Guidance on simultaneous reception of channels or RS”, Gothenburg, Sweden, August 20th – 24th, 2018