3GPP TSG-RAN WG1 Meeting #95	R1-1812155

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.6.1.2

Source:	Ericsson

Title:	PUSCH Enhancements for NR URLLC

Document for:	Discussion, Decision

# 1	Introduction

In June 2018’s RAN Plenary meeting, the eURLLC study item [1] was approved. The objective of the study item is to study reliability and latency performance supported by NR Rel. 15 and identify further enhancements, if needed, to achieve the requirements. This study item aims to investigate methods to further improve reliability and reduce latency for different use cases (such as factory automation, transport industry and electrical power distribution) that have different requirements, which are potentially stricter than the requirements considered in Rel-15. In this contribution, we discuss our view how to enhance PUSCH transmission to meet the URLLC requirements.

# 2	Discussion

## 2.1	Reliability

In Rel-16 eURLLC SI different relevant use cases can be considered with potentially different reliability requirements. In some use cases, a very strict reliability of 1-10-6 is required as for the example mentioned in [1]. It is worth noting that techniques for enhancing reliability can be done at different layers in the protocol stack. Requiring overall transmission reliability of 1-10-6 does not necessarily mean that all the solutions must come from the physical layer. For example, NR supports higher layer reliability enhancement in the form of PDCP duplication. With PDCP duplication, the reliability requirement on the physical layer can be relaxed.

In NR Rel-15, a new CQI table for CQI report corresponding to 10-5 BLER target was introduced. This aims to support URLLC DL transmission with high reliability requirement. Moreover, a new MCS table supporting new MCS entries with low spectral efficiency values was introduced to support very robust PDSCH and PUSCH transmissions. These PHY reliability enhancements made in NR Rel. 15 can be considered sufficient for eURLLC.

## 2.2	Latency

In terms of latency, NR Rel. 15 supports data transmission with shorter duration than a slot. PDSCH/PUSCH mapping Type B allows a transmission to start in any symbol in a slot, which is highly desirable from a latency viewpoint. For PDSCH mapping Type B, transmission durations of 2, 4, and 7 symbols are supported, while for PUSCH mapping Type B, arbitrary symbol durations from 1 to 14 symbols are supported. These features serve as the key elements to enable low latency transmission required for URLLC.

However, there still exist some limitations in terms of scheduling flexibility in NR Rel-15 to fully enable ultra-low latency transmission. One example is the restriction on scheduling across the slot border. For URLLC services with strict latency budget, it is highly desirable that data can be transmitted as soon as possible. It could happen for example that UL data for an UL transmission is ready to be transmitted (after some processing time at the UE) in a symbol that is too close to the slot border. Since NR Rel.15 does not allow transmissions to cross the slot border, the UE has to wait until the beginning of the next slot to transmit. This can lead to an increased latency which exceeds the allowed budget. Moreover, this restriction is extended to Rel. 16 at least for the grant-based transmission based on the following agreement in the previous meeting:

Agreements (RAN1#94bis):

- One PUSCH transmission instance is not allowed to cross the slot boundary at least for grant-based PUSCH.

See Figure 1, for example, for an illustration of high alignment delay when the arrival of data with 7-symbol duration is too close to the slot border. In the case of a 7-symbol transmission, this alignment delay will occur in 50% of UL transmissions assuming data arriving uniformly. The problem is especially severe for the UL transmission where UE is power-limited since increasing bandwidth does not help to improve the performance.

Slot n

Slot n+1

7os

Alignment delay

Figure 1: Illustration of long alignment delay due to transmission across slot border restriction

An alternative to waiting until the next slot is to schedule multiple transmissions with shorter duration so that the transmission can start already in the present slot. Although NR Rel. 15 supports slot aggregation where a transmission can be repeated over multiple slots, there is a limitation that the TB repetition in the next slots needs to have the same resource allocation as the transmission in the first slot. Therefore, the repetition of any transmission less than 14 symbols across multiple slots will have time gaps between them. See for example Figure 2 for an illustration of mini-slot aggregation, where 4os mini-slot allocation is repeated in two adjacent slots, separated by the 10os time gap between the mini-slots. Although the alignment delay is reduced, the overall latency is not improved with this approach as the receiver in most cases need to accumulate all the repetitions to be able to achieve the desired reliability.

4os

Slot n

Slot n+1

4os

Figure 2: Illustration of slot aggregation in NR Rel. 15 when applied to repetition of short transmissions

To support truly ultra-low latency transmission for eURLLC in Rel. 16 it is proposed that PUSCH scheduling across slot border can be supported. This can come in the forms of different scheduling flexibilities to allow low latency transmission across slot border without excessive delay.

Reduce the alignment delay at the slot boundary by supporting 2-segmented transmissions before and after the slot boundary.

One possible solution to support PUSCH transmission across slot border is to use two-PUSCH transmission (See Fig. 3). That is, a UE can expect to receive an UL grant or a configured UL grant which assign resources in time domain crossing the slot border. The UE then interprets that PUSCH transmission is split into two PUSCH transmissions.

UL data with N-symbol duration is configured or scheduled to cross the slot border.

Slot n

Slot n+1

N os

Slot n

Slot n+1

N1
os

UL data is split into two repetitions. The first PUSCH starts at the configured or assigned starting symbol and ends at the end of the present slot. The second PUSCH starts at the beginning of the subsequent slot and ends at the symbol corresponding to the original configured or scheduled length.

N2
os

Figure 3: Illustration of two-repetition PUSCH transmission

A simple signaling method can, e.g., be based on an implicit signaling by allowing direct indicators of start symbol (S) and allocation length (L) in the time-domain resource allocation to result in S+L > 14. In this case, the first PUSCH starting at the configured or scheduled starting symbol and lasting until the end of the first slot, and the second PUSCH starting immediately in the subsequent slot until the end of scheduled symbol, or symbol 14, whichever comes first. The same TB can be used for both segments of PUSCH transmission and RV can follow a pre-configured RV sequence. For TBs requiring transmission in more than two slots, similar segmentation of PUSCH transmission is applied.

Another approach under consideration is based on mini-slot repetition. Couple of considerations should be taken with respect to mini-slot repetition. Firstly, from the performance perspective, splitting the PUSCH into two PUSCH has an advantage due to improved coding gain in one of the segments as compared to the repetition-based solutions. Secondly, the DMRS overhead in each repetition creates unnecessary additional overhead. Therefore, additional mechanism should be considered to reduce DMRS overhead. Thirdly, repetition-based solution would not guarantee that the symbols around the slot boundary are fully utilized for the PUSCH transmission to reduce the delay. Depending on the data arrival and the allocated PUSCH resource, the repetition factor should be dynamically adapted. Since in Rel-15, slot-aggregation is RRC configured, introducing this feature implies that the dynamic repetition should be supported in Rel-16 to make the feature meaningful.

In addition, the reliability can be improved by frequency hopping. However, consideration should be taken into account whether frequency hopping results in fragmented spectrum, impacting the total system performance. Therefore, frequency hopping should be dynamically enabled or disabled.  Moreover, frequency hopping if enabled, can be performed based on the existing inter-slot and intra-slot frequency hopping. In some cases, it may however not be desirable to have a hopping position in asymmetric fashion with respect to PUSCH allocation. In that case, it is possible to consider a hopping pattern where the hopping position is based on intra-slot frequency hopping of either of the repetitions with some rule, e.g., the slot where there is larger number of symbols.

Based on the above discussion, we propose the following:

Consider the following alternatives for PUSCH scheduling across slot border:

  - Alt 1) Methods based on two PUSCH transmissions across the slot boundary based on implicit signaling by using the start symbol (S) and allocation length (L) in the time-domain resource allocation and S+L > 14.

    - The first PUSCH transmission starts at symbol S until the end of the slot.

    - The second PUSCH transmission starts at the beginning of next slot including the remaining symbols.

    - FFS on same or configured RV for two PUSCHs

  - Alt 2) Methods based on mini-slot repetition

    - Support dynamic repetition

    - Support Dynamic frequency hopping.

    - FFS on how to reduce DMRS overhead

  - Study whether Alt 2 is beneficial with respect to performance and signalling overhead as compared to Alt 1.

## 2.3 Transport block size determination when repeating mini-slots

When scheduling a PUSCH transmission, the target code rate and modulation order is determined from the MCS index signaled in DCI typically. The transport block size is then calculated from the target code rate, modulation order, number of layers, and the allocated resources as described in Section 6.1.4.2 of TS 38.214.

In the following, the issues of the Rel-15 procedure are analyzed, when (mini-)slot aggregation is applied.

### 2.3.1 Reduced scheduling flexibility

In the case of slot aggregation of Rel-15, the transport block size is determined using parameters for the first slot, and the same transport block size is then used in each of the aggregated slots. The same approach can be used for mini-slot repetition, where the transport block size is determined by the amount of allocated resources in the first mini-slot, together with a signaled target code rate and modulation order. One drawback of this is that it can affect scheduling flexibility. With repetitions, if a TB needs to be transmitted with low MCS indices, very large bandwidth is required. In some cases, it is not even possible to schedule the given TB using some MCS indices since the required bandwidth would be too large. We illustrate this problem by considering three different ways of transmitting an 8 OS long PUSCH, either

- by 4 repetitions of a 2 OS long PUSCH, or

- by 2 repetitions of a 4 OS long PUSCH, or

- a single 8 symbol long PUSCH.

(a) and (b) represents the case where the TBS is determined based on parameters for the first transmission, then the TB is repeated multiple times. (c) represents the case where the TBS is determined based on the total amount of resource used for the TB.

We consider three different target packet sizes from the evaluation assumptions, 100 bytes, 250 bytes, or 1370 bytes. We assume a BW of 40 MHz and SCS = 60 kHz, giving a maximum number of PRBs equal to 106 for CP-OFDM. We examine all MCS indices in MCS table 5.1.3.1-3 from TS 38.214 (low SE 64QAM table) and find the number of PRBs needed to support the target TBS. Sometimes the same number of PRBs gives close to equal TBS when using adjacent MCS indices. In this case we only select the MCS index with the smallest spectral efficiency, corresponding to the highest reliability.

| Target TBS = 100 bytes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2 OS, 4 repetitions |  | 4 OS, 2 repetitions |  | 8 OS, 1 repetition |  |
|  | MCS | NPRB | MCS | NPRB | MCS | NPRB |
| Smallest MCS | 11 | 88 | 6 | 92 | 2 | 95 |
| Largest MCS | 28 | 15 | 28 | 5 | 24 | 3 |

| Target TBS = 250 bytes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2 OS, 4 repetitions |  | 4 OS, 2 repetitions |  | 8 OS, 1 repetition |  |
|  | MCS | NPRB | MCS | NPRB | MCS | NPRB |
| Smallest MCS | 17 | 96 | 10 | 90 | 6 | 99 |
| Largest MCS | 28 | 36 | 28 | 12 | 26 | 6 |

| Target TBS = 1370 bytes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2 OS, 4 repetitions |  | 4 OS, 2 repetitions |  | 8 OS, 1 repetition |  |
|  | MCS | NPRB | MCS | NPRB | MCS | NPRB |
| Smallest MCS | Not possible |  | 23 | 100 | 15 | 98 |
| Largest MCS | Not possible |  | 28 | 67 | 28 | 29 |

In the examined cases, basing the TBS determination on the number of resources available in the first repetition leads to worse flexibility in the available {MCS, NPRB} combinations for achieving the target TBS, as compared to basing TBS determination on the total number of resources. For example, for all three TB sizes above, option (a) provides less {MCS, NPRB} flexibility than option (c).

Another advantage of basing the TBS on the total number of resources available is that it is possible to change the TBS both by changing the number of PRBs allocated as well as the number of OFDM symbols. It is not as easy to change the TBS when using (mini-)slot aggregation and Rel-15 TBS determination procedure, where the number of OS in the first transmission may need to stay fixed to keep the alignment delay low. Changing the number of repetitions only changes the total transmission length, but does not change the number of OS in the first transmission, which is used to determine the TBS in Rel-15.

Basing the TBS determination on the allocated resources in the first transmission can lead to inflexible scheduling, and poor usage of the MCS table.

TBS determination is enhanced to be based on the total amount of occupied resources when (mini-)slot aggregation is applied.

### 2.3.2 Mismatch in modulation order and base graph

When (mini-slot) aggregation with K repetition is used, the transmission uses {RMCS,K, QMCS,K, K}, where RMCS,K is the code rate signaled by the MCS index, QMCS,K is the modulation order signaled by the MCS index, K is the number of repetitions.

Alternatively, if assuming the MCS is selected according to the total amount of occupied resources, the TBS would be sent with {RMCS,1, QMCS,1, 1}.

When applying the Rel-15 approach, RMCS,1 could deviate so much from RMCS,K, such that QMCS,1 QMCS,K. When this happens, the link performance will suffer significantly. To illustrate this, we compare two cases:

- 4 repetitions of a 2OS PUSCH, and

- 1 repetition of an 8 OS PUSCH.

For (B), the first OFDM symbol is occupied by DMRS, followed by 7 OFDM symbols occupied by PUSCH payload. For (B), DMRS is artificially made to occupy only the first OFDM symbol of the first repetition, so that the same DMRS overhead (=1 os) is used in both cases. The target TBS of 32 bytes is used. The BLER performance of (A) vs (B) is shown in Figure 1 below, using the assumed transmission parameters.

Figure 1: performance degradation when improper modulation order is used in (mini-)slot aggregation

We see that (A) with 4 repetitions performs about 1.5 dB worse than (B) with 1 repetition. The reason is, (A) uses {RMCS,K, QMCS,K=64QAM, K=4}, (B) uses {RMCS,1 RMCS,K/4, QMCS,1=QPSK, 1} which is the right choice when taking into account all occupied resources.

Similar problem can occur to the base graph selection as well, since base graph selection is partly determined by the target code rate in the MCS. If the target code rate is above a certain threshold, depending on TBS, the base graph switches from BG2 to BG1. Since the mother code rate of BG1 is 1/3, and the mother code rate of BG2 is 1/5, this will also impact performance negatively due to circular buffer repetition being used below rate 1/3 for BG1 instead of fresh parity bits.

When (mini-)slot aggregation is used, basing the TBS determination on the allocated resources in the first transmission may lead excessively high target code rate, resulting in modulation order and base graph mismatch.

# Conclusion

In this contribution, we discussed our view how to enhance PUSCH transmission to meet the URLLC requirements and made the following observations:

Observation 1	Basing the TBS determination on the allocated resources in the first transmission can lead to inflexible scheduling, and poor usage of the MCS table.

Observation 2	When (mini-)slot aggregation is used, basing the TBS determination on the allocated resources in the first transmission may lead excessively high target code rate, resulting in modulation order and base graph mismatch.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reduce the alignment delay at the slot boundary by supporting 2-segmented transmissions before and after the slot boundary.

Proposal 2	Consider the following alternatives for PUSCH scheduling across slot border:

  - Alt 1) Methods based on two PUSCH transmissions across the slot boundary based on implicit signaling by using the start symbol (S) and allocation length (L) in the time-domain resource allocation and S+L > 14.

	The first PUSCH transmission starts at symbol S until the end of the slot.

	The second PUSCH transmission starts at the beginning of next slot including the remaining symbols.

	FFS on same or configured RV for two PUSCHs

	Alt 2) Methods based on mini-slot repetition

	Support dynamic repetition

	Support Dynamic frequency hopping.

	FFS on how to reduce DMRS overhead

	Study whether Alt 2 is beneficial with respect to performance and signalling overhead as compared to Alt 1.

Proposal 3	TBS determination is enhanced to be based on the total amount of occupied resources when (mini-)slot aggregation is applied.

# References

RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell.

3GPP RAN1#94b, Chairman’s note.

R1-1812168 – Reliability Evaluation of Rel-15 Enabled URLLC Use Cases, Ericsson

R1-1812169 – Latency Evaluation of Rel-15 URLLC, Ericsson

3GPP TS 38.212, NR; Multiplexing and channel coding, RAN#81, V15.3.0, September 2018.