3GPP TSG RAN WG1 Meeting #95	R1-1812199

Spokane, USA, November 12 – 16, 2018

Agenda Item:	7.2.3.2

Source:	Huawei, HiSilicon

Title:	System performance evaluation in multi-hop IAB network

Document for:	Discussion and Decision

Introduction

Coverage holes due to NLoS or blockage is one of the biggest challenges for commercial deployment of mmWave NR. Many field tests show that macro cell only deployment in typical urban environment may not provide decent coverage even with very high EIRP. IAB, as a promising technology for mmWave deployment, is expected to provide a low cost coverage enhancement and capacity improvement solution.

In this paper, we present some system-level evaluations results to prove the benefits of introducing IAB for mmWave deployment in terms of user perceived throughput (UPT) with burst traffic. Furthermore, dynamic/flexible TDM with SDM resource allocation between backhaul and access, cross-link interference and backhaul link high order modulation are investigated.

System performance evaluation summary

Evaluation assumption and parameters

The evaluation methodology and assumption are based on the agreement in [1], and the detailed parameters applied are listed in the appendix. Specifically, in the evaluation, multi-hop topology is assumed with 3 IAB nodes deployed randomly within each sector.  Max. RSRP criteria together with max. 4 hop number and 2 IAB node connectivity degree constraint [2] is used to build the topology among IAB nodes. Actually, the absolute system performance largely depends on the topology, frame structure and scheduler etc. applied. In this paper, we mainly study the relative performance with the same topology, frame structure and scheduler assumptions as much as possible.

Three resource allocation schemes in the evaluation are illustrated in Figure 1, i.e. static TDM, dynamic TDM, and dynamic TDM with flexible SDM.

- Static TDM slot allocation

With static TDM scheme, a predefined TDM slot allocation for parent backhaul link is applied, and the access link is not scheduled in backhaul slot even if the slot is not fully occupied.

- Dynamic TDM slot allocation

The TDM slot number and location for parent backhaul link can be flexibly configured according to the backhaul transmission capacity requirement, and also for each specific configuration, the backhaul slot can be used for access link by dynamic scheduling, if the backhaul transmission is not scheduled in that slot.

- Dynamic TDM slot allocation with flexible SDM

In this scheme, the SDM scheme between parent backhaul and access links is applied on top of the dynamically allocated backhaul transmission slot, according to the scheduling decision at IAB node.

(a) Static TDM scheme

(b) Dynamic TDM scheme

(c) Dynamic TDM + SDM scheme

Figure 1: Resource allocation schemes in the evaluation

System performance with dynamic/flexible resource allocation

In LTE relay, backhaul link is multiplexed with access in semi-static TDM manner. The time slot allocation pattern for backhaul link is predefined according to the MBSFN subframe configuration and the access link is not allowed to occupy the backhaul slot even though the backhaul slot is not utilized. While the TDM in NR IAB is much more flexible, without the specific MBSFN subframe configuration constraint. In addition, the multi-beam systems in NR mmWave creates an opportunity to deploy IAB with flexible SDM between access and backhaul links. As a result, transmission delay will be reduced.

In the evaluation, dynamic TDM slot allocation between backhaul and access link with and without SDM based multiplexing are evaluated, in multi-hop scenario. As a comparison, the static TDM based slot allocation scheme is also evaluated. Table 1 illustrates DL 50% UPT performance for burst traffic with various resource allocation schemes, where different traffic loadings are considered. As the UEs in outage are also included in the CDF for UPT, and the 50% UPT for baseline is almost zero. When IAB nodes are deployed, the outage UEs can be served by an IAB node and UPT performance are significantly improved. Consequently, the relative UPT performance gain can be very high. From the results, it is observed that:

- Better UPT can be achieved with the introduction of IAB nodes.

- Compared to static TDM, dynamic TDM scheme brings significant performance improvement due to the better resource utilization.

- For SDM scheme, IAB node can utilize the beams unavailable to backhaul links to serve UEs in backhaul transmission slot, so the transmission opportunities for access link can be increased compared with other resource allocation schemes. Thus end-to-end transmission delay can be further reduced.

Table 1:  DL 50% UPT performance comparison

| IAB Donor Node Resource Utilization | DL 50% UPT performance gain, baseline: w/o IAB nodes |  |  |
| --- | --- | --- | --- |
|  | Static TDM | Dynamic TDM | Dynamic TDM+ SDM |
| RU = 18% | 1585% | 1870% | 2144% |
| RU = 50% | 2469% | 5164% | 5982% |
| RU = 75% | 132% | 6013% | 8139% |

Moreover, to strive for providing ubiquitous service in NR, e.g., 4K HD video service, a guaranteed bit rate (GBR) of 12~40 Mbps for each user should be provided. Here we select 20 Mbps and 35 Mbps as the GBR requirement to evaluate the different resource allocation schemes. Table 2 presents the percentage of users satisfying the required GBR. The simulation results show that dynamic TDM can support up to 24%~28% and 20%~15% more users than the case without IAB node deployment respectively, and dynamic TDM+SDM can support up to 26%~35% and 23%~26% more users.

Table 2: DL GBR performance comparison

| IAB Donor Node Resource Utilization | GBR(Mbps) | DL Percentage of users satisfying a given GBR |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | W/o IAB nodes | Static TDM | Dynamic TDM | Dynamic TDM+ SDM |
| RU = 18% | 20 | 32% | 55% | 56% | 58% |
|  | 35 | 23% | 31% | 43% | 46% |
| RU = 50% | 20 | 23% | 28% | 51% | 54% |
|  | 35 | 15% | 12% | 34% | 41% |
| RU = 75% | 20 | 14% | 17% | 41% | 49% |
|  | 35 | 7% | 10% | 22% | 33% |

Observation 1: Introducing IAB nodes can significantly improve UPT performance for mmWave.

Observation 2: System performance evaluation shows that compared to static TDM, dynamic TDM reduces transmission delay due to the better resource utilization, and SDM further improves the UPT by utilizing the beams unavailable to backhaul links to serve UEs.

High order modulation for backhaul link

Table 3 demonstrates the downlink 50% UPT performance comparison with various modulation orders on backhaul link. Only relative gain is given with the no-IAB node deployment as the baseline. Table 4 shows the percentage of users satisfying the required GBR. Obviously, both UPT and GBR performance have significant improvement with higher modulation. In addition, dynamic TDM with SDM benefits more from the increased backhaul capacity.

The evaluation shows that the end-to-end system performance is highly dependent on the backhaul link capacity. If backhaul capacity is constrained, congestion will happen in the intermediate IAB nodes and data couldn’t reach destination timely, e.g., the decreasing performance of static TDM at high RU. With the increasing of modulation order, the congestion can be alleviated and the performance can be improved. For dynamic TDM with SDM, the access link transmission opportunity is increased since the access links can share the same backhaul slots in different beam directions, therefore more gain can be reaped if backhaul link allows higher transmission capacity.

It is necessary to support high modulation backhaul transmission.

Table 3:  DL 50% UPT performance comparison with 1024QAM/256QAM/64QAM backhaul

| IAB Donor Node Resource Utilization | Maximum MCS for backhaul link | DL 50% UPT performance gain, baseline: w/o IAB nodes |  |  |
| --- | --- | --- | --- | --- |
|  |  | Static TDM | Dynamic TDM | Dynamic TDM+ SDM |
| RU = 18% | 64QAM | 1297% | 1697 % | 1899 % |
|  | 256QAM | 1585% | 1870% | 2144% |
|  | 1024QAM | 1715% | 1962% | 2242% |
| RU = 50% | 64QAM | 1076% | 4281% | 4849% |
|  | 256QAM | 2469% | 5164% | 5982% |
|  | 1024QAM | 3324% | 5774% | 6805% |
| RU = 75% | 64QAM | 15% | 4374% | 5747% |
|  | 256QAM | 132% | 6013% | 8139% |
|  | 1024QAM | 270% | 7491% | 10445% |

Table 4:  DL GBR performance comparison with 1024QAM/256QAM/64QAM backhaul

| IAB Donor Node Resource Utilization | GBR (Mbps) | Maximum MCS for backhaul link | Percentage of users satisfying a given GBR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | W/o IAB nodes | Static TDM | Dynamic TDM | Dynamic TDM+ SDM |
| RU = 18% | 20 | 64QAM | 32% | 49% | 56% | 57% |
|  |  | 256QAM |  | 55% | 56% | 58% |
|  |  | 1024QAM |  | 57% | 58% | 60% |
| RU = 50% | 20 | 64QAM | 23% | 15% | 34% | 35% |
|  |  | 256QAM |  | 28% | 51% | 54% |
|  |  | 1024QAM |  | 38% | 53% | 57% |
| RU = 75% | 20 | 64QAM | 14% | 14% | 35% | 46% |
|  |  | 256QAM |  | 17% | 41% | 49% |
|  |  | 1024QAM |  | 19% | 44% | 53% |
| RU = 18% | 35 | 64QAM | 23% | 13% | 40% | 43% |
|  |  | 256QAM |  | 31% | 43% | 46% |
|  |  | 1024QAM |  | 39% | 44% | 49% |
| RU = 50% | 35 | 64QAM | 15% | 11% | 29% | 38% |
|  |  | 256QAM |  | 12% | 34% | 41% |
|  |  | 1024QAM |  | 15% | 38% | 44% |
| RU = 75% | 35 | 64QAM | 7% | 9% | 17% | 30% |
|  |  | 256QAM |  | 10% | 22% | 33% |
|  |  | 1024QAM |  | 11% | 25% | 36% |

Observation 3: High order modulation on backhaul link (e.g., 256QAM, 1024QAM) can significantly reduce the transmission delay and higher user perceived throughput can be obtained with dynamic resource allocation.

Cross-link interference performance

In this section, we study the downlink performance and focus on the impact of CLI to IAB node’s MT (i.e., DL backhaul link), which is generated by other IAB node’s DU as shown in Figure 2(a) and (b). In order to understand the impact of different interfering source better, the CLI is separated into two cases according to the link types, corresponding to the interference illustration in figure 2 (a) and figure 2(b):

Case 2-1:  CLI from interfering IAB node’s backhaul transmission to interfered IAB node’s MT

Case 2-2:  CLI from interfering IAB node’s access transmission to interfered IAB node’s MT

(a)  Case 2-1:  CLI from DL backhaul to DL backhaul      (b) Case2-2: CLI from DL access to DL backhaul

Figure 2: CLI from DU to MT

Figure 3(a) shows the downlink 50% UPT performance, with static TDM between access and backhaul link applied. Three different traffic loadings are simulated, corresponding to different donor node resource utilizations as indicated in the Figure 3. The CLI of case 2-1 and case 2-2 are eliminated separately so that their impact on performance can be observed individually. The UPT performance with all CLI taken into consideration is used as the baseline, and the following observation could be made:

The CLI level from interfering access transmission is higher than the interference from the interfering backhaul transmission. With lower traffic loading, only 0.02% and 3.93% UPT performance improvements benefit from totally CLI mitigation, while in high traffic loading around 3.2% and 20.37% improvements can be achieved.

(a) 50% UPT performance							(b) RU on access and backhaul link

Figure 3: UPT performance with static TDM resource allocation

We further analyze the per-hop RU on access and backhaul link at a higher donor node resource utilization. From the evaluation results in Figure 3 (b), it can be found that even with high traffic loading the RU for the 2nd and 3rd hop IAB node is rather low (44.29%, and 12.25%), which are much lower than the RU (88.44%) of the 1st hop, meaning that congestion happens in the 1st hop IAB node and the downlink data cannot reach the UE, thus leads to even lower RU on access links. Therefore, congestion in the intermediate IAB nodes becomes the bottleneck of system performance.

With dynamic TDM resource allocation, the time slot for backhaul link can dynamically be allocated according to the backhaul link transmission requirement. Therefore, the backhaul link congestion can be alleviated to some extent. Figure 4 gives the RU and UPT evaluation results with dynamic TDM resource allocation at high donor node resource utilization. As expected, the RU and UPT performance are greatly improved although the congestion still exists.

It should be noted that by alleviating the congestion using dynamic TDM slot allocation, the impact of CLI also becomes more significant. In Figure 4(a), it can be observed that the 50% UPT performance gain at high traffic loading is increased from 20.37% to 35.63% by eliminating case 2-2 CLI. This implies that CLI become more pronounced when congestion at the IAB nodes is alleviated.

(a) 50% UPT performance                               	(b) RU on access and backhaul link

Figure 4: UPT performance with dynamic TDM resource allocation

Observation 4: The inter-IAB node CLI impact on the UPT performance is noticeable especially at high traffic load.

Observation 5: With multi-hop topology, congestion easily happens at the intermediate IAB nodes.

Observation 6: The impact of CLI among IAB nodes becomes more pronounced when congestion at the IAB nodes is alleviated by dynamic resource allocation.

Conclusions

In this contribution, we further evaluate the system performance in multi-hop IAB deployment in terms of UPT and inter-IAB node interference for mmWave band, by system evaluation. We make the following observations:

Observation 1: Introducing IAB nodes can significantly improve UPT performance for mmWave.

Observation 2: System performance evaluation shows that compared to static TDM, dynamic TDM reduces transmission delay due to the better resource utilization, and SDM further improves the UPT by utilizing the beams unavailable to backhaul links to serve UEs.

Observation 3: High order modulation on backhaul link (e.g., 256QAM, 1024QAM) can significantly reduce the transmission delay and higher user perceived throughput can be obtained with dynamic resource allocation.

Observation 4: The inter-IAB node CLI impact on the UPT performance is noticeable especially at high traffic load.

Observation 5: With multi-hop topology, congestion easily happens at the intermediate IAB nodes.

Observation 6: The impact of CLI among IAB nodes becomes more pronounced when congestion at the IAB nodes is alleviated by dynamic resource allocation.

- References

RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #93.

R1-1805924, IAB system evaluation methodology and preliminary results, Huawei, HiSilicon, RAN1#93, Busan, May 2018.

TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, Dec.2017.

TR 38,802, Study on New Radio Access Technology.

- Appendix

Table A-1: Evaluation scenarios for IAB

| Attribution | Assumption |
| --- | --- |
| Network Layout | Hexagonal cellular network (ISD = 500m) with multi-hop relaying |
| Number of TRPs | 19 macro TRPs and 57*Nr rTRPs where Nr is the number of rTRPs per sector. The value of Nr is {3}. |
| Deployment of RN | Random |
| UE distribution | Uniform random deployment |
| Node selection for UE | Max RSRP |
| Topology building method | Maximum RSRP, together with the following constraints. - Maximum 4 hops - Maximum 2 node degree for each IAB node[2] |
| Carrier Frequency | In-band backhaul: 30GHz backhaul and access |
| Large-scale channel parameters | - Macro-to-UE: 5GCM UMa - Micro-to-UE: UMi-Street canyon - Macro-to-Macro: 5GCM UMa (hUE =25m) - Macro-to-Micro: 5GCM UMa (hUE =10m) - Micro-to-Micro: UMi-Street canyon (hUE =10m) - UE-to-UE: UMi-Street canyon (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 of TR38.802 |
| Fast fading parameters | - Macro-to-UE: 5GCM UMa - Micro-to-UE: UMi-Street canyon - Macro to macro: 5GCM UMa O-to-O (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro to micro: 5GCM UMa O-to-O; ASA and ZSA statistics updated to be the same as ASD and ZSD for UMi-Street canyon; ZoD offset = 0 - Micro to Micro: UMi-Street canyon O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - UE to UE: UMi-Street canyon; ASD and ZSD statistics updated to be the same as ASA and ZSA. Dual mobility support. |
| UE number per sector | 30 (80% indoor/ 20% outdoor) |
| Carrier frequency | 30GHz |
| Subcarrier spacing | 120kHz |
| Slot length | 0.25ms with 14 symbols |
| TDD UL/DL configuration | D:U = 3:2 for baseline(without IAB node) |
| MIMO | MU-MIMO |
| Scheduling | PF |
| HARQ | CC |
| MCS | Up to 1024QAM |
| Receiver | MMSE-IRC |
| Traffic model | FTP3 with 0.5Mbyte packet |
| Codebook for analog beamforming | DFT-based, no oversampling |
| Penetration loss | 50% high loss, 50% low loss |
| Metric | Burst traffic: end-to-end UPT |

Table A-2: Antenna configuration for IAB

| Attribution | Assumption |
| --- | --- |
| gNB height | 25 m |
| RN height | 10 m |
| UE height | 3D distributing [3] |
| TRP Tx power | 3 dBm |
| RN Tx power | 33 dBm |
| UE Tx power | 23dBm |
| TRP antenna configuration | (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ |
| RN antenna configuration for each side/sector | (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ |
| UE antenna configuration | (M,N,P,Mg,Ng) = (2,4,2,1,2) , (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (0, 0)λ, Θmg,ng=90, Ω0,0 uniformly distributed in [0, 360] degrees, Ω0,1=Ω0,0+180 |
| RN antenna pattern | See Table A.2.1-6 of TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 of TR 38.802 |