3GPP TSG RAN WG1 Meeting #95		R1-1812335

Spokane, USA, Nov 12th – 16th, 2018

Source:	vivo

Title:	Remaining issues on requirements for NR positioning

Agenda Item:	7.2.10.1

Document for:	Discussion and Decision

- Introduction

In RAN1#94bis [1], the requirements and evaluation methodology have been agreed at least for NR RAT-dependent positioning.  The requirements for NR positioning can be found in Applendix. However, there is still something unclear about the positioning requirements. In this contribution, the requirements related issues are discussed.

- Remaining issues on requirements

The positioning requirements of commercial use case are listed below:

- As a starting point for commercial use cases, the following requirements are considered as performance targets for RAT dependent solutions, which are subject to further analysis in terms of performance / complexity tradeoffs of NR positioning radio-layer solutions

  - Horizontal positioning error < [3]m for [80]% of UEs in indoor deployment scenarios

  - Horizontal positioning error < [10]m for [80]% of UEs in outdoor deployments scenarios

  - Vertical positioning error < [3]m for [80]% of UEs in indoor deployment scenarios

  - Vertical positioning error < [3]m for [80]% of UEs in outdoor deployment scenarios

  - End to end latency < [1]s

According to above requirements, we can see that NR should support different requirements  for different deployment scenarios (indoor or outdoor). However, the deployment scenarios are not clear enough. For indoor scenarios, does is mean the UEs and the gNBs are required to be indoors or only the UEs are required to be indoors. While for outdoor cases, does it require the gNBs and the UEs to be outdoors or only require the gNBs to be outdoors. In addition, it seems that the indoor positioning via outdoor gNBs is precluded in this requirements. It is not reasonable to exclude this case because for most real deployment situations where indoor gNBs are not deployed, outdoor gNBs can be used to locate UEs.  For this case, the requirement of outdoor deployment scenario can be considered as a baseline.

We are also confused about the vertical positioning requirements of [3]m for indoor deployment scenarios. A typical evaluation scenario is indoor office and the gNB antenna height is 3m. Therefore, it makes no sense to evaluate the vertical positioning error based on the requirement of 3m. A higher requirement of vertical positioning error for evaluation is needed. In our views, 2m is preferred.

Proposal 1:

- Clarify distribution of indoor and outdoor UEs for indoor and outdoor deployment scenarios, e.g. indoor UE with indoor gNBs, indoor UE with outdoor gNBs or outdoor UE with outdoor gNBs.

Proposal 2:

- Adding scenario and requirements for indoor UE positioning via outdoor gNBs, and the following requirements is supported.

  - Horizontal positioning error <[10]m for [80]% of UEs.

  - Vertical positioning error < [3]m for [80]% of UEs

Proposal 3:

- Support vertical positioning error < [2]m for [80]% of UEs in indoor deployment scenarios.

- Conclusion

In this contribution, we discuss NR positioning requirements with the following proposals:

Proposal 1:

- Clarify distribution of indoor and outdoor UEs for indoor and outdoor deployment scenarios, e.g. indoor UE with indoor gNBs, indoor UE with outdoor gNBs or outdoor UE with outdoor gNBs.

Proposal 2:

- Adding scenario and requirements for indoor UE positioning via outdoor gNBs, and the following requirements is supported.

  - Horizontal positioning error <[10]m for [80]% of UEs.

  - Vertical positioning error < [3]m for [80]% of UEs

Proposal 3:

- Support vertical positioning error < [2]m for [80]% of UEs in indoor deployment scenarios.

References

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 94bis, Chengdu, China, 8 –12th October, 2018

Appendix

Agreement:

- Regulatory requirements are considered as a minimum performance targets for NR Positioning studies

- Additional requirements based on commercial use cases can be used as input performance targets that are subject to further analysis in terms of performance / complexity tradeoffs in different evaluation scenarios

Agreement:

- For regulatory use cases, the following requirements are considered as a minimum performance targets for NR positioning

  - Horizontal positioning error <= 50m for 80% of UEs

  - Vertical positioning error [<5 m] for [80%] of UEs

    - Note: The regulatory requirements refer to floor level vertical accuracy

  - End to end latency and TTFF < 30 seconds

- As a starting point for commercial use cases, the following requirements are considered as performance targets for RAT dependent solutions, which are subject to further analysis in terms of performance / complexity tradeoffs of NR positioning radio-layer solutions

  - Horizontal positioning error < [3]m for [80]% of UEs in indoor deployment scenarios

  - Horizontal positioning error < [10]m for [80]% of UEs in outdoor deployments scenarios

  - Vertical positioning error < [3]m for [80]% of UEs in indoor deployment scenarios

  - Vertical positioning error < [3]m for [80]% of UEs in outdoor deployment scenarios

  - End to end latency < [1]s

- Note: This does not eliminate more or less demanding commercial use cases.

Agreement:

- At least CDFs of horizontal and vertical (vertical error not necessarily applicable to all solutions and/or scenarios) positioning errors are used as a performance metrics in NR positioning evaluations

  - At least the following percentiles of positioning error are analyzed 50%, 67%, 80%, 90%

Physical layer latency, UE power consumption, scalability/capacity, network deployment complexity, availability, UE and gNB complexity can be considered as important design factors for NR positioning solutions and can be evaluated analytically for proposed solutions.