3GPP TSG RAN WG1 Meeting #95		R1-1812396

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.7.2

Source:	ZTE

Title:	Evaluation results of User experience data rate in IMT-2020 self-evaluation

Document for:	Discussion

Introduction

There is an approved Study Item on Self Evaluation towards IMT-2020 submission in the RAN#75 meeting, which mainly focuses on evaluated RAN technologies based on Rel-15 and beyond to satisfy all ITU-R IMT-2020 requirements including eMBB scenario.

At RAN#77 meeting, the general work plan of self evaluation is approved [1]. Moreover, “[ITU-R AH 01] Calibration for self-evaluation”[2] has been set up for discussion on calibration and collection of calibration results. It is observed that the calibration results are well aligned according to the results collected so far for InH_x, UMa_x and RMa_x in all IMT-2020 defined test environments.

Moreover, At RAN1 #92, some features of system-level simulation in eMBB have been approved. In this contribution, some evaluation results on User experience data rate are shown.

Evaluated methodology on User experience data rate

On the one hand, in ITU-R, based on ITU-R reports M.2410, User experience data rate’s definition and values can be obviously found, which proponents should follow them. More specificity, User experience data rate can be defined as

Ruser = W × SEuser                                         (1)

which,

W denote the channel bandwidth,

SEuser is 5th percentile user spectral efficiency, and

Ruser is user Experience Data Rate.

Moreover, its values can be seen in Table 2-1; as well as it is noticeable seen that the evaluation of A User experience data rate is appreciated with Dense Urban – eMBB test environment.

| Test environment | Minimum values |
| --- | --- |
| Dense Urban – eMBB | Downlink: 100 Mbit/s |
|  | Uplink: 50 Mbit/s. |

Table 2-1

Observation 1: Based on ITU-R report, User experience data rate can be evaluated in Dense Urban – eMBB test environment; and the minimum requirements are 100 Mbit/s in downlink and 50 Mbit/s in uplink, respectively.

Furthermore, the high-level assessment method of User experience data rate also be approved in ITU-R report M.2412 [3], which is analytical for single layer configurations, while is simulation for multi-layer configurations.

On the other hand, 3GPP has defined the bandwidth supported by NR in FR1 (Frequency Range 1). Moreover, specific supported bandwidth can be found in below table. Meanwhile, NR is able to support aggregation of 16 component carriers in Rel-15.

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | [160] | 216 | 270 | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | [78] | 106 | 133 | 162 | 217 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | [38] | 51 | 65 | 79 | 107 | 135 |

Table 2-2: Maximum transmission bandwidth configuration NRB in FR1

Evaluation results

One layer configuration

Generally, obtaining User experienced data rate can be based on the evaluated results [4] of 5th percentile user spectral efficiency for the configuration A of Dense urban-eMBB test environment.

| Dense Urban-eMBB | Channel model | Bandwidth (MHz) | User experienced data rate （Mbit/s） | Required Bandwidth for obtaining User experience data rate in M.2410 |
| --- | --- | --- | --- | --- |
| 32TXRU TDD | Model A | 400 | 106.81 | 375MHz |
|  | Model B | 480 | 109.53 | 439MHz |
| 64TXRU TDD | Model A | 400 | 118.46 | 338MHz |
|  | Model B | 400 | 110.69 | 362MHz |

Table 3-1 Downlink evaluated results of User experience data rate in TDD

Table 3-1 shows minimum bandwidth to meet ITU-R requirement of User experience data rate in DL TDD below 6GHz; while at least 400MHz bandwidth is seen as actual deployment for satisfied minimum DL requirement of User experience data rate in ITU. More specifically, in model A, aggregating at least 4 component carriers can achieve ITU-R requirements, e.g. aggregating 4 of 100MHz; while in model B, it is necessary to up to 5 aggregating component carriers.

Observation 2: NR can meet User experience data rate in Downlink Dense Urban-eMBB test environment.

Conclusion

In this document, we provide our considerations on both Mobility and Reliability of evaluation towards IMT-2020 submission.

Observation 1: Based on ITU-R report, User experience data rate can be evaluated in Dense Urban – eMBB test environment; and the minimum requirements are 100 Mbit/s in downlink and 50 Mbit/s in uplink, respectively.

Observation 2: NR can meet User experience data rate in Downlink Dense Urban-eMBB test environment.

References

[1] RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al.

[2] Report ITU-R M.2410 - Minimum requirements related to technical performance for IMT-2020 radio interface(s).

[3] Report ITU-R M.2412 - Guidelines for evaluation of radio interface technologies for IMT-2020.

[4] R1-1812274, ZTE, “Updated DL spectral efficiency in the eMBB usage scenario for IMT-2020 self evaluation”, RAN1#95, USA, Spokane, 2018