3GPP TSG RAN WG1 Meeting #95	R1-1812434

Spokane, USA, November 12th – 16th,2018

Title:	Considerations on UL reference signals and channels design for NR-U

Source: 	ZTE

Agenda Item:	7.2.2.3.2

Document for:	Discussion and Decision

# Introduction

The SID on NR-U (NR-based Access to Unlicensed Spectrum) [1] aims to identify and evaluate solutions and techniques for next generation wireless systems operating on unlicensed bands. Besides, coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements should also be studied.

In RAN1 #94 meeting [2], the following agreement have been reached for NR-U:

Agreement:

- For scenarios in which a block-interlaced waveform is used for PUCCH/PUSCH, it has been identified that from FDM-based user-multiplexing standpoint it can be beneficial to have UL channels on a common interlace structure, at least for PUSCH, PUCCH, associated DMRS, and potentially PRACH

  - Note: This is only from a user-multiplexing perspective. Other aspects of PRACH design need to be considered, i.e., timing estimation accuracy, miss detection rate, PAPR, RACH capacity, transmission power

- For scenarios in which a contiguous allocation for PUSCH and PUCCH is used, it is beneficial to use contiguous resource allocation for PRACH

- FFS: Potential LBT blocking due to TA difference between FDM’d PUSCH, PUCCH, and PRACH

Agreement:

- For scenarios in which a block-interlaced waveform is used for UL transmission, a PRB-based block-interlace design has been identified as beneficial at least for 15 and 30 kHz SCS, and potentially for 60 kHz SCS

  - Link budget limited cases with given PSD constraint

    - It is observed that power boosting gains decrease with increasing SCS

  - As one option to efficiently meet the occupied channel bandwidth requirement

  - Comparatively less specification impact than Sub-PRB interlace design

- Design for 60 kHz requires further discussion, e.g., sub-PRB vs. PRB-based block interlace designs

- The following has been observed for sub-PRB block interlace designs

  - In some scenarios sub-PRB interlacing can be beneficial in terms of power boosting

    - FFS: scenario details, e.g., small resource allocations

  - Sub-PRB interlace design has at least the following specification impact:

    - Reference signal design (e.g., DMRS)

    - Channel estimation aspects

    - Resource allocation

Agreement:

- It has been identified as beneficial to support a block-interlaced structure in which the number of interlaces (M) decreases with increasing SCS, and the nominal number of PRBs per interlace (N) is similar for each SCS (in a given bandwidth) at least for 15 and 30 kHz SCS, and potentially 60 kHz depending on supported interlace design

- FFS: M and N for each supported SCS

- FFS: 60 kHz in case a sub-PRB interlace is introduced

Agreement:

- From a RAN1 perspective it has been identified that supporting a non-uniform interlace structure in which the number of PRBs per interlace is allowed to be different for different interlaces is beneficial from a spectrum utilization point of view

  - FFS: Exact number of PRBs per interlace for supported value(s) of M and N

    - Note: M is the number of interlaces and N is the nominal number of PRBs per interlace in a given bandwidth

  - FFS: Whether or not there are issues in the interlace design in the resource allocation to 2^n1*3^n2*5^n3 in the case of DFT-s-OFDM

In RAN1 #94bis meeting [3], the following agreement have been reached for NR-U:

Agreement:

- Within a 20 MHz bandwidth, the following candidate PRB-based interlace designs have been identified where M is the number of interlaces and N is the number of PRBs per interlace in a 20 MHz bandwidth. Where two values are listed for N, it means that some interlaces have one more PRB than others (non-uniform interlace design):

  - 15 kHz:

    - M = 12, N = 8 or 9

    - M = 10, N = 10 or 11

    - M = 8, N = 13 or 14

  - 30 kHz:

    - M = 6, N = 8 or 9

    - M = 5, N =  10 or 11

    - M = 4, N = 12 or 13

  - 60 kHz:

    - M = 4, N = 6

    - M = 3, N = 8

    - M = 2, N = 12

  - 60 kHz (assuming 26 PRBs is agreed by RAN4 in a 20 MHz bandwidth):

    - M = 4, N = 6 or 7

    - M = 2, N = 13

    - M = 3, N = 8 or 9

- It is up to RAN4 to investigate whether or not the non-uniform interlace structure has an impact on MPR/A-MPR requirements for PUSCH

Agreement:

Capture the following in TR 38.889

- Both PRB and sub-PRB interlacing for 60 kHz have been studied. For sub-PRB interlacing the following aspects have been considered:

  - Power boosting potential depending on resource allocation size

  - PUSCH DMRS configuration aspects

  - Channel estimation performance

  - Number of REs per interlace unit

Agreement:

For carriers with bandwidth larger than 20 MHz, two candidate interlace designs have been identified.

- Alt-1: Same interlace spacing for all interlaces regardless of carrier BW.

  - This alternative uses Point A as a reference for the interlace definition

- Alt-2: Interlacing defined on a sub-band (20 MHz) basis. (Note: Possible interlace spacing discontinuity at edges of sub-band).

In this contribution, we discuss potential UL physical layer channels and reference signals design for NR-U, including PUSCH, PRACH, PUCCH, and SRS for NR-U.

# Potential UL physical channels and reference signals design

## PUSCH

As NR support non-slot based scheduling, PUSCH can start from any symbol and the duration can be from 1 to 14 symbols. RAN1 had also agreed NR-U supports both PDSCH/PUSCH mapping Type-A and Type-B. Therefore, the UE can transmit PUSCH from the nearest permitted symbol after a successful channel access, then the resource utilization can be improved. However, the start symbol is dynamically DCI indicated, this position may not be well matched with the nearest symbol where UE LBT succeeds. Some further study can be performed for slot-aggregation transmission or multiple slot scheduling scheme similar to LAA can be considered to improve the MCOT utilization efficiency. Besides, in order to reduce UL channel access delay due to LBT, multiple opportunities can be configured to UE. For example, multiple time and frequency domain PUSCH resources can be indicated to UE by PDCCH.

Proposal 1: some enhancement to reduce channel access delay due to LBT should be studied for PUSCH transmission. Such as:

- multiple slot scheduling scheme

- multiple time and frequency domain  PUSCH resources can be indicated to UE by PDCCH

## PRACH

Considering LBT requirement in unlicensed carrier, the four-step PRACH procedure needs to succeed in at least four LBTs, and any LBT failure can lead to a significant random access delay and resource overhead. Therefore, some enhanced methods need to be considered for PRACH procedure in NR-U in order to decrease the expected random access delay caused by potential LBT failures. For instance, for four-step PRACH, Msg1 and partial Msg3 content can be used as a short control signal transmitted by UE and the response message for Msg1 and partial Msg3 can be transmitted in NR licensed carrier, or a single RACH procedure with multiple RACH candidates in frequency domain, or multiple parallel RACH procedures in frequency and/or time domain. More details can be found in our companion RAN2 contribution [4]. Alternatively, a higher priority LBT mechanism should be considered to quickly complete random access procedure, e.g., Cat-2 LBT. More details can be found in our companion contributions [5][6]. In addition to the enhancement method the above mention, the method of configuration multiple RACH occasion per SSB and increasing the number of RACH transmission opportunities can also be considered. Wherein, the former has already supported in Rel-15.

Proposal 2: Some enhanced methods should be considered for PRACH procedure as follows:

- Single RACH procedure with multiple RACH candidates.

- Multiple parallel RACH procedures.

- LBT of higher priority, e.g., Cat-2 LBT.

- Configuration multiple RACH occasion per SSB.

- Increasing the number of RACH transmission opportunities.

Furthermore, the OCB (occupied channel bandwidth) regulation for RACH preamble also needs to be satisfied. In Rel-15, NR supports both long and short sequence RACH preambles. The length of long sequence is 839, while the length of short sequence is 139. SCS supported for long sequence are 1.25 KHz and 5 KHz; SCS supported for short sequence are 15 KHz and 30 KHz for below 6 GHz. For instance, assuming 20MHz carrier bandwidth below 6GHz frequency band, the occupied bandwidth with 15 and 30 KHz SCS are 2.16MHz and 4.32MHz respectively, which obviously do not fulfil the ETSI OCB requirement as shown in Figure 1. Therefore, in order to meet the OCB requirement, some enhancement methods can be considered, e.g., repeat M times preamble in frequency domain as shown in Figure 2, larger subcarrier intervals of preamble, etc. For the latter, it need to introduce new SCS of PRACH for below 7GHz, e.g., SCS 60KHz, 120KHz, etc.

Apart from this, considering compatibility with other UL signal waveform, block interleaved FDMA (B-IFDMA) can also be considered to satisfy the OCB requirement. However, the correlation properties of PRACH preamble using the B-IFDMA have some issues in timing estimation, because there are many false peaks in the correlation properties. Another possible approach which meets the OCB requirement is to introduce sub-PRB block interlace design for PRACH waveform. This structure especially for larger SCS is beneficial for better power utilization under PSD limit. However, given the same SCS and number of subcarriers in an interlace, the ICI issue for sub-PRB block interlace would be more serious than that for B-IFDMA structure. Besides, it may introduce great standard effort such as design new sub-PRB based resource allocation (e.g., design the number of RE per interlace unit, or design uniform / non-uniform sub-PRB based RA), new UL RS pattern in frequency domain, channel estimation performance degrading for SCS 60KHz or 120KHz, etc.

Figure 1. An example of preamble’s OCB not satisfying regulatory requirement

Figure 2. Repeat M times preamble in frequency domain

Proposal 3: In order to meet the OCB requirement for RACH preamble, some enhancements should be considered, e.g., repeat M times preamble in frequency domain, larger subcarrier intervals of preamble.

## PUCCH

UCI such as ACK/NACK, SR, and CSI can be transmitted in the PUCCH. In standalone or dual connectivity operation, the PUCCH transmission over unlicensed carrier should be supported. Currently in NR, both short-duration PUCCH and long-duration PUCCH are supported to cater to different requirements of UCI payload and cell coverage. For PUCCH, the frequency resource allocation is semi-statically configured per BWP and the UL active BWP can be viewed as the UE nominal channel bandwidth. Therefore, PUCCH should find a way to occupy 70% or 80% of the BWP bandwidth.

For NR PUCCH format 0 and format 1, as only 1 or 2 bits UCI information is carried and occupy only 1 RB, this may not well match the unlicensed carrier characteristic as once UE perform LBT success, as it can send all the UCI information or it to combine delayed HARQ-ACK bits with current ones. As such, NR-U UEs would be often reporting HARQ-ACK using codebooks larger than 2 bits to improve the transmission efficiency. For PUCCH format 2, as it adopts CP-OFDM waveform and it can allocate from 1 to 16 RBs, therefore, it can meet the OCB requirement more easily. For PUCCH format 3, it can also allocate up to 16 RBs, but these RB should be contiguous and this have an impact on user multiplexing. For PUCCH format 4, multi-cluster transmission should be considered to meet the OCB requirement. As PUCCH format 4 occupying only 1 RB, it should be discussed how to configure UCI payload in each cluster of the interlaced unit. For instance, two methods can be considered.

Alt1: same coded UCI bits can be repeated over multiple clusters.

Although this can incur small specification impact, but the PAPR or CM should be considered.

Alt2: different coded UCI bits can be distributed over multiple clusters.

If PUCCH format 0 or 1 is introduced for NR-U, multi-cluster transmission method such as format 4 given above can also be applied, and these two methods should be further evaluated. Besides, the sequence design is another important factor on PUCCH performance. Furthermore, if multi-cluster transmission or interlaced structure for PUCCH is used, then frequency hopping is not need.

Proposal 4:

- NR PUCCH format 2 can be used in NR-U easily, and should be supported for NR-U.

- NR PUCCH format 3 can also be supported for NR-U.

- If PUCCH format 0, 1 and 4 is adopted for NR-U, necessary enhancements should be considered, such as interlaces allocation and sequence design for multiple RBs.

## SRS

Same as LTE, NR supports SRS-based frequency-selective scheduling and periodic/aperiodic SRS transmission. NR also supports SRS-based downlink beamforming and semi-persistent SRS transmission, which are not supported in LTE. In NR-U, SRS transmission in the configured time resource would be subject to the results of the LBT. Thus, the configuration of SRS resource and resource mapping should be as flexible as possible to transmit SRS in time when the channel is detected as idle. Based on this, the aperiodic SRS with DCI trigger should be supported for NR-U carrier while periodic SRS is not suitable for NR-U. In addition, SRS transmission should also meet the OCB regulatory requirements, only wide-band and BWP/sub-band SRS could be supported in NR-U. In our opinion, BWP or sub-band can be considered as the nominal channel bandwidth for UEs in NR-U.

Proposal 5: Rel-15 NR SRS can be used as a baseline and more flexible SRS transmission method should be considered due to LBT in the NR-U carrier.

# Conclusion

In this contribution, we discuss some potential solutions and techniques for NR-U UL reference signal design and physical channel design, and have the following proposals:

Proposal 1:  some enhancements to reduce channel access delay due to LBT should be studied for PUSCH transmission. Such as:

- multiple slot scheduling scheme

- multiple time and frequency domain  PUSCH resources can be indicated to UE by PDCCH

Proposal 2: Some enhanced methods should be considered for PRACH procedure as follows:

- Single RACH procedure with multiple RACH candidates.

- Multiple parallel RACH procedures.

- LBT of higher priority, e.g., Cat-2 LBT.

- Configuration multiple RACH occasion per SSB.

- Increasing the number of RACH transmission opportunities.

Proposal 3: In order to meet the OCB requirement for RACH preamble, some enhancements should be considered, e.g., repeat M times preamble in frequency domain, larger subcarrier intervals of preamble

Proposal 4:

- NR PUCCH format 2 can be used in NR-U easily, and should be supported for NR-U.

- NR PUCCH format 3 can also be supported for NR-U.

- If PUCCH format 0, 1 and 4 is adopted for NR-U, necessary enhancements should be considered, such as interlaces allocation and sequence design for multiple RBs.

Proposal 5: Rel-15 NR SRS can be used as a baseline and more flexible SRS transmission method should be considered due to LBT in the NR-U carrier.

# Reference

- RP-170828, New SID on NR-based Access to Unlicensed Spectrum, Qualcomm, March 6 - 9, 2017, RAN#78

- Draft report of 3GPP TSG RAN WG1#94, Gothenburg, Sweden, August 20th-24th, 2018

- Draft report of 3GPP TSG RAN WG1#94bis, Chengdu, China, October 8th - October 12th, 2018

- R2-1816829, considerations on 4-step RACH procedure for NR-U, ZTE

- R1-1812436, Discussion on initial access and mobility for NR-U, ZTE

- R1-1812435, Discussion on Channel Access Mechanism for NR-U, ZTE