3GPP TSG RAN WG1 Meeting #95	R1-1812686

Spokane, USA, Nov 12 – Nov 16, 2018

Agenda Item:	7.2.10.4

Source:	Huawei, HiSilicon

Title:	Additional considerations on channel modeling and deployment

Document for:	Discussion and decision

# Introduction

At RAN1#94b, the simulation assumptions for NR positioning were agreed in [1]. During the meeting, a modification to the channel model in TR 38.901 [2] was discussed. In our companion contribution [3], we propose to adjust the angle of the first cluster to the LOS direction as per the discussion in RAN1#94b. However, additional changes of the channel model for positioning evaluations may be needed. These changes are described in this contribution.

# Channel Model

## Delay adjustment for cluster removal

In Step 6 of sub-clause 7.5 in [2], the following procedure is described for removing the cluster with small power:

Remove clusters with less than -25 dB power compared to the maximum cluster power. The scaling factors need not be changed after cluster elimination.

Since the delay generation of each cluster is prior to the cluster removal, the delay is not affected by the procedure. The problem comes when the first cluster, whose delay is normalized to zero in Step 5, is removed. In such a case, the remaining first cluster has an extra delay in addition to the propagation delay, which may cause RSTD estimation error. In order to solve the problem, we suggest to subtract the delay of the first remaining cluster from the delays of all remaining clusters.

Proposal 1: Subtract the delay of the first remaining cluster from the delays of all remaining clusters if the first cluster is removed due to the low power.

## Angle adjustment for the first cluster

In [3], we propose to have the first cluster having the LOS direction. In addition, the first cluster should have a direct path transmitting from the LOS direction but suffering from penetration loss due to obstruction in case of NLOS. Thus, this cluster should have a behavior similar to a LOS cluster:

- Small number of rays or single ray within the cluster

- Small intra-cluster angle spread or zero intra-cluster angle spread

- Random initial phase  and cross polarization power ratios

The easiest adaptation is to use a single intra-cluster ray angle (e.g. AOA, AOD, ZOA and ZOD ) for first cluster, which implies that the intra-cluster angle spread is 0, without changing the random initial phase and XPR coefficients.

Proposal 2: Consider using a single intra-cluster ray angle within the first cluster for NLOS conditions.

# Considerations on indoor deployment

It is well-known that the deployment geometry of BSs will have an impact on the positioning accuracy. Generally, the Geometrical Dilution of Precision (GDOP) is used to assess the dependency between the error of measurements (e.g. angles and TOAs) and the relative geometry of UEs and BSs. To enhance the positioning performance, it is desirable to optimize the deployment w.r.t the GDOP because a large GDOP implies high sensitivity to the measurement error.

For the outdoor scenarios (i.e. UMa and UMi), the geometry of BSs are usually modeled as hexagon deployment because it is basically in line with practical case. For the indoor deployment, however, there is no representative deployment geometry. As shown in Fig. 1, different deployments may have different GDOP which leads to difference positioning performance. In RAN1#94b, the requirements for indoor positioning were agreed to be <[3m] for 80% of UEs with a 12-site indoor geometry in [2], but the impact of GDOP is not considered in the evaluation methodology. It is not clear whether the proposed requirements can still be met with less available sites or geometry of poorer GDOP.

Fig. 1 GDOP comparison of different in indoor deployments

Proposal 3: Consider the GDOP impact of deployment geometry in the evaluation.

# Conclusions

In this contribution, we discuss our considerations on channel modeling and indoor deployment. Based on the discussion, we have the following proposals and observations.

Proposal 1: Subtract the delay of the first remaining cluster from the delays of all remaining clusters if the first cluster is removed due to the low power.

Proposal 2: Consider using a single intra-cluster ray angle within the first cluster for NLOS conditions.

Proposal 3: Consider the GDOP impact of deployment geometry in the evaluation.

# References

3GPP, Chairman's Notes RAN1 94b final, RAN1#94b, Chengdu, China, Oct. 8-12, 2018.

TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 15), Jun. 2018.

R1-1812235, Remaining details on evaluation methodology and simulation results, Huawei, HiSilicon, RAN1#95, Spokane, US, Nov. 12-16, 2018.