3GPP TSG RAN WG1 Meeting #95	R1-1812752

Spokane, USA, 12th – 16th November 2018

Agenda Item:	7.2.10.3

Source: 	Sony

Title:	Considerations on OTDOA positioning in NR

Document for:	Discussion / Decision

# Introduction

A new study item “Study on NR positioning support” was approved In RAN plenary #80 [1] . The objective of this study item is to evaluate potential solutions to address NR positioning requirements as defined in TR 38.913, TS 22.261, TR 22.872 and TR 22.804 while considering E911 requirements by analysing positioning accuracy (including latitude, longitude and altitude), availability, reliability, latency, network synchronization requirements and/or UE/gNB complexity to perform positioning, and taking into account a preference to maximize synergy where possible with existing positioning support for E-UTRAN.

The second objective of the NR positioning support study includes the following:

- Study and evaluate potential solutions of positioning technologies based on the above identified requirements, evaluation scenarios/methodologies [RAN1]

  - The solutions should include at least NR-based RAT dependent positioning to operate in both FR1 and FR2 whereas other positioning technologies are not precluded.

  - Minimum bandwidth target (e.g. 5MHz) of NR with scalability is supported towards general extension for any applications.

In RAN1 meeting #94bis, the following agreements on potential techniques for NR positioning were made [2] :

Agreement:

The RAT dependent solutions considered for study include

- Downlink based solutions

- Downlink and uplink based solutions

- Uplink based solutions

# In this contribution, we discuss our view on the Observed Time Different of Arrival (OTDOA) technique as one of the downlink-based positioning solutions in NR.

# Discussion

Observed Time Difference of Arrival (OTDOA) is a Radio Access Technology (RAT) dependent downlink based positioning technique. OTDOA was introduced in 3GPP LTE rel.9. It has been proven to meet e911 regulatory requirements (e.g. 50 m horizontal accuracy with at least 67% availability requirements). Thus, it has been widely deployed in LTE networks. The basic OTDOA operation in LTE is shown in Figure 1. A user equipment (UE) receives reference signals from multiple eNBs (serving cell and neighbor cells) and then performs timing difference of arrival (TDOA) measurements for each eNB and then calculates Reference Signal Time Difference (RSTD). The RSTD measurement results are transmitted using the LTE Positioning Protocol (LPP) from the UE to a location server (LS) via the serving cell eNB. The LS then performs the positioning estimation based on measurement results from at least three eNBs using triangulation. The positioning estimation result is within measurement error range depending on the RSTD measurement quality.

Figure 1: Illustration of OTDOA technique

Considering the maturity of the OTDOA technique as well as its widespread adoption in LTE, we should consider to use OTDOA in LTE as the baseline for OTDOA in NR.

Proposal 1: OTDOA is supported as a RAT dependent downlink-based positioning solution in NR.

In LTE, Positioning Reference Signal (PRS) is one of the reference signals that is used to facilitate UE positioning determination based on the OTDOA method. PRS has the following properties:

  - PRS generation

PRS is generated based on a specific sequence generation as defined in TS 36.211. It is based on a length-31 gold sequence. The PRS sequence depends on the cell-id and frame/slot timing information.

  - Resource element mapping

The PRS is mapped to complex-valued QPSK modulation symbols and the mapping of the PRS signals to resource elements is defined according to TS 36.211.

  - PRS scheduling

PRS are transmitted in dedicated sub-frames (known as PRS occasions) and these PRS occasions may consist of several subframes. For example the number of PRS subframe within a PRS occasion can be 1, 2, 4, and 6 subframes. , The PRS occasions are transmitted with periodicities of 160, 320, 640 or 1280 ms.

There are many other PRS properties that have been considered in LTE, such as PRS muting, network synchronization, etc. For the OTDOA in NR positioning, we should consider to revisit the aforementioned PRS properties in LTE and whether to completely re-use or modify those properties for potential positioning improvements. The introduction of a new signal, a so-called NR-PRS, should be studied during the NR positioning study item phase.

The usage of any existing NR reference signal should be studied as well. This was also the case in LTE where the existing reference signals, such as CRS and/or PSS/SSS can be used for positioning. The UE can decide which signals are to be used for positioning estimation purposes. However, depending on the cell deployment, the aforementioned reference signals are expected to have higher interference levels than the PRS signal. The PRS has been designed to have lower levels of interference, for example, with PRS transmission from a single antenna port with a re-use factor of 6 (typically higher than CRS) and PRS muting operation.

The existing NR reference and synchronization signals that can be considered for positioning purposes are CSI-RS, TRS, and SSB. The properties of these signals may not the same as the above LTE signals (CRS, PSS/SSS). Thus, their usage should also be studied during the NR positioning study item phase.

Proposal 2: RAN1 to consider the possible downlink positioning reference signals (i.e. CSI-RS, TRS, SSB, and potentially NR-PRS signal) and investigate the usability of each signal.

NR rel-15 has many new physical layer features in comparison to LTE. NR can accommodate wider bandwidth and operates in various frequency ranges. In sub 6 GHz (FR1), the maximum bandwidth is 100 MHz and in millimeter wave range (FR2) the maximum bandwidth is 400 MHz. Carrier aggregation operation can further extend the bandwidth. In FR2, operation with beam forming, with beams consecutively transmitted in multiple directions (facilitated by MIMO antenna scheme), is commonly used. Each beam may have a narrow beam pattern with high gain. For example, the existing NR rel-15 specifications can accommodate up to 64 beams for synchronization signal block (SSB) transmission. Considering there are many significant improvements in NR, we should exploit such NR features for potential OTDOA positioning improvements.

Proposal 3: Exploit NR features (e.g. wider bandwidth, MIMO beam-forming operation, densify network) for potential OTDOA positioning improvements.

In LTE, the reference signal that can be used for positioning purposes is transmitted by the eNB under the assumption that the eNB has an omni-directional/sector antenna, at least there is no indication of the beam aspect of the antenna being used in transmitting the PRS. Similarly, the UE is expected to use an omni-directional or relatively wide antenna for receiving the beam carrying the reference signal.

As indicated above, particularly in FR2, NR is expected to operate with one or multiple narrow beams in order to compensate for the path loss in mmWave/FR2 frequency bands. This property means that LTE- like transmission of PRS reference signals cannot be fully adopted in NR. RAN1 needs to study, for example, identifying a proper scheduling mechanism to accommodate beam-forming positioning reference signal transmission on those multiple beams to cover a wider cell range These include the resource mapping aspects and beam-forming arrangement aspects. As illustrated in Figure 2, a UE needs to receive multiple beam-forming signals from gNBs or TRPs for the triangulation in OTDOA. In addition, the UE is expected to receive a beam-forming signal that can indicate a certain direction. This can be further exploited in order to improve the positioning accuracy. This operation can be facilitated by introducing beam-identity on each beam-formed signal.

Figure 2: Illustration of OTDOA with beam-forming signal from gNBs.

Proposal 4: On beamforming operation with potentially new NR-PRS, study at least the following:

  - Resource mapping of positioning reference signals.

  - Beam arrangement (e.g. beam sweeping operation).

  - Beam identity

In our previous contribution [3] we have summarized NR positioning requirements. The requirements are targeting numerous use-cases/scenarios. There are multiple dimensions of performance metric (horizontal accuracy, vertical accuracy, availability, latency, etc) and various requirement levels (sub-1m, 10m, 50m, etc).

It may be difficult to identify one technical solution that fits to all use-cases and scenarios. In OTDOA, an accurate positioning result typically requires wide bandwidth and many reference signal (e.g. PRS) subframes. Many reference signal subframes may increase the latency. Yet, some use-cases require high accuracy and short latency. In practice, we consider the majority of use-cases would not require high accuracy and require reasonable latency.

Based on the above observation, an adaptive/flexible approach to positioning techniques may be beneficial. A flexible approach can include the combination of RAT dependent and RAT independent techniques. Another consideration is the ability to flexibly configure OTDOA positioning with a goal of achieving the positioning requirements.

Proposal 5: Adaptive positioning reference signal transmission is supported in order to simultaneously support demanding requirements (high accuracy and low latency) for some UEs and relaxed requirements for typically many UEs.

# Conclusion

In this contribution, we have made the following proposals:

Proposal 1: OTDOA is supported as a RAT dependent downlink-based positioning solution in NR.

Proposal 2: RAN1 to consider the possible downlink positioning reference signals (i.e. CSI-RS, TRS, SSB, and potentially NR-PRS signal) and investigate the usability of each signal.

Proposal 3: Exploit NR features (e.g. wider bandwidth, MIMO beam-forming operation, densify network) for potential OTDOA positioning improvements.

Proposal 4: On beamforming operation with potentially new NR-PRS, study at least the following:

  - Resource mapping of positioning reference signals.

  - Beam arrangement (e.g. beam sweeping operation).

  - Beam identity

Proposal 5: Adaptive positioning reference signal transmission is supported in order to simultaneously support demanding requirements (high accuracy and low latency) for some UEs and relaxed requirements for typically many UEs.

# References

- RP-181399, “New SID: Study on NR positioning support”

- Chairman’s notes RAN1 meeting 94bis, Chengdu, October 2018

- R1-1810646, “Considerations on NR Positioning Requirements”, October 2018.