3GPP TSG RAN WG1 Meeting #95 		R1-1812853

Spokane, USA, November 12th – 16th, 2018

## Agenda item:	7.2.6.1.1

Source: 	AT&T

Title: 	On Potential Enhancements to PDCCH for URLLC

Document for:	Discussion/Decision

# Introduction

In RAN #80, a new study item on Physical layer enhancements for NR URLLC was approved.  The objective of the study item is to investigate different URLLC L1 improvements to further improve reliability/latency. In the RAN1#94 meeting, the following agreements were made.

Agreements:

Further evaluate the potential PDCCH enhancements for NR Rel-16 URLLC.

- Further evaluate PDCCH reliability

- Further evaluate PDCCH blocking

  - Companies describe the resource utilization

- Complexity should be considered

- Latency of the enhancement(s) should be considered

Agreement:

- Study further how to enable more than one PUCCH for HARQ-ACK transmission within a slot.

Agreements:

Study further whether/how to enable enhanced reporting procedure/feedback for HARQ-ACK.

- Enhanced HARQ-ACK multiplexing on PUSCH and PUCCH

- Finer indication for HARQ feedback timing, e.g. symbol-level, half-slot, etc.

  - Note: this may be related to more than one PUCCH for HARQ-ACK tx within a slot

- Other enablers are not precluded

Agreements:

Study the need for enhanced CSI reporting/measurement mechanisms. E.g.,

- DMRS based CSI

- A-CSI on PUCCH

- Trigger by DL assignment

- Enhanced CSI reporting mode

- Other approaches are not precluded

In this contribution, we provide detailed analysis of PDCCH enhancements for NR URLLC.

# Motivation for PDCCH Enhancements for NR URLLC

In this section we provide mathematical analysis for PDCCH reliability. Let  PR is the reliability or the probability  of correct reception of the NR URLLC packet at the application layer. Then for downlink packets transmission, the PR depends on

- Probability of correct reception of downlink control channel

- Probability of correct reception of PDSCH

- Probability of correct reception of HARQ-ACK

Since these three events are independent dent from the network point of view, the ioint probability of correct reception at the application layer is given by

Where Pdci is the probability of error for DCI, Ppdsch is the probability of transport block error for PDSCH and PHARQ-ACK is the probability of error for HARQ-ACK

Since we are interested in the probability of error for downlink control channel Pdci

Since Pdci is the probability and can’t be greater than unity, from the above equation we can conclude that

Similarly

Hence to satisfy the above equation

Table 1 shows the Pdci           for a given Ppdsch. It can be observed that for a reasonable performance to meet the NR URLLC target requirements the PDCCH performance has to be enhanced significantly.

Table 1 Required probability of block error rate for PDCCH for downlink data transmission

| PR in percentage | Pdci |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ppdsch |  |  |  |  |  |  |
|  | 0.1 | 0.01 | 0.001 | 0.0001 | 0.00001 | 0.000001 | 0.0000001 |
| 99.9999 | NA | NA | NA | NA | NA | NA | 0.0000009 |
| 99.999 | NA | NA | NA | NA | NA | 0.000009 | 0.0000099 |
| 99.99 | NA | NA | NA | NA | 0.00009 | 0.000099 | 0.0000999 |

Figure 1 shows the FER for PDCCH with 2 receiving antennas.  Typically the cell edge UE in NR will have the SNR of -7 dB. Hence for that SNR, it can observed that NR URLLC services can’t be satisfied with the conventional downlink control channel.  One way to improve the performance of PDCCH is to increase the aggregation level to 16 or more. However, it should be noted that higher aggregation levels are not always possible for all bandwidths. Hence we recommend RAN1 should study schemes to enhance the PDCCH performance.

Figure 1 FER performance of PDCCH with 2 receiving antennas

Proposal 1:  RAN1 should study schemes for enhancing the performance of PDCCH

# PDDCH Enhancements

For PDSCH transmission, NR Release 15 provides two DCI formats format1-0 and format 1-1. However, in our view we need to revisit the contents for NR-URLLC applications. For example many entries are not applicable for URLLC or we can optimize the number of bits for compact DCI design. In this section, we focus on the redundancy version indication in downlink control channel for URLLC.

## Indication of redundancy version in DCI

In section 7.3.1.2 of TS 38.212, it is mentioned that each transport block

For transport block 1:

-	Modulation and coding scheme – 5 bits

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

For transport block 2 (only present if Number-MCS-HARQ-DL-DCI equals 2):

-	Modulation and coding scheme – 5 bits as defined in Subclause x.x of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

That is the signaling overhead for the redundancy version is 2 bits per each transport block. Note that the RV indication in the DCI structure is taken from the legacy LTE/HSPA DCI structure.  However, we would like to mention that subblock interleaving and rate matching procedure are different for LDPC and turbo codes. For LDPC codes the starting position of each redundancy version is indicated in Table 6, and also shown as circular buffer in Figure 18.

Table 6: Starting position of different redundancy versions,

|  | Base graph 1 | Base graph 2 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

RV3

RV0

RV2

RV1

Figure 18 Redundancy versions using circular buffer

In NR, for each transmission, the network needs to inform to the UE which redundancy version it is currently scheduling.  From simulations, we observed that the only few redundancy versions (RV0 and RV3) are self-decodable for all code rates.  From network perspective, we observed that first transmission always uses RV0, since the systematic bits are sent in RV0 and it is beneficial to send RV0 in the first transmission. However, if we send RV0 in initial transmission irrespective of the deployment or the UE channel characteristics, we feel that there is no need to indicate RV0 in the first transmission (new transmission).

For the first transmission, the network can use the following structure to indicate RV0.

The first transmission is indicated using a bit called New Data Indicator (NDI) for each TB. This implies that for the first transmission, without indicating RV explicitly, we can use NDI bit so that UE knows that this is first transmission and use RV0 for this transmission.  i.e. whenever the NDI bit is toggled from previous successful transmission, it implies that RV0 is being used for the transmission. Hence we can save 2 bits for each TB for the first transmission.

DCI without RV for each TB, NDI=1 indicate RV0

Figure 19 Downlink control channel structure for the first transmission

For all the other (re) transmissions, conventional downlink structure as shown in Figure 20 is reused to indicate the RV, i.e. same structure as that of TS 38.212

Part 2

RV

Part 1

Figure 20 Downlink control channel for re transmissions.

In another technique, we can use joint encoding of NDI and RV as below. That is instead of using separate fields for RV and NDI for each TB, the bits are jointly encoded as shown in Table 7. In this case we can save 1 bit overhead for each TB in the DCI.

Table 7: Joint encoding of NDI and RV for DCI

| Joint Indication | RV | NDI |
| --- | --- | --- |
| 00 | RV0 | 1 |
| 01 | RV1 | 0 |
| 10 | RV2 | 0 |
| 11 | RV3 | 0 |

In another technique the network uses RV0 for every transmission i.e. HARQ-Chase combining, then the network can use a single bit to indicate this rather than using 2 bit to indicate RV.  i.e. introduce a new bit to differentiate  HARQ-CC or HARQ-IR. If that bit is set to 0 (HARQ-CC) means, RV0 is used for the current transmission, however when that bit is set to 1 (HARQ-IR), then additional 2 bits are used for each TB to indicate the RV for the current transmission. Hence for CC, only 1 bit is used, while for IR, 3 bits are used to indicate RV.  Hence we propose

Proposal 2: For NR-URLLC, to reduce the DCI payload, for the first transmission, RV is implicitly indicated as part of NDI

Proposal 3: Joint encoding of NDI and RV is used to reduce the DCI payload for NR-URLLC

Proposal 4: Adaptive payload structure should be used for indicating RV for NR-URLLC

## Indication of redundancy version using RRC

As explained in above section, for URLLC applications, we can save 2 bits for each TB if we can avoid the RV indication.  Since the reliability of URLLC is high say 1% or 0.1%. In these cases, the probability of packet pass is very high. Hence most of the packets pass in the first attempt itself. In these cases, indicating RV0 in the first transmission does not carry any new information to the UE. In addition, we believe that for URLLC applications, retransmitting RV0 does not penalize the performance compared to re transmitting RV1, RV2 or RV3. Hence we propose that the network can indicate RV 0 for all transmissions or multiple RV states through RRC signaling. By doing this, we can save 2 bits in each transmission for majority of the UEs. The method provides flexibility to the network vendor if the network believes HARQ-IR provides gains for a particular UE, they can enable multi state RV such that the gains can be obtained with 2 bit overhead for indicating RV.

Hence we propose

Proposal 5: Indication of single state RV0 (HARQ-CC) or multiple state RV (HARQ-IR, RV0, 1, 2, 3) is done using RRC signaling to reduce the payload of DCI

# Conclusions

In this contribution we described our views on PDCCH enhancements for NR URLLC.

Based on our observations, we have the following proposal:

Proposal 1:  RAN1 should study schemes for enhancing the performance of PDCCH

Proposal 2: For NR-URLLC, to reduce the DCI payload, for the first transmission, RV is implicitly indicated as part of NDI

Proposal 3: Joint encoding of NDI and RV is used to reduce the DCI payload for NR-URLLC

Proposal 4: Adaptive payload structure should be used for indicating RV for NR-URLLC

Proposal 5: Indication of single state RV0 (HARQ-CC) or multiple state RV (HARQ-IR, RV0, 1, 2, 3) is done using RRC signaling to reduce the payload of DCI