3GPP TSG RAN WG1 Meeting #95			R1-1812919

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.4.1.5

Source:	Apple

Title:	Considerations on NR V2X scheduling mechanism

Document for:	Discussion/Decision

# Introduction

NR Rel-16 V2X Study Item was approved in RAN#80 with detailed objective and scope in [1]. NR Rel-16 V2X is targeted to provide further enhancement to the existing LTE Rel-14/15 V2X by supporting enhanced V2X services. The enhanced V2X services and their requirement can be found in [2,3]. NR V2X needs to cover both the Uu interface and PC5 interface design, additionally, coexistence and joint operation with LTE V2X. From the spectrum perspective, NR V2X needs to support both the licensed and unlicensed spectrum which requires sidelink design.

In terms of RAN1 design goal, we think there are the following major difference between NR V2X and LTE V2X

- LTE V2X only targets for broadcast transmission, while NR V2X needs to support broadcast, groupcast and unicast transmission

- LTE V2X only targets for periodic traffic, while NR V2X needs to support periodic and aperiodic traffic

- NR V2X needs to provide higher throughput, lower latency ad higher reliability compared to LTE V2X

- NR V2V needs to support mmW band (FR2)

One fundamental problem NR V2X needs to resolve is resource allocation schemes, which is also one of the goal for NR RE-16 V2X SI as listed in [1]

Sidelink design [RAN1, RAN2]: - Identify technical solutions for a NR sidelink design to meet the requirements of advanced V2X services, including - Study the support of sidelink unicast, sidelink groupcast and sidelink broadcast - Study NR sidelink physical layer structures and procedure(s) - Study sidelink synchronization mechanism - Study sidelink resource allocation mechanism (also including objective 3) - Study sidelink L2/L3 protocols NOTE: Only the performance of advanced V2X use cases will be evaluated in the design of NR sidelink.

NR V2X already agreed to support at least two modes of resource allocation in [4]. Mode 1 is similar to LTE V2X mode 3 which is gNB based sidelink resource allocation while mode 2 is similar to LTE V2X mode 4 which is UE based resource allocation.

Agreements: At least two sidelink resource allocation modes are defined for NR-V2X sidelink communication - Mode 1: Base station schedules sidelink resource(s) to be used by UE for sidelink transmission(s) - Mode 2: UE determines (i.e. base station does not schedule) sidelink transmission resource(s) within sidelink resources configured by base station/network or pre-configured sidelink resources - Notes: - eNB control of NR sidelink and gNB control of LTE sidelink resources will be separately considered in corresponding agenda items. - Mode-2 definition covers potential sidelink radio-layer functionality or resource allocation sub-modes (subject to further refinement including merging of some or all of them) where - UE autonomously selects sidelink resource for transmission - UE assists sidelink resource selection for other UE(s) - UE is configured with NR configured grant (type-1 like) for sidelink transmission - UE schedules sidelink transmissions of other UEs RAN1 to continue study details of resource allocation modes for NR-V2X sidelink communication

In this contribution, we focus on the enhancement to NR V2X scheduling

# NR V2X Mode 1 Resource Allocation

Spatial reuse is one of the most important techniques to improve the capacity. For typical macro cellular deployment, the transmit power of the base station tower is very high which results in limited spatial reuse but provide ubiquitous coverage and mobility support. On the other hand, for small cell deployment or other RAT like WIFI, station transmit power is limited which provide high capacity by improving spatial reuse. For typical V2X use case, the transmit power is limited which allows the frequency domain resource to be re-used if two UEs are not close to each other. In this section, we consider the spatial reuse of Freq/Time resources based on UE distance/direction knowledge at gNB. With such knowledge, gNB can better assign Time/Freq domain resource based on UE’s relative distance and direction to avoid resource conflict and improve spectrum efficiency.

To support such functionality, V2X UE can be configured to send location information back to gNB through periodic report or event triggered report. For periodic update, the report can be carried over on the periodic PUSCH resource allocation configured by gNB. The event based update can also be sent if certain criteria are met, e.g., position has changed more than a preconfigured threshold, or it has moved to another location zone. Additionally, gNB can also send location info update request to pull from UE if certain timer expired without position update.

To improve the coverage of such kind of information, Road side unit (RSU) can help to relay the location info from PC5 monitoring to gNB through either backhaul or uU interface. Figure 1 illustrates the concept of enhanced spatial reuse based on location/beam information update.

Figure 1 Enhanced spatial reuse NR V2X Mode 1 resource allocation

Proposal 1: NR V2X Mode 1 resource allocation to consider supporting UE to update its location and beam information in order to achieve enhanced spatial reuse.

# NR V2X Mode 2 Resource Allocation

Similar as LTE V2X mode 4, NR V2X mode 2 allows the UE to autonomously select the resource for PSSCH transmission. The resource selection can be done from the resource pool configured by NW. In order to support Mode 2 operation, CCA procedure such as Listen Before Talk (LBT) is one of the most prominent solutions. Unlike NRU design, NR V2X design is required to provide more reliable and lower latency communication due to safety concern and high device movement speed. Therefore, optimization of Mode 2 resource allocation is very important

For LTE V2X mode 4, UE has to sense the channel for a while before using that resource which may lead to low efficiency and possible resource collision (reliability issue). The legacy sensing based approach cannot fully avoid collision. Therefore, we proposal to provide more resource allocation information in order to assist UE in selecting the right resources and avoid collision.

We introduce the concept of Resource Occupancy Message (ROM) which includes the frequency and time domain resource allocation for each UE, i.e. the RBs and symbol allocations, the periodicity to repeat this reservation, and the remaining time for this reservation. Every UE broadcasts ROM periodically if UE decides to occupy the channel after success of CCA procedure. For new UE synced to this system, UE will sense the channe to collect all existing UE’s ROM to find available resource. Figure 2 illustrates the concept of ROM.

Figure 2 Resource Occupancy Message (ROM) for collision reduction

Proposal 2: NR V2X Mode 2 resource allocation to consider supporting UE to broadcasts its resource allocation after success of CCA procedure for collision reduction

# Conclusion

Compared to LTE Rel-14/15 V2X, NR V2X is targeted to support large varieties of services that require broadcast/group/unicast communication, generate both periodic an a-periodic traffic, require higher data rate, lower latency and higher reliability. Resource allocation schemes are very important for sidelink operation. In this contribution, we consider two resource allocation schemes, one for Model and one for Mode 2. We have the following proposals

Proposal 1: NR V2X Mode 1 resource allocation to consider supporting UE to update its location and beam information in order to achieve enhanced spatial reuse.

Proposal 2: NR V2X Mode 2 resource allocation to consider supporting UE to broadcasts its resource allocation after success of CCA procedure for collision reduction

# References

- RP-182111, “Revised SID: Study on NR V2X”, LGE, RAN#81, Gold Coast, Australia, September 10-13, 2018

- 3GPP TR 22.886, “Study on enhancement of 3GPP Support for 5G V2X Services”, V15.2.0, Jun 2018

- 3GPP TS 22.186, “Enhancement of 3GPP support for V2X scenarios; Stage 1”, V15.3.0, June 2018

- Chairman notes, 3GPP TSG RAN WG1 Meeting #94, Gothenburg, Sweden, August 20th – 24th, 2018