3GPP TSG RAN WG1 Meeting #95  			R1-1812943

Spokane, USA, November 12th – 16th, 2018

Agenda item:	6.2.1.6

Source:	Samsung

Title:	Discussion on MPDCCH performance improvement for MTC

Document for:	Discussion and Decision

Introduction

In RAN1#94Bis Meeting, there are following agreements on MPDCCH performance improvement by using CRS for MTC [1]:

Agreement

CRS for improving channel estimation on MPDCCH in idle mode is supported. FFS the details.

Agreement

For localized MPDCCH, the following mappings are supported between CRS and DMRS:

- Predefined mapping

- Mapping based on CSI report

FFS the details.

Agreement

For distributed MPDCCH, mapping between CRS and DMRS is defined in specifications.

- FFS the details (e.g. number of PRBs over which mapping applies, fixed or configurable)

Agreement

Power offset between CRS and DMRS antenna ports of MPDCCH is signaled by eNB. FFS the signaling details (e.g. provisioning for default power offset value).

In this contribution, some issues are discussed, including mapping between CRS and DMRS for localized MPDCCH, mapping between CRS and DMRS for distributed MPDCCH, and power offset between CRS and MPDCCH DMRS.

Discussion

Mapping between CRS and DMRS for localized MPDCCH

In the last meeting, one agreement for localized MPDCCH is that predefined mapping and mapping based on CSI report are supported between CRS and DMRS, and FFS the details. For predefined mapping, explicit signaling is required to configure the used mapping in a predefined set of mappings between CRS and DMRS. A precoder with dimension of 4*1 can be used to reflect the mapping between CRS and DMRS, i.e., channel coefficients estimated on 4 CRS ports can be multiplied by the precoder to reconstruct the channel experienced by one DMRS port. Thus, a set of precoders should be defined in specifications. The configurable precoder can be selected from existing codebook for PMI reporting. The size of the predefined set can be further discussed, e.g., 16, 32, or 64.

Proposal #1: A set of precoders is defined to support predefined mapping.

For mapping based on CSI report, it is used only in CE Mode A due to no CSI reporting in CE Mode B. When using mapping based on CSI report, the latest reported PMI should be used for the mapping between CRS and DMRS. The applicable time point of reported PMI should be clarified. For example, assuming PMI is reported in subframe n, UE can assume the reported precoder is applied for mapping between CRS and DMRS in subframe n+4. The reported precoder is used until a new reported PMI is applied.

Proposal #2: Applied time point of reported PMI should be clarified for mapping based on CSI report.

Mapping between CRS and DMRS for distributed MPDCCH

In the last meeting, one agreement for distributed MPDCCH is that mapping between CRS and DMRS is defined in specifications, and FFS the details (e.g. number of PRBs over which mapping applies, fixed or configurable). In order to use a unified mapping for both localized MPDCCH and distributed MPDCCH, precoder with dimension of 4*1 can also be used to reflect the mapping between CRS and DMRS. Different from localized MPDCCH, the used precoder can be [1 0 0 0] to enable a one-to-one mapping between CRS and DMRS.

For distributed MPDCCH, since pursuing antenna diversity gain is an objective of the design, a predefined pattern of precoder cycling should be applied in time and frequency domain. However, PRB bundling or subframe bundling, i.e., using the same precoder within the bundled PRBs or subframes, can be used to improve performance of channel estimation. Therefore, the granularity of precoder cycling should not be per PRB or per subframe. To provide flexibility, the number of PRBs using the same precoder can be configurable, e.g., 2, 3 or 6. Similarly, the number of subframes using the same precoder can be configurable, e.g., 4 or 8.

Proposal #3: Number of PRBs using the same precoder can be configurable.

Proposal #4: Number of  subframes using the same precoder can be configurable.

Power offset between CRS and MPDCCH DMRS

When using CRS for improving channel estimation of MPDCCH DMRS, power offset between MPDCCH DMRS port and CRS port should be known by UE. In the last meeting, one agreement is that power offset between CRS and DMRS antenna ports of MPDCCH is signaled by eNB, and FFS the signaling details (e.g. provisioning for default power offset value). To save signaling overhead, the signaling for indication of power offset can be optional. A default value can be used if power offset is not indicated, e.g., 0dB. For MPDCCH USS and Type0-MPDCCH CSS, power offset should be indicated in UE-specific RRC signalling. For CSS used in RRC-idle mode, e.g., Type1-MPDCCH CSS (for paging) and Type2-MPDCCH CSS (for RAR), power offset can be indicated in system information.

Proposal #5: A default value can be used if power offset is not indicated, e.g., 0dB.

Proposal #6: For MPDCCH  USS and Type0-MPDCCH  CSS,  power offset can be indicated in UE-specific RRC signaling.

Proposal #7: For MPDCCH CSS used in RRC-idle mode, power offset can be indicated in system information, e.g., Type1-MPDCCH  CSS and Type2-MPDCCH  CSS.

- 3. Conclusion

In this paper, we discussed some issues on MPDCCH performance improvement by using CRS, including mapping between CRS and DMRS for localized MPDCCH, mapping between CRS and DMRS for distributed MPDCCH, and power offset between CRS and MPDCCH DMRS. Based on above discussion, we have the following proposals:

Proposal #1: A set of precoders is defined to support predefined mapping.

Proposal #2: Applied time point of reported PMI should be clarified for mapping based on CSI report.

Proposal #3: Number of PRBs using the same precoder can be configurable.

Proposal #4: Number of  subframes using the same precoder can be configurable.

Proposal #5: A default value can be used if power offset is not indicated, e.g., 0dB.

Proposal #6: For MPDCCH  USS and Type0-MPDCCH  CSS,  power offset can be indicated in UE-specific RRC signaling.

Proposal #7: For MPDCCH CSS used in RRC-idle mode, power offset can be indicated in system information, e.g., Type1-MPDCCH  CSS and Type2-MPDCCH  CSS.

- 4. Reference

- Final Chairman’s notes for RAN1#94Bis Meeting