3GPP TSG-RAN WG1 Meeting #95			R1-1813191

Spokane, WA, USA, November 12 – 16, 2018

Title:	[DRAFT] Reply LS on TSN requirements evaluation

Response to:	R1-1812110 (R2-1816043) LS on TSN requirements evaluation

Release:	Release 16

Work Item:	FS_NR_IIOT

Source:	Nokia [RAN1]

To:	RAN2

Cc:	RAN3, SA1, SA2

Contact Person:

#### Name:	Sigen Ye

E-mail Address:	sigen.ye@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 would like to thank RAN2 on the LS TSN requirements evaluation. RAN1 has discussed the following request from RAN2:

- provide feedback on whether 0.5 ms latency target can be achieved using current NR specification and/or enhancements considered as part of L1 URLLC enhancements SI. - provide feedback on what the achievable time synchronization accuracy over Uu interface, considering the synchronicity requirements of TSN networks as mentioned in TR 22.804

RAN1 has performed some latency and reliability analysis for the use case I provided in the LS. The 0.5 ms latency target can be achieved with physical layer reliability of 10-4 (which translates into 10-8 reliability assuming PDCP packet duplication) under certain assumptions (e.g. FDD, subcarrier spacing of 30 kHz or higher, no HARQ retransmissions, aggressive UE and gNB processing time, isolated cell with small service area) using NR Rel-15 specifications and/or potential enhancements in Rel-16.

For the time synchronization on Uu interface, a UE timing synchronization accuracy of [472.5ns] for 15kHz SCS, [338.5ns] for 30kHz and [255ns] for 60kHz SCS can be achieved based on NR Rel-15. In Rel-16, an improved UE timing synchronization accuracy could be achieved through tighter UE requirements and timing advance enhancements. Note that these numbers do not contain the effects of the granularity & accuracy of the absolute timing indication information by the gNB which are outside of the RAN1 study scope.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above into consideration.

3. Date of Next TSG-RAN WG1 Meetings:

3GPP RAN WG1 ad hoc		21 – 25 Jan 2019   	 	Taipei, Taiwan

3GPP RAN WG1#96 		25 Feb – 1 Mar 2019    	Athens, Greece