3GPP TSG RAN WG1 Meeting #95 R1-1813301 Spokane, USA, November 12th – 16th, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on 38.213 regarding number of BDs and CCEs for DL BWP. Source to WG:; NTT DOCOMO, INC.. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2018-11-02. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; At the RAN1#94bis meeting, following conclusion has been made: Conclusion: - It is understood that the BD/CCE limit is based on the current active DL BWP However, it has not been yet clarified how to count the number of BDs and CCEs for serving cells not being active. For the serving cells not being active, there are no active DL BWPs. Since UE is able to count the BDs/CCEs based on the active DL BWP, the UE should also be able to count the BDs/CCEs based on the active serving cells. In other words, there is no reason to count the BDs/CCEs for non-active serving cells. Therefore, our proopsal is to count the BDs/CCEs based on the active DL BWP of the active serving cells.. Summary of change:; Clarify that BD/CCE counting is according to the active DL BWPs of active serving cells.. Consequences if not approved:; gNB and UE cannot have common understanding on the numbers of BDs and CCEs. Clauses affected:; 10.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

<<Unchanged part omitted>>

Table 10.1-2 provides the maximum number of monitored PDCCH candidates, , for an active DL BWP with SCS configuration  for a UE per slot for operation with a single serving cell.

Table 10.1-2: Maximum number  of monitored PDCCH candidates per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of monitored PDCCH candidates per slot and per serving cell |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 10.1-3 provides the maximum number of non-overlapped CCEs, , for an active DL BWP with SCS configuration  that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell.

CCEs for PDCCH candidates are non-overlapped if they correspond to

-	different CORESET indexes, or

-	different first symbols for the reception of the respective PDCCH candidates.

Table 10.1-3: Maximum number  of non-overlapped CCEs per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of non-overlapped CCEs per slot and per serving cell |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

If a UE

-	is capable for operation with carrier aggregation with a maximum of 4 downlink cells or indicates through pdcch-BlindDetectionCA a capability to monitor PDCCH candidates for  downlink cells, and

-	is configured with  active downlink cells with active DL BWPs having SCS configuration  where  or , respectively, and

-	for cross-carrier scheduling over  active downlink cells, including a scheduling cell, from the  downlink cells where the scheduling cell and scheduled cell(s) have DL BWPs with same SCS configuration

the UE is expected to be able to monitor  PDCCH candidates and  non-overlapped CCEs per slot on the active DL BWP of the scheduling cell. The case of  corresponds only to self-scheduling for the scheduling cell.

If a UE

-	indicates through pdcch-BlindDetectionCA a capability to monitor PDCCH candidates for  downlink cells, and

-	is configured with  active downlink cells with active DL BWPs having SCS configuration  where , and

the UE is expected to be able to monitor a total of  PDCCH candidates and a total of  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the  downlink cells.

<<Unchanged part omitted>>