3GPP TSG RAN WG1 Meeting #95                                                 R1-1813316

Spokane, USA, November 12th – 16th, 2018

Source:	NTT DOCOMO, INC.

Title:	Discussion on enhancements to support NR backhaul links

Agenda Item:	7.2.3.1

Document for: 	Discussion and Decision

- Introduction

At the RAN1#94bis meeting, enhancements to support NR backhaul links for NR IAB were discussed, and RAN1 made a number of agreements [1].

In this contribution, we discuss possible enhancements to support NR backhaul links for NR IAB including IAB node initial access procedure, backhaul link selection based on discovery/measurement and transmission timing of IAB node.

- Discussion on IAB node initial access (Stage 1)

At the RAN1#94 meeting, RAN1 made the following agreements regarding IAB node initial access procedure [2]. There are two FFS points.

Agreements: - In case of SA deployments, initial IAB node discovery by the MT (Stage 1) follows the Rel.15 procedure for cell search and initial access based on SSBs available for access UEs without additional required specification support. - FFS: how to support NSA deployment - FFS whether Stage 2 enhancements (if any) can be applied to Stage 1

On the other hand, as described in TR38.874, there are the following requirements regarding SA and NSA for NR IAB [3]. Therefore, how to support NSA Stage 1 initial access should be further studied.

- 1: SA and NSA shall be supported for the access link. For an NSA access link, relaying is applied to the NR path. Relaying of the LTE path is contingent on the support of backhauling of LTE traffic. - 2: Both NSA and SA shall be studied for the backhaul link. Backhaul traffic over the LTE radio interface is excluded from the study. - 3: For NSA access- and backhaul links, the study shall consider EN-DC with priority. However, other NSA options shall not be precluded from the study.

2.1	How to support NSA stage 1 initial access

There would be a possible scenario where a new IAB node is deployed on NW/frequency layer supporting NSA only, e.g., on higher frequency to expand the coverage. Even in NW/frequency layer supporting NSA only, IAB node initial access should be supported for “plug and play” deployment of new IAB node. In such a scenario, we think that there are following three possible approaches for stage 1 initial access.

Alt.1: The new IAB node for NSA deployment supports LTE MT functionalities, and it uses LTE initial access and EN-DC procedures for stage 1 initial access.

Alt.2: IAB donors and IAB nodes on NSA deployment provide functionalities/signals/channels (same as in SA deployment) so that the new IAB node follows same initial access procedure as access UE even in NSA deployment.

Alt.3: IAB donors and IAB nodes on NSA deployment provide modified functionalities/signals/channels (different from SA deployment) so that the new IAB node can perform stage 1 initial access.

In Alt.1, the new IAB node needs to support LTE MT functionalities, although the LTE MT functionalities are not necessary at all for IAB operation after IAB node initial access. In addition, after stage 1 initial access, when the IAB node needs to change its backhaul link due to sudden blocking, the IAB node would need to perform LTE initial access and EN-DC procedures again and it causes quite long BH switching delay. Therefore, we think that implementing LTE MT functionalities on IAB node for NSA deployment seems inefficient and this approach is not attractive.

In Alt.2, since IAB donor and existing IAB node need to provide functionalities/signals/channels as described below for supporting IAB node initial access, i.e., the same procedure for access UE, system efficiency and corresponding performance would be degraded due to unnecessary overhead. In addition, since it is NSA deployment, IAB donor and existing IAB node need to prevent an initial access from UEs, i.e., some mechanism to allow only IAB node initial access is necessary.

- For standalone initial access of UE, transmission periodicity of SS/PBCH block and RMSI (SIB1) basically needs to be short such as equal to or less than 20 ms. However, new IAB node initial access will rarely occur and even for discovery/measurement on backhaul links, transmission periodicity does not need to be so short since fixed IAB node is assumed.

- RMSI (SIB1) includes not only RACH configuration information but also many other information such as other SI scheduling that would not be necessary for new IAB node initial access. Except for information necessary to establish initial connection to IAB donor or node, RMSI (SIB1) information would not need to be broadcasted since IAB node would be able to obtain necessary information after connection is established.

In Alt.3, we can design modified functionalities/signals/channels for IAB node initial access so that resource/signaling/procedure overhead for stage 1 initial access in NSA deployment can be minimized.

Figure 1: Signals/channels provided by IAB donor and existing IAB node in SA/NSA operation

Therefore, as initial access procedure of IAB node, it is beneficial to consider some modification from initial access procedure of access UE. For example, the following modifications can be considered.

- For initial access, IAB node assumes longer SS/PBCH block periodicity than 20 ms so that IAB donor/node operating NSA for access UE can transmit SS/PBCH block with long periodicity.

- RMSI for IAB node initial access is also transmitted with longer periodicity than SS/PBCH block periodicity. Broadcasted RMSI for IAB node initial access would be used only for IAB node initial access, and hence transmission periodicity can be even longer than 160ms.

- RMSI for IAB node initial access transmitted by IAB node/donor supporting NSA for access UE contains only minimum necessary information to establish initial connection to IAB node/donor, such as RACH configuration.

- Presence/absence of RMSI for IAB node (not for UE) associated with SS/PBCH block is indicated by different way from RMSI presence/absence indication for UE. Otherwise, initial access UE may misunderstand that detected SS/PBCH block has associated RMSI.

Proposal 1: For NR IAB in NSA deployment, at least the following modifications to support IAB node initial access procedure should be supported.

- For IAB node initial access, IAB node assumes longer SS/PBCH block periodicity than 20 ms.

- RMSI for IAB node initial access can be transmitted with longer periodicity than SS/PBCH block periodicity.

- RMSI for IAB node initial access transmitted by IAB node/donor supporting NSA for access UE contains only minimum necessary information to establish initial connection to IAB node/donor, such as RACH configuration.

- Presence/absence of RMSI for IAB node (not for UE) associated with SS/PBCH block is indicated by different way from RMSI presence/absence indication for UE.

- Discussion on IAB node discovery and measurement (Stage 2)

At the RAN1#94bis meeting, RAN1 made following agreements regarding IAB node discovery and measurement [1].

Agreements: - Solution 1-B means SSB, that may get muted, for inter-IAB cell search and measurement in stage 2 is not on the currently defined sync raster for a SA frequency layer, while for a NSA frequency layer the SSBs are transmitted outside of the SMTC configured for access UEs - Solution 1-A means SSB for inter-IAB cell search in stage 2 is on the currently defined sync raster for a SA frequency layer, while for a NSA frequency layer the SSBs are transmitted inside of the SMTC configured for access UEs Agreements: An IAB node should not mute SSB transmissions targeting UE cell search and measurement when doing inter-IAB cell search in stage 2 - For SA, means that SSBs transmitted on the currently defined sync raster follows the currently defined periodicity for initial access - Means that Solution 1-B implies SSB, that may get muted, for inter-IAB stage 2 cell search is at least TDM with SSB used for UE cell search and measurements Agreements: - Solution 1-A and Solution 1-B are both supported - Enhancements for off-raster SSB, e.g. new periodicities and time-domain mapping can be considered Agreements: - CSI-RS can be used for inter-IAB detection in synchronous network - Extended CSI-RS periodicities can be considered.

The following aspects should be further considered as part of the study as described in the TR [4].

- Enhancements to Rel-15 CSI-RS and SSB measurement configurations and required coordination

- Enhancements for off sync-raster SSB, e.g. new periodicities and time-domain mapping

- Extended periodicities for CSI-RS for IAB node discovery and measurement

3.1	Enhancements to Rel-15 CSI-RS and SSB measurement configurations and required coordination

Although the fixed deployment of IAB node is assumed, BH link (i.e., parent node/donor and/or beam) for the child IAB node may need to be changed e.g., in following cases.

- Ex1: The current BH link is blocked and the link quality is degraded.

- Ex2: A new IAB node/donor is deployed and it can provide better BH link than current parent node/donor.

- Ex3: A certain issue (e.g., BH link blocking and/or congestion) is occurred at the current parent node/donor.

Therefore, for stage 2 IAB node discovery, each IAB donor and node supporting parent node functionality should transmit CSI-RS or SSB with beam sweeping to cover all supported directions so that IAB child node can detect the candidate BH link(s) from all possible beams. Since the fixed deployment of IAB node is assumed, the backhaul link quality would not fluctuate frequently. So, for stage 2 IAB node discovery, SSB or CSI-RS transmission (if it is dedicated for stage 2 IAB node discovery) could apply quite long periodicity.

On the other hand, for detected candidate BH link(s) as well as current BH link, frequent measurement of RS would be beneficial to achieve timely BH link switching. We consider that for this candidate BH link measurement, sweeping all beams is not necessary and periodic transmission of only some beams that are detected by surrounding IAB nodes is enough and efficient, as shown in Figure 2. In order to select the transmitting beams for BH link measurement, results of stage 2 IAB node discovery at a IAB node should be reported/indicated to the parent node and/or surrounding nodes. Detailed configurations for stage 2 IAB node discovery, those for stage 2 IAB node measurement, and detailed mechanism of reporting the stage 2 IAB node discovery results are left for WI phase.

Proposal 2: Rel-16 IAB should support separate configurations between the transmission/measurement of SSB and/or CSI-RS for stage 2 IAB node discovery and those for stage 2 IAB node measurement.

- At least for stage 2 IAB node discovery, longer transmission/measurement periodicity and longer duration of measurement window than those for Rel-15 SSB and CSI-RS should be supported.

- Reporting the results of the stage 2 IAB node discovery, e.g., detected RS index(es) and its quantity(es), from IAB node to its parent node and/or surrounding node(s) should be supported.

- Detailed configurations for transmission/measurement of SSB and/or CSI-RS and mechanism for reporting stage 2 IAB node discovery results are left for WI phase.

Figure 2: Separate RS transmissions for candidate BH link detection and measurement

3.2	Enhancements for off sync-raster SSB

For off sync-raster SSB, we think at least same enhancements as proposed in previous sub-section, e.g., longer periodicity, should be considered. In addition, since off sync-raster SSB has no impact on Rel-15 UE, following enhancements can be considered.

- Time domain mapping pattern: We think that Rel-15 time-domain mapping pattern for SSB within 5ms half frame does not need to be changed even for off sync-raster SSB. But as IAB node/donor would support large number of narrow beams for backhaul link coverage, it would be beneficial to support over 64 beams for off sync-raster SSB. For such large number of SSB beams, transmitting SSBs in multiple half frames (e.g., within a radio frame) with different beams can be considered as shown in Figure 3.

- Wideband measurement: combining SSB and FDMed CSI-RS or frequency domain repetition of SSB for discovery and measurement can improve one-shot detection/measurement performance (e.g., accuracy of RSRP/SINR) so that longer transmission periodicity can be applied to SSB/CSI-RS transmission. IAB node measurement bandwidth can be wider than that of UE.

Proposal 3: For off sync-raster SSB, following enhancements should be supported.

- Over 64 Tx beams for off sync-raster SSB within a burst which is longer than 5ms

  - QCL assumption/indication between SSBs within a burst (especially SSBs with same index in different half frames)

  - Actually transmitted SSB indication for over 64 SSBs (e.g., bitmap longer than 64bits)

- SSB measurement window duration longer than 5ms

- SSB transmission/measurement periodicity longer than 160ms

Figure 3: Possible enhancement for off sync-raster SSB for larger number of Tx beams

- Discussion on BH link management and route switching

In case of Ex1 described in Section 3.1, since the child IAB node cannot communicate with current parent node/donor in case of blocking, the BH link switching procedure should be initiated by child IAB node in such case. For example, beam recovery procedure or random access procedure for current parent node (in case of beam switching within the current node) or new parent node (in case of parent node switching) can be considered. Basically, there would be only one LOS path between two nodes, and if the LOS path is blocked, it would be better to switch parent node rather than just switching the beam for the current parent node. Therefore, in order to minimize the BH link switching delay, IAB RLM and/or BFR mechanism should cover not only beams of the current parent node but also other beams of surrounding node(s) that are detected in stage 2 IAB node discovery. In such case, stage 2 IAB node measurement discussed in Section 3.1 can be covered by the IAB RLM and/or BFR mechanism.

Proposal 4: IAB RLM and/or BFR mechanism should cover not only beams of the current parent node but also other beams of surrounding node(s) that are detected in stage 2 IAB node discovery.

On the other hand, in case of Ex2 and Ex3 described in Section 3.1, network side (i.e., current parent node/donor) can initiate the BH link switching for the child IAB node since the current BH link is available. For such cases, the BH link condition notification mechanism is important and can be considered as a trigger/indication for BH link switching.

As discussed in our contribution [2], it is beneficial for each child IAB node to know not only BH link condition information of candidate parent nodes but also traffic load condition information such as resource utilization or expected throughput of candidate parent nodes for case of Ex3 as shown in Figure 4.

Proposal 5: For appropriate backhaul link selection, it is beneficial for IAB node/donor to support at least one of followings.

- If each IAB node decides its backhaul link, each IAB node/donor transmits (e.g., broadcasts) information related to link quality and traffic load on backhaul link(s) to its IAB donor.

- If IAB donor decides backhaul links for all child IAB nodes, each IAB node reports information related to link quality and traffic load on backhaul link(s) including its measurement results on IAB node discovery signal of parent IAB node and measurement results reported from child IAB node(s).

Figure 4: Example of IAB node backhaul link selection scenario

- Discussion on IAB node synchronization and timing alignment

At the RAN1#94bis meeting, RAN1 made following agreements regarding transmission timing of IAB node [1].

Agreements: - For Timing and Synchronization - For case #1 & case #7, if DL TX and UL RX are not well aligned at the parent node, additional information about the alignment is needed for the child node to properly set its DL TX timing for OTA based timing & synchronization - Case #7 to be supported if and only if compatible with release 15 Ues - Further check w.r.t. compatibility - Support of case #6 is FFS - No other cases are supported

Case #6 and #7 are defined as below in the TR[4].

- Case #6 (Case#1 DL transmission timing + Case #2 UL transmission timing):

  - The DL transmission timing for all IAB nodes is aligned with the parent IAB node or donor DL timing

  - The UL transmission timing of an IAB node can be aligned with the IAB node’s DL transmission timing

- Case #7 (Case#1 DL transmission timing + Case #3 UL reception timing):

  - The DL transmission timing for all IAB nodes is aligned with the parent IAB node or donor DL timing

  - The UL reception timing of an IAB node can be aligned with the IAB node’s DL reception timing

  - If DL TX and UL RX are not well aligned at the parent node, additional information about the alignment is needed for the child node to properly set its DL TX timing for OTA based timing & synchronization

Figure 5: Example of IAB node transmission timing alignment Case #7

Figure 6: Example of IAB node transmission timing alignment Case #6

In Case #7, as shown in Figure 5, the UL reception timing such as UL from child node (LC,UL) and UL from access UE (LA,UL) can be aligned with DL reception timing, i.e., DL from parent node/donor (LP,DL), so that FDM/SDM between LP,DL and {LC,UL and/or LA,UL} are possible. In terms of the compatibility with Rel-15 UE, as the propagation delay of LP,DL would be longer than that of LA,UL in most cases, the timing alignment between UL reception from Rel-15 UE and DL reception from parent node/donor would not be available due to a necessity of negative TA. Hence, for Rel-15 UE, a normal UL TA handling can be applied. From Rel-15 UE perspective, Case #7 should have no problem since all DL transmission timing of base stations/IAB nodes are aligned and the normal UL TA handling is applied.

In Case #6, as shown in Figure 6, the UL transmission timing towards parent node (LP,UL) can be aligned with DL transmission timing such as DL for child node (LC,DL) and DL for access UE (LA,DL) so that FDM/SDM between LP,UL and { LC,DL and/or LA,DL} are possible. For this UL transmission timing at a IAB node, its parent node needs to support specific UL reception timing which is delayed from a normal UL reception timing at the parent node. Therefore, Case #6 is possible at the IAB node only when its parent node configures/indicates Case #6 UL transmission timing for LP,UL. We think that Case #6 also has no problem in terms of the compatibility with Rel-15 UE.

BH link DL/UL transmission timing may not provide any impact on access UE procedure since UE is not required to receive BH link DL/UL signal/channel. In addition, for potential CLI, whether some transmission timing case(s) cause CLI issue or not would be dependent on deployment scenario and configuration for operation. Therefore, multiple cases can be supported and preferred case can be selected according to deployment scenario, operation and IAB node implementation (e.g., which case is supported). For efficient CLI management, it would be necessary to apply the same transmission timing case across IAB nodes in an area. Therefore, the indication of transmission timing case for IAB node coordination should be supported.

Proposal 6: For IAB node transmission timing, Case #6 is supported in addition to Case #1/#7 and preferred case can be selected in operation according to IAB node coordination based on deployment scenario, operation and IAB node implementation.

- Details on IAB node coordination for transmission timing case alignment (e.g., indication of transmission timing case, report of IAB node capability on support of transmission timing case(s)) are left for WI phase.

- Conclusion

In this contribution, we discussed on possible enhancements to support NR backhaul links for NR IAB including IAB node initial access (Stage 1), IAB node discovery and measurement (Stage 2), BH link management and route switching, and IAB node transmission timing. Based on the discussion, we made following proposals.

Proposal 1: For NR IAB in NSA deployment, at least the following modifications to support IAB node initial access procedure should be supported.

- For IAB node initial access, IAB node assumes longer SS/PBCH block periodicity than 20 ms.

- RMSI for IAB node initial access can be transmitted with longer periodicity than SS/PBCH block periodicity.

- RMSI for IAB node initial access transmitted by IAB node/donor supporting NSA for access UE contains only minimum necessary information to establish initial connection to IAB node/donor, such as RACH configuration.

- Presence/absence of RMSI for IAB node (not for UE) associated with SS/PBCH block is indicated by different way from RMSI presence/absence indication for UE.

Proposal 2: Rel-16 IAB should support separate configurations between the transmission/measurement of SSB and/or CSI-RS for stage 2 IAB node discovery and those for stage 2 IAB node measurement.

- At least for stage 2 IAB node discovery, longer transmission/measurement periodicity and longer duration of measurement window than those for Rel-15 SSB and CSI-RS should be supported.

- Reporting the results of the stage 2 IAB node discovery, e.g., detected RS index(es) and its quantity(es), from IAB node to its parent node and/or surrounding node(s) should be supported.

- Detailed configurations for transmission/measurement of SSB and/or CSI-RS and mechanism for reporting stage 2 IAB node discovery results are left for WI phase.

Proposal 3: For off sync-raster SSB, following enhancements should be supported.

- Over 64 Tx beams for off sync-raster SSB within a burst which is longer than 5ms

  - QCL assumption/indication between SSBs within a burst (especially SSBs with same index in different half frames)

  - Actually transmitted SSB indication for over 64 SSBs (e.g., bitmap longer than 64bits)

- SSB measurement window duration longer than 5ms

- SSB transmission/measurement periodicity longer than 160ms

Proposal 4: IAB RLM and/or BFR mechanism should cover not only beams of the current parent node but also other beams of surrounding node(s) that are detected in stage 2 IAB node discovery.

Proposal 5: For appropriate backhaul link selection, it is beneficial for IAB node/donor to support at least one of followings.

- If each IAB node decides its backhaul link, each IAB node/donor transmits (e.g., broadcasts) information related to link quality and traffic load on backhaul link(s) to its IAB donor.

- If IAB donor decides backhaul links for all child IAB nodes, each IAB node reports information related to link quality and traffic load on backhaul link(s) including its measurement results on IAB node discovery signal of parent IAB node and measurement results reported from child IAB node(s).

Proposal 6: For IAB node transmission timing, Case #6 is supported in addition to Case #1/#7 and preferred case can be selected in operation according to IAB node coordination based on deployment scenario, operation and IAB node implementation.

- Details on IAB node coordination for transmission timing case alignment (e.g., indication of transmission timing case, report of IAB node capability on support of transmission timing case(s)) are left for WI phase.

References

- 3GPP RAN1#94bis, Chairman’s note, Oct. 2018.

- 3GPP RAN1#94, Chairman’s note, August 2018.

- 3GPP, TR38.874 v0.5.0, Sept. 2018.

- 3GPP, R1-1812078, AT&T, “TP for 38.874 on PHY Enhancements for NR IAB,” Oct. 2018.