3GPP TSG RAN WG1 Meeting #95	R1-1813667

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.8.6

Source:	Huawei, HiSilicon

Title:	Evaluation results of PAPR reduction for CSI-RS and DMRS

Document for:	Discussion and decision

Introduction

On low PAPR RS design, it has been agreed as WA that [1]

Working Assumption - For PDSCH DMRS and PUSCH DMRS for CP-OFDM, DMRS enhancements are specified in Rel.16 to reduce the PAPR to the same level as for data symbols for all port combinations given by 38.212 - For the Rel-16 DMRS enhancement, each CDM group can be configured with different cinit - For Type 1, the two cinit (configured by nSCID=0,1, respectively) in Rel-15 are used for port(s) in each of the two CDM groups, respectively - For Type 2, introduce the CDM group index in cinit - FFS: How CDM group index is derived? - For Type 1 and Type 2, simultaneously use dynamic TRP selection (or MU-MIMO pairing with different nSCID) and CDM group specific cinit is supported - The following solution categories are precluded - Modification of OCC - Modification to PN sequence generation, such as subsampling a longer sequence - Note: Concerns raised by MediaTek that preclusion of the above solutions will negatively impact power imbalance issue - Carefully consider backward compatibility issues and the total number of cinit configured per UE - For PUSCH/PUCCH DMRS for pi/2 modulation, new DMRS sequences are specified in Rel.16 to reduce the PAPR to the same level as for data symbols - Carefully consider channel estimation performance and cross correlation performance - For the next meeting: - CSI-RS PAPR reduction - Whether to specify a solution to reduce the PAPR to the same level as for data symbols for all CSI-RS configurations given by 38.211 - Power imbalance issues - Power imbalance between PAs, between OFDM symbols, between RE in same OFDM symbol - Whether is it in scope of WI and if so, whether to specify a solution

In this paper, we provide some analysis for the time domain π/2 BPSK DMRS [2].

DM-RS for Pi/2-BPSK DFT-S-OFDM

In NR R15, it has been agreed to support pi/2 BPSK modulation of data for DFT-s-OFDM waveform as well as to support frequency domain spectrum shaping (FDSS) of that modulation. And for DFT-s-OFDM DMRS sequence design, ZC sequences and CGSs is used for both pi/2 BPSK and other modulation. However, a relatively higher PAPR compared to pi/2 BPSK data has been observed with R15 sequences.

In the following we present some evaluation results to show the PAPR increase. In the simulations, 4 RBs and 18 RBs scheduled for PUSCH are considered.

Figure 1 CCDF of PAPR for NR QPSK CGSs and data with filter parameter α=0.28

It can be found in the figures that ZC sequence and CGS used for 4RBs and 18RBs in NR R15 get worse PAPR performance (more than 1.7dB) than pi/2 BPSK data with FDSS when filter parameter α=0.28.

Observation 1: For 4RBs and 18RBs, ZC sequence and CGS used in R15 have higher PAPR (more than 1.7dB) than pi/2 BPSK data with FDSS ( α=0.28).

To avoid these PAPR increases, in previous meetings, a RS design based on pi/2 BPSK DFT-S-OFDM is proposed in [2]. In the design, the time domain Gold/CGS sequences are processed sequentially by pi/2 BPSK, DFT precoding, (FDSS) and IFFT as shown in Figure 2.

Figure 2 pi/2 BPSK DFT-S-OFDM based DMRS generation for pi/2 BPSK with filtering

As discussed in the contribution, the sequence can provide proper PAPR and cross correlation performance. However, as Gold/CGS sequences are generated in time domain and modulated through DFT precoding, as in Figure 3 (2RB CGS sequence and 16-PRB Gold sequence are considered), large power fluctuations occurs in frequency domain that may lead to severe unbiased channel estimation and performance loss.

Figure 3 Amplitude of sequences in frequency domain for 2RB and 16RB [2]

Observation 2: For 2RBs and 16RBs, large frequency fluctuations are observed with time domain sequences in [2].

Summary of proposals

The observations in this paper are summarized as follows.

Observation 1: For 4RBs and 18RBs, ZC sequence and CGS used in R15 have higher PAPR (more than 1.7dB) than pi/2 BPSK data with FDSS ( α=0.28).

Observation 2: For 2RBs and 16RBs, large frequency fluctuations are observed with time domain sequences in [2].

- References

3GPP, “RAN1#94bis Meeting Chairman’s Notes”, Chengdu, China, October 8th - 12th, 2018.

Qualcomm, “Lower PAPR reference signals”, R1-1811280, Chengdu, China, October 8th - 12th, 2018.

3GPP TS 38.211: "NR; Physical channels and modulation," v15.2.0, June 2018.