3GPP TSG RAN WG1 Meeting #95	R1-1813680

Spokane, United States, November 12th – 16th, 2018

Agenda Item:	7.2.12

Source:	Huawei, HiSilicon

Title:	Existing literature and measurement review for industrial factory environment

Document for:	Discussion and decision

Introduction

In 3GPP RAN#81 meeting, a new study item (SI) was approved [1] to study on channel model for indoor industrial scenarios. It was recognized that the current 3GPP channel model in TR 38.901 [2] contains a common channel model with scenario-specific model parameters and settings for scenarios such as Urban Macro, Urban Micro, Rural Macro, and Indoor Hotspot (InH). However, it is noted that InH is based on indoor office / shopping mall environment. To address industrial scenarios that exhibit more diverse and unique environmental features, see the LS from 5G-ACIA in RP-181521 [3], it is needed that the InH in TR 38.901 should be extended to cover additional characteristics of industrial scenarios.

In this contribution, we provide an overview of existing literature and new measurement at different frequencies and link configurations in industrial environment.

Existing literature and measurements review

In [4] and [5], comprehensive reviews of radio wave propagation in an industrial environment are presented. The reported channel models and measurement results, at different frequencies and link configurations, of path loss, noise, interference, multipath propagation, and time-varying channel conditions are discussed, and the factors that influence them are highlighted.

A number of narrowband indoor channel measurements in various industrial settings have been conducted over the past few years. In [6], measurements were done in an industrial facility at 2.4 GHz. Path-loss exponents of 1.6 and 3.73 for line-of-sight (LOS) and non-LOS (NLOS) scenarios were found, respectively. Three other indoor industrial measurements with LOS scenario, path-loss exponents of 1.1 for the chemical pulp and cable factories, and 1.86 for nuclear power plant were reported in [7]. Luo et al. [8] reported path-loss parameters at 2.4 and 5.8 GHz from a test-bed emulating an oil rig installation with LOS and NLOS scenarios. Measurement results of excess path loss from highly reflective environments (an oil refinery and automobile assembly plant) for 25 MHz–18 GHz signals were also reported in [9]. In [10], path-loss parameters for various industrial topographies, at 900, 2400, and 5200 MHz, are reported. Similar results obtained from three different industrial indoor environments, at 900, 1600, and 2450 MHz, are presented in [11]. Gungor et al. [12] reported pathloss measurement results from electric power environments for smart grid applications.

Observation 1: The reported measured path-loss parameters show significant variations with the environment, frequency, and link configuration

Compared with other environments, such as home and office areas, the presence of noise in industrial environments is high due to the broad operating temperatures, strong vibrations, and excessive electromagnetic noise [13]. Interference from other wireless systems working on the same frequency band might also be present [14]. Characterizing these effects is therefore important for designing robust wireless network for industry factory environment.

Few works have been done in the past to describe the noise in an industrial environment. The model parameters need to be extracted from measurements for the considered propagation environment.

Observation 2: Measurements in various industrial facilities with different frequencies and link configurations are required to further validate the noise characteristics in industrial environments

The multipath property of a wireless channel depends on the characteristics of the environment as well as on the properties of the antennas involved. The various measurement results show that the multipath components in industrial environments arrive in clusters [15]–[19]. Various distributions have been proposed for the small-scale amplitude fading of the multipath components in industrial environments.

Observation 3: It is unclear which envelope distributions to use for the received signal and multipath taps as various distributions have been suggested by different measurement campaigns. Further study is needed to characterize the envelope distribution of the received signal as well as the multipath taps.

Conclusion

According to the above discussions, we have the following observation:

Observation 1: The reported measured path-loss parameters show significant variations with the environment, frequency, and link configuration

Observation 2: Measurements in various industrial facilities with different frequencies and link configurations are required to further validate the noise characteristics in industrial environments

Observation 3: It is unclear which envelope distributions to use for the received signal and multipath taps as various distributions have been suggested by different measurement campaigns. Further study is needed to characterize the envelope distribution of the received signal as well as the multipath taps.

These observations on characteristic differences between different literatures may lead to the different propagation parameters and model, such as path-loss, shadow fading, LoS probability, delay spread, angular spread, blockage etc. Thus, we have the following proposals:

Proposal 1: Study the potential influence of channel model for different frequency band

Proposal 2: New measurements in various industrial facilities with different frequencies and link configurations are required

References

RP-182138, “New SI proposal: Study on Channel Modeling for Indoor Industrial Scenarios”, Ericsson, September 2018.

3GPP TR 38.901 v15.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”.

RP-181521, “LS on Channel Model for Indoor Industrial Scenarios”, 5G-ACIA, September 2018.

M. Cheffena."Propagation Channel Characteristics of Industrial Wireless Sensor Networks." IEEE Antennas & Propagation Magazine 58.1(2016): 66-73.

Cheffena M. Industrial wireless sensor networks: channel modeling and performance evaluation [J]. Eurasip Journal on Wireless Communications & Networking, 2012, 2012(1): 1-8.

D. Sexton, M. Mahony, and M. Lapinski, “Radio channel quality in industrial wireless sensor networks,” in Proc. IEEE Sensors Industry Conf., Houston, TX, Feb. 8–10, 2005, pp. 88–94

S. Kjesbu and T. Brunsvik, “Radiowave propagation in industrial environments,” in Proc. IEEE 26th Annu. Conf. Industrial Electronics Society, Nagoya, Aichi, Japan, Oct. 2–28, 2000, pp. 2425– 2430.

S. Luo, N. Polu, Z. Chen, and J. Slipp, “RF channel modeling of a WSN testbed for industrial environment,” in Proc. IEEE Radio Wireless Symp., Phoenix, Jan. 16–19, 2009, pp. 375–378.

K. A. Remley, G. Koepke, C. Holloway, D. Camell, and C. Grosvenor, “Measurements in harsh RF propagation environments to support performance evaluation of wireless sensor networks,” Sensor Rev., vol. 29, no. 3, pp. 211–222, 2009.

E. Tanghe, W. Joseph, L. Verloock, L. Martens, H. Capoen, K. V. Herwegen, and W. Vantomme, “The industrial indoor channel: Large-scale and temporal fading at 900, 2400, and 5200 MHz,” IEEE Trans. Wireless Commun., vol. 7, no. 7, pp. 2740–2751, July 2008.

Y. Ai, M. Cheffena, and Q. Li, “Radio frequency measurements and capacity analysis for industrial indoor environments,” in Proc. 9th European Conf. Antennas Propagation, Lisbon, Portugal, Apr. 13–17, 2015, pp. 1–5.

V. C. Gungor, B. Lu, and G. P. Hancke, “Opportunities and challenges of wireless sensor networks in smart grid,” IEEE Trans. Ind. Electron., vol. 57, no. 10, pp. 3557–3564, Oct. 2010.

K. S. Low, W. N. N. Win, and J. E. Meng, “Wireless sensor networks for industrial environments,” in Proc. Int. Conf. Computational Intelligence Modelling, Control Automation, Vienna, Austria, Nov. 28–30, 2005, vol. 2, pp. 271–276.

N. Azmi, L. M. Kamarudin, M. Mahmuddin, and A. Zakaria, “Interference issues and mitigation method in WSN 2.4 GHz ISM band: A survey,” in Proc. 2nd Int. Conf. Electronic Design, Penang, Malaysia, Aug. 19–21, 2014, pp. 403–408

J. Karedal, S. Wyne, P. Almers, F. Tufvesson, and A. F. Molisch, “A measurement-based statistical model for industrial ultra-wideband channels,” IEEE Trans. Wireless Commun., vol. 6, no. 8, pp. 3028–3037, Aug. 2007.

E. Tanghe, W. Joseph, J. D. Bruyne, L. Verloock, and L. Martens, “The industrial indoor channel: Statistical analysis of the power delay profile,” Int. J. Electron. Commun., vol. 64, no. 9, pp. 806–812, Sept. 2012.

E. Tanghe, D. P. Gaillot, M. Lienard, L. Martens, and W. Joseph, “Experimental analysis of dense multipath components in an industrial environment,” IEEE Trans. Antennas Propagat., vol. 62, no. 7, pp. 3797–3805, July 2014.

Y. Ai, M. Cheffena, and Q. Li, “Power delay profile analysis and modeling of industrial indoor channels,” in Proc. 9th European Conf. Antennas Propagation, Lisbon, Portugal, Apr. 13–17, 2015, pp. 1–5

P. Agrawal, A. Ahlen, T. Olofsson, and M. Gidlund, “Characterization of long term channel variations in industrial wireless sensor networks,” in Proc. IEEE Int. Conf. Communications, Sydney, Australia, June 10–14, 2014, pp. 1–6