3GPP TSG-RAN WG1 Meeting #95	R1-1813855

Spokane, WA, USA, 12th – 16th November 2018

Agenda Item:	7.1.1.1

Source:	Ericsson

Title:	Summary of synchronization signals and PBCH

Document for:	Discussion, Decision

# Introduction

This is the summary for discussion on maintenance of synchronization signals and PBCH in agenda item 7.1.1.1 based on the views expressed by companies in the contributions listed in the appendix and providing proposals for agreement.

# ssb-PositionsInBurst

## Introduction of new parameter

In a reply LS from RAN2 (R1-1812111), RAN2 suggest to solve the issue of ssb-PositionsInBurst mismatch discussed in RAN1#94 by adding a new parameter ssb-PositionsToRateMatchAround in ServingCellConfig in a backwards compatible manner. This parameter can then be used for rate matching purposes only, without affecting the SSB-to-PRACH mapping. The companies expressing their views stated the following if this new parameter should be included :

- Yes: Huawei, NTT DOCOMO(Only if deemed essential), Nokia(for dedicated PDSCH only), Samsung, ZTE and vivo

- No: Intel(Can be introduced in a later release)

Further discussion is needed.

## Behaviour for PDCCH monitoring

Nokia pointed out that since RAN1 and RAN2 have agreed that a UE can expect the ssb-PositionsInBurst content is the same whether provided by ServingCellConfigCommon or SystemInformationBlockType1, there is no need to distinguish these two when specifying the PDCCH monitoring behaviour.

Proposal:

- Endorse the draft CR to 38.213 labelled “section 3.1” in R1-1813616

# MIB

NEC suggested to change the MasterInformationBlock to MIB in 38.213 since the latter is used by RAN2.

Proposal:

- Endorse the draft CR to 38.213 in R1-1813020

# CORESET#0 presence

OPPO stated that the specification text in 38l.213 section 13 is not absolutely clear when it comes to the UE determining that the Type0-PDCCH common search space is not present.

The contribution was withdrawn so no discussion needed.

# Appendix: Contributions used as basis for the summary

| R1-1812178 | Remaining issues on downlink signals and channels | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1812282 | Draft CR on broadcast signals and channels | vivo |
| R1-1812463 | Maintenance for Downlink signal and channels | Intel Corporation |
| R1-1812814 | Maintenance for downlink signals and channels for initial access | OPPO |
| R1-1812895 | Maintenance for Downlink signals and channels | ZTE |
| R1-1812953 | Remaining Issues on Synchronization and System information | Samsung |
| R1-1813020 | Draft CR on RRC parameter name for master information block | NEC Corporation |
| R1-1813122 | Maintenance for Downlink signals and channels | Ericsson |
| R1-1813296 | Maintenance for Downlink signals and channels | NTT DOCOMO, INC. |
| R1-1813616 | On issues related to DL signals and channels | Nokia, Nokia Shanghai Bell |