3GPP TSG RAN WG1 Meeting #95			                                R1-1813858

Spokane, US, November 12th –16th, 2018

Source:               ZTE, Sanechips

Title:                    Complexity Analysis of NOMA Receivers

Agenda item:      7.2.1.2

Document for:    Discussion and Decision

Background

During the RAN1#94bis, the following complexity analysis templates as well as example values are agreed to perform detailed complexity analysis of the receiver [1]

Computation complexity approximation formulae

| Receiver component | Detailed component | Computation in parametric number of usages, O(.) analysis, [impact factor] |  |  |
| --- | --- | --- | --- | --- |
|  |  | MMSE-IRC/hard-IC | ESE |  |
|  |  |  | ESE+SISO | Enhanced ESE+SISO |
| Detector (complexity in #complex multi.) | UE detection | ONAPDMRS∙NREDMRS∙Nrx | ONAPDMRS∙NREDMRS∙Nrx | ONAPDMRS∙NREDMRS∙Nrx |
|  | Channel estimation | ONUE∙NRECE∙NREDMRS∙Nrx | ONUE∙NRECE∙NREDMRS∙Nrx | ONUE∙NRECE∙NREDMRS∙Nrx |
|  | Rx combining, if any |  |  | O(NREdataNUENrx) or O(NREdataNUENrx3) |
|  | Covariance matrix calculation, if any | Note 1 |  |  |
|  | Demodulation weight computation, if any | Note 2 |  |  |
|  | UE ordering, if any | Note 3 |  |  |
|  | Demodulation, if any | ONiterIC∙NREdata∙Nrx |  |  |
|  | Soft information generation, if any | ONiterIC∙Nbit |  |  |
|  | Soft symbol reconstruction, if any |  |  |  |
|  | Message passing, if any |  |  |  |
|  | Others |  |  |  |
| Decoder (complexity in #addtion/comparision) | LDPC decoding | A:NiterIC∙NiterLDPC∙dvNbit+2(Nbit-Kbit) C : NiterIC∙NiterLDPC∙(2dc -1)∙(Nbit-Kbit) | A:Niterouter∙NUE∙NiterLDPC∙dvNbit+2(Nbit-Kbit) C : Niterouter∙NUE∙NiterLDPC∙(2dc -1)∙(Nbit-Kbit) | A:Niterouter∙NUE∙NiterLDPC∙dvNbit+2(Nbit-Kbit) C : Niterouter∙NUE∙NiterLDPC∙(2dc -1)∙(Nbit-Kbit) |
| Interference cancellation (complexity in #complex multi) | Symbol reconstruction(Including FFT operations for DFT-S-OFDM waveform), if any | ONUE∙NREdata∙Nrx |  |  |
|  | LLR to probability conversion, if any |  | ONiterouter∙NUE∙Nbit | ONiterouter∙NUE∙Nbit |
|  | Interference cancellation |  | O6∙Niterouter∙NUE∙NREdata∙Nrx | O6∙Niterouter∙ρNUE∙NREdata |
|  | LDPC encoding, if any | Buffer shifting: NUE⋅(Nbit-Kbit)/2 Addition: NUE⋅(dc-1)(Nbit-Kbit) |  |  |
|  | Others |  |  |  |

Note 1: Two options for covariance matrix calculation

Option 1: ONUE∙NREdata∙Nrx∙NSF2/2NREadj

Option 2: NitrNrxNsf2NREDMRS2Nsf+NUENRxNREDMRS

Note 2: Three options for demodulation weight calculation

Option 1: ONREdata∙Nrx∙NSF3+NiterIC∙NREdata∙Nrx∙NSF2/NREadj

Option2: NitrNREadj2NrxNsf2NUE+NrxNsf3NREData, NREadj=NNsf

Option 3:  ONREdata∙Nrx∙NSF3+NiterIC∙NREdata∙Nrx∙NSF2+NREdata∙NUENUE+1∙Nrx∙NSF2/2/NREadj

Note 3: Two options for UE ordering

Option 1: ONREdata∙Nrx∙NSF∙NUE2/NREadj,SINR

Option 2: ONREdata∙Nrx∙NSF∙NUE2NREadj,SINR+NUE2logNUE/2

For ESE-SISO receiver,

The LLR probability and interference cancellation can be further elaborated as follows,

ESE-LLR: 5.5 NitrNUENREDataNRx.

Mean-variance update: 2.5 NitrNUENREDataNRx+0.5NitrNUENREData

| Receiver component | Detailed component | Computation in parametric number of usages, O(.) analysis, [impact factor] |
| --- | --- | --- |
|  |  | EPA+SISO receiver |
| Detector (complexity in #complex multi. per user per resource element) | UE detection | ONAPDMRS∙NREDMRS∙Nrx |
|  | Channel estimation | ONUE∙NRECE∙NREDMRS∙Nrx |
|  | Rx combining, if any |  |
|  | Covariance matrix calculation, if any |  |
|  | Matrix inversion |  |
|  | Equalization weight computation, if any |  |
|  | Demodulation weight computation, if any |  |
|  | UE ordering, if any |  |
|  | Equalization |  |
|  | Demodulation, if any |  |
|  | Soft information generation, if any |  |
|  | Soft symbol reconstruction, if any | O6∙Niterouter∙Niterdet∙NUE∙NREdata∙du∙2Qm/NSF |
|  | Message passing, if any | O8∙Niterouter∙Niterdet∙NUE∙NREdata∙du/NSF |
|  | Others |  |
| Decoder (complexity in #binary add/comp per user per coded bit) | LDPC decoding | A:Niterouter∙NUE∙NiterLDPC∙dvNbit+2(Nbit-Kbit) C : Niterouter∙NUE∙NiterLDPC∙(2dc -1)∙(Nbit-Kbit) |
| Interference cancellation (complexity in #complex multi per user per resource element) | Symbol reconstruction(Including FFT operations for DFT-S-OFDM waveform), if any | Additional for DFT-s-OFDM: O(Niterouter∙Niterdet∙NUE∙NREdata* log2(NFFT)) |
|  | LLR to probability conversion, if any | ONiterouter∙NUE∙NREdata∙Qm∙2Qm/NSF |
|  | Interference cancellation |  |
|  | LDPC encoding, if any |  |
|  | Others |  |

| Category | Parameter | Notation | Value |
| --- | --- | --- | --- |
| General | Number of receive antennas | Nrx | 2 or 4 |
|  | Number of data resource elements | NREdata | 864 |
|  | Number of users | NUE | 12 |
| MMSE and EPA related | Spreading length | NSF | 4 |
| MMSE-hard IC specific | Number of decoding for MMSE-hard IC | NiterIC | NiterIC=NUE for IRC; [1.5-3]∙NUE=[18-36] for hard-IC |
| Channel coding related | Average column weight of LDPC PCM | dv | 3.43 |
|  | Average row weight of LDPC PCM | dc | 6.55 |
|  | Number of information bits in a code block | Kbit | 176 |
|  | Number of coded bits of a block | Nbit | 432 |
|  | Number of inner iterations of LDPC decoding | NiterLDPC | 20 |
| Soft IC specific | Number of outer iterations between detector and decoder | Niterouter | 5 (for ESE), 3 (for EPA) |
| EPA specific | Number of inner iterations inside detector | Niterdet | 3 |
|  | Number FN nodes (or resource elements) connected to each user | du | 2 |
|  | Number of user connected to one resource element | df | 6 |
|  | Modulation order | Qm | 3 |
| User detection & channel estimation related | Maximal number of DMRS antenna ports | NAPDMRS | 12 |
|  | Total number of DMRS REs for initially estimated channel | NRECE | 12 |
|  | Total number of REs for DMRS, e.g., length of DMRS sequence | NREDMRS | 24 |

In addition, for EPA SISO receiver, there are 2 other different versions of complexity analysis. In this paper, we will consider different versions of complexity analysis and strive to provide a comprehensive quantitative analysis of different receivers.

Quantitative Analysis of NOMA Receivers

In this section, quantitative analysis of the NOMA receivers is based on the agreed computation complexity templates and parameter tables, together with a few additional parameters not yet reached consensus such as NREadj. Different options/versions of Rx computational complexity are considered.  The ratios, if reported, are rounded with 0.5 granularity.

UE Detection and Channel Estimation Complexity

The computation complexity of UE detection and channel estimation parts can be computed directly from the agreed table of example values,

Table 1 Complexity of UE detection and channel estimation

| UE detection | 576 |
| --- | --- |
| Channel estimation | 6912 |

UE detection and channel estimation parts do not account for significant portion of the total Rx complexity, as to be illustrated in the detailed quantitative analysis in the following sections..

MMSE IRC/hard-IC

| Option1 | Option2 | Option3 |
| --- | --- | --- |

Figure. 1 Complexity of MMSE hard-IC receivers and MMSE IRC receivers for 2Rx, under different options

| Option1 | Option2 | Option3 |
| --- | --- | --- |

Figure. 2 Complexity of MMSE hard-IC receivers and MMSE IRC receivers for 4Rx, under different options.

For the complexity of MMSE hard-IC, there are three options in a number of entries of agreed template. More detailed computation for each component can be found in Appendix A1.

For the complexity of MMSE-IRC, in the detector part, the number of decoding is tuned to Nue since no additional decoding attempts beyond the number of UEs are needed. Moreover, IC is turned off in MMSE-IRC.

For the complexity of MMSE hard-IC, the number of outer iterations is set to 1.5 which is the upper bound observed in the link level evaluation shown in Appendix A2.

Regarding the number of adjacent REs, depending on spreading factors, typical range seems between 24 and 144.  As shown in Appendix A3, the performance difference between NREadj=48 and NREadj=4 is very small. Under this setting, the complexity ratios of MMSE hard IC versus MMSE IRC are approximately {2, 2.5, 3.5} for Option 1, Option 2 and Option 3, respectively with 2 Rx antennas, and {1.5, 2, 3.5} respectively with 4 Rx antennas. With the varying of NREadj, the ratios are within the range of { 1.5~2, 2~2.5, 3~4} or  { 1~2, 1.5~2.5, 3~3.5} with 2 or 4 receive antennas.

ESE + SISO based

For the complexity of ESE-SISO based receivers, there exist 2 different versions of receiver complexity analysis i.e. ESE and enhanced-ESE. Figure 3/4 evaluates the complexity of ESE-SISO based receivers.

Regarding the number of adjacent REs over which demodulation weights are reused, when NREadj=48 , ratios of the  3 options of ESE-SISO receiver versus MMSE IRC are {18,5.5,7.5} under 2 receive antennas and {10.5, 2 ,7} under 4 receive antennas respectively.  With the varying of NREadj, the ratios of option 1 ESE with respect to MMSE IRC are within the range of 4.5~22 or 1.5~17.5 under 2 or 4 receive antennas,  the ratios of MF  e-ESE with respect to MMSE IRC are within the range of 1.5~7 or 0.5~3 under 2 or 4 receive antennas, and the ratio of MMSE  e-ESE with respect to MMSE IRC are within the range of 2~9 or 1~11.5 under 2 or 4 receive antennas, respectively.

| ESE | enhanced ESE with MF/MMSE combining |
| --- | --- |
| [4.5, 22] | MF e-ESE [1.5,7] MMSE e-ESE [2, 9] |

Figure. 3 Complexity of ESE/enhanced-ESE and MMSE IRC for 2Rx, under different options.

| MF-ESE | enhanced ESE with MF/MMSE combining |
| --- | --- |
| [1.5, 17.5] | MF e-ESE [0.5, 3] MMSE e-ESE [1, 11.5] |

Figure 4 Complexity of ESE/enhanced-ESE and MMSE IRC for 4Rx, under different options

EPA + SISO based

For the complexity of EPA-SISO based receivers, there are 3 versions of receiver complexity analysis. Figure 5/6 evaluates the complexity of EPA-SISO based receivers.

Regarding the number of adjacent REs over which demodulation weights are reused, when NREadj=48 , ratios of the  3 options of EPA-SISO receiver versus MMSE IRC are {50,33,15} under 2 receive antennas and {14, 15,11} under 4 receive antennas respectively.  With the varying of NREadj, the ratios of option 1 EPA with respect to MMSE IRC are within the range of 7~78 or 1.5~28 under 2 or 4 receive antennas,  the ratios of option 2 EPA with respect to MMSE IRC are within the range of 8.5~40 or 2~22 under 2 or 4 receive antennas, and the ratio of option 3 EPA with respect to MMSE IRC are within the range of 2~23.5 or 1~23 under 2 or 4 receive antennas.

| Option 1 EPA [7，78] | Option 2 EPA [8.5，40] | Option 3 EPA [2，23.5] |
| --- | --- | --- |

Figure.5 Complexity of EPA and MMSE IRC  for 2Rx, under different options.

| Option 1 EPA [1.5，28] | Option 2 EPA [2，22] | Option 3 EPA [1，23] |
| --- | --- | --- |

Figure.6 Complexity of EPA and MMSE IRC for 4Rx, under different options

Decoding Complexity

The decoding complexity can be quantified by the number of decoding attempts for NUE, i.e., NiterIC for the MMSE hard-IC receivers. While for the soft IC receiver, this quantification can be done via the number of outer iterations.

Table 2 Decoding Complexity for Receivers

| Rx Types Decoding Complexity | MMSE-IRC/hard-IC | EPA SISO | ESE SISO |
| --- | --- | --- | --- |
| Total number of Usages | NiterIC | Niterouter∙NUE | Niterouter∙NUE |

Depending on whether parallel or serial interference cancellation is employed, the number of outer iterations may vary for MMSE hard-IC receiver and is agreed within [1.5, 3] from the table of example values for calculation. Meanwhile, the number of outer iterations is as agreed to be 3 and 5 respectively for EPA-SISO and ESE-SISO. The decoding complexity ratio is reported accordingly as below,

Table 3 Decoding Complexity Ratios for Receivers

| Rx Types Decoding Complexity | MMSE IRC | MMSE hard-IC | EPA SISO | ESE SISO |
| --- | --- | --- | --- | --- |
| Ratio of number of usages | 1 | [1.5, 3] | 3 | 5 |

Conclusion

Observation 1 Choosing appropriate values for the number of adjacent REs to which the same demodulation weights are applied ,i.e. NREadj  can be considered to control the receiver complexity of  MMSE-IRC/hard-IC.

Observation 2  According to the example values of parameters for computation complexity calculation, with the varying of  NREadj,  the ratios of detector plus IC between different receivers are reported as follows,

  - MMSE hard-IC with respect to MMSE IRC : [1,4] @ 2Rx and 4Rx

  - ESE-SISO with respect to MMSE IRC

    - ESE-SISO with respect to MMSE-IRC:  [4.5,22] @ 2Rx  and [1.5,17.5]@4Rx

    - enhanced ESE-SISO with respect to MMSE-IRC: [1.5,9] @ 2Rx and  [0.5,11.5]@4Rx

  - EPA-SISO with respect to MMSE IRC :

    - option 1 EPA-SISO with respect to MMSE-IRC:[7,78] @ 2Rx and [1.5,28]@4Rx

    - option 2 EPA-SISO with respect to MMSE-IRC:[8.5,40] @ 2Rx and [2,22] @ 4Rx

    - option 3 EPA-SISO with respect to MMSE-IRC:[2,23.5] @ 2Rx and [1, 23] @ 4Rx

Observation 3  According to the example values of parameters for computation complexity calculation, the ratios of complexity of decoding between different receivers are reported as follows,

  - MMSE hard-IC with respect to MMSE IRC :1.5~3

  - EPA-SISO with respect to MMSE IRC : 3

  - ESE-SISO with respect to MMSE IRC : 5

Proposal 1 Capture the above observations and the complexity comparison figures, Figure.1-Figure.6, into TR 38.812.

References

[1] Chairman’s notes for 3GPP RAN1#94b

[2] 3GPP, R1-1811361, Receiver for uplink NOMA, NTT DOCOMO, RAN1#94bis.

[3] 3GPP, R1-1810203, Complexity analysis of NOMA receivers, ZTE, RAN1#94bis

[4] 3GPP, R1-1811938, Summary of NOMA Receiver Complexity Analysis, ZTE, RAN1#94bis.

[5] 3GPP, R1-1813518, NOMA Receiver Complexity Analysis, Zhejiang University, RAN1#95

[6] 3GPP, R1-1812188, Discussion on the design of NOMA receiver, Huawei, RAN1#95

Appendix

A1 Different options in MMSE hard-IC/IRC receiver

In the complexity analysis of MMSE hard-IC/IRC  different options in certain number of entries are considered. The break-down computation is as follows,

Table A1 Complexity analysis of different options for MMSE hard-IC receiver under 2 or 4 Rx

| # Rx Type # Options |  | # MMSE IRC | # MMSE hard-IC |
| --- | --- | --- | --- |
| Covariance Calculation | Option1-1 ONUE∙NREdata∙Nrx∙NSF2/2NREadj | 6912 or 27648 | 6912 or 27648 |
|  | Option1-2 O(NitrNrxNsf2NREDMRS2Nsf+NUENRxNREDMRS) | #### 768 or 1920 | #### 1152 or 2880 |
| #### Demodulation Weight Calculation | Option 2-1 ONREdata∙Nrx∙NSF3+NiterIC∙NREdata∙Nrx∙NSF2/NREadj | 23040 or 129024 | 29952 or 156672 |
|  | Option2-2 : O(NitrNREadj2NrxNsf2NUE+NrxNsf3NREData) | 9248 or 73856 | 13872 or 110784 |
|  | Option2-3 ONREdata∙Nrx∙NSF3+NiterIC∙NREdata∙Nrx∙NSF2+NREdata∙NUENUE+1∙Nrx∙NSF2/2/NREadj | NA | 119808 or 516096 |
| UE Ordering | Option 3-1 ONREdata∙Nrx∙NSF∙NUE2/NREadj,SINR | 0 | 6912 or 13824 |
|  | Option 3-2 ONREdata∙Nrx∙NSF∙NUE2NREadj,SINR+NUE2logNUE/2 | 0 | 7170 or 14082 |
| Demodulation | ONiterIC∙NREdata∙Nrx | 20736 or 41472 | 31104 or 62208 |

Notes

1: NREadj and NREadj,SINR are set to be 48 and 144.

2: For UE ordering, the Option 3-3 can also obtain satisfying results with the decoding attempts inferior to 1.5[2]. Therefore 1.5 is adopted in the evaluations for all the MMSE hard-IC receiver calculations for the analysis.

3. Option 1 is the combination of option sets{1-1, 2-1, 3-1}, option 2 is the combination of option sets{1-2, 2-2, 3-1} and option 3 is the combination of option sets {1-1, 2-3, 3-2}.

Table A2 Different Versions of complexity analysis of MMSE hard-IC Receiver under 2 or 4 Rx

| # Option Sets Rx Type | 1.1+2.1+3.1 | 1.2+2.2+3.1 | 1.1+2.3+3.2 |
| --- | --- | --- | --- |
| MMSE-IRC | 55,872 or 203,328 | 35,936 or 122,432 | NA |
| MMSE-hardIC | 103,392 or 309,600 | 81,552 or 238,944 | 193,506 or 669,282 |

Assigning the values from the table of example values to the complexity analysis table, the complexity analysis of  different versions of ESE/EPA plus SISO is provided in the following tables, .

Table A3 Different Versions of complexity analysis of ESE-SISO receiver under 2 or 4 Rx

| ESE | e-ESE |  |
| --- | --- | --- |
|  | MF combining | MMSE combining |
| 648,000 or 1,270,080 | 202,176 or 222,912 | 264,384 or 844,992 |

Table A4 Different Versions of complexity analysis of EPA-SISO receiver under 2 or 4 Rx

| option 1 | option 2 | option 3 |
| --- | --- | --- |
| 2,799,360 or 2,799,360 | 1,181,952 or 1,617,408 | 844,992 or 2,280,960 |

A2 Appropriate Number of Decoding Attempts

This value ranges from 1.5 to 3 in the table of example values. Apart from this particular example, 2 additional CDF curves of average number of decoding attempts @ operating points with enhanced hard SIC receiver [3] are provided as follows. 2Rx and TDL-C channels are assumed for all these examples.

(1.1) 12UE, 20byte, {coderate = 0.40}, SNRperUE = -4dB, BLER=0.0905

(1.2) 10UE, 60byte, {coderate = 0.57}, SNRperUE = 5dB, BLER=0.1009

(1.3) 10UE, 75byte, {coderate = 0.72}, SNRperUE = 11dB, BLER=0.0955

| (1.1)TDL-C 300ns, 12UE, 20byte, ICE | (1.2)TDL-C 300ns, 10UE, 60byte, ICE | (1.3)TDL-C 300ns, 10UE, 75byte, ICE |
| --- | --- | --- |

Figure.A1 Average Decoding Attempts NiterIC under ICE

(2.1) 12UE, 20byte, {coderate = 0.40}, SNRperUE = -4dB, BLER=0.0905

(2.2) 10UE, 60byte, {coderate = 0.57}, SNRperUE = 5dB, BLER=0.1009

(2.3) 10UE, 75byte, {coderate = 0.72}, SNRperUE = 11dB, BLER=0.0955

| (2.1)TDL-C 300ns, 12UE, 20byte, RCE | (2.2)TDL-C 300ns, 10UE, 60byte, RCE | (2.3)TDL-C 300ns, 10UE, 75byte, RCE |
| --- | --- | --- |

Figure. A2 Average Decoding Attempts NiterIC under RCE

A3 Values of NREadj in MMSE hard-IC/IRC receiver

The number of adjacent REs to which the same demodulation weights are applied can be written as NREadj = Nsf*Nadj, where Nadj denotes the number of adjacent spreading units. Both performance comparisons under ICE and RCE with To/Fo settings have been provided to see whether there would be any performance degradation between using the same channel for all the OFDM symbols(NREadj=4*12=48) to calculate MMSE demodulation weights. For cases with To/Fo, the To/Fo could be compensated after the equalization/de-spreading processes. For example, we can employ the channel on the first OFDM symbol to calculate the weight and compensate the phase rotation for the de-spread symbols. This compensation process would be common to all receivers either before or after de-spreading in the receiver. Evaluation results assuming different numbers of adjacent REs are provided for information below.

| - 12UE, 20byte {code rate = 0.40}, ICE | - 10UE, 60byte {code rate = 0.57}, ICE | (3) 10UE, 75byte {code rate = 0.72}, ICE |
| --- | --- | --- |
| (4) 24UE, 20byte {code rate = 0.1}, RCE | (5) 8UE, 60byte {code rate = 0.19}, RCE | (6) 16UE, 80byte {code rate = 0.29}, RCE |

Figure. A3  BLER performance of different NREadj  for TDL-C 300ns with non-zero To/Fo

A4 Considerations on DFT-S-OFDM waveform and higher order modulation

Considering DFT-S-OFDM waveform, additional complexity increase is shown with different types of options for the table of example values,  the numbers are rounded in the accuracy of  thousand.  It can be observed that the additional complexity of DFT-S-OFDM could lead to up to 60% complexity increase. for option 3 EPA.

For EPA receiver, higher order modulation results are given in Figure. A2, it can be observed that with the scaling of modulation order, the complexity of EPA would increase noticeably with option 1 and option 2 EPA.

Table A5  Complexity analysis for EPA under different waveforms

| Waveform Different options of Rx | CP-OFDM waveform | DFT-S-OFDM waveform |
| --- | --- | --- |
| Option 1 EPA | 2,800,000 | 3,375,000 |
| Option 2 EPA | 1,180,000 | 1,758,000 |
| Option 3 EPA | 845,000 | 1,421,000 |

Figure. A4 Complexity of EPA plus SISO with respect to Modulation Order

A5 Complexity of different options of Receivers

| ESE | enhanced ESE with MF/MMSE combining |
| --- | --- |
| [1.5, 18] | MF e-ESE:[0.5,5.5] MMSE e-ESE:[0.5,7.5] |
| [1.5,17.5] | MF e-ESE [0.5,3] MMSE e-ESE [1,.11.5] |

Figure. A5 Complexity of ESE/enhanced-ESE and MMSE IRC for 2Rx, under different options.

| MF-ESE | enhanced ESE with MF/MMSE combining |
| --- | --- |
| [0.5, 13] | MF e-ESE:[0.5,2.5] MMSE e-ESE:[0.5,8.5] |
| [1.5,17.5] | MF e-ESE [0.5,3] MMSE e-ESE [1,.11.5] |

Figure. A6 Complexity of ESE/enhanced-ESE and MMSE IRC  for 4Rx, under different options

| Option 1 EPA | Option 2 EPA | Option 3 EPA |
| --- | --- | --- |
| [7,78] | [3,33] | [2,23.5] |
| [20.5,94] | [8.5,40] | [6,28.5] |

Figure.A7 Complexity of EPA and MMSE IRC  for 2Rx, under different options.

| Option 1 | Option 2 | Option 3 |
| --- | --- | --- |
| [1.5,28] | [1,17] | [1,23] |
| [3,38] | [2,22] | [2.5,31] |

Figure.A8 Complexity of EPA and MMSE IRC  for 4Rx, under different options

A6 Complexity of EPA-PIC receiver

Table A6 Algorithm and complexity of EPA-PIC receiver

| Receiver component | Detailed component and algorithm |  | Complexity |
| --- | --- | --- | --- |
| Detector | 0: Initialization | Set a=1, IΔ→kxk=α=2-Qm | Marginal |
|  | 1: EPA-based detector for K users (Rx combining, Covariance matrix calculation, Demodulation weight computation, Message passing) | 1.0: Initialize mean and variance from FN to VN as μl→k0=0,ξl→k0=Inf, l=1,…,L, k∈F(l). Set t = 1, ptxk=α=IΔ→kxk=α. | Marginal |
|  |  | 1.1: If t >, move to step 2.1 Else, VN Update: For k=1,…,NUE: For l∈Vk : - Compute μklt and ξklt as μklt=a∈Xkptxk=ααl ξklt=a∈Xkptxk=α¦αl-μklt¦2 where αl is l-th element of L-dimensional vector α∈Xk. | For the mean value, 2*2Qm*du real multiplications are need per block per UE per inner iteration. For the variance ξklt=a∈Xkptxk=ααl-μklt2 =a∈Xkptxk=ααl2-μklt2, 2Qm*du real multiplications are need per block per UE per inner iteration. |
|  |  | 1.2: FN Update: For l=1,…,NSF: - Perform chip-by-chip MMSE as mlpost=mlpri+ Σlpri HlHHlΣlpriHlH+σ2I-1yl-Hlmlpri Σlpost=Σpri-ΣlpriHlHHlΣlpriHlH+σ2I-1HlΣlpriH where mlpri=μk→lt¦k∈F(l)∈Cdf×1 and Σlpri=diag(ξk→lt¦k∈F(l))∈Cdf×df. - Covariance matrix calculation ONiterouter∙NREdata12Nrx2df+Niterdet∙12Nrxdf - Demodulation weight calculation ONiterouter∙NREdata∙Niterdet∙Nrx3+Nrx2df - Demodulation ONiterouter∙NREdata∙Niterdet∙3Nrxdf |  |
|  |  | 1.3 Message Passing： - From VN to FN 1ξk→lt=1ξklt-1ξl→kt-1 μk→ltξk→lt=μkltξklt-μl→kt-1ξl→kt-1 - From FN to VN 1ξl→kt=1ξklt-1ξk→lt μl→ktξl→kt=μkltξklt-μk→ltξk→lt | Let a=ξkltξl→kt-1-ξklt, then ξk→lt=ξl→kt-1 a, and μk→lt=μklt+a(μklt-μl→kt-1) Thus 1 real division (count as 1 complex multiplication) and 3 real multiplications are needed for each message passing. 1 :4 ratio is used to translate real multiply to complex multiply. |
|  |  | 1.4: Update ptxk ptxk=α=IΔ→kxk=αn∈VkCNxkn=αn;μn→kt,ξn→kta∈XkIΔ→kxk=αn∈VkCNxkn=αn;μn→kt,ξn→kt, α∈Xk, Equivalent to Update t = t +1 and restart from step 1.1 | For each UE and each block and each inner iteration, - To calculate the euclidean distance, need 2Qm*2*du real multiplications. - To calculate , need du real divisions (count as du complex multiplications). - To calculate , need 2Qm*du real complex multiplications. - 2Qm times of table look-up (exp) are needed (count as 2Qm complex multiplications). 5.To multiply the probabilities, 2Qm real complex multiplications are needed. |
|  | 2. Soft information generation | 2.1: Calculate the LLR of coded bits of user k | For each UE and each constellation, Qm real division (count as Qm complex multiplications) and Qm times of table look-up (count as Qm complex multiplication) are needed per block per outer iteration. |
| Decoder | 3. LDPC decoding | 3.1: LDPC decoding based on the LLR of coded bits obtained in step 2.1 and obtain the LLR of coded bits |  |
| Interference cancellation | 4. LLR to probability conversion | 4.1: If a >, stop Else Calculate IΔ→kxk=α from as , restart from step 1.0 | For each UE and each constellation, Qm times of table look-up (count as Qm complex multiplication) are needed per block per outer iteration. |

A7 Clarification on complexity of MMSE hard IC

  - Clarification on demodulation weight calculation

This weight update only needs to be calculated for -1 rounds for demodulation purpose, instead of   rounds as elaborated in Section 3.2 [5].  Therefore the option 3 for demodulation weight calculation in Table 8-1 of [4]  is flawed and the correction in Proposal 3 in [6] makes it more reasonable.

However, it would be better to attribute the SINR generation item in the corrected option 3[6] to UE ordering component and the following sub-section provides further clarification and complexity analysis on demodulation weight calculation and UE ordering.

  - Clarification on UE ordering

UE ordering can be done after obtaining whose calculation is shown as follows:

where ∑ is the sum of all the matrix elements and . Notice that  is already calculated during the calculation of Ry and the matrix Q is a Hermitian matrix. The calculation can be further divided into 2 parts: (a) The multiplication for each element in the diagonal whose cost 1 real multiplication (b) The calculation of the real parts for other elements whose cost is 2 real multiplications.  An example of this calculation method is shown in Figure. 7 when N = 3 for information.

Figure.A 9. An example of calculation of Tk when N = 3.

  - Overall Complexity Calculation

The following table shows the detailed complexity analysis for the demodulation weight calculation and UE ordering components.

Table A7 Complexity Analyisis break-up for Demodulation Weight  Calculation

|  | Formulas | Complexity | Note |
| --- | --- | --- | --- |
| Demodulation weight |  |  |  |
|  |  |  | =Hermitian, in (NUE-1) rounds. The division is done for , which is of lower order and therefore omitted. |
|  |  |  | in NICNUE rounds. |
| Total |  |  | Items with lower order is omitted in the O(·) notation. |
| O(·) |  |  |  |

Table A8 Complexity Analyisis break-up for  UE ordering

|  | Formulas | Complexity | Note |
| --- | --- | --- | --- |
| UE ordering |  |  | for NrxNSF diagonal elements for NUE~2 times in (NUE -1)rounds. |
|  |  |  | for non-diagonal elements for NUE~2 times in (NUE -1)rounds. |
| Total |  |  | can be seen as a coefficient and omitted, in the O(·) notation. The quick sorting is omitted. |
| O(·) |  |  |  |