3GPP TSG RAN WG1 Meeting #95	R1-1813870

Spokane, US, November 12th – 16th, 2018

Source:	NTT DOCOMO, INC.

Title:	Receiver for uplink NOMA

Agenda Item:	7.2.1.2

Document for: 	Discussion and Decision

- Introduction

In RAN1 #94 meeting, the template of receiver computation complexity was agreed [1]. Based on the template, the computation complexity of receivers including MMSE-hard IC, (enhanced) ESE-soft PIC and EPA (hybrid hard and) soft PIC were discussed during RAN1 #94bis. The overall computation complexity estimate and example values of parameters for computation complexity calculation has been agreed [2].

Agreements: The following table for computation complexity analysis of the receiver as the starting point, entries can be updated till RAN1#94bis. Table I Template of Receiver Computation Complexity breakup Receiver component; Detailed component; Computation in parametric number of usages, O(.) analysis, [impact factor]. Receiver type 1; Receiver type 2; …. Detector; UE detection. Channel estimation. Rx combining, if any. Covariance matrix calculation, if any. Demodulation weight computation, if any. UE ordering, if any. Demodulation, if any. Soft information generation, if any. Soft symbol reconstruction, if any. Message passing, if any. Others. Decoder; LDPC decoding. Interference cancellation; Symbol reconstruction (Including FFT operations for DFT-S-OFDM waveform), if any. LLR to probability conversion, if any. Interference cancellation. LDPC encoding, if any. Others. Agreements: - Table 8 (and its subtables & notes) and Table 9 in R1-1811938 are agreed - To be captured in 38.812

In this contribution, we show analysis and comparison of different types of receivers. Same notations with [3] used in this paper are listed below.

Notations of parameters [3]:

- NiterIC	: average number of decoding, demodulation and IC attempts for MMSE-hard IC

- Niterouter	: number of outer iterations between detector and soft-output LDPC decoder

- Niterdet 	: number of inner iterations of detection/interference cancellation

- NiterLDPC	: number of inner iterations of LDPC decoding

- NUE	: number of UEs

- Nrx	: number of Rx antennas

- Nbit	: number of encoded bits

- Kbit	: number of information bits

- NSF	: spreading factor

- NREdata	: total number of REs for data

- NREDMRS	: total number of REs for DMRS, e.g., length of DMRS sequence

- NRECE	: total number of DMRS REs for initially estimated channel

- NAPDMRS	: maximal number of DMRS antenna ports

- NREadj 	: number of adjacent REs to which the same demodulation weights are applied

- NREadj,SINR: number of adjacent REs to which the same SINR is assumed

- dv 	: average column weight (e.g., variable degree) of LDPC parity check matrix

- dc 	: average row weight (e.g., check degree) of LDPC parity check matrix

- Qm 	: modulation order, e.g., 3 for 8-point constellation, 4 for 16-point constellation

- du 	: number FN nodes (or resource elements) connected to each user

- df 	: number of users connected to each resource element

- Complexity analysis

The high-level block diagram of multi-user receiver is shown in Fig. 1.

Figure 1 A high-level block diagram of multi-user receiver

Generally, three main data processing units are needed for multi-user receiving, detector, decoder and interference cancellation. The complexity impact from all these three unites needed to be considered when comparing the complexity of different types of receivers. The template of receiver complexity including more detailed components were agreed in RAN1 #94. Based on the analysis in Appendix 1, the complexity of three main receivers including MMSE-hard IC, EPA-hybrid PIC and ESE-soft PIC is provided in Table 1. Besides, three options of the complexity of MMSE-hard IC receiver can be obtained based on the agreements in RAN1 #94bis and are provided in Table 8 in Appendix 2, which are related to different algorithms of MMSE-hard IC receiver.

Table 1 Receiver Computation Complexity

| Receiver component | Detailed component | Computation in parametric number of usages, O(.) analysis, [impact factor] |  |  |
| --- | --- | --- | --- | --- |
|  |  | MMSE-hard IC | EPA-hybrid PIC | ESE-soft PIC |
| Detector | UE detection |  |  |  |
|  | Channel estimation |  |  |  |
|  | Rx combining, if any |  |  |  |
|  | Covariance matrix calculation, if any |  |  |  |
|  | Demodulation weight computation, if any |  |  |  |
|  | UE ordering, if any |  |  |  |
|  | Demodulation, if any |  |  |  |
|  | Soft information generation, if any |  |  |  |
|  | Soft symbol reconstruction, if any |  |  |  |
|  | Message passing, if any |  |  |  |
|  | Others |  |  |  |
| Decoder | LDPC decoding |  |  |  |
| Interference cancellation | Symbol reconstruction(Including FFT operations for DFT-S-OFDM waveform), if any |  |  |  |
|  | LLR to probability conversion, if any |  |  |  |
|  | Interference cancellation |  |  |  |
|  | LDPC encoding, if any |  |  |  |
|  | Others |  |  |  |

- Complexity comparison

The example values of parameters for computation complexity calculation agreed in RAN1 #94bis [2] are provided in Table 2.

Table 2 Example values of parameters for computation complexity calculation

| Category | Parameter | Notation | Value |
| --- | --- | --- | --- |
| General | Number of receive antennas | Nrx | 2 or 4 |
|  | Number of data resource elements | NREdata | 864 |
|  | Number of users | NUE | 12 |
| MMSE and EPA related | Spreading length | NSF | 4 |
| MMSE-hard IC specific | Number of decoding for MMSE-hard IC | NiterIC | NiterIC=NUE for IRC; [1.5-3]∙NUE=[18-36] for hard-IC |
| Channel coding related | Average column weight of LDPC PCM | dv | 3.43 |
|  | Average row weight of LDPC PCM | dc | 6.55 |
|  | Number of information bits in a code block | Kbit | 176 |
|  | Number of coded bits of a block | Nbit | 432 |
|  | Number of inner iterations of LDPC decoding | NiterLDPC | 20 |
| Soft IC specific | Number of outer iterations between detector and decoder | Niterouter | 5 (for ESE), 3 (for EPA) |
| EPA specific | Number of inner iterations inside detector | Niterdet | 3 |
|  | Number FN nodes (or resource elements) connected to each user | du | 2 |
|  | Number of user connected to one resource element | df | 6 |
|  | Modulation order | Qm | 3 |
| User detection & channel estimation related | Maximal number of DMRS antenna ports | NAPDMRS | 12 |
|  | Total number of DMRS REs for initially estimated channel | NRECE | 12 |
|  | Total number of REs for DMRS, e.g., length of DMRS sequence | NREDMRS | 24 |

Number of decoding for MMSE-hard IC and EPA-hybrid PIC depends on the evaluation cases and SNR. To compute the complexity, we firstly evaluate the number of decoding with the example values in Table 2. In Figure 2, the number of decoding per UE at 95% CDF for UGMA (with sequences in Table X.5-4 in [6] and equal power) with MMSE-hard IC receiver and SCMA with EPA-hybrid PIC is provided. It can be found that the maximum number of decoding per UE at 95% CDF for all configurations is 1.6 for MMSE-hard IC and 2.6 for EPA-hybrid PIC, i.e., NiterIC=1.6∙NUE for MMSE-hard IC and Niterouter=2.6 for EPA-hybrid PIC.

More evaluation results on the number of decoding for UGMA with MMSE-hard IC receiver are provided in [9], which shows that power assignment can further reduce the number of decoding and hence reduce receiver complexity.

(a) TDL-C 300ns, MMSE-hard IC                                   (b) TDL-A 30ns, MMSE-hard IC

(c) TDL-C 300ns, EPA-hybrid PIC                                   (d) TDL-A 30ns, EPA-hybrid PIC

Figure 2 Number of decoding per UE at 95% CDF

Then, the complexities of MMSE-hard IC receiver with NiterIC=1.6∙NUE and , EPA-hybrid PIC receiver with Niterouter=2.6 and ESE-soft PIC receiver with Niterouter=5 can be computed based on the equations in Table 1 and Table 8, and are provided in Table 3.

Table 3 Receiver complexity of MMSE-hard IC, EPA-hybrid PIC and ESE-soft PIC

| Receiver component | MMSE-hard IC |  |  |  | EPA-hybrid PIC |  | ESE-soft PIC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Option 1 in Table 8 |  | Table 1 |  |  |  |  |  |
|  | Nrx=2 | Nrx=4 | Nrx=2 | Nrx=4 | Nrx=2 | Nrx=4 | Nrx=2 | Nrx=4 |
| Detector | 93932 | 292926 | 246643 | 728294 | 1072282 | 1457165 | 266688 | 533376 |
| IC | 20736 | 41472 | 20736 | 41472 | 61171 | 81907 | 466560 | 466560 |
| Decoder | 251670 | 251670 | 251670 | 251670 | 408963 | 408963 | 3864527 | 3864527 |
| Overall complexity | 366337 | 586067 | 519049 | 1021436 | 1542416 | 1948035 | 4597775 | 4864463 |

Taking the complexity of MMSE-hard IC in Table 1 as baseline, we can compute the normalized overall complexities of MMSE-hard IC, EPA-hybrid PIC and ESE-soft PIC based on Table 3.

Table 4 Normalized overall complexity of MMSE-hard IC, EPA-hybrid PIC and ESE-soft PIC

| Number of receive antennas | Nrx=2 | Nrx=4 |
| --- | --- | --- |
| MMSE-hard IC in Option 1 of Table 8 | 0.706 | 0.574 |
| MMSE-hard IC in Table 1 | 1 | 1 |
| EPA-hybrid PIC in Table 1 | 2.972 | 1.907 |
| ESE-soft PIC in Table 1 | 8.858 | 4.762 |

Observation 1: With example values of parameters for computation complexity calculation in Table 2, the complexity of EPA-hybrid PIC and ESE-soft PIC is always higher than the complexity of MMSE-hard IC.

The differences between the complexity of MMSE-hard IC, EPA-hybrid PIC and ESE-soft PIC include the number of outer iterations and the number of bits for decoding, etc. For example, the number of decoding required by EPA-hybrid PIC and ESE-soft PIC is usually larger than MMSE-hard IC. This will induce large difference of decoding complexity. Besides, for bit-level NOMA scheme (i.e., bit-level spreading), the number of bits for decoding is , which will consume higher complexity of decoding than NOMA with spreading (number of coded bits is ).

As the simplest type of advanced receivers, MMSE-hard IC receiver has been well studied and is suitable for all existing NOMA schemes and could be implemented at the gNB easily. Therefore, MMSE-hard IC receiver can be used as a baseline for performance comparison of NOMA schemes.

Proposal 1: MMSE-hard IC receiver has lowest complexity and should be considered as a baseline for performance comparison of NOMA schemes.

- Conclusion

In this contribution, we analyzed and compared the complexity of the receiver candidates for uplink NOMA. According to the discussions, we have following proposals:

Observation 1: With example values of parameters for computation complexity calculation in Table 2, the complexity of EPA-hybrid PIC and ESE-soft PIC is always higher than the complexity of MMSE-hard IC.

Proposal 1: MMSE-hard IC receiver has lowest complexity and should be considered as a baseline for performance comparison of NOMA schemes.

References

- Chairman notes, 3GPP RAN1 #94.

- Chairman notes, 3GPP RAN1 #94bis.

- R1-1811938, ZTE, “Summary of NOMA Receiver Complexity Analysis”, Oct. 2018

- R1-1808050, Huawei, HiSilicon, “Discussion on the design of NoMA receiver”, Aug. 2018

- R1-1800123, ZTE, “Receiver Algorithms of Linear-spreading based NOMA”, Jan. 2018

- R1-1811364, NTT DOCOMO, INC., “Clarifications of UGMA design”, Oct. 2018

- R1-164390, Huawei, HiSilicon, “Transceiver implementation and complexity analysis for SCMA”, May 2016

- R1-1811195, InterDigital Inc., “Complexity Evaluation of ESE-based IDMA Receivers”, Oct. 2018

- R1-1813309, NTT DOCOMO, INC., “Receiver complexity reduction by UE-specific power assignment”, Nov. 2018

Appendix 1: Detailed algorithm and complexity of receivers

- MMSE-SIC receiver

The structure of MMSE-SIC receiver is provided in Figure 3. In conventional MMSE-SIC receiver, the signals of  users are detected one by one based on the order and  iterations are needed. To further improve the performance of SIC receiver, some enhanced SIC receiver such as SIC receiver with CRC check and SIC receiver with outer iterations are proposed. For SIC receiver with CRC check, the detected signals are cancelled from the received signal only if the CRC check is correct. If CRC check is wrong, the detected signals are not cancelled and are considered as interference for the detection of the remaining users. For enhanced SIC receiver with outer iterations, signals of failed users will be detected again in the next iteration [5].

Figure 3 Illustration of conventional MMSE-SIC receiver

The algorithm and complexity analysis of enhanced MMSE-SIC receiver are provided in Table 5. The notations are listed below.

- ,               : set of constellation points mapped from the Qm coded bits with 1 and 0 at the j-th position

- : channel vector of user k

- : spreading sequence used by user k with unit norm

- : set of undetected (including unsuccessfully detected) users in t-th iteration

- : number of users being successfully decoded before t-th iteration and

- : effective channel vector (including spreading and/or scrambling if any) for the i-th modulated symbol of the user to be detected in t-th iteration

- : effective channel matrix of NUE-NSuc,t undetected users in t-th iteration

- : remaining received signal after interference cancellation before t-th iteration

Table 5 Algorithm and complexity of MMSE-SIC receiver

| Receiver component | Detailed component and algorithms |  | Complexity |
| --- | --- | --- | --- |
| Detector | 0: Initialization | 0.1: Set t=1, NSuc,1=0, , 0.2: Compute instantaneous received powers of NUE users across allocated resources, |  |
|  | 1: User ordering for undetected users in t-th iteration | 1.1: If NUE - NSuc,t = 0, all NUE users are successfully detected, and stop Else, UE ordering based on instantaneous received power, and set n=1 | Marginal |
|  | 2: MMSE detection for user with n-th maximum metric in set , i.e., Rx combining, covariance matrix calculation, and Demodulation weight computation | 2.1: Calculate covariance matrix and MMSE detector for i-th adjacent REs , i = 1… |  |
|  |  | 2.2: Estimate the modulated symbols of the user |  |
|  | 3: Soft information generation | 3.1: Calculate the LLR of coded bits based on the estimated symbols |  |
| Decoder | 4: LDPC decoding | 4.1: LDPC decoding based on the LLR of coded bits obtained in 3.1 | 1:4 ratio is considered for real multiply and complex multiply |
| Interference cancellation | 5: Symbol reconstruction and Interference cancellation | If CRC check is false, t = t + 1, NSuc,t = NSuc,(t-1) If n =, stop. else n = n+1 and restart from step 2 | Marginal |
|  |  | If CRC check is true, 5.1: Reconstruct the received symbols (encoding modulation spreading pass channels) 5.2: Delete the reconstructed symbols from the remaining received signals . Update t = t + 1, NSuc,t = NSuc,(t-1) + 1, delete the UE index from and restart from step 1 |  |

* Based on analysis in [6], the complexity of MMSE-SIC receiver under DFT-s-OFDM and CP-OFDM is similar

In MMSE-SIC receiver, the data of NUE users are detected one by one. Therefore, the complexity of MMSE-SIC can be obtained from Table 5 and summarized in Table 1.

- EPA-hybrid PIC receiver

The signals of multiple users are detected jointly by EPA receiver. The structure of EPA receiver is provided in Figure 4, where function node (FN) denotes resource element (RE) and variable node (VN) denotes UE or data layer. EPA receiver is performed based on the tanner graph constructed by the codebook and the FN and VN connecting in the tanner graph makes a FN-VN pair. The message passes and updates between FN-VN pairs. After enough iterations of message updates at FNs and VNs, the LLR for coded bits is calculated and outputs at the VN, and is used as LLR input for the decoder.

Figure 4 Illustration of MPA/EPA receiver

To reduce the complexity, EPA receiver employs complex Gaussian distribution to approximate the message update function in MPA and only the mean and variance of complex Gaussian distribution need to be computed, passed and updated at each iteration, which reduces the exponential complexity to linear complexity [7]. The algorithm of EPA-based detector as well as the definition of notations used in the algorithm can be found from [4] and the complexity is discussed in Table 6.

Table 6 Algorithm and complexity of EPA-hybrid PIC receiver

| Receiver component | Detailed component and algorithm |  | Complexity |
| --- | --- | --- | --- |
| Detector | 0: Initialization | Set a=1, IΔ→kxk=α=2-Qm | Marginal |
|  | 1: EPA-based detector for K users (Rx combining, Covariance matrix calculation, Demodulation weight computation, Message passing) | 1.0: Initialize mean and variance from FN to VN as μl→k0=0,ξl→k0=Inf, l=1,…,L, k∈F(l). Set t = 1, ptxk=α=IΔ→kxk=α. | Marginal |
|  |  | 1.1: If t >, move to step 2.1 Else, VN Update: For k=1,…,NUE: For l∈Vk : - Compute μklt and ξklt as μklt=a∈Xkptxk=ααl ξklt=a∈Xkptxk=α¦αl-μklt¦2 where αl is l-th element of L-dimensional vector α∈Xk. - Compute the mean μk→lt and variance ξk→lt as 1ξk→lt=1ξklt-1ξl→kt-1 μk→ltξk→lt=μkltξklt-μl→kt-1ξl→kt-1 |  |
|  |  | 1.2: FN Update: For l=1,…,NSF: - Perform chip-by-chip MMSE as mlpost=mlpri+ Σlpri HlHHlΣlpriHlH+σ2I-1yl-Hlmlpri Σlpost=Σpri-ΣlpriHlHHlΣlpriHlH+σ2I-1HlΣlpriH where mlpri=μk→lt¦k∈F(l)∈Cdf×1 and Σlpri=diag(ξk→lt¦k∈F(l))∈Cdf×df. - For k∈F(l): Given the posterior mean μklt¦k∈F(l)=mlpostand variances ξklt¦k∈F(l)=diag(Σlpost) of xkl, compute the mean μl→kt and variance ξl→kt as 1ξl→kt=1ξklt-1ξk→lt μl→ktξl→kt=μkltξklt-μk→ltξk→lt |  |
|  |  | 1.3: Update ptxk ptxk=α=IΔ→kxk=αn∈VkCNxkn=αn;μn→kt,ξn→kta∈XkIΔ→kxk=αn∈VkCNxkn=αn;μn→kt,ξn→kt, α∈Xk Update t = t +1 and restart from step 1.1 |  |
|  | 2. Soft information generation | 2.1: Calculate the LLR of coded bits of user k | 1:4 ratio is considered for real multiply and complex multiply |
| Decoder | 3. LDPC decoding | 3.1: LDPC decoding based on the LLR of coded bits obtained in step 2.1 and obtain the LLR of coded bits |  |
| Interference cancellation | 4. Symbol reconstruction | If CRC check is true, 4.1: Reconstruct the received symbols (encoding modulation spreading pass channels) 4.2: Delete the reconstructed symbols from the remaining received signals. |  |
|  | 5. LLR to probability conversion | 5.1: If a >, stop Else Calculate IΔ→kxk=α from as , restart from step 1.0 |  |

The overall complexity of EPA-hybrid PIC can be obtained from Table 6 and summarized in Table 1.

- ESE-soft PIC receiver

The general structure of ESE-soft PIC receiver is provided in Figure 5, where πk and π-1k denote interleaver and deinterleaver of user k, respectively. The signals of multiple users are detected together and the detected results for each user are used for further decoding. Generally, multiple iterations are required for ESE receiver. Because that the elementary signal estimator is not optimized for interference cancellation, and the decoders play a main role for interference cancellation. The elementary signal estimator has to use the external information to output a more reliable detected results, such that information must be exchanged between elementary signal estimator and decoder several times.

Figure 5 Illustration of ESE-soft PIC receiver

When modulation order Qm=2, i.e., QPSK, is used, the algorithm of chip-by-chip ESE detector as well as the definition of notations used in the algorithm can be found in [8], and the complexity is discussed in Table 7.

Table 7 Complexity of ESE-soft PIC receiver

| Receiver component | Detailed component and algorithm |  | Complexity |
| --- | --- | --- | --- |
| Detector | 0: Initialization | Initialize t=1, eDECRexkj=0, eDECImxkj=0, j=1, 2, …,NREdata | Marginal |
|  | 1: ESE-based detector for K users (Rx combining, interference cancellation) | 1.0: If t >, stop Else, compute ExkRe(j),ExkIm(j), VarxkRe(j), and VarxkIm(j) for j=1, 2, …,NREdata and k=1, 2, …,NUE ExkRe(j)=tanheDECRexkj2 VarxkRe(j)=1-ExkRej ExkIm(j)=tanheDECImxkj2 VarxkIm(j)=1-ExkImj | 1:4 ratio is considered for real multiply and complex multiply |
|  |  | 1.1: Estimate mean and the variance of interference for j=1, 2, …, NREdata ErlRej=k=1NUEhk, lRe(j) ExkRe(j)-hk, lIm(j)ExkIm(j) ErlImj=k=1NUEhk, lRe(j) ExkIm(j)-hk, lIm(j)ExkRe(j) ERehk, l(j) ζk, l(j)=hk, lRe(j)ErlRe(j)+hk, lIm(j)ErlIm(j)-hk,l(j)2ExkRe(j) VarrlRe(j)=k=1NUEhk, lRe(j)2 VarxkRe(j)+ hk, lIm(j)2 VarxkIm(j) +σ2 VarrlIm(j)= k=1NUEhk, lIm(j)2 VarxkRe(j)+ hk, lRe(j)2 VarxkIm(j)+σ2 ψRej= k=1NUEhk, lRe(j)hk, lIm(j)VarxkRe(j)- VarxkIm(j) VarRehk, l(j)ζk, lj= hk, lRe(j)2VarrlRej+hk, lIm(j)2VarrlImj+2 hk, lRe(j)hk, lIm(j)ψRej-hk, l(j)4VarxkRe(j) EImhk,ljζk,lj=hk,lReErImj-hk,lImErRej-hk,lj2ExkRej ψImj=k=1NUEhk, lRe(j)hk, lIm(j)VarxkIm(j)- VarxkRe(j) VarImhk,l(j)ζk,l(j)=hk,lRe(j)2VarrIm(j) + hk,lIm(j)2VarrRe(j)+2hk,lRejhk,lImjψImj-hk,lj4VarxkImj |  |
|  | 2. LLR generation | 2.1: Calculate ESE LLRs eESERexkImjl=2hk, lRe(j)2Rehk, l (j) rl(j)-ERehk, l jζk, l(j)VarRehk, l(j)ζk, l(j) eESExkImjl=2hk,lj2Imhk, l (j) rl j-EImhk,ljζk,ljVarImhk,l(j)ζk,l(j) |  |
|  |  | 2.2: LLR combining eESExkRej=l=1NrxeESExkRejl eESExkImj=l=1NrxeESExkImjl |  |
| Decoder | 3. LDPC decoding | 3.1: LDPC decoding based on the LLR of coded bits obtained in step 2.2 and obtain LLR of coded bits eDECRexkj, eDECImxkj 3.2: Update t = t+1, and restart from step 1.0 |  |

Appendix 2: Three options of MMSE-hard IC complexity based on agreements in RAN1 #94bis [2]

Table 8 Computation complexity approximation formulae of MMSE-hard IC

| Receiver component | Detailed component | Computation in parametric number of usages, O(.) analysis, [impact factor] |  |  |
| --- | --- | --- | --- | --- |
|  |  | MMSE-hard IC |  |  |
|  |  | Option 1 | Option 2 | Option 3 |
| Detector (complexity in #complex multi.) | UE detection | ONAPDMRS∙NREDMRS∙Nrx | ONAPDMRS∙NREDMRS∙Nrx | ONAPDMRS∙NREDMRS∙Nrx |
|  | Channel estimation | ONUE∙NRECE∙NREDMRS∙Nrx | ONUE∙NRECE∙NREDMRS∙Nrx | ONUE∙NRECE∙NREDMRS∙Nrx |
|  | Rx combining, if any |  |  |  |
|  | Covariance matrix calculation, if any | ONUE∙NREdata∙Nrx∙NSF2/2NREadj Note: Option 1 in [2] | NitrNrxNsf2NREDMRS2Nsf+NUENRxNREDMRS Note: Option 2 in [2] | ONUE∙NREdata∙Nrx∙NSF2/2NREadj Note: Option 1 in [2] |
|  | Demodulation weight computation, if any | ONREdata∙Nrx∙NSF3+NiterIC∙NREdata∙Nrx∙NSF2/NREadj Note: Option 1 in [2] | NitrNREadj2NrxNsf2NUE+NrxNsf3NREData NREadj=NNsf Note: Option 2 in [2] | ONREdata∙Nrx∙NSF3+NiterIC∙NREdata∙Nrx∙NSF2+NREdata∙NUENUE+1∙Nrx∙NSF2/2/NREadj Note: Option 3 in [2] |
|  | UE ordering, if any | ONREdata∙Nrx∙NSF∙NUE2/NREadj,SINR Note: Option 1 in [2] | ONREdata∙Nrx∙NSF∙NUE2/NREadj,SINR Note: Option 1 in [2] | ONREdata∙Nrx∙NSF∙NUE2NREadj,SINR+NUE2logNUE/2 Note: Option 2 in [2] |
|  | Demodulation, if any | ONiterIC∙NREdata∙Nrx | ONiterIC∙NREdata∙Nrx | ONiterIC∙NREdata∙Nrx |
|  | Soft information generation, if any | ONiterIC∙Nbit | ONiterIC∙Nbit | ONiterIC∙Nbit |
|  | Soft symbol reconstruction, if any |  |  |  |
|  | Message passing, if any |  |  |  |
|  | Others |  |  |  |
| Decoder (complexity in #addtion/comparision) | LDPC decoding | A:NiterIC∙NiterLDPC∙dvNbit+2(Nbit-Kbit) C : NiterIC∙NiterLDPC∙(2dc -1)∙(Nbit-Kbit) | A:NiterIC∙NiterLDPC∙dvNbit+2(Nbit-Kbit) C : NiterIC∙NiterLDPC∙(2dc -1)∙(Nbit-Kbit) | A:NiterIC∙NiterLDPC∙dvNbit+2(Nbit-Kbit) C : NiterIC∙NiterLDPC∙(2dc -1)∙(Nbit-Kbit) |
| Interference cancellation (complexity in #complex multi) | Symbol reconstruction(Including FFT operations for DFT-S-OFDM waveform), if any | ONUE∙NREdata∙Nrx | ONUE∙NREdata∙Nrx | ONUE∙NREdata∙Nrx |
|  | LLR to probability conversion, if any |  |  |  |
|  | Interference cancellation |  |  |  |
|  | LDPC encoding, if any | Buffer shifting: NUE⋅(Nbit-Kbit)/2 Addition: NUE⋅(dc-1)(Nbit-Kbit) | Buffer shifting: NUE⋅(Nbit-Kbit)/2 Addition: NUE⋅(dc-1)(Nbit-Kbit) | Buffer shifting: NUE⋅(Nbit-Kbit)/2 Addition: NUE⋅(dc-1)(Nbit-Kbit) |
|  | Others |  |  |  |

For users with velocity 3km/h, the channels in all symbols in one subcarrier are nearly the same, which indicates that the matrix inversion can be reused in all symbols. Therefore, is used for complexity evaluation of MMSE-hard IC. Besides,  is used for UE ordering in calculation of complexity of option 1 in Table 8.