3GPP TSG RAN WG1 Meeting #95  		R1- 1813897

Spokane, WA, USA, November 12th – 16th, 2018

Agenda item:	7.2.8.4

Source: 	Qualcomm Incorporated

Title: 	Full Tx power for UL transmissions

Document for:	Discussion/Decision

# Introduction

In RAN #80 meeting [1], the following WI was approved as one of work items for NR MIMO Enhancements.

  - Specify enhancement to allow full power transmission in case of uplink transmission with multiple power amplifiers (assume no change on UE power class)

In RAN #94bis meeting [2], the following agreements were made on this WI:

Consider the following potential solutions and other solutions (such as combination of the solutions below) for UL full power transmission. Decision will be made in RAN1#95:

Option 1: Refinement/adjustment of UL codebook is supported

•             1-1: Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs

•             1-2: Introduce additional scaling factor for uplink codebook

Option 2: UE transparently apply a small cyclic or linear delay

Option 3: Power control mechanism to be modified to support UL full power transmission without precluding the use of full rated PA(s)

•             Note: Full rated PA refers to a PA having power not lower than that of the power class

Option 4: Up to UE implementation (no specification impact)

In this contribution, enhancements to allow full power transmission for uplink PUSCH transmissions is addressed in Section 2. The enhancement to allow full power transmission with simultaneous PUSCH and PUCCH transmission is addressed in Section 3. Conclusions are drawn in Section 4.

# Full Tx power transmission for PUSCH

## Full Tx power transmission for PUSCH with non-coherent antenna

In NR Rel. 15, two key restrictions can potentially prevent a UE from transmitting PUSCH at full power. The first issue arises due to restrictions on transmit precoder codebook for UEs with non-coherent or partially coherent antennas (see Tables 7.3.1.1.2-2 to 7.3.1.1.2-5 in 38.212), and the second is due to the power allocation rule in 38.213, Section 7.1 This section focuses on addressing these two issues.

Issue 1: Restriction on TPMIs for UEs with non-coherent or partially-coherent antennas

In NR Rel. 15, for PUSCH transmission using codebooks, when UEs have non-coherent antennas or partially coherent antennas, the set of TPMIs are restricted so that the precoder for a layer only spans a coherent subset of antennas (see Tables 7.3.1.1.2-2 to 7.3.1.1.2-5 in 38.212). For example, if a UE has two non-coherent antennas, then the precoders are restricted to be either  or . Such a restriction can prevent such UEs from transmitting at full power. For example, suppose the UE is a class-3 UE, powered by 2 PAs, each with a 20 dBm output rating, then the UE cannot transmit at full power in the scenario described above.

Issue 2: Uplink power allocation

In NR Rel. 15, 38.213, Section 7.1 states that

“For PUSCH, a UE first scales a linear value  of the transmit power  on UL BWP , as described in Subclause 12, of carrier  of serving cell , with parameters as defined in Subclause 7.1.1, by the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme. The resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted.”

This rule requires scaling the linear value PPUSCH of the transmit power PPUSCH by the ratio of non-zero PUSCH ports to total configured ports, then divides the resulting power equally among non-zero PUSCH ports. Such an approach can prevent the UE from transmitting at full power. For example, consider a UE with 2 non-coherent antenna ports that is assigned the precoder , in this case, the first antenna port is assigned PPUSCH/2 transmit power (linear value) to transmit PUSCH. Thus, for a class-3 UE that is powered by 2 PAs, each with a 23 dBm output rating, the maximum transmit power for this precoder is 3 dB below the maximum power the UE can transmit at.

Resolving Issue 1 helps a class-3 UE with 2 PAs of 20 dBm output rating each to transmit at full power, while resolving Issue 2 helps a class-3 UE with 2 PAs having 23 dBm output rating each to transmit at full power. In what follows, we make proposals to address the above two issues.

### Proposal to enable wider set of TPMIs for UEs with non-coherent or partially-coherent antennas

As noted in the prior section, restricting precoders to non-zero entries within coherent subsets of antenna ports can prevent a UE from transmitting at full power. Additionally, such a restriction does not take full advantage of all available transmit chains at the UE. Using all available chains can potentially bring more diversity gains in addition to power and help improve performance. One crucial observation in this context is that transparent diversity schemes such as small cyclic delay diversity (S-CDD) do not require coherence across antenna ports. This motivates us to consider schemes that exploit precoding within coherent set of antenna ports while deploying diversity schemes across non-coherent set of antenna ports.

In particular, we envision a two-step process wherein the UE first precodes the transmit signal using the assigned precoder, followed by the application of a transparent diversity scheme across the non-coherent sets of antennas. This process is illustrated in the following two figures for a UE with 2 non-coherent antenna ports and a UE with 4 pairwise-coherent antenna ports.

Figure 1: Precoding + CDD for a UE with 2 non-coherent antenna ports

Figure 2: Precoding + CDD for a UE with 4 pairwise coherent antenna ports

As illustrated in Figure 2, the precoder assigned to a UE with 4 pairwise coherent antenna ports can be interpreted to contain two 2×1  precoders, one spanning the first pair of coherent antenna ports and the second spanning the second pair of coherent antennas. With this interpretation, the precoding operation can be seen as synthesizing two virtual antenna ports that are non-coherent with each other. In the second step, a transparent diversity scheme such as cyclic delay diversity (CDD) is used across these two virtual antenna ports to synthesize a single virtual port for uplink transmission.

Such a scheme is observed to show performance gains compared to current Rel. 15-compliant transmission of codebook-based PUSCH transmission. In the following figure the proposed scheme is compared against a 1 or 2 layer codebook-based  PUSCH transmission for a UE with 4 pairwise-coherent antenna ports. Note that for 1 layer transmission, only 2 of the 4 antenna ports are used while the two layer transmission can potentially use all four antenna ports.

Figure 3: Performance of the proposed scheme compared to Rel. 15-compliant precoding scheme

Based on these observations, we can consider allowing UEs with noncoherent antennas to use the following TPMIs:

- TPMIs 0 to 2 in 38.211 Table 6.3.1.5-1 (Single-layer transmission using two antenna ports),

- TPMIs 0 to 4, 8, and 12 in 38.211 Table 6.3.1.5-2 (Single layer transmission using four antenna ports with transform precoding),

- TPMIs 0 to 4, 8, and 12 in 38.211 Table 6.3.1.5-3 (Single layer transmission using four antenna ports without transform precoding),

- TPMI 0 to 6 in 38.211 Table 6.3.1.5-5 (Two layer transmission using four antenna ports without transform precoding),

- and TPMI 0 to 1 in 38.211 Table 6.3.1.5-6 (Three layer transmission using four antenna ports without transform precoding).

Similarly, we can consider allowing UEs with partially coherent antennas (pairwise coherence) to use the following TPMIs:

- TPMIs 0 to 19, in 38.211 Table 6.3.1.5-2 (Single layer transmission using four antenna ports with transform precoding)

- TPMIs 0 to 19 in 38.211 Table 6.3.1.5-3 (Single layer transmission using four antenna ports without transform precoding).

Based on these observations we make the following proposal

Proposal 1: for UE with non-coherent or partially coherent antenna ports, allow the following PUSCH precoders with non-zero entries that span across non-coherent antenna ports with up to UE implementation e.g. a small cyclic or linear delay may be applied across non-coherent antenna ports.

- Introduce a new codebookSubset = “noncoherentWithCyclicDelay” in 38.212 Section 7.1 for UEs with noncoherent antennas to include:

  - TPMIs 0 to 2 in 38.211 Table 6.3.1.5-1 (Single-layer transmission using two antenna ports),

  - TPMIs 0 to 4, 8, and 12 in 38.211 Table 6.3.1.5-2 (Single layer transmission using four antenna ports with transform precoding),

  - TPMIs 0 to 4, 8, and 12 in 38.211 Table 6.3.1.5-3 (Single layer transmission using four antenna ports without transform precoding),

  - TPMI 0 to 6 in 38.211 Table 6.3.1.5-5 (Two layer transmission using four antenna ports without transform precoding),

  - and TPMI 0 to 1 in 38.211 Table 6.3.1.5-6 (Three layer transmission using four antenna ports without transform precoding).

  - For Tables not listed above, set of allowed TPMI remains same as that for codebookSubset “nonCoherent” in 38.212

- Introduce a new codebookSubset = “partialAndNonCoherentWithCyclicDelay” in 38.212 Section 7.1 for UEs with partial and noncoherent antennas to include:

  - TPMIs 0 to 19, in 38.211 Table 6.3.1.5-2 (Single layer transmission using four antenna ports with transform precoding)

  - TPMIs 0 to 19 in 38.211 Table 6.3.1.5-3 (Single layer transmission using four antenna ports without transform precoding).

  - For Tables not listed above, set of allowed TPMI remains same as that for codebookSubset “partialAndNonCoherent” in 38.212

- Support of this feature is subject to UE capability per band-band combination

One key question that emerges when considering such a scheme is on whether or not it is possible to sound the virtual port to the gNB. It is important for the gNB to have knowledge of this virtual port either through explicit sounding or through implicit reconstruction of the virtual port based on individual antenna ports. Aspects on sounding this virtual port and other associated issues are deferred to Section 2.2.

### Proposal for enhancing uplink power allocation

Given proposal 1, it is clear that for UEs with CDD capability, full power transmission can be achieved when precoders with all non-zero entries are assigned to the UE. For example, for a UE with 2 noncoherent Tx, a precoder  can be used to allow full power transmission with S-CDD. The remaining issue is on how to allow UEs to transmit at full power with precoders having zero entries, e.g. precoder  or .

Enabling full power transmission with precoders having zero entries is heavily dependent on a UE’s PA output rating. Given a power class 3 UE without CDD capability, if equipped with two noncoherent Tx/PA as an example, the situation can be summarized in the following table. For a power class 3 UE with CDD capability, the situation is more complex. For example, for a UE with two 23dBm PAs, when precoder is scheduled, UE can either use single PA to achieve full power or use two PAs via CDD to achieve full power transmission. The former is more power efficient while the latter can provide some diversity gain. NR Rel-16 should allow flexible UE implementation to dynamically switch between the two approaches for tradeoff between performance optimization and UE battery power saving.

In summary, due to heavy dependence on UE PA implementation, whether and how to support full power transmission with precoders with zero entries should be up to UE implementation. In other words, if precoder [1,0] is scheduled to a UE, it is up to that UE’s implementation to keep or remove “the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme”, i.e., to transmit with up to 20Bm or 23dBm.

However, the UE’s behavior should be consistent across different PUSCH transmissions using the same precoder. If  the UE keeps the ratio for a precoder P, then the UE should keep the ratio for all PUSCH transmissions with precoder P. If UE removes the ratio for a precoder P, then UE should always remove the ratio for PUSCH transmissions with precoder P. UE is required to maintain consistent PHR on different occasions of PUSCH transmissions with same precoder so that the network can maintain a stable power control loop.

Table 1: Full power transmission with different PA settings (assume without S-CDD capability)

| PA setting | S-CDD with precoder [1,1] with full power transmission | Precoder [1,0] with full power transmission | Precoder [0,1] with full power transmission |
| --- | --- | --- | --- |
| 20dBm+20dBm | no support | no support | no support |
| 20dBm+23dBm | no support | no support | support |
| 23dBm+20dBm | no support | support | no support |
| 23dBm+23dBm | no support | support | support |

Table 2: Full power transmission with different PA settings (assume with S-CDD capability)

| PA setting | S-CDD with precoder [1,1] with full power transmission | Precoder [1,0] with full power transmission | Precoder [0,1] with full power transmission |
| --- | --- | --- | --- |
| 20dBm+20dBm | support | Support via CDD | Support via CDD |
| 20dBm+23dBm | support | Support via CDD | Support via CDD or single PA |
| 23dBm+20dBm | support | Support via CDD or single PA | Support via CDD |
| 23dBm+23dBm | support | Support via CDD or single PA | Support via CDD or single PA |

Proposal 2: For PUSCH precoders with zeros entries

- It is up to UE implementation to decide whether and how to support full power transmission.

- UE is required to maintain consistent PHR on different occasions of PUSCH transmissions with same precoder

## SRS-related aspects to support full transmit power

Given the precoding+CDD scheme proposed in Section 2.1, we need to ensure that SRS-based sounding enables the gNB to reconstruct the virtual port. Two options exist: explicit sounding of the virtual port or implicit reconstruction at the gNB based on sounding the individual antenna ports.

Figure 4 Explicit sounding of the virtual port. UE is assumed to have 4 tx. antennas and pairwise coherent. [a b c d]T is the precoder and τ indicates the delay used for CDD.

Given the typical sequence of operations wherein SRS is sounded first in uplink, followed by UL grant transmission that includes the TPMI and MCS to use for a subsequent PUSCH transmission, it is easy to see that this sequence precludes explicitly sounding the virtual port --- lack of knowledge of the TPMI at the time of SRS sounding makes it impossible to sound the virtual port. Additionally, the lack of coherence between the antenna ports further dilutes the value of explicitly sounding the virtual port.

As an alternate, we propose to instead sound the original antenna ports as they are, but with the additional delay corresponding to the usage of CDD. Figures 5 shows how the individual ports are sounded for the scenario considered in Figure 4.

Figure 5 Sounding individual antenna ports with additional delay on the second pair of ports to reflect usage of CDD. UE is assumed to have 4 pairwise-coherent tx. antennas.

To understand how the gNB can make use of these modified SRS ports, consider the scenario presented in Figure 6 where we consider a UE with 2 noncoherent antennas powered by a 20dBm PA each.

Figure 6 UE is assumed to have 2 non-coherent tx. antennas. Individual antenna ports are sounded with additional delay on the second port to reflect usage of CDD.

With S-CDD as an option, there are three potential rank 1 TPMIs for the gNB to choose from:  . The last TPMI requires using S-CDD to be applied across the two noncoherent  tx. chains as discussed in Section 2.1.1. Focusing on a single tone k, denote the channel measured on port P0 as h0k and the channel from the second port as h1kej2πθk , where the additional phase offset reflects the delayed sounding. A virtual SRS port which matches the precoder can be synthesized at gNB by h0k+ejϕ(h1kej2πθk), where the additional phase ϕ is added to reflect the fact that the two ports are non-coherent. By selecting the best channel among h0k,h1kej2πθk, h0k+ejϕ(h1kej2πθk) gNB selects the best TPMI among . Note that since precoders are chosen to only span a coherent set of antennas, adding delay to SRS ports has no impact on precoder selection.

An additional advantage of this approach to sounding SRS is that the current framework provided by NR Rel. 15 for SRS sounding via SRS resource sets and SRS resources can continue to be used with no additional changes.

In summary, based on the above observations, no additional SRS resource or SRS resource set need to be added in NR Rel-16 to support the precoding + CDD scheme on PUSCH for full power transmission with non-coherent or partially coherent antennas.

Observation 1: To support full power PUSCH transmission with noncoherent or partially coherent antennas, there is no need to define new SRS ports for the allowed new PUSCH ports with small cyclic or linear delay.

Proposal 3: To sound for PUSCH with the allowed precoders in Proposal 1, UE transparently applies a small cyclic or linear delay to a subset of existing SRS ports defined for codebook-based PUSCH. The same amount of delay is applied in SRS sounding and corresponding PUSCH transmission.

## Considerations on link adaptation

Adding a delay to one or more of the SRS ports raises a couple of key questions pertaining to uplink link adaptation. First, is on the question of whether adding a delay to one more SRS ports impacts uplink precoder selection. The second is on the question of whether it is possible to reliably predict the uplink CQI based on the individual SRS ports sounded as described above.

On precoder selection, we note that since precoders are chosen to only span a coherent set of antennas, adding delay to SRS ports has no impact on precoder selection. For example, in Figure 4, the choice of the precoder [a b]T spanning the first pair of ports and the precoder [c d]T spanning the second pair of ports is not impacted since the first pair of ports are transmitted without any delay while the second pair is transmitted with the same delay.

On CQI prediction, and its impact on link adaptation, we show that we can indeed reliably reconstruct the effective channel of the virtual port in uplink using SRS port sounding as described above. For the gNB to be able to reliably reconstruct the channel of the virtual port, the delay used for CDD and the relative phase across non-coherent antenna ports are required. Assuming the delay used for CDD is reflected in the SRS ports as described earlier, the only unknown is the random relative phase difference across the antenna ports. Exact knowledge of this variable allows a faithful reconstruction of the channel on the virtual port, while the lack of this knowledge only allows a coarse reconstruction of the channel.

Since the base station does not having access to the random relative phase difference apriori, only a coarse estimate can be formed. However, simulations (Figure 7) show that there is very little performance gap between the case where the base station is provided this value via a genie or the case where base station assumes a random relative phase difference to predict uplink CQI. Further, simulations (Figure 7) also show that embedding the delay associated with CDD in the SRS port is a safer alternative to allowing the gNB to assume a certain CDD delay which can lead to a mismatch between the actual CDD delay chosen by the UE and the value assumed by the gNB.  Thus, these results show that it is indeed possible to get a reasonably accurate estimate of CQI using a random value for the relative phase without compromising on performance.

Figure 7 Performance with link adaptation using CQI computed using either genie information on the relative phase difference or or some nominal value for the relative phase difference

# Simultaneous PUCCH+PUSCH transmissions across noncoherent antennas

To maximize network scheduling flexibility, for UE with non-coherent antennas, e.g., a power class 3 UE equipped with two non-coherent 20dBm PAs, NW may schedule a PUSCH transmission with precoder  and a simultaneous PUCCH transmission for certain use cases. One typical use case can be illustrated as in Figure 6. Consider a DL heavy TDD configuration where there is one UL slot out of 10 slots, similar to LTE TDD configuration 5. Since it is UL light scenario, for a cell centre UE, for power saving purpose, eNB schedules a rank-1 PUSCH with TPMI  in slot n. Therefore, only 1 Tx antenna is active with this TPMI where the other Tx antenna is turned off so far. In later DL slots, e.g., in slot n+4, eNB scheduled a PDSCH for the same UE with HARQ-ACK scheduled to transmit in slot n+9. It makes much sense to use the other Tx antenna to transmit HARQ-ACK. Since these are two different channels on two Tx antenna ports, no coherence between the two Tx antenna has no impact at all to the two transmissions. Another approach is to multiplex HARQ-ACK on the rank-1 PUSCH. But the multiplexing approach does not work actually because the DL grant is later than the UL grant. Therefore, the UL grant scheduling PUSCH does not have resource provision for HARQ-ACK. Furthermore, dynamic beta offset indication cannot be applied for rate-matching for HARQ-ACK multiplexing, again due to the fact that DL grant is later than UL grant thus eNB does not know what the correct beta factor is without knowing the HARQ-ACK codebook size. Since multiplexing does not work well, simultaneous PUCCH and PUSCH transmission cross different antenna ports is a better solution for this scenario. Of course, if the sum of the ranks of the two transmissions exceeds the maximum UL rank supported by the UE or the sum power of the two transmissions exceeds Pc_max, UE need to drop one of the channels.

Figure 8: Simultaneous PUCCH+PUSCH transmissions cross two non-coherent antennas

Proposal 4: For UE with MIMO capability, support simultaneous intra-UE PUCCH and PUSCH transmissions cross non-coherent or partial coherent antenna ports on one OFDM symbol.

- Channel dropping is applied if the sum rank or sum power of the simultaneous transmissions exceeds UE capability

# Conclusions

In summary, we have the following proposals to address the full power uplink transmissions for Rel-16 MIMO enhancements.

Proposal 1: for UE with non-coherent or partially coherent antenna ports, allow the following PUSCH precoders with non-zero entries that span across non-coherent antenna ports with up to UE implementation e.g. a small cyclic or linear delay may be applied across non-coherent antenna ports.

- Introduce a new codebookSubset = “noncoherentWithCyclicDelay” in 38.212 Section 7.1 for UEs with noncoherent antennas to include:

  - TPMIs 0 to 2 in 38.211 Table 6.3.1.5-1 (Single-layer transmission using two antenna ports),

  - TPMIs 0 to 4, 8, and 12 in 38.211 Table 6.3.1.5-2 (Single layer transmission using four antenna ports with transform precoding),

  - TPMIs 0 to 4, 8, and 12 in 38.211 Table 6.3.1.5-3 (Single layer transmission using four antenna ports without transform precoding),

  - TPMI 0 to 6 in 38.211 Table 6.3.1.5-5 (Two layer transmission using four antenna ports without transform precoding),

  - and TPMI 0 to 1 in 38.211 Table 6.3.1.5-6 (Three layer transmission using four antenna ports without transform precoding).

  - For Tables not listed above, set of allowed TPMI remains same as that for codebookSubset “nonCoherent” in 38.212

- Introduce a new codebookSubset = “partialAndNonCoherentWithCyclicDelay” in 38.212 Section 7.1 for UEs with partial and noncoherent antennas to include:

  - TPMIs 0 to 19, in 38.211 Table 6.3.1.5-2 (Single layer transmission using four antenna ports with transform precoding)

  - TPMIs 0 to 19 in 38.211 Table 6.3.1.5-3 (Single layer transmission using four antenna ports without transform precoding).

  - For Tables not listed above, set of allowed TPMI remains same as that for codebookSubset “partialAndNonCoherent” in 38.212

- Support of this feature is subject to UE capability per band-band combination

Proposal 2: For PUSCH precoders with zeros entries

- It is up to UE implementation to decide whether and how to support full power transmission.

- UE is required to maintain consistent PHR on different occasions of PUSCH transmissions with same precoder

Proposal 3: To sound for PUSCH with the allowed precoders in Proposal 1, UE transparently applies a small cyclic or linear delay to a subset of existing SRS ports defined for codebook-based PUSCH. The same amount of delay is applied in SRS sounding and corresponding PUSCH transmission.

Proposal 4: For UE with MIMO capability, support simultaneous intra-UE PUCCH and PUSCH transmissions cross non-coherent or partial coherent antenna ports on one OFDM symbol.

- Channel dropping is applied if the sum rank or sum power of the simultaneous transmissions exceeds UE capability

# References

- 3GPP TSG RAN Meeting #80, RP-181453 “WI Proposal on NR MIMO Enhancements,” La Jolla, USA, June 11 – 14, 2018.

- 3GPP TSG RAN Meeting #94b, “Meeting Report,” Chengdu, China, October 8th – 12th, 2018