3GPP TSG RAN WG1 Meeting #95			R1-1814029

Spokane, United States, November 12th – 16th, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [draft] CR on overlapping PUCCHs including multi-slot PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - If in a slot, there can be overlapping between single-slot and/or multi-slot PUCCH resoruces, it is not clear which one should be resolved first. This results in misunderstanding between UE and gNB for determination of final PUCCH/PUSCH resource for transmission. The clarifcations are needed in Subclause 9 and 9.2.5 of 38.213. - Currenlty the overlapping PUCCH resources in subclause 9.2.6 is defined such that a multi-slot PUCCH resource is overlapped with other PUCCH resources. Based on the agreement, the overlapping PUCCH resources in this subclause should be defined as two or more PUCCH resources that overlap with each other in a slot within and one of them is at least a multi-slot PUCCH resource. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define an order such that among the overlapping PUCCH resources in a slot, overlapping with multi-slot PUCCHs is resolved first, in Subclause 9 and 9.2.5. - Update the current text in Subclause 9.2.6 to capture the agreement RAN1#94bis of overlapping of a multi-slot PUCCH and single-slot PUCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambigious UE bahvior and misalignment between gNB and UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9, 9.2.5, 9.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 9	UE procedure for reporting control information

< Unchanged parts are omitted >

If a UE would transmit on a serving cell a PUSCH without UL-SCH that overlaps with a PUCCH transmission on the serving cell that includes positive SR information, the UE does not transmit the PUSCH.

If a UE would transmit CSI reports on overlapping physical channels, the UE applies the priority rules described in TS 38 .214 for the multiplexing of CSI reports.

If a UE has overlapping resources for PUCCH transmissions in a slot and at least one of the PUCCH transmissions is with repetitions over multiple slots, the UE first follows the procedures described in Subclause 9.2.6 for resolving the overlapping among the resources for the PUCCH transmissions.

< Unchanged parts are omitted >

### 9.2.5	UE procedure for reporting multiple UCI types

This Subclause is applicable to the case that a UE has overlapping resources for PUCCH transmissions or for PUCCH and PUSCH transmissions and each PUCCH transmission is over a single slot. Any case that a PUCCH transmission is with repetitions over multiple slots is described in Subclause 9.2.6.

< Unchanged parts are omitted >

9.2.6	PUCCH repetition procedure

< Unchanged parts are omitted >

If a UE would transmit a first PUCCH over more than one slot and at least a second PUCCH over one or more slots, and the transmissions of the first PUCCH and any of the second PUCCHs would overlap in a third number of slots then, for the third number of slots and with UCI type priority of HARQ-ACK > SR > CSI with higher priority > CSI with lower priority

-	the UE does not expect the first PUCCH and any of the second PUCCHs to start at a same slot and include a UCI type with same priority

-	if the first PUCCH and any of the second PUCCHs include a UCI type with same priority, the UE transmits the PUCCH starting at an earlier slot and does not transmit the PUCCH starting at a later slot

-	if the first PUCCH and any of the second PUCCHs do not include a UCI type with same priority, the UE transmits the PUCCH that includes the UCI type with higher priority and does not transmit the PUCCHs that include the UCI type with lower priority

< Unchanged parts are omitted >