3GPP TSG RAN WG1 Meeting #95	R1-1814131

Spokane, USA, November 12th – 16th 2018

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	Outcome of offline session for CSI enhancement for MU-MIMO support

Document for:	Discussion and Decision

# Introduction

The offline session (Wednesday 11/14/2018) discussed various aspects on Type II rank 1-2 overhead reduction, (if applicable) each with alternatives for down selection in later RAN1 meetings.

Design aspects and remaining details for DFT-based compression

  - Frequency-domain (FD) compression unit

Offline agreement: In RAN1 NR-AH 1901, select one of the following alternatives for precoder/PMI FD compression unit, taking into account UPT vs. overhead and complexity

- Alt1. Subband (SB), wherein the SB size for precoder/PMI compression is the same as the CQI subband size

- Alt2. X resource blocks (RBs), different from CQI subband size. Two sub-alternatives

  - Alt2.1 X = 1

  - Alt2.2 X = CQI SB size / R where R>1 is a predetermined integer

    - Only one R value is supported. FFS: the value of R

  - Alt2.3 X = {2, 4} where X is higher-layer configured

Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2 quantization for evaluation purposes.

  - Basis subset selection for the 2L beams

Offline agreement: In RAN1 NR-AH 1901, select one of the following alternatives for basis subset selection scheme for each layer

- Alt1A. Common selection for all the 2L beams, wherein M coefficients are reported for each beam

  - Wf=fk0 fk1…fkM-1

  - W2  is composed of K=2LM linear combination coefficients

  - The value of M (applied to all 2L beams) is higher-layer configured

  - The size-K subset (chosen from the size-N3 basis set) is dynamically selected by the UE (hence reported with CSI)

- Alt1B. Common selection for all the 2L beams, but only a size- K0<2LM subset of coefficients are reported (not reported coefficients are treated as zero)

  - Wf=fk0 fk1…fkM-1

  - W2  is composed of K=2LM linear combination coefficients, but (K-K0) of its coefficients are zero

  - The value of M (applied to all 2L beams) is higher-layer configured

  - For evaluation, companies should state their assumption on size-K0 subset selection (applied to all 2L beams), e.g.

    - The value of K0 is fixed or higher-layer configured, and the subset is dynamically selected by the UE (hence reported with CSI), or

    - The size-K0 subset and its size are dynamically selected by the UE (hence reported with CSI)

- Alt2. Independent selection for all the 2L beams, wherein Mi coefficients are reported for the i-th beam (i=0, 1, …, 2L-1)

  - Wf=Wf0,…,Wf2L-1, where Wf(i)=fki,0 fki,1…fki,Mi-1, i.e. Mi frequency-domain components (per beam) are selected

  - W2  is composed of K=i=02L-1Mi linear combination coefficients

  - The value of K (applied to all 2L beams) is higher-layer configured

  - For evaluation, companies should state their assumption on size-Mi subset selection (applied to the i-th beam), e.g. for i=0, 1, …, 2L-1

    - The size-Mi subset and the value of Mi are dynamically selected by the UE (hence reported with CSI)

    - The size-Mi subset is dynamically selected by the UE (hence reported with CSI), but the value of Mi is determined by a predefined rule in specification

    - The size-Mi subset is dynamically selected by the UE (hence reported with CSI), but the value of Mi is higher-layer configured

  - The size-Mi subset can be chosen either from the fixed basis set or from a beam-common UE-selected intermediate subset of the fixed basis set

Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2 quantization for evaluation purposes.

Offline agreement: In RAN1 NR-AH 1901, select one of the following alternatives for DFT basis oversampling factor(s) O3:

- Alt1. O3 = 4

- Alt2. O3 = 1 (critically sampled)

- Alt3. O3 is fixed for and depends on a given length of the DFT vector (N3) and/or bandwidth part, exact dependence is FFS

Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2 quantization for evaluation purposes.

Offline agreement: In RAN1 NR-AH 1901, companies are encouraged to evaluate the following alternatives for compression basis (Wf) subset selection scheme across different layers when RI=2. Select one of the following alternatives in RAN1#96:

- Alt1. Basis subset selection for the 1st is the same as that for the 2nd layer

- Alt2. Basis subset selection for the 1st can be different from 2nd layer

Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2 quantization for evaluation purposes.

  - Other details on DFT-based compression

Offline agreement: In RAN1 NR-AH 1901, companies are encouraged to study the following issues for finalizing the remaining details on DFT-based compression in RAN1#96:

- Supported values for the number of FD compression units before compression, or the DFT vector length (N3), by considering, e.g.

  - Whether one compression is performed across the entire CSI reporting band or a segment of the CSI reporting band

- Supported values for the number of FD components after compression (M for common selection or {Mi} for independent selection)

Offline agreement: For each layer, the following alternatives for quantizing each of the coefficients in W2 are to be studied for down selection in RAN1#96:

- Alt1A. Rel.15 3-bit amplitude; Rel.15 QPSK and 8PSK co-phasing

- Alt1B. Rel.15 3-bit amplitude; Rel.15 QPSK, Rel.15 8PSK, and new 16PSK co-phasing

- Alt2A. Rel.15 3-bit wideband amplitude for each beam, 2/3-bit differential amplitude for FD coefficients; Rel.15 QPSK and 8PSK co-phasing

- Alt2B. Rel.15 3-bit wideband amplitude for each beam, 2/3-bit differential amplitude for FD coefficients; Rel.15 QPSK, Rel.15 8PSK, and new 16PSK co-phasing

- Alt2C. Rel.15 3-bit wideband amplitude + Rel.15 QPSK and 8PSK wideband co-phasing for each beam, 2/3-bit differential amplitude and co-phasing for FD coefficients;

- Alt3. A-bit amplitude for each of 2L beams, B-bit amplitude for each of M FD components, 1-bit differential amplitude and 8PSK co-phasing for each of the 2LM FD coefficients

- Alt4. For each beam,

  - B0-bit amplitude and C0-bit phase per coefficient for the P0 strongest coefficients,

  - B1-bit amplitude and C1-bit phase per coefficient for the P1 2nd strongest coefficients, …

  - …

  - BQ-1-bit amplitude and CQ-1-bit phase per coefficient for the PQ-1 Qth strongest coefficients

  - Alternatively, amplitude/phase can be replaced with real/imaginary

- Alt5. Special case of Alt4: Q=2, B0=C0=3; B1=C1=2 on amplitude/phase