3GPP TSG-RAN WG1 Meeting #95	R1-1814278

# Spokane, WA, USA, November 12 - 16, 2018

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | V15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft] Determination of slots used for Multi-slot PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 02.11.2018 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | The procedure for determining slots for a multi-slot PUCCH transmission in unpaired spectrum does not address the case of overlap with SS/PBCH block in flexible symbols or in case the UE is not provided TDD-UL-DL-ConfigurationCommon. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If multi-slot PUCCH transmission in a slot overlaps a SS/PBCH block in flexible symbols or in case the UE is not provided TDD-UL-DL-ConfigurationCommon, the PUCCH transmission in that slot is deferred. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambigious UE behaviour in case a multi-slot PUCCH transmission in a slot overlaps an SS/PBCH block in a flexible symbols or in case the UE is not provided TDD-UL-DL-ConfigurationCommon, the UE doesn’t transmit PUCCH in this slot, but it is not clear if the UE cancels the PUCCH transmission of that slot, or defers it to a later slot. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This change has issolated impact, it impacts the case when a multi-slot PUCCH transmisison overlaps a SS/PBCH block in flexible symbols or in case the UE is not provided TDD-UL-DL-ConfigurationCommon. If the network implements this change, but the UE doesn’t the UE might drop a PUCCH transmission and not defer it, this leads to fewer transmissions for PUCCH combining, the network can detect this with a DTX detector. |  |  |  |  |  |  |  |  |

### 9.2.6	PUCCH repetition procedure

<omitted parts>

An SS/PBCH Block symbol is a symbol indicated to the UE by ssb-PositionsInBurst in SystemInformationBlockType1 or ssb-PositionsInBurst in ServingCellConfigCommon.

For unpaired spectrum, the UE determines the  slots for a PUCCH transmission starting from a slot indicated to the UE as described in Subclause 9.2.3 and having

-	an UL symbol, as described in Subclause 11.1, or flexible symbol that is not SS/PBCH Block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and

-	consecutive UL symbols, as described in Subclause 11.1 or flexible symbols that are not SS/PBCH Block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4

For paired spectrurm, the UE determines the  slots for a PUCCH transmission as the  consecutive slots starting from a slot indicated to the UE as described in Subclause 9.2.3

If a UE would transmit a PUCCH over a first number  of slots and the UE would transmit a PUSCH over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in Subclause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots.

<omitted parts>