3GPP TSG RAN WG1 Meeting #95 R1-1814281 Spokane, USA, November 12th - 16th, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR1 on default value for msg3-DeltaPreamble. Source to WG:; ZTE. Source to TSG:; R1. Work item code:; NR_newRAT-Core; Date:; 2018-11-16. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; For Rel-15 UL power control-nonCA, the higher layer parameters msg3-DeltaPreamble is optional RRC parameter without default values in TS 38.331. Default values are needed in case the parameter is not configured to the UE.. Summary of change:; Providing 0dB as a default value for msg3-DeltaPreamble.. Consequences if not approved:; Incomplete and incorrect specifications for UL power control in NR Rel-15. Clauses affected:; 7.1.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

### 7.1.1	UE behaviour

<Unchanged parts are omitted>

-	If a UE is not provided P0-PUSCH-AlphaSet or for a PUSCH transmission scheduled by a RAR UL grant as described in Subclause 8.3, , , and  , where the parameter preambleReceivedTargetPower [11, TS 38.321] (for ) and msg3-DeltaPreamble (for ) are provided by higher layers, or = 0 dB if msg3-DeltaPreamble is not provided, for carrier  of serving cell

<Unchanged parts are omitted>