3GPP TSG RAN WG1 Meeting #95 R1-1814281 Spokane, USA, November 12th - 16th, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR2 on power scaling for codebook transmission. Source to WG:; ZTE. Source to TSG:; R1. Work item code:; NR_newRAT-Core; Date:; 2018-11-16. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; For Rel-15 UL power control-nonCA, the number of configured antenna ports for PUSCH transmission does not have one definition in NR. The UE behaviour of PUSCH power scaling is not clear.. Summary of change:; The ‘number of configured antenna ports for the transmission scheme’ in Section 7.1 in TS38.213 is replaced as “the maximum number of SRS ports in one SRS resource', for codebook transmission.. Consequences if not approved:; gNB is unaware of the power scaling among the PUSCH ports used by UE for PUSCH transmission in codebook based operation. Clauses affected:; 7.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

## 7.1	Physical uplink shared channel

For a PUSCH transmission on active UL BWP , as described in Subclause 12, of carrier  of serving cell , a UE first calculates  a linear value  of the transmit power , with parameters as defined in Subclause 7.1.1. If the PUSCH transmission is scheduled by DCI format 0_1 and when the higher layer parameter txConfig in PUSCH-Config is set to 'codebook', the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the the maximum number of SRS ports in one SRS resource supported by the UE. When the higher layer parameter txConfig in PUSCH-Config is set to 'codebook', the UE splits the resulting scaled power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power.