3GPP TSG RAN WG1 Meeting #95			                                R1-1814350

Spokane, US, November 12th –16th, 2018

Source:                ZTE

Title:                    Summary of NOMA Receiver Complexity Analysis

Agenda item:      7.2.1.2

Document for:    Discussion and Decision

Background

During the RAN1 #94bis meeting, the order-wise computation complexity formulae and an example value table were agreed. Several companies have submitted complexity counts based on these agreed tables [1][2][3][8] [15][16] [17]. Complexity analysis results are collected based on company's contributions and some observations are drawn accordingly.

Calculation of NOMA receiver complexity

For the complexity of MMSE IRC/hard-IC receiver, several options have been agreed to calculate the demodulation weight which cover both serial and parallel cancellation in MMSE hard-IC receiver. Option 1 complexity are provided in [1][3][17]. Option 2 complexity are provided in [8][15].  Option 3 complexity are provided in [2]. For MMSE IRC receiver, three different versions are provided in [1]. They are further revised into two. These two options adopt the two different methods of covariance matrix calculation in the agreed table of order-wise computation complexity analysis respectively.

For the computational complexity analysis of ESE SISO based receiver, ESE receiver complexity is provided in [1][8][16] and enhanced ESE with both MMSE and MF signal combining is provided in [1].

For the computational complexity analysis of EPA SISO based receiver, 3 options based on the agreed table are provided in [1][3][17]

For channel decoder, statistics of decoding attempts for different receivers are provided in [1][2][4][17].

In the agreed parameter value table, the following two parameters can have wide range.

- NREadj 	: number of adjacent REs to which the same demodulation weights are applied

- NREadj,SINR: number of adjacent REs to which the same SINR is assumed

MMSE-IRC/hard-IC

In [1][2][6][12], link level results are provided to compare the performance with different values of NREadj,. Many results show that under certain circumstances, performance degradation is negligible even when NREadj is relatively large.. Typical values used in the simulation are {4, 24, 48, 144}. Therefore, the following complexity calculation of MMSE IRC/hard-IC receiver focuses on these values. In [1][3][17], NREadj,SINR= 48 or 144 have been proposed. The complexity analysis for three options of MMSE hard-IC and two options of MMSE IRC receivers are collected in Figure.1-3

Figure. 1 Option 1 complexity calculation of MMSE hard-IC based detector/IC

Figure. 2 Option 2 complexity calculation of MMSE IRC/ hard-IC based detector/IC

Figure. 3 Option 3 complexity calculation of MMSE hard-IC receiver

ESE based

The complexity analysis for ESE SISO based receivers are collected in Figure.4

Figure. 4 Complexity calculation of ESE/enhanced-ESE based detector/IC

EPA based

Figure. 5 Complexity calculation of EPA based detector/IC

Channel decoder

It has been proposed in [1][5][8] that the number of usages can be used to quantify the decoding complexity. Maximal decoding attempts under various simulation cases are provided in [1][4][17] for MMSE hard-IC receiver which ranges from 1.2 to 1.6, while the maximal number of decoding attempts for EPA SISO is 2.6 as provided in [17]. In the evaluation of MMSE hard-IC and ESE decoding complexity, 2 and 5 has been used in [1][3][8][16]. These proposed values are roughly aligned with the agreed example values. Hence, the channel decoder complexity can be reflected in following the table with the agreed values or range of values,

Table 1 Ratio of decoding complexity of various receivers

| Rx Types Decoding Complexity | MMSE IRC | MMSE hard-IC | ESE SISO | EPA SISO |
| --- | --- | --- | --- | --- |
| Ratio by number of usages | 1 | 1.5~3 | 5 | 3 |

UE detection and channel estimation

The calculation of UE detection and channel estimation is provided in [1][2]. Compared to the overall detector plus IC complexity provided, the UE detection and channel estimation don't account for significant portion.

Table 2 Computation complexity of UE detection and channel estimation

| Detailed Component | Computation Complexity |
| --- | --- |
| UE detection | 576 |
| Channel estimation | 6912 |

Text Proposal for NOMA related receivers to TR38.812

[R1-1813547] propose to include add the description of MPA under Section 6.1 in the TR38.813. We can check it during the offline discussion.

- MPA

MPA (message passing algorithm) detector passes the conditional probability back and forth between every FN (function node, representing RE) and VN (variable node, representing data layer) edge in the factor graph of a NOMA scheme. During each iteration, the values (probabilities) on FNs and VNs are updated respectively. After a number of iterations (inner loops), the LLRs for the coded bits are calculated based on the current probabilities and then input to the channel decoder. The MPA detector can be combined with any interference cancellation (IC) types, such as soft PIC, hard SIC/PIC, and hybrid soft and hard PIC.

Conclusion

Discuss further offline using the proposals/observations in R1-1813963 as a starting point – for the ratio of complexity, refine the ranges by considering adding references, and further dividing the ranges into some sub-ranges

Agreements:

- To capture Figures 1~5 & Table 1 (subject to further update) as in R1-1813963 into the TR

  - Discuss further offline how to incorporate the proposals in R1-1813920 & R1-1814076 into updating Figures 1~5 & Table 1

  - To also have chip-level receiver as another basis for complexity comparison

    - Discuss further offline details

  - Discuss further offline whether or not to correlate (if so, how) the complexity analysis with the performance for different baseline assumptions

  - Discuss further offline performance vs. N (# of adjacent REs) tradeoff

According to the numerical analysis submitted to RAN1#95, the different options of receiver complexity calculations have been chosen,

The options chosen for calculations can be listed in the following,

MMSE hard-IC demodulation weight calculation option 1(Table in TR38.812) chosen by R1-1813858, R1-1814043, R1-1813870

MMSE hard-IC demodulation weight calculation option 2(Table in TR38.812) chosen by R1-1813157

MMSE hard-IC demodulation weight calculation option 3(Table in TR 38.812) chosen by R1-1813920

EPA option 1(Table in TR 38.812) R1-1813858

EPA option 2(Table in TR 38.812)  R1-1813920

EPA option 3(Table in TR 38.812) R1-1814043 R1-1813870

  - To also have chip-level receiver as another basis for complexity comparison

    - Discuss further offline details

The chip-level receiver,i.e. chip MMSE hardIC and chip MMSE IRC could be obtained from tuning SF = 1 and number of iterations into 1 in option 1 MMSE hardIC demodulation weight calculation option.

The updates from R1-1814043, R1-1813870.

With NREadj=1 ,

  - Option 1 MMSE hard-IC with respect to MMSE IRC

MMSE hard-IC  with SF = 1 with respect to MMSE-IRC with SF =1:  6.7 @2Rx, 3.3~5.6 @4Rx[TDoc# to be added])

  - Option 2 MMSE hard-IC with respect to MMSE IRC

    - MMSE hard-IC  with SF = 4 with respect to MMSE-IRC with SF =1:  6.7 @2Rx, 3.3~5.6 @4Rx[TDoc# to be added])

  - Option 3 MMSE hard-IC with respect to MMSE IRC

    - MMSE hard-IC  with SF = 4 with respect to MMSE-IRC with SF =1: @2Rx, @4Rx[TDoc# to be added])

With NREadj=4 ,

  - Option 1 MMSE hard-IC with respect to MMSE IRC

    - MMSE hard-IC  with SF = 4 with respect to MMSE-IRC with SF =4:  1.3~4.8 @2Rx, 1.2~1.6 @4Rx

[TDoc# to be added])

  - Option 2 MMSE hard-IC with respect to MMSE IRC

    - MMSE hard-IC  with SF = 4 with respect to MMSE-IRC with SF =4: 1.3~4.8 @2Rx, 1.2~1.6 @4Rx

[TDoc# to be added])

  - Option 3 MMSE hard-IC with respect to MMSE IRC

    - MMSE hard-IC  with SF = 4 with respect to MMSE-IRC with SF =4:

With NREadj=24 ,

…..

With NREadj=48

…

With NREadj=144 ,

….

The complexity analysis results are reported as follow,

With NREadj=1 ,

  - ESE-SISO with respect to MMSE IRC with SF = 1:

    - ESE-SISO with respect to MMSE-IRC:  6.7~10.4 @2Rx, 2.8~4.7 @4Rx

    - enhanced ESE-SISO-MF with respect to MMSE-IRC:  2.1 @2Rx, 0.7 @4Rx

    - enhanced ESE-SISO-MMSE with respect to MMSE-IRC:  2.8 @2Rx, 2.4 @4Rx

  - EPA-SISO with respect to MMSE IRC with SF = 1 :

    - option 1 EPA-SISO with respect to MMSE-IRC:  28.5 @2Rx, 7.8 @4Rx

    - option 2 EPA-SISO with respect to MMSE-IRC:  8.7~12.1 @2Rx, 4.5~6.4 @4Rx

    - option 3 EPA-SISO with respect to MMSE-IRC:  8.6~12.2 @2Rx, 4.1~6.9  @4Rx

With NREadj=4 ,

  - ESE-SISO with respect to MMSE IRC with SF = 4:

    - ESE-SISO with respect to MMSE-IRC:  1.7~2.6 @2Rx, 0.5~0.9 @4Rx

    - enhanced ESE-SISO-MF with respect to MMSE-IRC:  0.5 @2Rx, 0.1 @4Rx

    - enhanced ESE-SISO-MMSE with respect to MMSE-IRC:  0.7 @2Rx, 0.4 @4Rx

  - EPA-SISO with respect to MMSE IRC with SF = 4 :

    - option 1 EPA-SISO with respect to MMSE-IRC:  7.1 @2Rx, 1.5 @4Rx

    - option 2 EPA-SISO with respect to MMSE-IRC:  5.9~8.2 @2Rx, 1.7~2.4 @4Rx

    - option 3 EPA-SISO with respect to MMSE-IRC:  2.2~3.1 @2Rx, 0.8~1.3  @4Rx

With NREadj=24 , the computation complexity of ESE/EPA-SISO is higher than MMSE IRC receiver.

  - ESE-SISO with respect to MMSE IRC with SF = 4:

    - ESE-SISO with respect to MMSE-IRC:  7~11 @2Rx, 2.7~3.4 @4Rx

    - enhanced ESE-SISO-MF with respect to MMSE-IRC:  2.3 @2Rx, 0.6 @4Rx

    - enhanced ESE-SISO-MMSE with respect to MMSE-IRC:  2.9 @2Rx, 2.3 @4Rx

  - EPA-SISO with respect to MMSE IRC with SF = 4 :

    - option 1 EPA-SISO with respect to MMSE-IRC:  30.1 @ 2Rx, 7.5@4Rx

    - option 2 EPA-SISO with respect to MMSE-IRC:  16.2~22.6 @ 2Rx, 4.4~6.1@4Rx

    - option 3 EPA-SISO with respect to MMSE-IRC:  9.1~12.9 @ 2Rx, 4~6.7@4Rx

With NREadj=48 , the computation complexity of ESE/EPA-SISO is higher than MMSE IRC receiver.

  - ESE-SISO with respect to MMSE IRC with SF = 4:

    - ESE-SISO with respect to MMSE-IRC:  10.3~16.1 @2Rx, 4.6~7.8 @4Rx

    - enhanced ESE-SISO-MF with respect to MMSE-IRC:  3.3 @2Rx, 1.1 @4Rx

    - enhanced ESE-SISO-MMSE with respect to MMSE-IRC:  4.3 @2Rx, 3.9 @4Rx

  - EPA-SISO with respect to MMSE IRC with SF = 4 :

    - option 1 EPA-SISO with respect to MMSE-IRC:  44.3 @2Rx, 12.9 @4Rx

    - option 2 EPA-SISO with respect to MMSE-IRC:  19.7~27.4 @2Rx, 7.5~10.5 @4Rx

    - option 3 EPA-SISO with respect to MMSE-IRC:  13.5~19 @2Rx, 6.9~11.5 @4Rx

With NREadj=144 , the computation complexity of ESE/EPA-SISO is higher than MMSE IRC receiver.

  - ESE-SISO with respect to MMSE IRC with SF = 4:

    - ESE-SISO with respect to MMSE-IRC:  15.1~23.5 @2Rx,4.6~7.8 @4Rx

    - enhanced ESE-SISO-MF with respect to MMSE-IRC:  4.8 @2Rx, 6.3 @4Rx

    - enhanced ESE-SISO-MMSE with respect to MMSE-IRC:  2.1 @2Rx, 7.6 @4Rx

  - EPA-SISO with respect to MMSE IRC with SF = 4 :

    - option 1 EPA-SISO with respect to MMSE-IRC:  64.7 @2Rx, 24.7 @4Rx

    - option 2 EPA-SISO with respect to MMSE-IRC:  22.9~31.9 @2Rx, 14.3~20.2 @4Rx

    - option 3 EPA-SISO with respect to MMSE-IRC:  19.7~27.7 @2Rx, 13.2~22 @4Rx

Discuss further offline performance vs. N (# of adjacent REs) tradeoff

From [1], the simulation results show that the no performance degradation could be observed following the simulation assumptions for the cases simulated following the assumptions in template I.

From [2], the simulation results show non-negligeable performance degradation could be observed, in particularly considering high mobility.

From [6], the simulation results following the assumptions in template I has no performance degradation while the simulation results under higher mobility exibit certain performance loss.

From [15], the simulation results following the assumptions in template I has no performance degradation while the simulation results under higher mobility exibit accpetable performance loss.

References

- 3GPP,R1-1812173, Complexity analysis of NOMA receivers ZTE, Sanechips, RAN1#95

- 3GPP,R1-1812188, Discussion on the design of NOMA receiver, Huawei, HiSilicon, RAN1#95

- 3GPP,R1-1812476, NOMA receiver structure and complexity analysis, Intel Corporation, RAN1 #95

- 3GPP,R1-1812553, Discussion on the receiver types, LG Electronics, RAN1#95

- 3GPP, R1-1811175, Receiver design for NOMA, Ericsson, RAN1#94bis.

- 3GPP,R1-1812610, Discussion on NOMA receivers, CATT,RAN1#95

- 3GPP,R1-1812967, Receivers for NOMA, Samsung,RAN1#95

- 3GPP,R1-1813157, Receiver considerations for UL NOMA, Nokia, Nokia Shanghai Bell,RAN1#95

- 3GPP,R1-1813212, Performance Evaluation of NOMA Receivers for IDMA, InterDigital, Inc, RAN1#95

- 3GPP,R1-1813255, NoMA receiver design and complexity, Ericsson,RAN1#95

- 3GPP,R1-1813306, Receiver for uplink NOMA,NTT DOCOMO, INC, RAN1#95

- 3GPP,R1-1813406, Receivers for NOMA, Qualcomm Incorporated,RAN1#95

- 3GPP,R1-1813518, NOMA Receiver Complexity Analysis, Zhejiang University,RAN1#95

- 3GPP,R1-1813544, On receiver architecture benchmarks for OMA and NOMA, Orange,RAN1#95

- 3GPP, R1-1813160,Complexity Analysis of MMSE-based Hard IC Receive, Nokia, Nokia Shanghai Bell, RAN1#95

- 3GPP,R1-1813161,Complexity Analysis of ESE-based Soft IC Receiver, Nokia, Nokia Shanghai Bell, RAN1#95

- 3GPP,R1-1813870, Receiver for uplink NOMA,NTT DOCOMO, INC, RAN1#95

Appendix

| Source Number | References |
| --- | --- |
| Source 1 | [1] |
| Source 2 | [2] |
| Source 3 | [3] |
| Source 4 | [8] |
| Source 5 | [17] |

- NiterIC average number of IC iterations.

- NUE average number of UEs processed per iteration.

- NUE+ average number of UEs per IC iteration excluding the first iteration.

- NUES average number of successfully decoded UE per IC iteration.

- NUEF average number of UEs that were not successfully decoded per IC iteration.

Table 1 Total complexity of the LMMSE MUD with hybrid IC across all iterations

| Receiver component | Detailed component | Computation in parametric number of usages, O(.) analysis, [impact factor] |
| --- | --- | --- |
|  |  | LMMSE with Hybrid IC |
| Detector | UE detection | ONAPDMRS∙NREDMRS∙Nrx |
|  | Channel estimation | ONUE∙NRECE∙NREDMRS∙Nrx |
|  | Rx combining, if any |  |
|  | Covariance matrix calculation, if any | O(NSF2.Nrx2.NUE.NREdata/NSF/Nschsym) +O((NiterIC-1)NSF2.Nrx2.NUE+.NREdata/NSF) |
|  | Demodulation weight computation, if any | O(NSF2.Nrx2.NUE.NREdata/NSF/Nschsym) + O(NSF3.Nrx3.NREdata/NSF/Nschsym) +O(NiterIC-1.NSF2.Nrx2.NUE+.NREdata/NSF)+O(NiterIC-1.NSF3Nrx3.NREdata/NSF) |
|  | UE ordering, if any |  |
|  | Demodulation, if any | O(NSF.Nrx.NUE.NREdata/NSF/Nschsym) +ONiterIC-1.NSF.Nrx.NUE+2.NREdata/NSF +ONiterIC-1.NSF.Nrx.NUE+.NREdata/NSF |
|  | Soft information generation, if any | Real: O(NiterIC.NUE+.Nbit) |
|  | Soft symbol reconstruction, if any | O(NiterIC.NUES.NREdata.Nrx) |
|  | Soft symbol reconstruction, if any |  |
|  | Message passing, if any |  |
|  | Others |  |
| Decoder | LDPC decoding | A: NiterIC.NUE.NiterLDPC.dvNbit+2(Nbit-Kbit) C: NiterIC.NUE.NiterLDPC.(2dc -1)∙(Nbit-Kbit) |
| Interference cancellation | Symbol reconstruction(Including FFT operations for DFT-S-OFDM waveform), if any |  |
|  | LLR to probability conversion, if any | Real: O(NiterIC.NUEF.2Qm.NREdata/NSF) |
|  | Interference cancellation | Complexity of addition ignored |
|  | LDPC encoding, if any | Buffer shifting: NiterIC.NUES⋅(Nbit-Kbit)/2 Addition: NiterIC.NUES⋅(dc-1)(Nbit-Kbit) |