3GPP TSG-RAN WG1 Meeting #95 	R1-1813704

# Spokane, Washington, USA November 12th – 16th 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft 36.213 CR on edge PRB restriction for flexible starting PUSCH PRB for Rel-15 LTE-MTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  |  | Date: |  | 2018-11-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The new PUSCH resource allocation with flexible starting PRB allows allocation outside the 6-PRB narrowbands. Some concerns have been raised that this may require RAN4 to revisit A-MPR requirements and that this might jeopardize the timely and successful conclusion of the work in December 2018. To address these concerns, one possibility could be to disallow resource allocations that would contain (one or two) edge PRBs that are not part of any narrowband. Such a restriction would reduce the scheduling flexibility to some extent, but since these edge PRBs are in practice likely to be occupied e.g. by PUCCH transmissions, the resulting impact of the restriction may be insignificant. The restriction is in line with the RAN1#94bis agreements on frequency hopping that deprioritize the special cases which involve edge PRBs and may help simplify the specification of the frequency retuning and frequency hopping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For flexible starting PUSCH PRB, disallow the PUSCH resource allocation cases that contain one or more edge PRBs not belonging to any narrowband. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential risk that RAN4 is required to revisit A-MPR requirements (see above) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1, 8.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This draft CR is based on TS 36.213 V15.3.0 plus agreed CR in R1-1812088. |  |  |  |  |  |  |  |  |

### 8.1.1	Uplink resource allocation type 0

The resource allocation information for uplink resource allocation type 0 indicates to a scheduled UE a set of contiguously allocated virtual resource block indices denoted by . A resource allocation field in the scheduling grant consists of a resource indication value (RIV) corresponding to a starting resource block () and a length in terms of contiguously allocated resource blocks ( 1).

For a BL/CE UE,

-	uplink resource allocation type 0 is only applicable for UE configured with CEModeA, and

-	, if the UE in TDD is configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig; otherwise , and

-	if the UE is configured with higher layer parameters ce-PUSCH-FlexibleStartPRB-AllocConfig, PUSCH resource allocations containing PRB(s) not belonging to any narrowband are not allowed unless it is the center PRB in the uplink system bandwidth, and

-	if the UE is not configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig,  is always set to 6 in this subclause regardless of the system bandwidth.

For PDCCH/SPDCCH DCI format 7-0A/7-0B and NRBUL>15, VRB allocations for a UE vary from 4 VRB(s) up to  VRBs with an increment step of 4 VRBs. A type 0 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block  using  and a length in terms of virtually contiguously allocated resource blocks , where  is defined if configured by higher layer parameter resourceAllocationOffset; otherwise set to 0. The resource indication value is defined by:

if  then

else

where , and , and where,

1 and shall not exceed .

Otherwise, the resource indication value is defined by

if  then

if a BL/CE UE in TDD is configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig, then

else

else

### 8.1.3	Uplink resource allocation type 2

Uplink resource allocation type 2 is only applicable for BL/CE UE configured with CEModeB. The resource allocation information for uplink resource allocation type 2 indicates to a scheduled UE a set of contiguously allocated resource blocks within a narrowband as given in Table 8.1.3-1. If the UE is not configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocOffset,  else value of is given by the higher layer parameter, offsetCeModeB.

If the UE is configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocOffset and the value of the resource allocation field is '110' or '111', the allocated resource blocks with indices less than 0 and greater than 5 correspond to resource-blocks outside the allocated narrowband relative to resource block 0. The physical resource-block numbers are  with i=0 or i=1 for the resource allocation field of '110' or '111', respectively, where nNB,0 is the smallest physical resource-block number of the narrowband as defined in Subclause 6.2.7 of [3]. PUSCH resource allocations containing PRB(s) not belonging to any narrowband are not allowed unless it is the center PRB in the uplink system bandwidth.

Table 8.1.3-1: Resource block(s) allocation for BL/CE UE configured with CEModeB.

| Value of resource allocation field | Allocated resource blocks |
| --- | --- |
| '000' | 0 |
| '001' | 1 |
| '010' | 2 |
| '011' | 3 |
| '100' | 4 |
| '101' | 5 |
| '110' | and |
| '111' | and |