3GPP TSG RAN WG1 Meeting #95	R1-1813997

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.1.5

Source:	Huawei, HiSilicon

Title:	Additional NOMA SLS evaluations

Document for:	Discussion and Decision

- Introduction

In the email discussion [94b-NR-05] PHY abstraction methods for MMSE-Hard IC receiver, ESE-SISO receiver and a method applicable to both EPA-hybrid IC and MMSE-Hard IC type of receivers are approved in [1].

In this contribution, we provide additional SLS results using the link-to-system mapping method proposed in section 2.1.1 in [1] for MMSE hard IC type of receiver.

- PHY abstraction

In the Step 1 for calculating the post-processing SINR for PHY abstraction of MMSE-Hard IC receiver, using the same notation as in [1], for each of the ith target user with received signal and noise plus intra-/inter-cell interference given as

(1)

(2)

respectively, the weight of linear MMSE receiver can be then calculated as

(3)

with the covariance of zi

Rz,i=m=1,m≠iNhk,mhk,m*+Rnknk								(4)

where (.)* denotes Hermitian transpose and Rnknkis the covariance matrix of noise plus inter-cell interference. Then the corresponding post-processing (pp)-SINR of the ith user can be calculated as

(5)

In the above description it is not clear how inter-cell interference is actually modelled and reflected in the calculation of pp-SINR. In this contribution, we assume the inter-cell inference covariance matrix Rnknk is whitened, e.g. due to symbol level scrambling.

The remaining steps including effective SINR mapping, interference cancellation and modelling of realistic channel estimation are following the method given in section 2.1.1 in [1].

- System level simulation results for mMTC

The results of PDR vs. PAR, CDF of pp-SINR based on the above method are given in Figure 1. The evaluation assumptions are also given in the Appendix. Note the receiver for SCMA and LCRS is chip-MMSE with hard IC, and the receiver for MUSA is block-MMSE with hard IC.

From the results, it is observed that SCMA with 2 branches (ML-SCMA in the legend) performs the best among the NOMA schemes, 2.7% better than LCRS, and 7% better than MUSA with SF=2, and 105% better than MUSA with SF=4. The relative performance is similar to the results we presented in [2].

| (a) PDR vs. PAR | (b) CDF of pp-SINR |
| --- | --- |

Figure 1. PDR vs. PAR and CDF of pp-SINR

Table 1. PAR@1% target PDR

|  | PAR @1% PDR | Gain of SCMA with 2 branches over other NOMA schemes |
| --- | --- | --- |
| SCMA, 2 branches | 760 | / |
| SCMA, 1 branch | 710 | 7.0% |
| MUSA, SF=2 | 710 | 7.0% |
| MUSA, SF=4 | 370 | 105% |
| LCRS | 740 | 2.7% |

- Conclusion

In this contribution, we provide additional system level evaluation results for NOMA MMSE Hard IC receiver using the proposed method in [1]. We observe that when MMSE IC receivers are applied, SCMA with 2 branches (ML-SCMA in the legend) performs the best among the NOMA schemes, 2.7% better than LCRS, and 7% better than MUSA with SF=2, and 105% better than MUSA with SF=4.

References

- R1-1812096, “Email discussion to collect PHY abstraction methods”, ZTE, RAN1#94bis.

- R1-1813912, “SLS evaluations and observations in mMTC”, Huawei, HiSilicon, RAN1#95.

Appendix

Table 1. System-level evaluation assumptions

| Parameters | mMTC |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 1732m |
| Carrier frequency | 700MHz |
| Simulation bandwidth | 6 PRBs as starting point |
| Resource allocation | 1PRB+6ms per UE |
| Number of UEs per cell | 100 |
| Channel model | UMa in TR 38.901; The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for SLS with frequencies below 6 GHz. |
| UE Tx power | Max 23 dBm |
| BS antenna configurations | 2 Rx (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), 2 TXRU; dH = dV = 0.5λ; BS antenna downtilt: 92 |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi, 0dB cable loss |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 1Tx as starting point |
| UE antenna height | Follow the modelling of TR 38.901 |
| UE antenna gain | 0dBi as starting point |
| UE distribution | For mMTC: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell |
| UE power control | Open loop PC, P0=-110, a=1 |
| HARQ/repetition | maximum number of HARQ transmission=8 |
| Channel estimation | Realistic |
| BS receiver | Block-wise MMSE for MUSA, chip-wise MMSE for LCRS/SCMA |
| Traffic model | Packet arrival per UE: Poisson arrival with arrival rate λ; Packet size: 20~200 bytes Pareto + higher layer protocol overhead of 29 bytes, as defined in TR 45.820 to be the starting point. In the case of packet segmentation, use 5 bytes packet segmentation overhead for each TB in the SLS evaluation of the NOMA schemes. |
| TB size | 40 bytes |
| Packet dropping criterion | 8 HARQ transmission or 1s latency |