3GPP TSG RAN WG1 Meeting #96	R1-1901522

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.2.1.1

Source:	Huawei, HiSilicon

Title:	Initial access signal and channels in NR unlicensed band

Document for:	Discussion and decision

Introduction

In this contribution, we discuss NR-U DRS design including numerology, OCB requirement and DRS composition. Furthermore, extensive PRACH preamble evaluations are provided for the four agreed frequency resource allocation alternatives together with comparisons for different preamble sequences.

NR–U DRS design

DRS multiplexing in frequency domain

According to the ETSI regulation [4] the OCB requirement is at least 80 percent of the initial DL BWP for NR-U, which equals to 16 MHz. The bandwidth of an SSB is 7.2 MHz with 30 kHz SCS. The current transmission bandwidth of SSB cannot satisfy OCB requirement when only SSB is transmitted in NSA deployment. Multiplexing of RMSI/CSI-RS, if available, with SSB(s) in FDM manner could facilitate OCB requirement fulfillment of NR-U DRS. The following options can be considered.

Alt-1: If NR-U DRS consists of RMSI, frequency multiplexing of RMSI PDSCH with SSB(s), can be considered to meet the OCB requirement in sub-7GHz.

Alt-2: If CSI-RS is jointly transmitted with SSB(s), combinations of the quasi co-located CSI-RS and SSB is another possible alternative to meet the OCB requirement for unlicensed operation. However, such joint transmission may reduce CSI-RS configuration flexibility, and the transmitted CSI-RS could be meaningless if not configured to any UE for beam management or CSI reporting.

Alt-3: Duplicated SSB(s) transmission in frequency domain could provide additional coverage and meanwhile fulfill the OCB requirement.

To sum up, if NR-U DRS consists of RMSI/CSI-RS/paging, frequency multiplexing RMSI/CSI-RS with SSB(s) could be an efficient solution to the OCB requirement fulfillment. On the other hand, if NR-U DRS only consists of SSB(s), duplicated SSB(s) transmission in frequency domain could be considered to fulfill the OCB requirement.

Proposal 1: When NR-U DRS including RMSI/CSI-RS, multiplexing RMSI /CSI-RS with SSB(s) in FDM manner should be supported to meet the OCB requirement.

Proposal 2: When NR-U DRS only consists of SSB(s), duplicated SSB(s) transmission in frequency domain could be supported to fulfill the OCB requirement.

DRS multiplexing in time domain

LTE LAA Discovery Reference Signal (DRS) consists of CRS, PSS, SSS and optional CSI-RS. Similarly, introduction of NR-U DRS consisting of at least the SSB burst set was identified beneficial in SI phase. For NR-U Scell in CA mode, only SSB(s) are required in NR-U DRS. SSB pattern without gap is identified beneficial to reduce LBT impact when an NR-U DRS consists of multiple SSBs. An example is illustrated in Figure 1. 6 SSBs can be consecutively transmitted within 2 slots. Note that each SSB candidate location shall not cross the slot boundary, which means that only 12 symbols within one slot will be used for SSB transmission, and the remaining 2 symbols could be used for purpose of DL control/singling. While in DC or SA mode, the corresponding RMSI is jointly transmitted with SSB as NR-U DRS for initial access. Besides, CSI-RS could also be configured as part of NR-U DRS for the purpose of RLM/RRM measurements. To better accommodate SSB and the corresponding DL signal(s)/channel(s), a new DRS pattern is required, in which at least 7 symbols are allocated for SSB and other multiplexed channels/signals with same QCL assumption. As demonstrated in Figure 2, two alternatives can be considered. In proposed alternatives, Type-0 PDCCH corresponds to SSB 0 starts at symbol 0, and Type-0 PDCCH corresponds to SSB 1 starts at symbol 7. The only difference between two alternatives is that Alternative 2 supports 3 symbol CORESET configuration while Alternative 1 only supports 1or 2 symbol CORESET configuration. In this case, SSB pattern is symmetric with half slot granularity, which is beneficial for DRS transmission since DRS signal/channel composition will not change according to the starting position of DRS transmission, i.e., at symbol 0 or symbol 7 within a slot. In other words, the basic unit of DRS (could be referred to as DRS block) consists of SSB, the corresponding Type-0 PDCCH/PDSCH, OSI (optional) and paging (optional) with same QCL assumption. The duration of one DRS block depends on the composition of DRS (i.e., whether OSI/Paging/CSI-RS are included or not) and MCS(s) used for transmission. When OSI, Paging and CSI-RS are jointly transmitted with low MCS, the duration of a DRS block could be one slot. In this case, SSB position within the second half of the slot will be occupied by other channel(s)/signal(s). In addition, the corresponding Type-0 PDCCH associated with an SSB shall be jointly transmitted within the same slot to ensure its transmission will not be blocked due to LBT failure or COT expiration. If RMSI CORESET shall meet OCB requirement and be transmitted with the corresponding SSB in the same slot, then limited RMSI CORESET configurations are available in both time and frequency domain for NR-U compared with that specified in NR Rel-15. Furthermore, only pattern 1(TDM) is supported by NR-U for SSB and RMSI CORESET multiplexing. To summarize, fewer signaling bits are needed to indicate NR-U RMSI CORESET configuration. In order to reducing signaling overhead, it is beneficial that a new RMSI configuration table shall be considered which can include configuration indication in both time and frequency domain instead of reusing table that designed for NR Rel-15.

It was agreed in TR [2] that OSI and paging could be transmitted within NR-U DRS. It is beneficial to reduce the required number of channel access attempts and better fulfill the OCB requirement in NR-U. From another perspective, it is undesirable to always consist of all these channels/signals in NR-U DRS due to limited resources available for the DRS transmission.

In NR, PBCH within SSB can indicate the multiplexing pattern between SSB and RMSI CORESET. When additional signal (i.e. OSI, paging) also be included in DRS, it is intuitive to extend the configuration information in PBCH to carry the indication of inclusion for additional signal and the multiplexing pattern between SSB and those signals multiplexed in DRS.

Figure 1. Illustration of NR-U DRS in CA mode

Figure 2.  Illustration of NR-U DRS in DC/SA mode

Proposal 3: NR-U should support different SSB patterns depending on whether RMSI/OSI/paging are included in NR-U DRS.

- In NR-U LAA mode, an SSB pattern with 3 consecutive SSB locations per slot as shown in Figure 1 should be supported.

- In DC or SA mode, SSB pattern with 2 non-consecutive SSB locations per slot should be supported. Two sets of SSB candidate positions within a slot as shown in Figure 2 can be considered.

  - Alt 1: symbol (2, 3, 4, 5) and symbol (9, 10, 11, 12)

  - Alt 2: symbol (3, 4, 5, 6) and symbol (10, 11, 12, 13)

Proposal 4: NR-U should support monitoring of Type-0 PDCCH of the 2nd SSB position in a slot start at symbol#7.

Proposal 5: The corresponding Type-0 PDCCH associated with an SSB shall be jointly transmitted within the same slot.

Proposal 6: PBCH within SSB can be used to indicate the inclusion of RMSI, OSI and paging in DRS.

NR-U PRACH Design

In this section, we evaluate and compare various PRACH preamble schemes for NR-U, considering different frequency resource allocation alternatives (i.e., Alt 1-Alt 4 agreed in [3]). We follow the simulation assumptions agreed in [1], and the description of the detailed evaluation method can be found in Appendix A.

Numerical evaluations

The following points are important for properly evaluating PRACH performance:

Definition of SNR

For fair comparison between schemes, the actual SNR is defined as the ratio of the signal and noise power measured on each time-domain sample, and we define a nominal SNR value as

SNRnominal dB=SNRactual dB+ PAPRmax dB+23- PowerTx, max dB,

which takes into account the power back-off caused by the maximum PAPR, PAPRmax, of the particular PRACH preamble set and the gap between the maximum transmission power, PowerTx, max, and the corresponding upper limit of 23 dBm. Here the maximum transmission power of each scheme is determined by its frequency resource allocation under the power spectrum density (PSD) limitation of no higher than 10 dBm with granularity of 1 MHz. The nominal SNR at 1% mis-detection probability is used to compare performance between preamble schemes. For a given SCS, the same FFT size (i.e., sampling rate) is used for all schemes.

Detection threshold

It is observed that some PRACH scheme may not work well when the detection threshold is calculated based on pure noise input and the 0.1% target false alarm probability. This is due to that RAN1 changed the definition of mis-detection probability to also include the error event of false detection. In the evaluations, the detection threshold needs to be carefully selected for each scheme.

PRACH capacity

For fair comparison, the PRACH capacity is compared under the same overhead. The term PRACH capacity is defined as the maximum number of PRACH preambles that can be generated for the whole system, e.g., using NFDM different frequency locations allowable over the total bandwidth, NTDM different time locations, and different frequency domain sequences. The number of cells that each scheme can support is computed under the assumption of 64 PRACH preambles per cell (the same as LTE and NR). Also for PRB-interlaced schemes with Zadoff-Chu (ZC) sequence, it is possible to use cyclic shifts of the sequence to generate more preambles. In these evaluations, we have numerically verified that the cyclic shifts are dimensioned such that the pairwise cross-correlations between PRACH preambles generated by a common ZC sequence with different cyclic shifts are kept low within the timing detection window.

Maximum Coupling Loss

The maximum coupling loss (MCL) is calculated as

MCL dB=PowerTx,max dBm- SNRactual dB-NoiseFiguredB-NoiseFloordBm

where SNRactual is measured at 1% mis-detection probability, NoiseFigure=5 dB and NoiseFloor=-174+10log1020×106=-100.99 dBm.

Preamble Sequence

We evaluate different sequences:

- The NR length-139 ZC sequences (marked with yellow color in Tables 1-3 and B1-B3).

- ZC sequences of other lengths (marked with blue color in Tables 1-3 and B1-B3).

- Generic sequences proposed in [5] (marked with green color in Tables 1-3 and B1-B3).

Resource Allocation

For the PRB-interlaced schemes (Alt 1-2), the frequency resource allocation has to be judiciously chosen. In particular, the selected frequency resources determine the auto-correlation function (e.g., the zero-autocorrelation zone, ZAZ), the cross-correlation between the preambles and the PRACH capacity.

15 kHz SCS

For 15 kHz SCS, Table B-1 in Appendix B lists all the candidate PRACH schemes and their resource allocations. The performance is shown in Figure C-2 of Appendix C and summarized in Table 1 below. The difference between Alt2-ZC241A and Alt2-ZC241B is the selection of the non-uniform interlaces.

From Table 1, the following observations are made:

Observation 1: A PRB/RE interlaced PRACH (Alt 1-Alt 3) has lower mis-detection probability and smaller timing error than a PRACH with a contiguous frequency allocation (Alt 4) and length-139 ZC sequence.

Observation 2: The PRACH capacity is sufficient (i.e., a preamble reuse factor in the order of 100s) for the PRB interlaced schemes (Alt 1 and Alt 2).

Observation 3: Under PRB level interlaced mapping (Alt 1 and Alt 2), the generic sequence of [5, R1-1901530] has lower PAPR and mis-detection probability than the ZC sequence.

Observation 4: The NR length-139 ZC sequence has bad mis-detection performance under PRB level interlaced mapping (Alt 1) due to the fact that its low PAPR/correlation properties are destroyed, and it has large timing error as well as lower maximum transmission power under contiguous mapping (Alt 4) with/without repetition due to the narrow occupied bandwidth.

Observation 5: When the PRACH sequence is allocated contiguously in the frequency domain (Alt 4), a single long ZC sequence outperforms a short ZC sequence with/without repetition in a number of aspects including PAPR, mis-detection probability, timing error, PRACH capacity and MCL.

Table 1. Performance Comparison of different PRACH schemes with 15 kHz SCS.

| Scheme | Max. PAPR (dB) | Signal BW (MHz) | Max. Tx Power (dBm) | performance at 1% mis-detection probability |  |  |  | PRACH capacity | MCL (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Actual SNR (dB) | Nominal SNR (dB) | False Alarm | Timing Error |  |  |
| Alt1-ZC139 | 6.76 | 16.575 | 21.89 | -6.41 | 1.46 | <0.1% | Small | 3312 (48 cells) | 124.29 |
| Alt1-GS23x23 | 6.04 | 16.38 | 21.79 | -12.06 | -4.81 | <0.1% | Small | 12144 (184 cells) | 129.84 |
| Alt1-ZC641 | 7.22 | 18.90 | 22.47 | -10.46 | -2.71 | <0.1% | Small | 15360 (212 cells) | 128.92 |
| Alt2-ZC241A | 7.90 | 16.92 | 20 | -10.26 | 0.64 | <0.1% | Small | 7200 (105 cells) | 126.25 |
| Alt2-ZC241B | 8.28 | 17.73 | 21.25 | -8.33 | 1.70 | <0.1% | Small | 7200 (105 cells) | 125.57 |
| Alt2-GS19x19 | 6.49 | 16.83 | 21.60 | -11.98 | -4.09 | <0.1% | Small | 6156 (57 cells) | 129.57 |
| Alt3-ZC139 | 5.08 | 18.645 | 22.40 | -9.04 | -3.36 | <0.1% | Small | 3726 (54 cells) | 127.43 |
| Alt4-ZC139 | 3.10 | 2.085 | 13.17 | -10.08 | 2.85 | <0.1% | Large | 18768 (272 cells) | 119.24 |
| Alt4-ZC139x2 | 6.03 | 18.99 | 16.18 | -11.50 | 1.35 | <0.1% | Large | 2346 (34 cells) | 123.67 |
| Alt4-ZC1259 | 2.77 | 18.885 | 22.74 | -14.64 | -11.61 | <0.1% | Small | 27676 (419 cells) | 133.37 |

30 kHz SCS

For 30 kHz SCS, Table B-2 in Appendix B lists all the candidate PRACH schemes and their resource allocations. Their performance is compared in Figure C-3 of Appendix C and summarized in Table 2 below. It is noted that for 30 kHz SCS, it is impossible to design a scheme using uniform interlace based frequency resource allocation (i.e., Alt 1) that achieves satisfactory performance. This is because a uniform interlace based frequency resource allocation always generates insufficient ZAZ in this case, which in turn leads to high auto-/cross-correlation side-lobes within the required 2.02 μs timing detection window. Basically, Observations 1-5 made for 15 kHz SCS are also applicable here. In addition, we have the following observation.

Observation 6: If a Zero Auto-correlation Zone larger than the receiver timing detection window is not possible with uniform frequency resource mapping, it is beneficial to consider non-uniform frequency resource mapping, e.g.,

- Allocation of multiple PRB interlaces

- Allocation of a subset of PRBs from a set of PRBs obtained from multiple PRB interlaces

- Non-uniform sequence-to-subcarrier mapping within a PRB

Table 2. Performance Comparison of different PRACH schemes with 30 kHz SCS.

| Scheme | Max. PAPR (dB) | Signal BW (MHz) | Max. Tx Power (dBm) | performance at 1% mis-detection probability |  |  |  | PRACH capacity | MCL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Actual SNR (dB) | Nominal SNR (dB) | False Alarm | Timing Error |  |  |
| Alt2-ZC139 | 9.30 | 16.41 | 18.01 | -4.87 | 9.42 | <0.1% | Small | 7728 (104 cells) | 118.87 |
| Alt2-GS12x12 | 6.76 | 16.68 | 20.79 | -7.80 | 1.17 | <0.1% | Small | 4992 (52 cells) | 124.58 |
| Alt2-ZC241A | 7.60 | 17.28 | 20.38 | -11.41 | -1.19 | <0.1% | Small | 5760 (84 cells) | 127.78 |
| Alt2-ZC241B | 8.63 | 17.46 | 21.25 | -6.68 | 3.70 | <0.1% | Small | 5760 (84 cells) | 123.92 |
| Alt2-GS17x17 | 6.36 | 18.00 | 22.30 | -11.42 | -4.36 | <0.1% | Small | 4352 (68 cells) | 129.71 |
| Alt3-ZC139 | 5.08 | 16.59 | 21.89 | -7.24 | -1.05 | <0.1% | Small | 3312 (48 cells) | 125.12 |
| Alt4-ZC139 | 3.10 | 4.17 | 16.12 | -10.48 | -0.50 | <0.1% | Large | 13248 (184 cells) | 122.59 |
| Alt4-ZC139x2 | 6.11 | 18.18 | 19.13 | -11.75 | -1.77 | <0.1% | Large | 3312 (46 cells) | 126.87 |
| Alt4-ZC607 | 3.04 | 18.21 | 22.52 | -11.52 | -8.00 | <0.1% | Small | 15756 (242 cells) | 130.03 |

60 kHz SCS

For 60 kHz SCS, Table B-3 in Appendix B lists all the candidate PRACH schemes and their resource allocations. The performance is compared in Figure C-4 of Appendix C and summarized in Table 3 below. Similar to those for 30 kHz SCS, uniform interlace based frequency resource allocation (i.e., Alt 1) is infeasible here. A PRACH with 60 kHz SCS is anticipated to be useful for low-latency access and we have not observed any particular technical prohibitive issue which makes the PRACH preamble performance problematic. A contiguous allocation, as used in NR, would obviously also work. Whether 60 kHz SCS PRACH could be optional or not seems to be a future discussion.

Observation 7: The performance evaluations show that there is no issue in supporting a PRACH preamble with 60 kHz SCS.

Table 3. Performance Comparison of different PRACH schemes with 60 kHz SCS.

| Scheme | Max. PAPR (dB) | Signal BW (MHz) | Max. Tx Power (dBm) | performance at 1% mis-detection probability |  |  |  | PRACH capacity | MCL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Actual SNR (dB) | Nominal SNR (dB) | False Alarm | Timing Error |  |  |
| Alt2-ZC97A | 8.14 | 16.56 | 19.03 | -7.96 | 4.15 | <0.1% | Small | 5760 (84 cells) | 122.98 |
| Alt2-ZC97B | 7.42 | 16.20 | 19.41 | -7.02 | 3.99 | <0.1% | Small | 4608 (72 cells) | 122.42 |
| Alt2-ZC97C | 9.55 | 16.50 | 19.41 | -7.02 | 6.12 | <0.1% | Small | 3456 (48 cells) | 122.42 |
| Alt3-ZC139 | 5.08 | 16.62 | 21.89 | -4.40 | 1.79 | <0.1% | Small | 3312 (48 cells) | 122.28 |
| Alt4-ZC139 | 3.98 | 8.34 | 19.13 | -8.97 | -1.12 | <0.1% | Medium | 7728 (104 cells) | 124.09 |
| Alt4-ZC139x2 | 6.97 | 16.92 | 22.14 | -8.96 | -1.13 | <0.1% | Small | 3864 (52 cells) | 127.09 |
| Alt4-ZC283 | 3.45 | 16.98 | 22.21 | -7.97 | -3.73 | <0.1% | Small | 7896 (112 cells) | 126.17 |

Discussion of the results

The results show that when the frequency resources are allocated properly for PRB-interlaced schemes, there is no issue with the detection performance. Considering the maximum transmission power and PAPR, there can even be a gain over a preamble using a contiguous frequency resource. The main drawback of a PRB-interlaced preamble with ZC sequence is the higher PAPR. However, this is can be compensated for by achieving higher transmit power than a contiguous allocation and there is a net gain in performance. In terms of PRACH capacity, under the same overhead assumption, the numbers are comparable between the schemes (i.e., preamble reuse factor of up to several hundreds) and are sufficient. A single long ZC sequence mapped on a set of contiguous subcarriers (Alt 4) will obviously achieve the best performance provide much higher PRACH capacity and is preferable over repeating several shorter ZC sequences. However, these schemes can either not FDM PRACH with PUSCH/PUCCH or can only do so with complicated puncturing/rate matching (which is also the case for Alt 3). Since we have proven here that the performance of a well-designed PRB-interlaced preamble is sufficient, it should be adopted.

Proposal 7. The PRACH preamble in NR-Unlicensed is based on a PRB interlaced structure.

- FFS: Whether/how to map the preamble sequence to a subset of the REs within each PRB.

For 15 kHz, SCS it is possible to use a uniform PRB interlace, while it does not work well for 30 and 60 kHz due to the sidelobes appearing in the timing detection window. Several methods for suppressing the sidelobes are possible by means of irregular frequency resource allocation.

Proposal 8: It is beneficial to have an irregular frequency resource allocation for PRACH and the following methods will be further studied:

- Allocation of multiple PRB interlaces

- Allocation of a subset of PRBs from a set of PRBs obtained from multiple PRB interlaces

- Non-uniform sequence-to-subcarrier mapping within a PRB

Based on the evaluations, it is concluded that even if a ZC sequence is adopted, there is no advantage of constraining the length to be 139, especially if it is applied to Alt 1-Alt 2. The good properties that a ZC sequence achieves for a contiguous mapping (Alt 4) are not maintained for Alt 1 and Alt 2, and it is essentially a completely new waveform, with an associated new receiver. The sequence length is tightly coupled with the frequency resource allocated for the preamble which directly impacts the cross-correlation among preambles as well as the PRACH capacity. There is therefore no technical justification to arbitrarily lock the sequence length (e.g., to 139) before a good resource allocation has been found.

There is also no benefit of repeating a length-139 sequence (e.g., to fulfill OCB requirements) under Alt 4, instead a single long ZC sequence is better in all aspects (except that it complicates/prohibits FDM of PRACH and other channels). However, the results show that with PRB-interlacing, a ZC is inferior to the generic sequence presented in [5], which should be considered further.

Proposal 9: The preamble sequence in Proposal 1 of R1-1901530 is adopted.

- The sequence length is not constrained to be 139.

PRACH formats

The NR PRACH formats were agreed after considerable work and we see no reason to change the basic preamble structure, i.e., a CP followed by repetitions of the sequence (without CP in between) and potentially a Guard Time (GT) at the end. Therefore NR-U PRACH formats based on NR PRACH formats A and B should be the default. In particular is there no reason to change the preamble structure in order to increase the PRACH capacity, since there are methods for achieving several orders of magnitude increase under existing PRACH formats [6]. Introduction of a time period for LBT prior to the preamble in the RACH Occasions (ROs) may need more discussion.

Multiplexing between PRACH and PUSCH/PUCCH

PRACH transmission may be blocked by other UEs’ PUSCH/PUCCH transmissions if the UEs are sufficiently close such that they interfere with each other and the timing advance is larger than the RX-to-TX switch time. The agreed 300 m Inter Site Distance (ISD) assumed in the evaluations corresponds to a Round Trip Time (RTT) of 1.2 μs, which is smaller than the CP length for all SCSs and the expected switch time. Hence, the issue will not be severe and the gNB could handle it with implementation specific means.

Conclusions

In this contribution, we have made the following proposals:

For DRS:

Proposal 1: When NR-U DRS including RMSI/CSI-RS, multiplexing RMSI /CSI-RS with SSB(s) in FDM manner should be supported to meet the OCB requirement.

Proposal 2: When NR-U DRS only consists of SSB(s), duplicated SSB(s) transmission in frequency domain could be supported to fulfill the OCB requirement.

Proposal 3: NR-U should support different SSB patterns depending on whether RMSI/OSI/paging are included in NR-U DRS.

- In NR-U LAA mode, an SSB pattern with 3 consecutive SSB locations per slot as shown in Figure 1 should be supported.

- In DC or SA mode, SSB pattern with 2 non-consecutive SSB locations per slot should be supported. Two sets of SSB candidate positions within a slot as shown in Figure 2 can be considered.

  - Alt 1: symbol (2, 3, 4, 5) and symbol (9, 10, 11, 12)

  - Alt 2: symbol (3, 4, 5, 6) and symbol (10, 11, 12, 13)

Proposal 4: NR-U should support monitoring of Type-0 PDCCH of the 2nd SSB position in a slot start at symbol#7.

Proposal 5: The corresponding Type-0 PDCCH associated with an SSB shall be jointly transmitted within the same slot.

Proposal 6: PBCH within SSB can be used to indicate the inclusion of RMSI, OSI and paging in DRS.

For PRACH:

Proposal 7. The PRACH preamble in NR-Unlicensed is based on a PRB interlaced structure.

- FFS: Whether/how to map the preamble sequence to a subset of the REs within each PRB.

Proposal 8: It is beneficial to have an irregular frequency resource allocation for PRACH and the following methods will be further studied:

- Allocation of multiple PRB interlaces

- Allocation of a subset of PRBs from a set of PRBs obtained from multiple PRB interlaces

- Non-uniform sequence-to-subcarrier mapping within a PRB

Proposal 9: The preamble sequence in Proposal 1 of R1-1901530 is adopted.

- The sequence length is not constrained to be 139.

References

Chairman’s Notes RAN1 AdHoc Meeting, Taipei, Jan. 21-25, 2019.

Chairman’s Notes RAN1 Meeting #95, Spokane, USA, Nov. 12-16, 2018.

TR 38.889 v16.0.0 Study on NR-based access to unlicensed spectrum.

ETSI EN 301 893 V2.0.7 (2016-11) 5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU

Huawei, HiSilicon, “Generic design of spectral-domain modulation sequences for 5G NR-U PRACH waveforms,” R1-1901530, Athens, Greece, Feb. 25 - Mar. 1, 2019.

Huawei, HiSilicon, “Capacity shortfall solution for agreed NR PRACH formats”, R1-1719378, Reno, USA, Nov. 27 – Dec. 1, 2017.

- Appendix A: Simulation setting and evaluation method for PRACH in NR-U

We follow the simulation assumptions agreed in [1] with 100 ns delay scaling for the TDL-C channel and PRACH format A1 for all the candidate PRACH schemes. In addition, we assume 30 kHz SCS for the other channels (e.g., PUSCH/PUCCH) in the same system and set the maximum timing estimation error to be 50% of the corresponding normal CP length (i.e., 1.17μs). For the candidate PRACH SCS value of 15/30/60 kHz, the corresponding FFT size is set at 2048/1024/512 so as to keep the same sampling rate in the system. During the evaluation of each PRACH scheme, the UE randomly selects one preamble from a set of 64 pre-generated preambles and transmits it into the TDL-C channel with an initial timing offset uniformly distributed in [0, 1.15] μs corresponding to the maximum round-trip time delay with 300 meters inter site distance (ISD). This, plus the 0.87 μs maximum channel delay of the TDL-C channel with 100 ns delay scaling, implies that a timing detection window of 2.02 μs should be assumed at the receiver for all the PRACH schemes considered.

At the receiver, different signals ra(n) are received on different antenna ports/polarized directions a. Each received signal ra(n) of length N is correlated with a bank of filters that correspond to all the 64 candidate preambles skn (k=1, 2, …, 64) of the same length N. Namely, one gets the output of the circular cross-correlation from each correlator as

za,kl=n=0N-1ra(n)sk*(n+lN).

The output signals {za,kl} from different antenna ports/polarized directions are then non-coherently combined to form a power delay profile (PDP)

PDPkl=aza,kl2

for each time delay l and candidate preamble k. This PDP is then compared to a threshold for all l∈{0, 1,…,LW-1}, where  LW is the timing detection window length in samples. The receiver declares that a preamble k is detected if PDPkl is larger than the threshold for some l in the timing detection window. For each detected preamble k, a timing estimation is also obtained as

τ=argmaxl∈{0, 1,…,LW-1}PDPkl .

- Appendix B: Candidate PRACH schemes for NR-U

Tables B-1 to B-3 list all the candidate PRACH schemes for NR-U. Their corresponding frequency resource allocation are detailed in the tables. For those based on ZC sequence, their ZC sequence lengths LZC are given in the tables, and their cyclic shift gaps NCS are numerically optimized to achieve low pairwise cross-correlation within the timing detection window of 2.02 μs. The cyclically shifted sequences are then shortened (if necessary) to fit the available frequency resources. For those schemes based on generic sequences, the corresponding description can be found in [5]. The parameter NFDM represents the number of FDMed PRACH preambles assuming the total 20 MHz bandwidth can be adopted for PRACH in the whole cellular system. However, the frequency resource allocated to PRACH in each cell may be only a portion of the 20 MHz such that the other channels (e.g., PUSCH/PUCCH) can be accommodated in parallel. The parameter NTDM represents the number of TDMed PRACH preambles, which is set to be 1, 2 and 4 for 15, 30 and 60 kHz SCS to keep the same time resource for different SCS values. Both NFDM and NTDM will be used to calculate the achievable PRACH capacities of different schemes in Tables 1-3 of Section 3.

Table B-1. PRACH schemes for NR-U with 15 kHz SCS.

| Scheme | Frequency domain sequence generation | Frequency resource allocation |  | NTDM |
| --- | --- | --- | --- | --- |
|  |  | RE allocation | NFDM |  |
| Alt1-ZC139 | ZC sequence with LZC=139 and NCS=46 | 3 REs with indexes {0, 1, 2}, {3, 4, 5}, {6, 7, 8} or {9, 10, 11} in every other PRB | 8 | 1 |
| Alt1-GS23x23 | Generic sequence [5] with δ=48, t=A=23 | 23 REs with indexes {1, 2, …11, 24, 25,…35} or {13, 14,…23, 36, 37, …47} per subband of 48 REs | 2 | 1 |
| Alt1-ZC641 | ZC sequence with LZC=641 and NCS=53 | All the 12 REs of every other PRB | 2 | 1 |
| Alt2-ZC241A | ZC sequence with LZC=241 and NCS=40 | All the 12 REs of 20 PRBs in interlace-0, 3 of 10 interlaces | 5 | 1 |
| Alt2-ZC241B | ZC sequence with LZC=241 and NCS=40 | 6 REs with indexes {0, 1, 2, 3, 4, 5} or {6, 7, 8, 9, 10, 11} in all the 40 PRBs of interlace-0, 3, 5, and 8 of 10 interlaces | 5 | 1 |
| Alt2-GS19x19 | Generic sequence [5] with δ=60, t=A=19 | 19 REs with indexes {0, 1, …, 5, 12, 13, …, 17, 36, 37, …, 42} or {6, 7, …, 11, 18, 19, …, 23, 41, 42, …, 47} per subband of 60 REs | 3 | 1 |
| Alt3-ZC139 | ZC sequence with LZC=139 and NCS=46 | In every 9th RE of the whole bandwidth | 9 | 1 |
| Alt4-ZC139 | ZC sequence with LZC=139 and NCS=8 | 139 contiguous REs | 8 | 1 |
| Alt4-ZC139x2 | ZC sequence with LZC=139 and NCS=8, repeated twice | REs with indexes {3, 4, …141} in the first and last 12 PRBs | 1 | 1 |
| Alt4-ZC1259 | ZC sequence with LZC=1259 and NCS=55 | 1259 contiguous REs | 1 | 1 |

Table B-2. Candidate PRACH schemes for NR-U with 30 kHz SCS.

| Scheme | Frequency domain sequence generation | Frequency resource allocation |  | NTDM |
| --- | --- | --- | --- | --- |
|  |  | RE allocation | NFDM |  |
| Alt2-ZC139 | ZC sequence with LZC=139 and NCS=18 | All the 12 REs of 12 PRB with indexes {0, 2, 5, 10, 15, 20, 22, 25, 30, 35, 40, 45} | 4 | 2 |
| Alt2-GS12x12 | Generic sequence with δ=48, t=A=12 | 12 REs with Indexes {0, 1, 2, 3, 12, 13, 14, 15, 24 25, 26, 27}, {4, 5, 6, 7, 16, 17, 18, 19, 28, 29, 30, 31} or {8, 9, 10, 11, 20, 21, 22, 23, 32, 33, 34, 35} per subband of 48 REs | 4 | 2 |
| Alt2-ZC241A | ZC sequence with LZC=241 and NCS=40 | All the 12 REs of the 20 PRBs in interlace-0 and 2 of 5 interlaces | 2 | 2 |
| Alt2-ZC241B | ZC sequence with LZC=241 and NCS=40 | 6 REs with indexes {0, 1, 2, 3, 4, 5} or {6, 7, 8, 9, 10, 11} in each of the 40 PRBs from interlace-0, 1, 2, 3 of 5 interlaces | 2 | 2 |
| Alt2-GS17x17 | Generic sequence with δ=36, t=A=17 | 17 REs with indexes {0, 1, …, 11, 19, 20, …, 23} or {12, 13, …, 16, 24, 25, …, 35} per subband of 36 REs | 2 | 2 |
| Alt3-ZC139 | ZC sequence with LZC=139 and NCS=46 | 139 uniformly distributed REs with spacing of 4 REs | 4 | 2 |
| Alt4-ZC139 | ZC sequence with LZC=139 and NCS=11 | 139 contiguous REs | 4 | 2 |
| Alt4-ZC139x2 | ZC sequence with LZC=139 and NCS=11 | REs with indexes {3, 4, …141} in the first and last 12 PRBs | 1 | 2 |
| Alt4-ZC607 | ZC sequence with LZC=607 and NCS=46 | 607 contiguous REs | 1 | 2 |

Table B-3. Candidate PRACH schemes for NR-U with 60 kHz SCS.

| Scheme | Frequency domain sequence generation | Frequency resource allocation |  | NTDM |
| --- | --- | --- | --- | --- |
|  |  | RE allocation | NFDM |  |
| Alt2-ZC97A | ZC sequence with LZC=97 and NCS=18 | All the 12 REs of 8 PRBs with indexes {0, 4, 7, 9, 13, 15, 18, 22}, which belong to interlace-0 and 1 of 3 interlaces | 3 | 4 |
| Alt2-ZC97B | ZC sequence with LZC=97 and NCS=24 | 6 REs with indexes {0, 1, 2, 3, 4, 5} or {6, 7, 8, 9, 10, 11} in all the 16 PRBs from interlace-0 and 1 of 3 interlaces | 3 | 4 |
| Alt2-ZC97C | ZC sequence with LZC=97 and NCS=27 | 6 REs randomly selected from each of the 16 PRBs from interlace-0 and 1 of 3 interlaces | 3 | 4 |
| Alt3-ZC139 | ZC sequence with LZC=139 and NCS=46 | 139 uniformly distributed REs with spacing of 2 REs | 2 | 4 |
| Alt4-ZC139 | ZC sequence with LZC=139 and NCS=19 | 139 contiguous REs | 2 | 4 |
| Alt4-ZC139x2 | ZC sequence with LZC=139 and NCS=19 | REs with indexes {3, 4, …141} in the first and last 12 PRBs | 1 | 4 |
| Alt4-ZC283 | ZC sequence with LZC=283 and NCS=40 | 283 contiguous REs | 1 | 4 |

- Appendix C: Performance comparison of different PRACH schemes

The mis-detection probability is defined in the simulation assumptions agreed in [1] as “the ratio between the total number of transmitted preambles that are either not detected, or detected as a different preamble, or detected but with timing error greater than the maximum value (i.e., 50% of normal CP length), and the total number of transmitted preambles within an observation interval.”. According to this definition, a successful detection event is claimed when only the transmitted preamble is detected with timing error smaller than the tolerance, and all the other events should be regarded as mis-detection. These mis-detection events can be classified into the following error cases.

- Error case 1: The transmitted preamble is not detected. Note that in this case, the receiver may detect some ghost preambles that are not transmitted (i.e., false alarm);

- Error case 2: The transmitted preamble is detected together with some ghost preambles (i.e., false alarm);

- Error case 3: Only the transmitted preamble is detected, but its timing error is larger than the tolerance.

Principally, error case 1 is mainly caused by insufficient signal energy collected by the receiver (e.g., at low SNR and/or in channels with frequency offset/multi-path), error case 2 is mainly caused by the high cross-correlation between some preamble pairs, and error case 3 is mainly caused by the high auto-correlation sidelobes at certain non-zero time offset. Hence a good PRACH scheme should maintain low auto-/cross-correlation between the generated preambles. For illustration, we take the legacy NR PRACH scheme (i.e., Alt4-ZC139 for 15 kHz SCS in Table B-1) as an example. Figure C-1 below plots the probabilities of the above three error cases as well as the overall mis-detection probability. It can be seen that the mis-detection performance is dominated by Error case 1 at low SNR and by Error case 2 at high SNR.

Figure C-1. Mis-detection performance of the legacy PRACH scheme with 15 kHz SCS.

Figures C-2 to C-4 show the performance of all the candidate PRACH schemes listed in Tables B-1 to B-3, respectively.

(a) 												(b)

(c)

Figure C-2. Performance comparison of PRACH schemes with 15 kHz SCS.

(a) 												(b)

(c)

Figure C-3. Performance comparison of PRACH schemes with 30 kHz SCS.

(a) 												(b)

(c)

Figure C-4. Performance comparison of PRACH schemes with 60 kHz SCS.

- Appendix D: Related agreements

The following agreements had been achieved in AdHoc meeting [1]:

- UE assumes 30KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers.

- Support configuration by higher layers of 15 KHz or 30 KHz SCS for SS/PBCH block

- Include this agreement in a LS to RAN4 (cc RAN2) for inclusion in specs managed by RAN4

The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties:

- TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed)

- Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot

  - FFS start at symbol #6 of #7 or both

- FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions)

It was agreed in RAN1 #95 meeting that [2]:

- Support of Pattern 1 is recommended for multiplexing of SS/PBCH block(s) and CORESET(s)#0 in NR-U.

  - As one element to facilitate a NR-U DRS design without gaps in the time domain, the CORESET#0 configuration(s) and/or Type0-PDCCH common search space configuration(s) may need enhancements compared to NR Rel-15, such as additional time domain configurations of the common search space.

Note: Pattern 1 is understood as CORESET#0 and SS/PBCH block(s) occur in different time instances, and CORESET#0 bandwidth overlaps with the transmission bandwidth of the SS/PBCH block.

- Adopt the following text proposal to reflect the above

  - “Support of Pattern 1 is recommended for multiplexing of SS/PBCH block(s) and CORESET(s)#0 in NR-U, where Pattern 1 is understood as CORESET#0 and SS/PBCH block occur in different time instances, and CORESET#0 bandwidth overlaps with the transmission bandwidth of the SS/PBCH block.

  - As one element to facilitate a NR-U DRS design without gaps in the time domain, the CORESET#0 configuration(s) and/or Type0-PDCCH common search space configuration(s) may need enhancements compared to NR Rel-15, such as additional time domain configurations of the common search space.”

Note that this case does not occur in LTE or NR licensed, where a contiguously mapped ZC sequence is used, since it exhibits an ideal auto-correlation function.