3GPP TSG RAN WG1 Meeting #96	R1-1901529

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.2.2.5

Source:	Huawei, HiSilicon

Title:	NRU wideband BWP operation

Document for:	Discussion

Introduction

In RAN#82, the WID [1] of NR-based Access to Unlicensed Spectrum was approved. One of the objective is to specify the wideband operation procedure in RAN1 and RAN4.

- RAN1

  - Wide band operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with multiple serving cells, and wideband operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with one serving cell with bandwidth > 20MHz with potential scheduling constraint subject to input from RAN2 and RAN4 on feasibility of operating the wideband carrier when LBT is unsuccessful in one or more LBT subbands within the wideband carrier. For all wide-band operation cases, CCA is performed in units of 20MHz (at least for 5GHz). [RAN1]

- RAN4

  - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 2). [RAN4]

In RAN1 Ad Hoc meeting 1901[2], RAN1 decide to adopt both option 2 and option 3 in DL BWP as following.

Agreement:

- For wideband operation in DL with a single serving cell operation within a carrier with bandwidth larger than 20 MHz

  - Multiple BWPs can be configured, single BWP activated, gNB may transmit PDSCH on parts or whole of single active BWP where CCA is successful at gNB (i.e., option 2 and 3 from previous agreement)

    - FFS: Restrictions on supportable gaps and combinations of gaps between discontinuous blocks where

      - each block spans contiguous (one or) multiple successful LBT sub-bands

      - each gap spans one or multiple contiguous unsuccessful LBT sub-bands

    - FFS: Transmission bandwidth adaptation delay, potentially different delay for e.g., different number of supported gaps, different transmission bandwidths and different positions of the LBT sub-bands where transmissions occur

    - FFS: Limit on the occupied LBT sub-bands due to regulation and coexistence considerations (not intended to imply that regulation and coexistence considerations will not be addressed)

    - FFS: Whether/how to indicate gNB’s transmitted LBT sub-bands

    - FFS: Enhancements to PDCCH/PDSCH configuration/transmission for the parts of BWP where gNB does not transmit due to CCA failure

Send LS to RAN4 to inform above decision with the description that RAN1 requires RAN4’s feedback on the first three FFS parts in addition to what was requested in earlier LSs.

Agreement:

- Operation with multiple active BWPs for a carrier on unlicensed bands is not supported for DL or UL at least in Rel-16 NR-U WI.

  - Inform RAN2 of this decision

Considering multiple active BWP is not supported in R16, there are still 2 options for PUSCH remaining [3] to down select

- For UL operation, the following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz can be considered.

  - Option 1a: Multiple BWPs configured, multiple BWPs activated, transmission of PUSCH on one or more BWPs

  - Option 1b: Multiple BWPs configured, multiple BWPs activated, transmission of PUSCH on single BWP

  - Option 2: Multiple BWPs can be configured, single BWP activated, UE transmits PUSCH on a single BWP if CCA is successful at UE for the whole BWP

  - Option 3: Multiple BWPs can be configured, single BWP activated, UE transmits PUSCH on parts or whole of single BWP where CCA is successful at UE

In this contribution, we will further discuss the benefit to support dynamic transmission bandwidth in PUSCH (option 3). Meanwhile, the indication of transmitted LBT subband(s) and enhancements of PDCCH/PDSCH/PUCCH/PUSCH configuration and transmission are also investigated.

Dynamic transmission bandwidth adaptation in UL

When free of narrow band operation of coexistence systems cannot be guaranteed, subband LBT performed in units of 20MHz is necessary for transmitter to avoid using the subband already occupied. Option 3 can dynamically adapt transmission bandwidth based on LBT results while option 2 only allows transmission when all subband LBT succeed. In RAN1 Ad Hoc meeting 1901, both option 2 and option 3 were supported for PDSCH. gNB may decide how to transmit based on its own capability. In Figure 1, it is observed that system suffers even larger UPT loss in UL when wideband transmission is only allowed if all LBT subbands succeed channel contention. The simulation assumption is attached in appendix.

Figure 1 DL and UL UPT comparison of NRU with and without dynamic transmission bandwidth adaptation when coexists with 20MHz coexisting systems

Proposal 1: When an UL carrier with bandwidth larger than 20MHz is configured, NRU should support transmission on parts or whole carrier bandwidth where CCA is successful.

Enhancements on partial BW transmission and receiving

If option 3 is enabled, a single wideband BWP is activated for a UE within a component carrier, as illustrated in Figure 3. The bandwidth of wideband BWP can be in multiple LBT subband. For example, if the LBT subband is 20MHz in 5GHz band, the wideband BWP can consist of multiple 20MHz. The actual transmission bandwidth is subject to subband LBT success, which will result in dynamic transmission bandwidth within this active wideband BWP.

Figure 3 Option 3 for BWP operation in NR unlicensed carrier

Impacts on PDSCH/PDCCH

Considering the hardware restriction, it usually takes some time for the gNB to schedule and prepare the transmission in advance before LBT. The LBT subband(s) available for DL transmission might be different from those scheduled. Similar as the case in time domain, gNB should also avoid changing the pre-determined TBS at least at the beginning of the gNB’s COT, in order to avoid additional gap between end of LBT and start of transmission. For the rest of COT, gNB has enough time to re-schedule based on the acquired bandwidth. Several candidates can be considered for gNB to handle the mismatching between the scheduling and transmission at beginning of COT, similar as the discussion in time domain for flexible start point.

Alternative 1: gNB could schedule multiple PDSCHs, each of which is restricted within an LBT subband at beginning of COT, e.g. 1st slot/mini-slot, as plotted in Figure 4. If LBT on certain LBT subband fails, gNB just drop the PDSCH on that LBT subband. Each PDSCHs can be scheduled by individual PDCCH on the corresponding LBT subband. According to UE capability in NR R15, these PDSCH(s) should be scheduled to different UE respectively. NR-U gNB could also schedule 2 PDSCHs to the same UE on 2 LBT subbands, considering the latest agreement in multiple TRP/Panel transmission [2] is also applied to NR-U.

Alternative 2: gNB could schedule one PDSCH across multiple LBT subbands assuming all of them are subband(s). As CBG based feedback and retransmission is supported in NR R15, gNB could only retransmit the CBG affected by LBT subband(s) puncturing. Further enhancement can be considered to restrict mapping a CBG within the LBT subband. It could minimize the number of CBG affected due to LBT subband(s) puncturing, as plotted in Figure 4.

Figure 4 PDSCH and PDCCH in single wideband BWP

Proposal 2: The following two alternatives can be considered for PDSCH scheduling and transmission at beginning of a gNB acquired COT.

- Alternative 1: scheduling multiple PDSCHs, each located within a LBT subband. Do not transmit PDSCH(s) on the LBT subband(s) where CCA fails.

- Alternative 2: scheduling PDSCH assuming all LBT subbands are available. Puncturing the REs on the LBT subband(s) where CCA fails.

At beginning of a DL burst (1st slot/mini slot), a PDCCH should be restricted within a LBT subband in order to avoid partial puncturing of a DCI. It is also beneficial for a UE to monitor multiple PDCCH searching spaces in different LBT subbands in case CCA fails on part of LBT subbands of a wideband BWP. It is straightforward to configure a UE with PDCCH search spaces associating with CORESET(s) in different LBT subbands and each CORESET limited within a LBT subband. The number of configured CORESET (at most 3 CORESET in NR R15) might be increased considering the bandwidth of BWP and LBT subband. The number of configured search spaces and blind detection could remain same as NR R15. Self-scheduling (PDCCH and PDSCH in the same LBT subband) is also preferred considering UE cannot decoding PDSCH without PDCCH.  In the remaining of the DL burst, a UE can dynamically reduce PDCCH search spacing monitoring in frequency domain given the knowledge of transmitted LBT subband(s).

Proposal 3: At least at beginning of a DL burst, UE should be configured to monitor PDCCH search spaces associating with CORESETs on different LBT subbands. Each CORESET should be within a LBT subband.

- FFS: Maximum number of configured CORESETs and PDCCH search spaces

- FFS: Dynamic switching of PDCCH search spaces configuration in frequency domain in the COT

Impacts on PUSCH/PUCCH

The mismatch between resource allocation in UL grant and LBT outcome becomes even more severe, as gNB has no idea about the LBT result to be performed by UE later.  Similar as PDSCH, two alternatives are possible.

Alternative 1: gNB schedules multiple PUSCH(s) each located within a LBT subband based on PRB-based interlace design for one LBT subband (e.g. 20MHz). UE does not transmit PUSCH on the LBT subband(s) which fails LBT.

Alternative 2: gNB schedules single PUSCH based on PRB-based interlace design of the wideband BWP (e.g. 80MHz). UE punctures the PRB(s) on the LBT subband(s) which fails LBT. gNB is able to schedule partial interlace corresponding to part of LBT subbands when UE do not have enough data to fill the whole wideband interlace. Further enhancement can be considered to restrict mapping a CBG within the LBT subband.

Alternative 2 has the benefits of higher spectrum utilization (no guard tone) and less HARQ processes consumption compared with alternative 1. The detail interlace design can be referred to our companion tdoc [5]. gNB can identify the UL transmission bandwidth as well as of the start point of PUSCH by detecting the UL DMRS at beginning of slot(s).

Proposal 4: When dynamic transmission bandwidth adaption (option 3) is adopted to transmit PUSCH, gNB schedules single PUSCH based on PRB-based interlace design of the wideband BWP. UE punctures the PRB(s) on the LBT subband(s) which fails LBT.

A PUCCH resource should be restricted within a LBT subband in order to avoid partial puncturing of UCI due to LBT failure. When PUCCH format(s) of PRB-based interlace design is configured, it occupies PRB of a partial wideband interlace. Multiple PUCCH resource can be configured in different LBT subbands. UE can select one of the resource to feedback even if LBT failure occur on part of LBT subbands. Detail can refer to our companion tdoc [6].

Proposal 5: A PUCCH resource should be restricted within a LBT subband.

Contiguous or non-contiguous transmission in frequency domain

In order to minimize the switch delay, retuning of local oscillator and reconfiguration of RF chain (e.g. center frequency and bandwidth of analog filter) should be avoided. In such case, a wideband RF should be activated corresponding to the bandwidth of component carrier. Depending on the RF requirement of inband leakage at transmitter and ACS at receiver (to be discussed in RAN4), baseband (digital) filter(s) adjustment according to the transmission bandwidth as well as PRB nulling could be adopted. We discuss the RF issues in detail in our companion tdoc [7]. According to RAN4 response, it is feasible to specify RF requirement at least for transmissions spanning multiple contiguous LBT sub-bands. If WiFi requirement is adopted (i.e. -20dBr), only baseband processing is required. There is no difference whether the transmission is contiguous or non-contiguous. However, some restriction can still be considered which simplify UE implementation. For example, gNB could still transmit on multiple noncontiguous LBT subands, each contiguous part corresponding to a PDSCH to a different UE as illustrated in Figure 5(a). In UL, UE may transmit on the maximum contiguous portion of the bandwidth where LBT succeeds as illustrated in Figure 5(b).

Figure 5 Contiguous transmission when LBT fails in “internal” LBT sub-band

According to ETSI regulation [8], Load Based Equipment (LBE) may use any combination/grouping of 20 MHz Operating Channels when Type-A multiple carrier channel access mechanism is used. Load Based Equipment that uses a combination/grouping of 20 MHz Operating Channels that is a subset of bonded 40 MHz, 80 MHz or 160 MHz channels, may transmit on any of the 20 MHz Operating Channels when Type-B multi carrier channel access mechanism is used. The channel bonding restriction is only applicable to LBE device use Type-B multi carrier channel access. As long as its configured BWP is a subset of bonded channels, LBE device can transmit on any of the LBT subbands where CCA is successful without any restriction.

Observation 1: There is no restriction on the occupied LBT subbands for LBE device using Type-A multi carrier channel access mechanism

Observation 2: LBE device using Type-B multi carrier channel access mechanism can transmit on any of the LBT subbands where CCA is successful without any restriction as long as its configured BWP is a subset of bonded 40MHz, 80MHz and 160MHz channels as defined in regulation.

Indication of transmitted LBT subband

It is beneficial for the receiver to know about the actual transmission bandwidth. The receiver could adjust its baseband filter to improve ACS performance. If puncturing were adopted, the RE punctured could be excluded from decoding of initial transmission or combination of retransmission. The transmission bandwidth can be either indicated explicitly or implicitly. For example, UE could detect PDCCH DMRS on each LBT subband of the configured wideband BWP assuming gNB would transmit PDCCH on each transmitted LBT subband. The transmission bandwidth indication could also be carried in group common PDCCH. It can reduce times of blind detection and false detection rate. The GC-PDCCH can transmit at beginning of the COT (pre-indication) or transmit in the earliest pre-defined GC-PDCCH monitoring occasion within the COT (post-indication). Some guard tones between adjacent LBT subbands might be reserved at beginning of COT, which facilitate UE to reliably detect indication of actual transmission bandwidth indication.  In UL, gNB can acquire the transmitted bandwidth from UE by detecting front loaded DMRS of PUSCH on each LBT subband.

Proposal 6: gNB indicates transmitted LBT subband(s) in DL in GC-PDCCH at least at beginning of DL burst. gNB acquires transmitted LBT subband(s) in UL by detection of front loaded DMRS of PUSCH(s).

Premature active BWP switching due to Inactivity Timer

In NR-U an LBT-based transmission of DCI in a non-default active BWP may be delayed because the wireless channel is occupied, or the DCI transmission may collide with another transmission from a hidden node on the wireless channel and may not be successfully decoded or even recognized by the UE. In such cases, the inactivity timer may expire before it is reset by the DCI reception which would cause the UE to prematurely decide that the DL BWP is no longer active and switch back to the default unpaired DL/UL BWPs. If such premature switching happens, the UE would stop monitoring the CORESETs in the deactivated DL BWP and would not be expected to provide any requested UCI feedback with resources allocated on the deactivated UL BWP.  Therefore, one or more of the following solutions can be adopted in NR-U:

- Larger maximum timer value: One simple solution is that the gNB configures the UE to have a larger maximum inactivity timer value when operating in an NR-U cell. For example, if a maximum inactivity timer value x is typically used with active BWPs in the licensed band, when the UE is active on a BWP on an unlicensed CC, it is configured with a sufficiently larger maximum timer value y>x to provision for potential DCI latency/loss due to the unlicensed channel occupancy. Although this solution is simple, it becomes less efficient as the channel becomes less occupied.

- UE pauses timer during UL LBT failure: The UE can use same inactivity timer maximum value across different BWPs/unlicensed CCs while dynamically accounting for the amount of time the medium access has been blocked on the respective BWP, at least as sensed by the UE. This may be achieved by the UE pausing its inactivity timer on an active BWP for the time duration over which the uplink LBT fails. The inactivity timer therefore does not count during the blocking time associated with an uplink LBT failure. In this way, a BWP on a busy channel with frequent uplink LBT failure and will be effectively associated with a greater inactivity timer value compared to a BWP on a less busy channel and experiencing less frequent uplink LBT failure. Also, the behavior of the inactivity timer on a clear (mostly unoccupied) BWP falls back naturally to the intended behavior of the inactivity timer on a licensed BWP.

Observation 3: Due to the potential DCI latency/loss of detection in the unlicensed band, switching of the single active BWP to the default BWP may occur due to premature expiry of the associated Inactivity Timer.

Proposal 7: To avoid switching of the active BWP due to premature expiry of the respective Inactivity Timer, the UE should pause the timer when the medium access is blocked, at least as sensed by the UE.

Conclusions

In this contribution, we discussed wideband operation in NR-U mainly from RAN1 perspective. We had following observations and proposals.

Observation 1: There is no restriction on the occupied LBT subbands for LBE device using Type-A multi carrier channel access mechanism

Observation 2: LBE device using Type-B multi carrier channel access mechanism can transmit on any of the LBT subbands where CCA is successful without any restriction as long as its configured BWP is a subset of bonded 40MHz, 80MHz and 160MHz channels as defined in regulation.

Observation 3: Due to the potential DCI latency/loss of detection in the unlicensed band, switching of the single active BWP to the default BWP may occur due to premature expiry of the associated Inactivity Timer.

Proposal 1: When an UL carrier with bandwidth larger than 20MHz is configured, NRU should support transmission on parts or whole carrier bandwidth where CCA is successful.

Proposal 2: The following two alternatives can be considered for PDSCH scheduling and transmission at beginning of a gNB acquired COT.

- Alternative 1: scheduling multiple PDSCHs, each located within a LBT subband. Do not transmit PDSCH(s) on the LBT subband(s) where CCA fails.

- Alternative 2: scheduling PDSCH assuming all LBT subbands are available. Puncturing the REs on the LBT subband(s) where CCA fails.

Proposal 3: At least at beginning of a DL burst, UE should be configured to monitor PDCCH search spaces associating with CORESETs on different LBT subbands. Each CORESET should be within a LBT subband.

- FFS: Maximum number of configured CORESETs and PDCCH search spaces

- FFS: Dynamic switching of PDCCH search spaces configuration in frequency domain in the COT

Proposal 4: When dynamic transmission bandwidth adaption (option 3) is adopted to transmit PUSCH, gNB schedules single PUSCH based on PRB-based interlace design of the wideband BWP. UE punctures the PRB(s) on the LBT subband(s) which fails LBT.

Proposal 5: A PUCCH resource should be restricted within a LBT subband.

Proposal 6: gNB indicates transmitted LBT subband(s) in DL in GC-PDCCH at least at beginning of DL burst. gNB acquires transmitted LBT subband(s) in UL by detection of front loaded DMRS of PUSCH(s).

Proposal 7: To avoid switching of the active BWP due to premature expiry of the respective Inactivity Timer, the UE should pause the timer when the medium access is blocked, at least as sensed by the UE.

- References

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”

Chairman's Notes RAN1 AdHoc 1901

3GPP TR 38.889 V16.0.0 (2018-12)

R4-1816658, “Reply LS on wideband carrier operation for NR-U”, Ericsson

R1-1901524, “UL channels and signals in NR unlicensed band”, Huawei, Hisilicon

R1-1901527, “HARQ enhancement in NR unlicensed”, Huawei, Hisilicon

R1-1903062, “Inband leakage and blocking in wideband operation”, Huawei, Hisilicon

ETSI EN 301 893 V2.1.1 (2017-05)

- Appendix

Table 1 simulation assumption of wideband operation

| Simulation scenario | Indoor Sub-7GHz, 3+3 TRPs with indoor mixed office model (a-b-a = 20-40-20) |
| --- | --- |
| Carrier Frequency | 5GHz |
| Number of users per operator | 5 per gNB per 20MHz |
| Channel bandwidth | NR-U: 80MHz 802.11ac: 20MHz per AP randomly distributed within NR-U’s operating bandwidth |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | DL/UL mixed traffic, FTP3 with packet size of 0.5Mbyts for both DL and UL traffic. |