3GPP TSG RAN WG1 Meeting #96 R1-1901756 Athens, Greece, 25th February – 1st March, 2019 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on CORESET#0 after gNB response for recovery. Source to WG:; ZTE. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-2-15. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In TS38.213, the time offset K for updating the QCL assumption of CORESET#0 after gNB response for recovery is still open. The same value of 28 symbols, agreed for updating spatial relation of PUCCH, should be used. Taking into account that both CSI-RS and SSB can be configured for new candidate beam identification, QCL assumption of CORESET#0 may be determined by CSI-RS, rather than SSB only, when the PRACH is initialized by beam recovery procedure. However, according to the agreements for CORESET#0 in RAN1#95, CORESET#0 only can be associated with an SSB or a CSI-RS, which is QCL with an SSB. Consequently, in this case, the CSI-RS for new beam identification should be QCLed with an SSB. Besides, there is a redundant description of PDSCH reception after transmitting PRACH for recovery, and the UE behaviour for this case has been clearly described in TS 38.214.. Summary of change:; Adding the value of time offset K and a sentence to describe that the CSI-RS for new beam identification should be QCLed with an SSB. Removing a redundant description of “and for corresponding PDSCH reception”.. Consequences if not approved:; Unclear description of timeline of updating the QCL assumption of CORESE#0. The QCL assumption of CORESET#0 may be determined by one CSI-RS, which is NOT QCLed with any SSB.. Clauses affected:; 6. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

### 6	Link recovery procedures

A UE can be provided, for a serving cell, a set  of periodic CSI-RS resource configuration indexes by failureDetectionResources and a set  of periodic CSI-RS resource configuration indexes, a CSI-RS resource of which is QCLed with a SS/PBCH block, and/or SS/PBCH block indexes by candidateBeamRSList for radio link quality measurements on the serving cell. If the UE is not provided failureDetectionResources, the UE determines the set  to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-state for respective CORESETs that the UE uses for monitoring PDCCH and, if there are two RS indexes in a TCI state, the set  includes RS indexes with QCL-TypeD configuration for the corresponding TCI states. The UE expects the set  to include up to two RS indexes. The UE expects single port RS in the set .

<Unchanged part omitted>

The UE may receive by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in Subclause 8.1. For PRACH transmission in slot  and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index  provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot  within a window configured by BeamFailureRecoveryConfig. For PDCCH monitoring in a search space set provided by recoverySearchSpaceId, the UE assumes the same antenna port quasi-collocation parameters as the ones associated with index  until the UE receives by higher layers an activation for a TCI state or any of the parameters TCI-StatesPDCCH-ToAddlist and/or TCI-StatesPDCCH-ToReleaseList. After the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpaceId, the UE continues to monitor PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command for a TCI state or TCI-StatesPDCCH-ToAddlist and/or TCI-StatesPDCCH-ToReleaseList.

After 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where a UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE assumes same antenna port quasi-collocation parameters as the ones associated with index  for PDCCH monitoring in a CORESET with index 0.

<Unchanged part omitted>