3GPP TSG RAN WG1 Meeting #96 			                                                                 	R1-1901871

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.2.2.2

Source:	Charter Communications

Title:	Views on Random Access Enhancements

Document for:	Discussion/Approval

# Introduction

Following the recently concluded study item on the feasibility of using NR in unlicensed spectrum, the NR-U WI is in progress in RAN1 [1]. In TR 38.889, several proposals for enhancements of random access (RA) procedures across message 1/2/3/4 were listed without consensus and were left for finalization when specifications are developed [2]. In this contribution, we discuss the necessity of such enhancements for 4-step RA.

# NR-U 4-Step Random Access Procedure

## Preamble transmission

TR 38.889 contains the following summary and candidates for enhancing preamble (msg1) transmissions:

For potential RACH resource enhancements, the following options have been identified for NR-U, beyond the flexibility already available in Rel-15, but consensus was not achieved. These options may be further considered when specifications are developed:

-	Frequency-domain enhancement: Multiple PRACH resources across multiple LBT sub-bands/carriers for both contention-free and contention-based RA

-	Time-domain enhancements:

-	For connected mode UE, scheduling of PRACH resources via DCI.

-	Triggered PRACH within gNB acquired COT can use a new resource indicated by the DCI

-	For idle mode UE, scheduling of PRACH resources via paging

-	Note: potential inefficiency in network resource due to paging across multiple cells

-	Additional, new RACH resources are used immediately following detection of DRS transmission

-	Multiple PRACH transmissions before Msg2 reception in RAR window for initial access

-	Number of allowed transmissions is pre-defined or indicated, e.g., in RMSI

-	Group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain

In our view, the flexibility for PRACH resources already available in Rel-15 is largely sufficient and any enhancements need to be justified with significant performance benefits. Further comments on the candidates are as follows.

| Candidate | Comments |
| --- | --- |
| Multiple PRACH resources across multiple LBT sub-bands/carriers for both contention-free and contention-based RA | Multiple PRACH resources in frequency already supported in Rel-15, can be achieved by configuration |
| For connected mode UE, scheduling of PRACH resources via DCI | PDCCH order for RA with indication of preamble and RO already supported in DCI Format 1_0, additional indication of new resources would be a minor enhancement |
| For idle mode UE, scheduling of PRACH resources via paging | This would lead to wasteful over-allocation of PRACH resources within all cells of a tracking area. |
| Additional, new RACH resources are used immediately following detection of DRS transmission | If motivation is COT-sharing between DRS and PRACH, this can also be achieved by configuring PRACH resources within or following DRS transmission window |
| Multiple PRACH transmissions before Msg2 reception in RAR window for initial access | Unnecessary channel occupancy, UE energy expenditure, and collisions with PRACH transmissions from other UEs |
| Group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain | Already supported to an extent in Rel-15, per our understanding. |

Observation 1: The flexibility for PRACH resources already available in Rel-15 is largely sufficient and performance benefits of candidate enhancements in TR 38.889 are yet to be justified.

## Msg 2/3/4

TR 38.889 states the following with respect to msg 2/3/4 enhancements:

“Other candidate mechanisms that were identified without reaching consensus include preconfigured/pre-indicated/scheduled multiple opportunities in time and/or frequency domain in different LBT sub-bands for message 2/3/4 transmissions and/or reducing the latency of the RACH procedure.”

Msg 2 and msg 4 transmissions may be multiplexed with DRS, with unicast PDSCH, or sent in a standalone manner, and channel access methods for initiating a DL COT for the first two approaches have been defined in the TR. Instead of repeating msg 2 and msg 4 transmissions, it is better to define a channel access mechanism other than Cat-4 LBT for the standalone transmission case. Msg 3 transmissions have an associated HARQ process, thus additional measures for reliability are not essential.

Furthermore, the most effective method of reducing the latency of the RACH procedure is to adopt a 2-step RA procedure. Unlicensed spectrum operation is explicitly within the scope of the Rel-16 WI on 2-step RACH [3]:

“For unlicensed operation:

- After PRACH and PUSCH design enhancements are completed for NR-U in the Rel-16 NR-U WI, identify and specify the necessary modification of 2-step RACH design for its application in NR-U(RAN1/RAN2)”

Therefore, it is counter-productive to further optimize 4-step latency since its performance will be lower-bounded by the 2-step method.

Proposal 1:  De-prioritize latency enhancements for 4-step RA due to inclusion of 2-step RA in NR-U.

# Summary

In this contribution we examined the necessity of RA procedure enhancements for NR-U. The following observations and proposals ensued.

Observation 1: The flexibility for PRACH resources already available in Rel-15 is largely sufficient and performance benefits of candidate enhancements in TR 38.889 are yet to be justified.

Proposal 1:  De-prioritize latency enhancements for 4-step RA due to inclusion of 2-step RA in NR-U.

# Reference

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, RAN#82

- TR 38.889 v16.0.0, Study on NR-based access to unlicensed spectrum, (Release 16)

- RP-182894, WID: 2-step RACH for NR