3GPP TSG RAN WG1 #96	R1-1901944

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.4.1.2

Source: 	Fujitsu

Title:	Discussion on HARQ-ACK feedback for NR-V2X

Document for:	Discussion/Decision

Introduction

In NR V2X, HARQ-ACK feedback is beneficial for meeting the reliability requirement and, therefore, has been agreed to be supported for unicast and groupcast. In RAN1#95 and #Ad-Hoc 1901 meetings, some further agreements have been achieved for this feature.

In this contribution we continue our discussion on various aspects of HARQ-ACK feedback procedure for NR V2X, including HARQ-ACK transmitted via NR Uu, HARQ-ACK transmitted via NR sidelink, and some further details are also included in the discussion of these two main topics, respectively.

Discussion

HARQ-ACK transmitted via NR Uu

In RAN1 #95, the following agreement was made with respect to sending SL information to gNB for mode 1:

Agreements:

Study further whether to support UE sending to gNB information which may trigger scheduling retransmission resource in mode 1. FFS including

Which information to send

Which UE to send to gNB

Which channel to use

Which resource to use

And in RAN1 #Ad-Hoc 1901, the following agreement was reached.

Agreements:

- It is supported that in mode 1 for unicast, the in-coverage UE sends an indication to gNB to indicate the need for retransmission

  - At least PUCCH is used to report the information

    - If feasible, RAN1 reuses PUCCH defined in Rel-15

  - The gNB can also schedule re-transmission resource

  - FFS transmitter UE and/or receiver UE

    - If receiver UE, the indication is in the form of HARQ ACK/NAK

    - If transmitter UE, FFS

In this section, we try to further analyze the four FFSs in the agreement of RAN1 #95 one by one and give our proposals respectively. Meanwhile, the agreement of RAN1 #Ad-Hoc 1901 meeting is also taken into account in the discussion.

Which information to send

For Mode 1 where resources for sidelink transmission are scheduled by the base station, SL (sidelink) HARQ-ACK can be transmitted to the base station via NR Uu to facilitate retransmission scheduling. Reporting SL HARQ-ACK to the base station can be seen as a way of requesting retransmission resources from the base station. It is agreed in the last meeting that, if the receiver UE is finally agreed to send an indication to gNB to indicate the need for retransmission for unicast, the aforementioned scheme should be adopted. Another conceivable approach is the transmitter UE directly requests retransmission resource by reporting SR to gNB. However, this scheme may not be timely compared with reporting HARQ-ACK to gNB. This is due to the fact that a UE can directly follow SL HARQ-ACK timing to transmit HARQ-ACK, whereas a UE may have to wait for an available SR occasion to transmit SR.

Moreover, reporting HARQ-ACK to the base station can also facilitate retransmission scheduling. Since the base station has global control over resource allocation of in-coverage UEs, adaptive retransmission can be achieved with less collisions, more efficiency and higher flexibility. However, if the SR reporting scheme is adopted here, base station cannot adaptively schedule a retransmission.

Based on the above analysis, for unicast, we propose that only HARQ-ACK is considered as an indication to gNB to indicate the need for retransmission, regardless of whether the transmitter UE or the receiver UE is finally agreed to be responsible for reporting the indication; for groupcast, we have the same opinion.

Proposal 1: For Mode 1, only HARQ-ACK is considered as an indication to gNB to indicate the need for retransmission, regardless of whether the transmitter UE or the receiver UE is finally agreed to be responsible for reporting the indication.

Which UE to send to gNB

The HARQ-ACK report mechanism should be considered separately for unicast and groupcast due to there are different more concerned performance indicators in these two scenarios. For example, latency and reliability need to be considered more for unicast, but the resource overhead is more concerned in groupcast scenario.

- Unicast

In a unicast transmission, there is a transmitter UE which sends data to a receiver UE on sidelink. If the receiver UE is out-of-coverage or in RRC idle state (the transmitter UE is in-coverage and in RRC connected state, this is natural for Mode 1), there is only one option for this case: SL HARQ-ACK is sent by the transmitter UE to gNB, the reason is that the receiver UE cannot connect with the base station in this case. By the way, the ID of the receiver UE can be reported to gNB by the transmitter UE if the receiver UE is out-of-coverage or in RRC idle state.

If the receiver UE is also in-coverage and in RRC connected state like the transmitter UE, there are two options for this case: SL HARQ-ACK is sent to gNB by the transmitter UE or receiver UE. In our opinion, sending SL HARQ-ACK by the receiver UE is preferable because the receiver UE does not need to send the HARQ-ACK to the transmitter UE firstly, as described by figure 1a and 1b. Then the latency is lower than the source UE sending option.

Proposal 2: For a Mode 1 unicast SL transmission, if the receiver UE is in-coverage and in RRC connected state, SL HARQ-ACK is sent to gNB by the receiver UE; otherwise, SL HARQ-ACK is sent by the transmitter UE.

Figure 1a. HARQ-ACK transmitted to the gNB by receiver UE for unicast

Figure 1b. HARQ-ACK transmitted to the gNB by transmitter UE for unicast

- Groupcast

In groupcast scenario, there is a transmitter UE which sends data to multiple receiver UEs on sidelink. If the SL HARQ-ACK is sent by the receiver UEs to gNB, there are two possible options: same resource or separate resources are used for the multiple receiver UEs to report SL HARQ-ACK. It is difficult to coordinate the HARQ feedback timing and resource between the multiple receiver UEs if the former scheme is adopted; else if the latter one is adopted, the resource overhead may be unacceptable.  On the other hand, it is more desirable to make the transmitter UE take responsibility for reporting SL HARQ-ACK. The details are if the transmitter UE receives at least one “NACK” from one or more of the multiple receiver UEs during the period from the end of the SL transmission to the DCI indicated timing, “NACK” is reported by the transmitter UE to gNB; otherwise, the transmitter UE reports“ACK”  to gNB. Moreover, the base station is required to indicate the feedback timing and resource to the transmitter UE in both unicast and groupcast scenario.

Proposal 3: For a Mode 1 groupcast SL transmission, SL HARQ-ACK is sent to gNB by the transmitter UE.

Figure 2. HARQ-ACK transmitted to the gNB by transmitter UE for groupcast

- Which channel to use

In LTE and NR Uu, HARQ-ACK is conveyed to eNB/gNB by PUCCH and can also be conveyed by PUSCH if there are UL data to be transmitted as well. Thus, the most direct way is to reuse this scheme for reporting SL HARQ-ACK via NR Uu. It is agreed in the Ad-Hoc meeting that at least PUCCH is used to indicate gNB the need for retransmission. Some details may need to be further considered, for example, whether/how the Uu feedback information and SL HARQ-ACK can be multiplexed in a same PUCCH when a UE needs to transmit feedback on Uu and SL simultaneously, impact on the UCI drop mechanism when the UCI of SL is multiplexed with the UCI of Uu in a same PUCCH, etc.

Proposal 4: Besides PUCCH, SL HARQ-ACK can also be conveyed by PUSCH if there are UL data to be transmitted as well.

- Some details may need to be further considered.

- Which resource to use

In order not to increase the amount of work on specification, PUCCH resource indication and resource set selection mechanism in Rel-15 can be regarded as a starting point here. When SL HARQ-ACK is sent by transmitter UE, the PUCCH resource for reporting SL HARQ-ACK can also be directly indicated by the SL scheduling DCI; when SL HARQ-ACK is sent by receiver UE, the indication of the PUCCH resource for reporting SL HARQ-ACK needs to be forwarded by the transmitter UE to the receiver UE, and PSCCH can be used for conveying this indication. Some fields which are used to indicate the PUCCH resource for SL HARQ-ACK feedback should be included in the corresponding format of DCI which is used for NR V2X Mode 1’s scheduling.

Proposal 5: PUCCH resource indication and resource set selection mechanism in Rel-15 can be regarded as a starting point for reporting SL HARQ-ACK.

- When SL HARQ-ACK is sent by transmitter UE, the PUCCH resource for reporting SL HARQ-ACK is indicated by the SL scheduling DCI.

- When SL HARQ-ACK is sent by receiver UE, the indication of the PUCCH resource for reporting SL HARQ-ACK is indicated by the SL scheduling DCI and needs to be forwarded by the transmitter UE to the receiver UE.

As mentioned before, when a UE needs to transmit SL HARQ-ACK to gNB, it may also have some feedback information to be transmitted for Uu simultaneously, such as HARQ-ACK and/or CSI. Then the issue is whether the PUCCH resource for SL is independent of the PUCCH resource for Uu. In our opinion, for an NR V2X service, if the requirement for reliability is high, or on Uu there is a URLLC type of service, a more beneficial approach is to separate the PUCCH resource for SL from the resource for Uu; otherwise, SL HARQ-ACK can be multiplexed with the Uu feedback information in a same PUCCH resource.

Proposal 6: When transmitting SL feedback information to gNB, the following can be considered.

- The SL feedback information is transmitted on a PUCCH resource determined independently from the resource for Uu feedback information.

- The SL feedback information is multiplexed with the Uu feedback information in a same PUCCH resource.

HARQ-ACK transmitted via NR sidelink

In this section, we share some further considerations on HARQ-ACK transmitted via NR sidelink.

In NR V2X Mode 2, UEs can be out-of-coverage and thus HARQ-ACK will be transmitted to the source UE. It has been agreed to define a new physical channel PSFCH to carry HARQ-ACK. It can be feasible to also use other physical channels to convey HARQ-ACK. At least HARQ-ACK piggyback on PSSCH can be supported. Then once HARQ-ACK and data need to be transmitted to a same UE in a slot, both of them can be transmitted through PSSCH.

Proposal 7: As for conveying HARQ-ACK via physical channels other than PSFCH, at least HARQ-ACK piggyback on PSSCH can be supported.

In the last meeting, long-term measurement has been agreed and it is for further studied whether feedback of this measurement is needed. Also it has not been determined whether to support CSI feedback in NR V2X. In our view, CSI feedback could be useful at least for unicast where two UEs are not moving too fast. If CSI feedback is to be supported, CSI piggyback on PSSCH can also be supported.

In RAN1#AH1901 meeting, the following agreements have been agreed for determining the HARQ-ACK resources.

Agreements:

For determining the resource of PSFCH containing HARQ feedback, support that the time gap between PSSCH and the associated PSFCH is not signaled via PSCCH at least for modes 2(a)(c)(d) (if respectively supported)

FFS whether or not to additionally support other mechanism(s) for modes 2(a)(c)(d)

FFS for mode 1

According to the agreements, the fixed HARQ timing is at least supported for Mode 2. In addition to HARQ-ACK timing, the determination of PSFCH resources also includes the frequency domain. For HARQ-ACK transmission via sidelink, it can be also considered to allow a UE to autonomously select HARQ-ACK resources. In NR V2X Mode 2, a major characteristic is that a UE autonomously selects resources for sidelink transmission. More specifically, when a UE has PSCCH and/or PSSCH to transmit, sensing including SCI decoding and/or sidelink measurements can be used to avoid colliding and/or interfering with other concurrent sidelink transmissions. To achieve a similar benefit on collision/interference avoidance, if a UE needs to transmit HARQ-ACK, it can also be possible for the UE to autonomously select resources for HARQ-ACK transmission. When a HARQ-ACK-transmitting UE autonomously determines HARQ-ACK resources, the corresponding HARQ-ACK-receiving UE needs to blind detect HARQ-ACK bits.

In LTE V2X, the autonomous resource selection for PSCCH/PSSCH transmission is only based on the sensing results of the transmitting UE even if the interference environments of the transmitting and receiving UEs could be different. This is because the transmission is broadcast and thus it is difficult or unnecessary to take the surrounding environment of each receiving UE into account. However, HARQ-ACK feedback in NR V2X is a transmission towards one UE and thus it is basically in a manner of unicast. In this case, the LTE-V2X resource selection can be improved by also taking the receiving UE into account. It can be beneficial if the resource selection is based on the sensing results of both transmitting and receiving UEs. For example, the data-transmitting UE can indicate some preferred resources to the data-receiving UE. This will provide the data-receiving UE a freedom to select HARQ-ACK resources by taking the indicated resources into consideration.

Proposal 8: It can be considered to let a UE autonomously select resources to transmit HARQ-ACK.

When conveying HARQ-ACK via PSFCH, whether to also convey source ID and/or destination ID in PSFCH can be further considered. If a UE can have multiple unicast sessions, the UE may transmit HARQ-ACK to or receive HARQ-ACK from multiple UEs. In this case, a UE may need to be able to identify which HARQ-ACK is transmitted to himself and which UE the received HARQ-ACK comes from. The ability to distinguish HARQ-ACK is required especially when overlapped HARQ-ACK resources are scheduled due to independent HARQ scheduling of different unicast UE-pairs. In the last meeting, it has been agreed to include Layer-1 destination ID and Layer-1 source ID in SCI. For HARQ-ACK transmission, it can also be considered to associate source and/or destination ID with HARQ-ACK. How to convey these IDs via PSFCH may be related to the SFCI format design and thus can be further studied.

Proposal 9: It can be considered to support conveying source ID and/or destination ID in PSFCH.

Figure 3 Examples of potential power control issues

In the last meeting, it has been agreed to support open-loop power control which can be based on the pathloss of both Uu link and sidelink. For unicast, the target of power control is to improve efficiency and avoid excessive interference to others. However, when a UE has multiple unicast sessions with multiple UEs, some HARQ related issues may arise due to power control. Two examples are shown in Figure 3. In the left figure, a UE needs to transmit PSSCH and PSFCH towards two different UEs. Since these two UEs may have different distances with the transmitting UE, power control may yield different transmission power for PSSCH and PSFCH. This will require symbol-level power adjustments within a slot. Considering the impact on implementations, such fast power control should be avoided e.g. by following some predefined power allocation rules. As shown in the right figure, since PSSCH scheduling and HARQ timing of different UE pairs can be independently determined, a UE may need to transmit more than one PSFCH at the same time. In this case, how to allocate power to these PSFCHs needs to be specified.

Proposal 10: For power control, power allocation rules should be specified when a UE has PSFCH and/or PSSCH transmissions towards different UEs within a slot.

CBG-based (re)transmission and CBG-based HARQ-ACK have been introduced in NR Rel-15. One of the motivations is to support preemption where URLLC services can override eMBB services. For NR V2X, services with different requirements need to be supported and thus preemption can still be useful. To support preemption, CBG-based (re)transmission and CBG-based HARQ-ACK can also be supported in NR V2X. Additionally, in the complex interference environment of sidelink, channel conditions can vary a lot even within one slot. For example, since the duration of PSFCH may be smaller than that of a slot, PSFCH can have an impact on AGC accuracy and make only partial code blocks within a TB not received successfully. More details can be found in our companion contribution [1]. When only a partial TB fails due to e.g. the impact of PSFCH, CBG-based (re)transmission and CBG-based HARQ-ACK can also be used to save retransmission overhead.

In NR V2X, transmission will occur in a certain resource pool. It could be possible that there is no preemption within a resource pool or there is no aforementioned channel condition fluctuation within a resource pool. For such a resource pool, CBG-based (re)transmission and CBG-based HARQ-ACK may not need to be configured. Therefore, it can be considered to configure CBG-based (re)transmission and CBG-based HARQ-ACK on a per resource pool basis.

Proposal 11: It can be considered to introduce CBG-based (re)transmission and CBG-based HARQ-ACK.

Conclusions

In this contribution, we share some views on supporting HARQ-ACK feedback for NR V2X. Some design aspects are highlighted. The proposals are summarized as follows.

Proposal 1: For Mode 1, only HARQ-ACK is considered as an indication to gNB to indicate the need for retransmission, regardless of whether the transmitter UE or the receiver UE is finally agreed to be responsible for reporting the indication.

Proposal 2: For a Mode 1 unicast SL transmission, if the receiver UE is in-coverage and in RRC connected state, SL HARQ-ACK is sent to gNB by the receiver UE; otherwise, SL HARQ-ACK is sent by the transmitter UE.

Proposal 3: For a Mode 1 groupcast SL transmission, SL HARQ-ACK is sent to gNB by the transmitter UE.

Proposal 4: Besides PUCCH, SL HARQ-ACK can also be conveyed by PUSCH if there are UL data to be transmitted as well.

- Some details may need to be further considered.

Proposal 5: PUCCH resource indication and resource set selection mechanism in Rel-15 can be regarded as a starting point for reporting SL HARQ-ACK.

- When SL HARQ-ACK is sent by transmitter UE, the PUCCH resource for reporting SL HARQ-ACK is indicated by the SL scheduling DCI.

- When SL HARQ-ACK is sent by receiver UE, the indication of the PUCCH resource for reporting SL HARQ-ACK is indicated by the SL scheduling DCI and needs to be forwarded by the transmitter UE to the receiver UE.

Proposal 6: When transmitting SL feedback information to gNB, the following can be considered.

- The SL feedback information is transmitted on a PUCCH resource determined independently from the resource for Uu feedback information.

- The SL feedback information is multiplexed with the Uu feedback information in a same PUCCH resource.

Proposal 7: As for conveying HARQ-ACK via physical channels other than PSFCH, at least HARQ-ACK piggyback on PSSCH can be supported.

Proposal 8: It can be considered to let a UE autonomously select resources to transmit HARQ-ACK.

Proposal 9: It can be considered to support conveying source ID and/or destination ID in PSFCH.

Proposal 10: For power control, power allocation rules should be specified when a UE has PSFCH and/or PSSCH transmissions towards different UEs within a slot.

Proposal 11: It can be considered to introduce CBG-based (re)transmission and CBG-based HARQ-ACK.

- References

[1] R1-1901943, Discussion on sidelink physical layer structures, Fujitsu, RAN1 #96.