3GPP TSG RAN WG1 #96		R1-1901971

Athens, Greece, 25th February – 1st March, 2019

Source:	CATT

Title:	A clarification of NR PRACH resource mapping

Agenda Item:	7.1.1

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss a clarification of NR PRACH resource mapping requirement.

# Discussion

The following requirement for NR PRACH resource mapping is defined in Section 8.1 of TS 38.213 :

“A UE is provided a number  of SS/PBCH blocks associated with one PRACH occasion and a number  of contention based preambles per SS/PBCH block per valid PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB. If , one SS/PBCH block is mapped to  consecutive valid PRACH occasions and  contention based preambles with consecutive indexes associated with SS/PBCH block , , per valid PRACH occasion start from preamble index 0. If ,  contention based preambles with consecutive indexes associated with SS/PBCH block , , per valid PRACH occasion start from preamble index  where  is provided by totalNumberOfRA-Preambles and is an integer multiple of .

In above requirement, the sentence: “A UE is provided a number  of SS/PBCH blocks associated with one PRACH occasion and a number  of contention based preambles per SS/PBCH block per valid PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB”, the SS/PBCH blocks should be understood as a number  of actually transmitted SS/PBCH blocks, excluding other untransmitted SSBs in an SSB burst, although this is not explicitly mentioned.

For the sentence: “If ,  contention based preambles with consecutive indexes associated with SS/PBCH block , , per valid PRACH occasion start from preamble index  where  is provided by totalNumberOfRA-Preambles and is an integer multiple of ”, the requirement does not indicate how the index n for the SS/PBCH block is defined, which could result in different interpretations.

For example, assume Lmax=8, ssb-perRACH-Occasion = 4,CB-PreamblesPerSSB   = 8, 4 ROs in PRACH association period, and SSB0/2/3/4/7 are actually transmitted. There can be at least two interpretations for the mapping of the SSBs to PRACH occasions (ROs).

- Interpretation 1: the index n = (the sequence number of the actually transmitted SSBs in a burst) mod N. With this interpretation, we have n={0, 1, 2, 3, 0}. The mapping of SSB0/2/3/4/7 to the PRACH resource is shown in Figure 1. As shown in the figure, the actually transmitted SSBs are first mapped to RO0 and RO1 with some PRACH resources in RO1 remaining unused, and then the actually transmitted SSBs are mapped to RO2 and RO3 with some PRACH resources in RO3 remaining unused.

- Interpretation 2: the index n = (the circular sequence number of the actually transmitted SSBs in a mapping period) mod N. Then, the mapping of SSB0/2/3/4/7 to the PRACH resource is shown in Figure 2. With this interpretation, the actually transmitted SSBs are mapped to RO0/RO1/RO2/RO3 sequentially. Only RO3 has the remaining PRACH resources.

Figure 1: PRACH resource mapping with n =M mod N, where M is the sequence number of the actually transmitted SSBs.

Figure 2: PRACH resource mapping with n =M mod N, where M is the circular sequence number of the actually transmitted SSBs.

In our understanding, there is no clear superiority of the above two mapping methods, and the second interpretation of PRACH resource mapping in Figure 2 could be the understanding of most companies. Thus, we propose the clarification of the index n for the PRACH resource mapping based on the second interpretation in the companion draft CR.

# Summary

In this contribution, we discussed the potential ambiguity in PRACH resource mapping caused by the lack of a clear definition of the index n in Section 8.1 in TS 38.213. The proposed clarification is included in the companion draft CR.

# References

- 3GPP TS 38.213, V15.4.0, "NR; Physical layer procedures for control"