3GPP TSG RAN WG1 Meeting #96						R1-1901986

Athens, Greece, 25th February – 1st March 2019

Source:	CATT

Title:	UL Power Control for NR DC

Agenda Item:	7.2.13.1

Document for:	Discussion and decision

Introduction

In RAN1 AH#1901, the UL power control for NR DC was discussed with the following agreements,

Agreements:

- For Rel. 16 UEs and asynchronous NN-DC operation, where MCG has serving cells only in FR1 and the SCG has serving cells only in FR2, the uplink power control is performed independently across cell groups

  - This is under the assumption that for NR Rel. 16, no joint power limit across FR1 and FR2 is defined by RAN4.

- RAN1 has not identified any use case to support the case where SCG is fully in FR1 and MCG is fully in FR2 for both synchronous & asynchronous NN-DC operation. At the same time, if supported, RAN1 has not identified other RAN1 specification impact other than the power control aspect listed below and UE capability

  - If supported, power control is performed independently across the two cell groups.

Send an LS to RAN4 (cc RAN2) capturing the above – Kianoush (QC), R1-1901402, which is approved.

In this contribution, we discuss the UL power control for NR DC.

UL Power Control for NR DC

The power control for NR DC is to control the UL total Tx power from MCG and SCG.  Each cell in the cell group would perform the power control independently with the configured maximum power of the cell PCMAX,c.  The UL maximum power PCMAX,c for each cell group would take other cell’s Tx power into account as the MPR/A-MPR.   The UL power control for NR-DC follows NR power control formula of each component carrier based on fractional power control principle.   When the sum of the total UL Tx power from all cell groups exceeds the maximum power of the configured power for the technology, such as P_NR, or UE power class, the Tx power would be scaled.  The Tx power would be scaled only on the portion of the total power exceeding the maximum Tx power with the power scaling rule.

In LTE, the power scaling rule for the dual connectivity is to protect the MCG with the guarantee power and scale other component carriers.   The power scaling rule in LTE also consider the UL subframe alignment among component carriers.   The LTE PCM-1 and PCM-2 power controls are designed for synchronous and asynchronous dual connectivity, respectively.   The PCM-1 power control for synchronous dual connectivity in LTE is to have the UL subframes from all CGs fully aligned.   The power scaling rule for PCM-1 in LTE is to scale the whole subframe of the unprotected/de-prioritized CGs when the total power exceeds the power limit.  For LTE PCM-2, the power scaling applies to the portion of the subframes, which the subframes are overlapped among cell groups.

For NR DC, the power control is to determine the power scaling rule when the UL total Tx power exceeds the configured maximum power.  The UL total TX power for EN-DC would be determined by the alignment of the UL transmission from all cell groups.  The alignment of the UL transmissions among CGs would depend on the slot synchronization, the numerology, and the slot format.   The slot synchronization between MCG and SCG would depend on whether the gNB are synchronized and the cell sizes in the NR DC deployment.  If the gNBs are synchronized and the cell size is small, the UL slots could be aligned between MCG and SCG.    The power could be scaled with the assumption of slot alignment based on the power scaling rule when total power exceeds the maximum Tx power limit for synchronous NR DC deployment.   For asynchronous NR DC deployment, the power control would scale the UL Tx power on the portion of the slots exceeding the maximum configured power.

NR supports different numerologies and slot formats at different carrier when multiple carriers are configured with dual connectivity.  Even the slot is aligned in the synchronous NR DC deployment,  each cell group might have different slot format, which the number of UL symbols within a slot could be different even all component carriers have same numerology as shown in Figure 1.  When multiple CGs are scheduled to transmit PUSCHs, the number of symbols for PUSCH in each CG could be different.   Thus, the total Tx power could vary among symbols within a slot when multiple PUSCH from different CGs are transmitted with same numerology.

Figure 1: MCG and SCG have same numerology but different slot formats in synchronous and asynchronous NR DC

For cell groups with different numerologies, the slot length would be different as shown in Figure 2.  The UL total Tx power could vary when CGs with different numerologies are transmitted in the same time.  Since the Tx power control is independent configured the PCMAX,c for each CC, the UL total Tx power might exceed the total output power of the UE power class in the specific band.  UE has to scale down the  UL Tx power when the total output power over the power class of the specific band.

Figure 2: MCG and SCG have differnt numerologies in both synchronous and asynchronous NR DC

Dynamic power sharing among component carriers are the baseline assumptions with the configured maximum power PCMAX,c for NR based on the overbooking principle.  The maximum Tx power PCMAX,c for each cell from each CG can be individually configured up to the maximum power for the UE power class of the specific band  for dynamic power sharing among carriers in aggregation.    Each component carrier would have the Tx power controlled independently based on the configured component maximum power PCMAX,c if the sum of Tx power from all CGs is not over the total maximum power.  Due to different slot formats and numerologies as shown in Figure 1 and Figure 2, the UE output power might be contributed from Tx power of one CG at some OFDM symbols and more than one CG at other OFDM symbols.  The total power in some OFDM symbols might be over the maximum power of the UE power class due to the sum of Tx power from multiple cells from same or different CGs.  UE needs to scale down the Tx power of those OFDM symbols to meet the requirements and prevent the signal distortion from Tx power amplifier.     Since multiple PUSCH transmissions from MCG and SCG are independently scheduled by two gNBs, the gNB scheduler could not take into account the slot format and numerologies among scheduled PUSCH transmissions in NR DC to avoid the total Tx power exceeding the maximum power of the UE power class over all symbols.    If the UE output power at some OFDM symbols exceeds the maximum power of the UE power class, it would be UE implementation choice of scaling the transmit power with the power scaling rule based on the prioritization of each channel.

In RAN1#91, the power control for NR CA was agreed as follows,

# Agreement

- In Case 1, (CCs/uplinks configured for UE have same numerology and overlapping transmissions between different CCs/uplinks with same starting time and same PUSCH/PUCCH transmission duration and one or two PUCCH group(s)), when the UE is power limited due to simultaneous transmission on multiple serving cells,

  - PRACH of PCell > PUCCH/PUSCH with ACK/NACK and/or SR > PUCCH/PUSCH with other UCIs > PUSCH w/o UCI > SRS/PRACH of SCell

    - Within a same priority level, PCell is prioritized over SCell.

    - In case that transmission power exceeds Pcmax, Scaling/dropping is applied to the lowest priority first until the aggregated power is within Pcmax. Exact scaling or dropping is left to UE implementation.

    - Note: different priority of SRS used for carrier switching can be discussed further.

# Working  Assumption

- In Case 2, (CCs/uplinks configured for UE have same or different numerologies and partially overlapping transmissions between different CCs/uplinks and same/different transmission duration and one or two PUCCH group(s)), when the UE is power limited due to simultaneous transmission on multiple serving CCs/uplinks,

  - PRACH of PCell > PUCCH/PUSCH with ACK/NACK and/or SR > PUCCH/PUSCH with other UCIs > PUSCH w/o UCI > SRS/PRACH of Scell

    - Within a same priority level, PCell is prioritized over Scell

    - In case that transmission power exceeds Pcmax, Scaling/dropping is applied to the lowest priority first until the aggregated power is within Pcmax.

    - Note: different priority of SRS used for carrier switching can be discussed further

  - Scaling or dropping of the whole or part(s) of a transmission is left to UE implementation.

- Note: If the aggregated transmission power does not exceed Pc_max within any part of a transmission that overlaps with other transmission(s), the transmission is considered as non-power limited case.

- Note: power control with look-ahead is not required at UE.

If the UE output power at some OFDM symbols exceeds the configured maximum power, it would be UE implementation choice of scaling the UL Tx power on one or more UL channels based on the power scaling rule.   The physical channel priority used for NR CA power scaling should be reused for NR DC

Proposal 1: If the UE output power at some OFDM symbols exceeds the maximum power of the UE power class, it would be UE implementation choice of scaling the UL Tx power on one or more UL channels based on the power scaling rule.  The physical channel priority used for NR CA power scaling should be reused for NR DC

Conclusion

In this contribution, we the power control for NR DC.  The power control of each cell of any CG would be based on the Rel-15 power control of individual cell with configured maximum power of the cell.  The power sharing among cells from different cell groups for NR DC would be based on power overbooking principle.  The maximum Tx power PCMAX,c of each component carrier could be configured at the maximum power of the UE power class of specific band.   The Tx power could be dynamically shared among component carriers to fully utilize the UE power.   We have the following proposal:

- Proposal 1: If the UE output power at some OFDM symbols exceeds the maximum power of the UE power class, it would be UE implementation choice of scaling the UL Tx power on one or more UL channels based on the power scaling rule.   The physical channel priority used for NR CA power scaling should be reused for NR DC

References

- RP-182076, “WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements”, Ericsson, Nokia, NSB, Huawei

- R1-1721548, “Offline summary of UL power control – CA aspects”, Samsung

- R1-1900316,  “UL Power control for NR –DC”, CATT