3GPP TSG RAN WG1 Meeting #96														R1- 1902109

Athens, Greece, February 25th – March 1st, 2019

Agenda item:		7.2.2.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Channel access and co-existence for NR-U operation

Document for:		Discussion and Decision

# 1	Introduction

NR-U study item discussed and studied several aspects related to channel access procedures, including LBT mechanisms, LBT options for different DL and UL channels, receiver assisted LBT, directional LBT, etc. Multiple options and design principles for channel access procedure have agreed and been captured into TR 38.899 [1].

In RAN1 Ad-hoc 201901 [2], the following agreements were made:

Agreement:

- A gap (DLUL, ULUL, or UL DL) of a specific duration is created using one or more of:

  - Timing Advance

  - CP extension

    - max value of not more than one OFDM symbol

  - Shortening of DL or UL transmission duration by one or more OFDM-symbol(s) by puncturing or rate matching

  - Note: the mechanisms applied in each case may be different for different SCSs

- FFS: how to signal the way of creating the gap to the UEs

In this contribution, we discuss further details and remaining issues related to channel access procedure for NR unlicensed band operation.

# 2	Clarifications on Cat 2 LBT

In TR 38.899, Cat 2 LBT is mentioned in many places, for example to support DRS transmission with duty cycle less than 1/20, and for LBT when the gap between two consecutive transmissions is larger than 16 sec but not more than 25 µsec. However, the definition of Cat 2 LBT is not clearly defined.

The Cat 2 LBT derives from a 25 us LBT option defined in LTE-LAA. The 25 us duration consists of a duration Tf=16us immediately followed by one slot duration Tsl=9us and Tfincludes an idle slot duration Tslat start of Tf. The channel can be considered as idle if it is sensed to be idle during the both slot durations of Tsl=9us. Such channel access procedure has been agreed to be reused in NR-U, in order to support DRS transmission alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) when the DRS duty cycle ≤1/20, and the total duration is up to 1 ms.

However, such definition does not directly fit to the case with gap being less than 25 us. The duration of Cat 2 LBT should be in line with the gap duration. We suggest the following definition for Cat 2 LBT:

When Cat 2 LBT applies within a COT, the sensing duration consists of a duration 7us<Tf≤16us immediately followed by one slot duration Tsl=9us and Tf includes an idle slot duration Tsl at start of Tf. The channel can be considered as idle if it is sensed to be idle during the both slot durations of Tsl.

Proposal 1: The duration of Cat 2 LBT should be in line with the gap duration when the Cat 2 LBT is using within a COT.

# 3	Channel access procedure for a shared gNB COT

In SI phase, single and multiple DL to UL and UL to DL switching within a shared gNB channel occupancy time (COT) can be supported in NR-U. Channel access schemes for a DL burst following a UL burst within a gNB-initiated COT and channel access schemes for a UL burst within a gNB-initiated COT have been agreed. There are still two main open issues:

- How is channel access mechanism for the case where the gap between a DL and UL transmission may be larger than 25 us?

To solve the issue, one potential way is to make the gap of two consecutive transmissions always less than 25 us.  As agreed in RAN1 AH201901, a gap (DLUL, ULUL, or UL DL) of a specific duration is created using one or more of “CP extension”, “timing advance”, and “Shortening of DL or UL transmission duration by one or more OFDM-symbol(s)”.

In ETSI regulation, the operation for gap duration of two consecutive transmissions larger than 25 us and less than 100 us is not defined. We see two potential options to handle this case.

- Extending the sensing duration. If the channel is sensed to be continuously idle after the previous transmitting (DL or UL), the next transmission (DL or UL) is allowed within the COT initiated by a gNB having multiple switching points.

- Gap duration control. The gNB should control the gap of two consecutive transmissions not being larger than 25 us and less than 100 us.

For the case of gap duration larger than 100 us, Cat.2 LBT should be allowed to resume the next transmission, as ETSI regulation already allows such behavior.

- Any restriction for the duration of UL burst which is transmitted within a gNB-initiated COT subjects to no-LBT is required?

It is captured in TR 38.889, within a gNB-initiated COT, an UL burst for a UE consisting of one or more of PUSCH, PUCCH, PRACH, and SRS follows the channel access schemes in the following Table.

Table: Channel access schemes for a UL burst within a gNB-initiated COT as LBE device

| Cat 1 Immediate transmission | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| When the gap from the end of the DL transmission to the beginning of the UL burst is not more than 16 sec. Note: Maximum limits of the duration of the UL burst other than those already derived from MCOT duration limits should be further discussed when specifications are developed. | For any of the following cases: - When the gap between any two successive scheduled/granted transmissions in the COT is not greater than 25 sec - For the case where a UL transmission in the gNB initiated COT is not followed by a DL transmission in the same COT - Note: the duration from the start of the first transmission within the channel occupancy until the end of the last transmission in the same channel occupancy shall not exceed 20 ms. | N/A |

“Maximum limits of the duration of the UL burst other than those already derived from MCOT duration limits” is to be discussed in WI phase.

According to ETSI regulatory requirement [3], no-LBT can be used if the gap between initiating device and responding device is less than 16 us. “The Responding Device may proceed with such transmissions without performing a Clear Channel Assessment (CCA) if these transmissions are initiated at most 16 μs after the last transmission by the Initiating Device that issued the grant.” As there is no additional restriction on the duration of transmission from responding device other than MCOT duration limit, we think the duration of a UL burst within a gNB-initiated COT only needs to comply with MCOT limit and further limitations, if any, should be motivated with co-existence results.

If the limiting of the duration of transmissions using Cat 1 LBT is deemed as necessary, we think that Cat 1 framework (Immediate transmission) should support at least transmission of HARQ-ACK at the beginning of a UL burst, i.e. PUCCH or PUSCH transmissions having a duration of up to 1 ms. Furthermore, since FDM of different UL channels is applied, Cat 1 LBT should be applicable to all UL channels in the same way irrespective of the signals they carry. If the maximum limit of the duration is defined for Cat 1, it should be possible for the UE to perform one-shot LBT during the 16 sec gap, and when the channel is found to be free, use UL resources according to MCOT duration limit.

Observation 1: According to ETSI requirements, Cat 1 Immediate transmission does not involve additional restriction on the duration of transmission from responding device other than MCOT duration limit

Proposal 2: Cat 1 framework should support at least HARQ-ACK transmission at the beginning of the UL burst

Proposal 3: If UE performs one-shot LBT during the 16 sec gap, and the channel is found to be free, the UE can use UL resources according to MCOT duration limit.

# 4	Receiver assisted LBT

With the experience of LAA/eLAA channel access design, we find that energy detection based LBT procedure may suffer from the hidden node issue. As shown in Figure 2, the gNB A and the gNB B cannot sense each other due to being located outside of each other’s sensing range. However, the UE A is able to sense both gNBs. Before the gNB A transmits UL grant to the UE A, the gNB senses the channel to be idle with LBT category 4 procedure, although gNB B is transmitting data to UE B. After receiving the UL grant, UE A may sense the channel to be busy, when it performs 25 us one-shot LBT before its UL transmission. Hence, the scheduled UL resources are wasted due to failed UL LBT, which may degrade the UL performance dramatically and result in inefficient spectrum utilization.

Observation 2: Hidden nodes may reduce UL channel access probability in unlicensed spectrum.

Figure 2. Hidden node issue for UL transmission in unlicensed spectrum.

During study item phase, many companies expressed interest introducing in RTS/CTS-like procedure (know from e.g. Wi-Fi) to e.g. avoid issues with hidden nodes. On the other hand, some companies also point out possible issues related to such mechanisms, such as:

- Spatial reuse may be severely impacted by RTS/CTS-like mechanism.

- RTS/CTS type mechanism may not be very suitable for a use with multi-user scheduling, which may result in a long handshake chain.

- How to support RTS/CTS-like signaling in NR slot structure is not clear.

Therefore, it seems further study is needed before concluding on the possible benefits of RTS/CTS for NR-U.

Proposal 4: The benefits of RTS/CTS-like receiver assisted LBT schemes require further discussion and study.

One potential way to minimize the impact of hidden node issue in NR-U is to introduce UE assistance information, e.g., an energy detection report or a CSI-IM report. By evaluating the received measurement report, the gNB may be able to identify the UEs suffering from hidden nodes. As shown in Figure 3, in a gNB acquired COT, the gNB may trigger multiple UEs to perform a CCA operation (e.g., LBT Cat.2) and reports the level of received energy to the gNB. UEs, who fail LBT Cat.2, will not transmit a report to the gNB. Consequently, the gNB implicitly knows which UEs are interfered by hidden node. Furthermore, the gNB can evaluate the received energy detection reports, in order to schedule the UEs with low interference in the following COT.

Figure 3: energy detection report for hidden node issue

Furthermore, above mechanism can be used together with over-booked UL scheduling and two-stage scheduling. As shown in Figure 4, the gNB could schedule UL transmissions for different UEs on the same time-frequency resource. By evaluating the received energy detection reports, the gNB can decide and trigger which UE performs UL transmission on over-booked time-frequency resource.

Figure 4: over-booked UL transmission

By above method, UL channel access probability can be greatly increased, which will improve the UL resource utilization.

Proposal 5: Overbooked UL transmissions and UE reporting of channel sensing results can be considered as ways to increase UL access probability.

# 5	CWS maintenance for Cat.4 LBT

It is agreed in NR-U SI that “for CWS adjustment procedure in NR-U, in addition to aspects considered in LTE LAA, NR-U may additionally consider at least the following aspects: CBG based HARQ-ACK operation, NR scheduling and HARQ-feedback delays and processing times, wideband (>20 MHz) operation including BWPs, Configured grant operation”. Some detailed discussions are given in the following.

Consideration of reference slot

In LTE-LAA, CWS update for DL/UL is based on the ACK/NACK feedbacks of TB(s) in reference subframe(s), which should be defined in NR-U for the same purpose.

In LAA DL, the reference subframe for CWS update is the starting subframe of the most recent transmission on the carrier made by the eNB, for which at least some HARQ-ACK feedback is expected to be available. In LAA UL, the reference subframe is the first subframe or first two subframes (when the first subframe is a partial subframe) of a UL burst before n-3, where n is the UE receives an UL grant or an AUL-DFI.

Although we could follow the same design principle in LAA/eLAA/feLAA as much as possible for reference subframe determination in NR-U, two new aspects need to be considered.

- Slot structure

Flexible starting position is currently being discussed in both DL and UL, in order to increase the transmission opportunity of a DL transmission burst or a UL transmission burst. Furthermore, two potential ways could be used to support multiple starting positions within a slot: 1. Mini-slot (Type B mapping) based approach; 2. Puncturing based approach. In case of mini-slot based approach, there will be more than one ACK/NACK corresponding to multiple TBs in a slot. How to take multiple ACK/NACK into CWS adjustment metric should be considered. In the cased of puncturing based approach, partial TB in the slot may be punctured. Hence, a TB-level NACK can be always expected for such partial slot case. However, this NACK cannot reflect the contention situation, which is original intention of CWS adjustment. Therefore, no matter which way is used/supported, whether or how partial slot could be used as the reference slot for CWS adjustment should be further studied

- Processing time

In NR, the processing time is much smaller than that in LTE, the timing of n-3 should be revisited.

Observation 3: The determination of the reference slot for CWS adjustment should take slot structure (partial slot or full slot) and processing time into consideration.

Consideration of CBGs

In LTE LAA, the DL CWS is updated based on the NACK ratio Z of the HARQ-ACKs for the TBs in a DL reference subframe. In LTE eLAA, the UL CWS is updated based on the ACK/NACK feedback for the TB(s) in the UL reference subframe(s). In NR, separate HARQ operation is possible for the level of CBGs (code block groups) even in one TB. How to define the rule for CWS maintenance for CBG based HARQ-ACK operation should be studied.

To guarantee the consistency of CWS maintenance based on TB-level HARQ-ACK and CBG-level HARQ-ACK, it is beneficial to define a mapping rule from CBG-level HARQ-ACK to TB-level HARQ-ACK. The following alternatives can be considered.

- Alternative 1: If all CBG-level HARQ-ACKs corresponding a TB are ACK(s), TB-level HARQ-ACK feedback for CWS adjustment is counted as ACK, otherwise, NACK.

- Alternative 2: If the first CBG-level HARQ-ACKs corresponding a TB is ACK, TB-level HARQ-ACK feedback for CWS adjustment is counted as ACK, otherwise, NACK. This alternative is based on frequency first CB mapping.

Proposal 6: Both alternatives can be considered in CWS adjustment to support CBG based HARQ-ACK operation:

- Alternative 1: If all CBG-level HARQ-ACKs corresponding a TB are ACK(s), TB-level HARQ-ACK feedback for CWS adjustment is counted as ACK, otherwise, NACK.

- Alternative 2: If the first CBG-level HARQ-ACKs corresponding a TB is ACK, TB-level HARQ-ACK feedback for CWS adjustment is counted as ACK, otherwise, NACK.

Consideration of wideband BWP (> 20 MHz)

In LTE-LAA, the CWSs are maintained separately in each 20 MHz carrier. At least for a band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), it is agreed that NR-U LBT can be performed in units of 20 MHz. The most straightforward solution is that the CWS maintenance is also done separately in each 20 MHz sub-band.

In a wideband BWP, there may be TBs across multiple sub-bands. HARQ-ACK feedback should be taken into account in all corresponding sub-bands for CWS adjustment. If CBG based HARQ operation is supported in a wideband TB, HARQ-ACK feedback for CBGs within a sub-band are used for CWS adjustment in corresponding sub-band.

Proposal 7: The CWS maintenance is done separately in each 20 MHz sub-band.

# 6. 	Directional LBT

The need for multi-beam (multi-SSB) NR-U operation at sub-7 GHz is unclear. As discussed in [4], broadcast signaling (such as SSS/PSS/PBCH/RMSI) benefits from single beam operation in low frequency unlicensed spectrum (e.g., <7 GHz), leading to much higher efficiency compared to beamforming with beam sweeping operation. On unlicensed spectrum the total RF output power is normally restricted by EIRP limitation. For example, the RF output power EIRP limitation is 23 dBm and 24 dBm in 5 GHz unlicensed band, as regulated in ETSI 301.839 and FCC 15.247, respectively. This limitation captures both transmit power and antenna gain (beamforming gain + antenna element gain). This means directional transmission with high beamforming gain cannot lead to additional coverage in unlicensed spectrum, if the device is able to reach maximum output power with omni-direction.

Without support of beam sweeping for SSB, it is hard to guarantee efficient beamforming maintenance based on current NR framework. However, note that restricting the NR-U operation at sub 7 GHz to single SSB does not impose restrictions on using beamforming for PDSCH; on the contrary, gNB can make use of all its antennas with beamforming based on PMI feedback, beamformed CSI-RS, or SRS.

Observation 4: single-beam (omni-directional SSB) operation is more efficient for NR-U below 7 GHz

If only single-beam operation is supported in NR-U below 7 GHz, directional LBT (energy detection with specific phase array gain) is no needed. Moreover, even if beam-sweeping, is applied, the benefits of directional LBT are still unclear. Although directional LBT can potentially enhance spatial reuse, we also observe some issues caused by directional LBT that would need to be overcome:

- The hidden node problem becomes more server due to limited sensing area.

- The implementation complexity of directional LBT at both UE and gNB is considerable.

- The efficiency of COT sharing mechanism may be impacted, as the acquired COT may be only allowed to share to specific direction.

- Is it unclear how directional LBT would comply with regulatory framework for unlicensed spectrum (e.g. ETSI)

Based on the above, we see no benefits in introducing directional LBT for sub-7 GHz bands.

Proposal 8: Directional LBT is not supported for NR-U in sub-7 GHz bands

# 7. 	LBT for different channels and signals

LBT for DRS with transmission duration > 1 ms

In RAN1 #94bis, multi-beam configuration was discussed for DRS transmission. The duration of the whole DRS burst may be larger than 1ms when multi-beam operation is applied. The following two LBT options can be considered to support DRS transmission > 1ms.

- Option 1: Cat.4 LBT before the whole DRS burst

The gNB shall use Cat.4 LBT to acquire a COT for DRS burst transmission. The channel access priority class can be selected according to the DRS burst duration. For example, if the DRS burst duration is 2 ms, the gNB can use the highest channel access priority to perform Cat. 4 LBT.

- Option 2: Multiple one-shot CCAs, each of which initiates partial of DRS burst with duration less than 1ms

In this option, DRS burst can be divided into multiple portion, the duration of each portion is less than 1ms. Each DRS portion is subject to a one-shot CCA. If a one-shot CCA fails, the gNB can try a new attempt of transmitting corresponding DRS portion in the other candidate locations for such DRS portion within the DRS transmission window. The new attempt should also be subject to a one-shot CCA. Figure 3 illustrate an example of LBT for DRS transmission within the DRS transmission window of 5 ms assuming 30 kHz SCS, two SSBs per DRS transmission of 14 symbols, cyclic shifting of DRSs and 4 actually transmitted beams.

Figure 3: 4-beam DRS transmission in a 5ms DRS transmission window

In comparison with option 1, option 2 has stronger channel access capability to improve the DRS transmission opportunity in DMTC. On the other hand, to comply with the agreed limitations to duty cycle of DRS, it may be necessary to transmit DRS less often in this case.

Proposal 9: Potential LBT options to support DRS transmission with the DRS transmission burst longer than 1 ms are:

- Cat.4 LBT before the whole DRS burst

- Multiple one-shot CCAs, each of which initiates partial of DRS burst with duration less than 1ms; the use of such option for DRS transmission should comply with the duty cycle limitation.

LBT for UL channels

It is captured in TR 38.899:

- Applicability of a channel access scheme other than Cat 4 for the following signals / channels have been discussed and details are to be determined when the specifications are developed:

  - UL control information including UCI only on PUSCH, e.g. HARQ-ACK, Scheduling Request, and Channel State Information

  - Random Access

ETSI regulation defines a short control signalling transmission mode, which can be used to transmit control information without sensing the channel for the presence of other signals. However, there is strict limits of using Short Control Signalling Transmission [3].

- within an observation period of 50 ms, the number of Short Control Signalling Transmissions by the equipment shall be equal to or less than 50; and

- the total duration of the equipment's Short Control Signalling Transmissions shall be less than 2500 μs within said observation period

3GPP specifications for LTE LAA make use of the short control signaling clause in [3] when transmitting DRS, although instead of no-LBT, a single-shot 25 μs is applier prior to each transmission. Same rule was also agreed for NR-U DRS in RAN1 #85.

According to these limitations, the transmission of PUCCH seems too frequent to be used as Short Control Signalling Transmission. In principle, RACH related UL transmissions might fulfill the use conditions. related to SCS, but since RACH transmission occur fairly seldom and are not that time critical, a highest priority class Cat 4 LBT should suffice for those too. Therefore we conclude that NR-U UL signals do not need to apply SCS (no-LBT or 25 single-shot 25 μs LBT) to initiate a COT.

Proposal 10: Outside of a gNB acquired COT, all UL signals apply Cat 4 LBT. At least for PUCCH and RACH related signals, Cat4 with lowest channel access priority class value is applied.

# 8. 	LBT for Wideband (>20 MHz) operation

Different from LTE system, NR supports wider bandwidth operation. The maximum channel bandwidth per NR carrier is 100MHz for <6GHz and 400 MHz for 6~52.6GHz in Rel-15. There are hundreds of MHz of spectrum available in 5 GHz and 6 GHz unlicensed frequency bands and multiple GHz of spectrum available in the 60 GHz unlicensed band. Hence, NR-U shall also support wider bandwidth operation due to the benefit of higher spectrum utilization and lower baseband complexity. In the following, our discussion focuses on channel access for wider bandwidth operation in 5 GHz unlicensed spectrum.

As agreed in RAN1 92bis:

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

The LBT can be performed in units of 20 MHz in 5 GHz unlicensed bands.

To perform LBT on a wideband carrier, one potential way is to split a wideband carrier into multiple 20 MHz sub-bands. This is illustrated in Figure 3, considering subcarrier spacings of 15 kHz, 30 kHz, and 60 kHz. In ETSI regulation, two types of LBT mechanisms are defined for multi-channel operation.

- Type 1: Cat 4 LBT is performed on each 20 MHz operating channel individually.

- Type 2: Cat 4 LBT is performed on a selected primary channel. Other operating channels only need to perform a clear channel assessment (CCA) check of at least 25 us immediately before the transmission. And, the primary channel is a) chosen uniformly randomly, or b) arbitrarily determined and not changed more than once per second.

Figure 3. Possible NR BWs for 4k FFT and different subcarrier spacings. “20” denotes a 20 MHz sub-band

Proposal 11: Existing multi-channel LBT operation defined in ETSI or LTE-LAA can be used as baseline channel access mechanism for NR-U with wider bandwidth for sub-7 GHz unlicensed bands.

# 9. 	LBT and COT Sharing for configured grant

In “Enhancements to LTE operation in unlicensed spectrum”, both Type 1 and Type 2 UL channel access are supported for AUL transmission.

- AUL is allowed within the eNB acquired COT and Type 2 channel access (25 us one-shot LBT) should be performed before the AUL transmission starts.

- The UE could also use Cat.4 LBT to acquire its own COT to perform AUL transmission. Furthermore, this COT is allowed to be shared with the eNB for sending DL control signaling, including AUL-DFI or UL grant, to the UE which acquired the COT within remaining COT. The duration of the DL transmission is limited to up to 2 symbols.

We see that similar channel access procedures can be introduced in NR-U for UL transmission with configured grants. Further enhancements to LBT procedure (e.g., CWS adjustment) could also be considered due to new features in NR, such as faster processing time.

Proposal 12: UL transmissions with configured grants should use channel access procedure defined in LTE WI “Enhancements to LTE operation in unlicensed spectrum” as a baseline. Further enhancements to LBT procedure (e.g., CWS adjustment) could also be considered due to new features in NR.

Another enhancement for COT sharing is that DL broadcast signalling and scheduled DL data to the UE that initiated the COT could be also considered within a COT acquired by a UE. Such operation can improve the efficiency of channel usage in NR-U and also complies with regulatory requirements.

Proposal 13: DL broadcast signalling and scheduled DL data to the UE that initiated the COT could be also considered within a COT acquired by a UE. FFS: further details.

# 10. 	Conclusions

In this contribution, we have discussed different aspects related to channel access procedure for NR unlicensed. Based on the discussion, we make the following observations and proposals:

Clarifications on Cat 2 LBT

Proposal 1: The duration of Cat 2 LBT should be in line with the gap duration when the Cat 2 LBT is using within a COT.

Channel access procedure for a shared gNB COT

Observation 1: According to ETSI requirements, Cat 1 Immediate transmission does not involve additional restriction on the duration of transmission from responding device other than MCOT duration limit

Proposal 2: Cat 1 framework should support at least HARQ-ACK transmission at the beginning of the UL burst

Proposal 3: If UE performs one-shot LBT during the 16 sec gap, and the channel is found to be free, the UE can use UL resources according to MCOT duration limit.

Receiver assisted LBT

Observation 2: Hidden nodes may reduce UL channel access probability in unlicensed spectrum.

Proposal 4: The benefits of RTS/CTS-like receiver assisted LBT schemes require further discussion and study.

Proposal 5: Overbooked UL transmissions and UE reporting of channel sensing results can be considered as ways to increase UL access probability.

CWS maintenance for Cat.4 LBT

Observation 3: The determination of the reference slot for CWS adjustment should take slot structure (partial slot or full slot) and processing time into consideration

Proposal 6: Both alternatives can be considered in CWS adjustment to support CBG based HARQ-ACK operation:

- Alternative 1: If all CBG-level HARQ-ACKs corresponding a TB are ACK(s), TB-level HARQ-ACK feedback for CWS adjustment is counted as ACK, otherwise, NACK.

- Alternative 2: If the first CBG-level HARQ-ACKs corresponding a TB is ACK, TB-level HARQ-ACK feedback for CWS adjustment is counted as ACK, otherwise, NACK.

Proposal 7: The CWS maintenance is done separately in each 20 MHz sub-band.

Directional LBT

Observation 4: single-beam (omni-directional SSB) operation is more efficient for NR-U below 7 GHz

Proposal 8: Directional LBT is not supported for NR-U in sub-7 GHz bands

LBT for different channels and signals

Proposal 9: Potential LBT options to support DRS transmission with the DRS transmission burst longer than 1 ms are:

- Cat.4 LBT before the whole DRS burst

- Multiple one-shot CCAs, each of which initiates partial of DRS burst with duration less than 1ms; the use of such option for DRS transmission should comply with the duty cycle limitation.

Proposal 10: Outside of a gNB acquired COT, all UL signals apply Cat 4 LBT. At least for PUCCH and RACH related signals, Cat4 with lowest channel access priority class value is applied.

LBT for Wideband (>20 MHz) operation

Proposal 11: Existing multi-channel LBT operation defined in ETSI or LTE-LAA can be used as baseline channel access mechanism for NR-U with wider bandwidth for sub-7 GHz unlicensed bands.

LBT and COT Sharing for configured grant

Proposal 12: UL transmissions with configured grants should use channel access procedure defined in LTE WI “Enhancements to LTE operation in unlicensed spectrum” as a baseline. Further enhancements to LBT procedure (e.g., CWS adjustment) could also be considered due to new features in NR.

Proposal 13: DL broadcast signalling and scheduled DL data to the UE that initiated the COT could be also considered within a COT acquired by a UE. FFS: further details.

# References

- TR 38.899 Study on NR-based access to unlicensed spectrum

- Chairman’s notes RAN1 AH201901

- ETSI EN 301 893 V2.1.1

- R1-1812696, On DL Signals and Channels for NR-U, Nokia, Nokia Shanghai Bell