3GPP TSG RAN WG1 Meeting #96                                                                	     R1-1902146

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	6.2.1.2

Source:	Lenovo, Motorola Mobility

Title:	UL transmission in preconfigured resources for eMTC

Document for:	Discussion and decision

Introduction

In the RAN#80 plenary meeting, a new work item for Rel.16 eMTC is approved. One of the objectives is to specify the uplink transmission based on preconfigured resource for machine-type communications for BL/CE UEs.

Improved UL transmission efficiency and/or UE power consumption:

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance [RAN1, RAN2, RAN4]

  - Both shared resources and dedicated resources can be discussed

Note: This is limited to orthogonal (multi) access schemes

In the last RAN1 meeting, the following agreements are achieved for the design of uplink transmission based on preconfigured resource for UE power saving:

Agreement

For dedicated PUR in idle mode, the UE may skip UL transmissions.

- FFS: Resource release mechanism

- FFS: Whether or not to support mechanism to disable skipping by eNB

Agreement

- If multi-TB grant is not enabled, a dedicated PUR allocation is associated to only a single TB and single HARQ process

- FFS: if multi-TB grant is enabled/supported

- Agreement

- In idle mode, at least the following TA validation attributes are supported:

- Serving cell changes (serving cell refers the cell that the UE is camping on)

- Time Alignment Timer for idle mode

- Serving cell RSRP changes (serving cell refers the cell that the UE is camping on)

  - Based on RSRP measurement definition in existing Rel-15 TS36.214

Include in LS to RAN2, RAN4 to consider in their work.

- Agreement

- The UE can be configured to use at least these TA validation attributes:

- Time Alignment Timer for idle mode

- Serving cell RSRP changes

- Note: the configuration shall support disabling of the TA validation attributes

Include in LS to RAN2, RAN4

For further study:

TA validation attributes:

- Subscription based UE differentiation (or Stationary indication in held in subscription)

- Cell specific indication where TA is valid within that cell

Agreement

Include in LS to RAN2, RAN4:

RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode.

Agreement

For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in MPDCCH search space

- FFS: Details on the search space (for example USS, CSS)

- Agreement

- For dedicated PUR in idle mode, upon successful decoding by eNB of a PUR transmission, the UE can expect an explicit ACK

FFS: if ACK is sent on MPDCCH (layer 1) and/or PDSCH (layer 2/3)

Include in LS to RAN2, RAN4.

- Agreement

- For dedicated PUR in idle mode, upon unsuccessful decoding by eNB of a PUR transmission, the UE can expect

- an UL GRANT for retransmission on the MPDCCH, or

- FFS: a NACK, or

- FFS: no explicit ACK

Include in LS to RAN2, RAN4.

In this contribution, detailed considerations of transmissions in pre-configured UL resources, especially TA issues and HARQ related issue based on dedicated PUR in IDLE mode for Rel.16 MTC are presented.

Discussion

TA Validation

For uplink transmissions in PURs, TA validation is the basic procedure and only transmissions passing TA validation are allowed. During IDLE mode, the initial TA is validated using some criteria as agreed in RAN1 #95 meeting. At least 1) Serving cell changes; 2) Time Alignment Timer for idle mode; 3) Serving cell RSRP changes are supported in TA validation.  Furthermore, we give detailed analysis for some other attributions proposed previous meeting.

- Subscription based UE differentiation

The “Subscription Based UE differentiation” includes an indicator “stationary indicator” , which is ENUMERATED as {stationary, mobile…}. If UE reports itself as “stationary”, it means that UE has fixed relative distance with “serving” eNB, and UE can in principle keep TA validation timer as “indefinite”. So subscription based UE differentiation can be supported as an attribution for TA validation. However, the “stationary” indication doesn’t mean any change of the legacy procedure of measurement and related cell reselection.

Proposal 1:  Subscription based UE differentiation is supported as an attribution for TA validation.

- Cell specific indication where TA is valid within that cell

It is proposed that a cell specific indication is introduced for small cell [2]. If a UE is connected to a small cell (i.e. cell coverage of e.g. <700m), the UE could assume the TA is always valid while in that cell, which can be implemented by setting Time Alignment Timer “indefinite” or “zero” and is transparent to UE without any additional indication.

Proposal 2:  Cell specific indication where TA is valid within that cell is not necessary and which can be implemented by setting Time Alignment Timer “indefinite” or “zero”.

Dedicated PUR Issues

HARQ

- In the last meeting, it is agreed that if multi-TB grant is not enabled, a dedicated PUR allocation is associated to only a single TB and single HARQ process. However, the detail of the scheduling multiple TB is not stable at this stage, especially the HARQ, DCI design, which is even another story for transmission in IDLE mode. So we propose to move supporting multiple HARQ process number with multi-TB grant on PUR transmission to later release.

- Proposal 3: For uplink transmission based dedicated PUR, only single TB and single HARQ process are supported in Rel.16.

- When UE is being assigned with PUR resource, e.g., time/frequency resource, it is also necessary to  assign a dedicate RNTI for UE to monitor HARQ feedback. After transmission based PUR, UE start to monitor the (M)PDCCH with the dedicate RNTI to expect an explicitly ACK upon successful decoding by eNB or an UL GRANT for retransmission upon unsuccessful decoding by eNB.

- Proposal 4: After transmission based PUR, UE start to monitor the (M)PDCCH with the dedicate RNTI to expect an explicitly ACK upon successful decoding by eNB or an UL GRANT for retransmission upon unsuccessful decoding by eNB.

- In response to the ACK, the UE could then continue new transmission in next PUR resource, while in response to NACK (e.g., the UL GRANT for retransmission), the UE could then retransmit in configured new grant resource or wait till the next PUR transmission to send a retransmission if only supporting single HARQ process.

PUR Search Space

For connected mode UEs, UE specific MPDCCH can be used for monitoring HARQ feedback after uplink transmission. However, for idle mode, it is natural to use MPDCCH common search space as starting point as type 1 MPDCCH common search space for paging and type 2 MPDCCH common search space for RAR as shown in Figure 1 for UE power saving. Due to the similarity of uplink transmission based PUR and RACH procedure, it is preferable to use type 2 common search space as baseline. In order to further reduce blind detection for different UE (e.g., UE with different coverage/potential repetitions), dedicated (reduced) search space based type 2 common search space can be newly designed for different UEs.

Proposal 5: Due to the similarity of uplink transmission based PUR and RACH procedure and UE capability; it is preferable to use Type 2 common search space as baseline.

Figure 1: Type 2 common search space for RAR

Fallback

For transmission in PUR, fallback mechanism to RACH/EDT procedure was agreed in the last meeting, but the details is still FFS. The fallback mechanism can be categorized to two cases.  Firstly, the fallback to RACH/EDT is triggered by UE and no new signalling is required. For example, TA validation failure; transmission in PUR unsuccessfully decoded, e.g., ACK is not received during a preconfigured window or count to a preconfigured number.  In this case, UE can transmit preamble in the next RACH occasion if fallback decision made.

Secondly, the fallback to RACH/EDT is triggered by eNB and an indication signalling needs to be introduced, e.g., multiple PUR transmission failure, system load balance requirement, PUR resource unavailable, etc. Fallback indicator can be sent via dedicated RRC signalling when eNB finds UE’s condition is no longer applicable for PUR.

Proposal 6: Fallback to RACH/EDT can be triggered by eNB or UE for different cases.

Configuration

According to the definitions agreed for PUR in RAN1 #94bis, dedicated PUR is defined as PUSCH resource used by a single UE. For periodic resource configuration, the resource can be configured as periodic starting subframe and resource duration for each resource occasion as shown in Figure 2. For the periodic starting subframe of resource occasion, resource configuration of LTE SPS and NR type 1 configured grant uplink transmission can be the starting point as shown in Figure 3. For aperiodic resource configuration, multiple PUR resources can be configured with different periods to one particular UE.

The resource occasion number of PUR can be configured as a predefined value, the predefined value can be 1 to deal with traffic model of one shot uplink transmission and can also be zeros or infinite to indicate the PUR resource is always valid or reserved for UE.

Proposal 7:  PUR resource can be configured as periodic starting subframe and resource duration for each PUR resource occasion.

Figure 2 Periodic PUR resource configuration illustration

Figure 3 LTE SPS resource configuration illustration

Release PUR

Similar to LTE SPS release mechanism, it is agreed that release of the dedicated preconfigured resources are supported, details for NW triggered and UE triggered are FFS in RAN2#104.  There are several potential PUR release mechanisms: 1) PUR resource released right after the transmission until to preconfigured number. 2) Implicit PUR release mechanism after consecutive multiple missed transmissions to avoid the waste of allocated PUR. 3) eNB trigger re-configuration/release of PUR configured to a UE by dedicated RRC. 4) UE request PUR resource release via RACH/EDT procedure or on-going transmission based on PUR, which causes large UE power and should be studied further.

Proposal 8: PUR resource is released by 1) transmission until to preconfigured number; 2) consecutive multiple missed transmissions.

Proposal 9: eNB may trigger re-configuration/release of PUR configured to a UE by dedicated RRC and UE request PUR resource release via RACH/EDT procedure or on-going transmission based on PUR should be studied further.

Shared PUR Issues

In RAN1 #94bis, it was agreed that “In idle mode, dedicated PUR is supported”, while the “Support for CFS PUR” and the “Support for CBS PUR” remained FFS.

For CFS PUR, multiple UEs share the same configured time-frequency resource with orthogonal multiple access, e.g., MU-MIMO and CDM. For MU-MIMO, it can be transparently to configured multiple UE same time-frequency resource with UE-specific DMRS via dedicated PUR. Therefore, CFS PUR using MU-MIMO with UE specific DMRS pattern can be supported without any specification work.

For CBS PUR, contention based PUR is required to ensure the reliability of uplink transmission and solve collision problems. Furthermore, CBS PUR may cause more specification effort, waste more UE power consumption, and even bring more challenges for eNB receiver, so we propose that CBS PUR is not supported in Rel.16 eMTC.

Proposal 10: CFS PUR using MU-MIMO with UE specific DMRS pattern can be supported without any specification work.

Proposal 11: CBS PUR is not supported in Rel.16 eMTC.

Conclusions

In this contribution, considerations of multiple transport block transmission with DCI are provided. The following proposals are given.

Proposal 1:  Subscription based UE differentiation is supported as an attribution for TA validation.

Proposal 2: Cell specific indication where TA is valid within that cell is not necessary and which can be implemented by setting Time Alignment Timer “indefinite” or “zero”.

- Proposal 3: For uplink transmission based dedicated PUR, only single TB and single HARQ process are supported in Rel.16.

- Proposal 4: After transmission based PUR, UE start to monitor the (M)PDCCH with the dedicate RNTI to expect an explicitly ACK upon successful decoding by eNB or an UL GRANT for retransmission upon unsuccessful decoding by eNB.

Proposal 5: Due to the similarity of uplink transmission based PUR and RACH procedure and UE capability; it is preferable to use Type 2 common search space as baseline.

Proposal 6: Fallback to RACH/EDT can be triggered by eNB or UE for different cases.

Proposal 7:  PUR resource can be configured as periodic starting subframe and resource duration for each PUR resource occasion.

Proposal 8: PUR resource is released implicitly by 1) one shot resource allocation 2) consecutive multiple missed transmissions.

Proposal 9: eNB may trigger re-configuration/release of PUR configured to a UE by dedicated RRC and UE request PUR resource release via RACH/EDT procedure or on-going transmission based on PUR should be studied further.

Proposal 10: CFS PUR using MU-MIMO with UE specific DMRS pattern can be supported without any specification work.

Proposal 11: CBS PUR is not supported in Rel.16 eMTC.

- References

RP-181420, “New WID on Rel-16 MTC enhancements for LTE”, Ericsson, RAN#80

R1-1813725, “LTE-M Preconfigured UL Resources Summary RAN1 #95”, Sierra Wireless, RAN1#95

R1-1812120, “Support for transmission in preconfigured UL resources in LTE-MTC”, Ericsson, RAN1#95

R1-1812142, “UL transmission in preconfigured resource”, Huawei, HiSilicon, RAN1#95

R1-1812724, “LTE-M Pre-configured UL Resources Design Considerations”, Sierra Wireless, S.A. , RAN1#95

R1-1812940, “Discussion on transmission in preconfigured UL resources for MTC”, Samsung, RAN1#95