3GPP TSG RAN WG1 Meeting #96                                                                	     R1-1902147

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	6.2.1.3

Source:	Lenovo, Motorola Mobility

Title:	Design of scheduling of multiple DL/UL transport blocks for MTC

Document for:	Discussion and decision

Introduction

The contribution is the update from R1-1812788.

In the RAN#80 plenary meeting, a new work item for Rel.16 eMTC is approved. One of the objectives is to specify the scheduling improvement of DL and UL for machine-type communications for BL/CE UEs.

- Scheduling enhancement

  - Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast[RAN1, RAN2]

    - Enhancement of SPS can be discussed.

In the last RAN1 meeting, the following agreements are achieved for the design of scheduling multiple DL/UL transport blocks (TBs):

Agreement

For CEmodeA, the maximum number of scheduled transport blocks with one single DCI is 8 in the UL, 8 in the DL

Agreement

The following working assumption is confirmed

For CEmodeB, the maximum number of scheduled transport blocks with one single DCI is 4 in the UL, 4 in the DL

Agreement

For both UL and DL unicast, at least consecutive resource allocation in time is supported when multiple TBs are scheduled by one single DCI.

- Above applies only for valid subframes within the consecutive resource allocation in time

- FFS: Whether time gaps between two TBs is also supported

For future meetings in Rel-16 eMTC:

Further consider which Rel-14/15 features that should be possible to configure together with scheduling of multiple DL-UL transport blocks.

Agreement

For the DL unicast for a UE, when multiple TBs are scheduled by one DCI, the following parameter values are the same across all the TBs:

- Frequency-hopping flag, PMI confirmation (TM6-specific), Precoding information (TM6-specific), DM-RS scrambling / antenna ports (TM9-specific), Downlink assignment index (TDD-specific), PUCCH power control

- FFS: MCS, RV, Resource assignment, Number of PDSCH repetitions

Agreement

For the UL unicast, when multiple TBs are scheduled by one DCI, the following parameter values are the same across all the TBs:

- Frequency-hopping flag, TPC command

- FFS: MCS, RV, Resource assignment, Repetition number, Downlink assignment index (TDD-specific)

For next meeting

For unicast when multi-TBs are scheduled, companies are encouraged to bring in DCI designs which can support

- scheduling of initial and retransmission TBs within one DCI

- scheduling of initial TBs within one DCI, and retransmissions with one DCI

- scheduling of initial TBs within one DCI, and retransmission can only be scheduled by individual DCI

RAN1 tries to make a decision on which case is specified in the next meeting based on the trade-off between DCI overhead and scheduling flexibility comparisons of the three cases.

Agreement

For the case of single DCI scheduling multiple transport blocks with repetitions, scheduling of transport blocks repetitions is down selected between:

- Option 1: All the repetitions for one transport block are contiguously scheduled in valid UL/DL subframes

- Option 2: The repetitions for one transport block are interleaved with repetitions of all the other transport blocks

- Option 3: Option 1 and 2 are supported and eNB configures among them.

Agreement

The maximum number of TBs for multicast is 8.

In this contribution, detailed considerations of scheduling improvement for uplink and downlink, especially scheduling multiple DL/UL TBs with one DCI for unicast for Rel.16 MTC are presented.

Discussion

Scheduling multiple transport blocks for MTC

In RAN1-95 meeting, it is agreed that maximum number of scheduled TBs with one single DCI is 8 for CE mode A and 4 for CE mode B for both uplink and downlink. One of the motivations to support scheduling multiple TB is to reduce MPDCCH resources; however, it is observed that the maximum number of supported TBs has a significant impact on the size of the DCI, so companies propose the optimization of DCI. In the last meeting, some of the DCI fields are agreed to be the same across all the TBs except following fields:

Resource assignment –  5~9 bits

Number of PDSCH repetitions –  2 bits

MCS – 4 bits

HARQ ID – 3 bits

RV – 2 bits

New Data Indication – 1 bit

If we allow full flexibility across all TBs for above fields, this would result in a significant increasing of the DCI size, which decreases the DCI reliability. So we proposal most of fields above will be the same for different TBs as shown in Table 1. For initial transmission and retransmission scheduled by one DCI, the eNB schedules the TBs (re)transmission according to the channel state information, so all TBs should follow the same resource assignment, number of repetition number, etc. Furthermore, all TBs follows the same scheduling parameters, which is easier for resource alignment and scheduling interleaved transmission if supported

Table 1: Initial evaluation of impacted fields in DCI format 6-1A

| DCI field | Number of bits in legacy DCI format 6-1A | Interpretation when multiple transport blocks enabled via RRC |
| --- | --- | --- |
| Flag for 6-0/6-1 differentiation | 1 | Same for all TBs |
| Frequency-hopping flag | 1 | Same for all TBs |
| Resource assignment | 5-9 | Same for all TBs |
| Number of PDSCH repetitions | 2 | Same for all TBs |
| HARQ process number | 4 (FDD) 5(TDD) | Only some of HARQ process combination scheduling pattern supported, especially for TDD 1-8 [P0][P1]…[P7] 9-12 [P0,P1], [P2,P3]….[P6,P7] 13-14 [P0,P1,P2,P3], [P4,P5,P6,P7] 15 [P0,P1,P2,P3,P4,P5,P6,P7] |
| MCS | 4 | Same for all TBs |
| RV | 0 | Default RV order {0 2 3 1} |
| NDI | 8 | One NDI bit for each HARQ process |
| PMI confirmation (TM6-specific) | 1 | Same for all TBs |
| Precoding information (TM6-specific) | 2 or 4 | Same for all TBs |
| DM-RS scrambling / antenna ports (TM9-specific) | 2 | Same for all TBs |
| Downlink assignment index (TDD-specific) | 2 | Same for all TBs |
| PUCCH power control | 2 | Same for all TBs |
| SRS request | 1 | Same for all TBs |
| Ack/Nack offset | 2 | Same for all TBs |
| Number of MPDCCH repetitions | 2 | Same for all TBs |

Proposal 1:

For both DL and UL unicast for a UE, when multiple TBs are scheduled by one DCI, the following parameter values are the same across all the TBs:

- Resource assignment

- Number of PDSCH/PUSCH repetitions

- MCS

Scheduling pattern

In the last meeting, scheduling pattern of non-mixed scheduling and mixed scheduling are discussed. In order to achieve the scheduling flexibility and save MPDCCH overhead for retransmission for multiple TBs, mixed scheduling should be supported. That is, some of the TBs are for retransmission with some HARQ process while the other TBs are used for initial transmission of the other HARQ process, and scheduling of initial and retransmission TB within one DCI is supported.

Proposal 2: Scheduling of initial and retransmission TBs within one DCI is supported.

Interleaving transmission

When repetitions are used in PUSCH/PDSCH, interleaving transmission among TBs can introduce time diversity as shown in Figure 1. If interleaving transmission among TBs from different HARQ process is adopted, some of the interleaving issues should be further studied, e.g. interleaving pattern, interleaving period, GAP period, TB transmission sequence of multiple TBs, etc.

Figure 1 Interleaving transmission illustration

Proposal 3: Some of the interleaving issues should be further studied, e.g. interleaving pattern, interleaving period, GAP period, TB transmission sequence of multiple TBs, etc.

Cyclic repetition

Cyclic repetition is adopted in legacy CE Mode B for enabling symbol-level I/Q combining and frequency/timing offset tracking. So for the scheduling multiple TBs, even with interleaving transmission, the interleaving operation should be based upon 4 (FDD) absolute subframes where the same RV is adopted as shown in Figure 3.

Figure 2 cyclic repetition transmission illustration

Proposal 4: Cyclic repetition is supported even for TB interleaving transmission for CE mode B.

Transmission gap

For both UL and DL unicast, at least consecutive resource allocation in time is supported when multiple TBs are scheduled by one single DCI. Some companies proposed to introduce additional gap (for discontinuous transmission) among the transmission for time diversity gain. However, adding gap may cause some potential issues as: 1) resource fragments 2) transmission delay and transmission rate reduction 3) additional control overhead for gap configuration. 4) Non-continuous DMRS for channel estimation. 5) Limited time diversity gain if interleaving transmission applied. Therefore, it is preferred that the allocated resources of different TBs are consecutive in time domain and it is not necessary to introduce additional gap for scheduling multiple TB transmission.

Proposal 5: For unicast transmission, additional gap between two TBs in time domain should not supported for multiple TBs scheduling.

HARQ ACK/NACK feedback

For the single transport block, the PUCCH resource is derived via MPDCCH location in system bandwidth. For multiple transport blocks, PUCCH resource for ACK/NACK of multiple TBs can reuse the legacy method as well. E.g., the PUCCH resource for the first TB is firstly determined following the legacy method and PUCCH resources for the remaining TBs are derived from the already determined PUCCH resource for the first TB even with PUCCH repetition, e.g., the same PUCCH resources of contiguous available subframes.  PUCCH resources collision can be avoided by eNB scheduling. Furthermore, the ACK/NACK timing offset of multiple TB to the corresponding PDSCH should be further studied, e.g. minimal timing offset to PDSCH of 8ms should be guaranteed.

Figure 3 HARQ ACK/NACK feedback illustration

In the last RAN1 meeting, some companies pointed out that HARQ ACK/NACK bundling is beneficial in terms of resource saving. However, for the ACK/NACK bundling mechanism, if one of the TBs in the bundle fails decoding, all the TBs will be retransmitted regardless whether other TBs are correctly decoded or not. Even for the semi-persist channel in most of IoT scenarios, it is not an efficient way especially for TBs with larger repetition number. So simple ACK/NACK bundling mechanism is not a suitable solution for multiple transport blocks, especially when the number of ACK/NACK bits for bundling is big, e.g. 4 or 8.

Proposal 6: HARQ ACK/NACK resource can be implicitly derived from MPDCCH location even for scheduling multiple TBs.

Proposal 7: For independent ACK/NACK feedback, ACK/NACK timing offset of multiple TB to the corresponding PDSCH should be further studied.

Conclusions

In this contribution, considerations of multiple transport block transmission with DCI are provided. The following proposals are given.

Proposal 1:

For both DL and UL unicast for a UE, when multiple TBs are scheduled by one DCI, the following parameter values are the same across all the TBs:

- Resource assignment

- Number of PDSCH/PUSCH repetitions

- MCS

Proposal 2: Scheduling of initial and retransmission TBs within one DCI is supported.

Proposal 3: Some of the interleaving issues should be further studied, e.g. interleaving pattern, interleaving period, GAP period, TB transmission sequence of multiple TBs, etc.

Proposal 4: Cyclic repetition is supported even for TB interleaving transmission for CE mode B.

Proposal 5: For unicast transmission, additional gap between two TBs in time domain should not supported for multiple TBs scheduling.

Proposal 6: HARQ ACK/NACK resource can be implicitly derived from MPDCCH location even for scheduling multiple TBs.

Proposal 7: For independent ACK/NACK feedback, ACK/NACK timing offset of multiple TB to the corresponding PDSCH should be further studied.

- References

RP-181420, “New WID on Rel-16 MTC enhancements for LTE”, Ericsson, RAN#80

R1-1813725, “LTE-M Preconfigured UL Resources Summary RAN1 #95”, Sierra Wireless, RAN1#95

R1-1812120, “Support for transmission in preconfigured UL resources in LTE-MTC”, Ericsson, RAN1#95

R1-1812142, “UL transmission in preconfigured resource”, Huawei, HiSilicon, RAN1#95

R1-1812724, “LTE-M Pre-configured UL Resources Design Considerations”, Sierra Wireless, S.A. , RAN1#95

R1-1812940, “Discussion on transmission in preconfigured UL resources for MTC”, Samsung, RAN1#95