3GPP TSG RAN WG1 Meeting #96 R1-1902234 Athens, Greece, February 25th – March 1st, 2019 CR-Form-v11.2. [DRAFT] CHANGE REQUEST. 38.213; CR; rev; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on PDCCH monitoring for overlapped CORESETs. Source to WG:; Samsung. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-02-25. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; Clarify UE behavior on PDCCH monitoring for overlapped CORESETs configured with the same TCI-states with ‘QCL-TypeD’. Summary of change:; Describe UE behavior on PDCCH monitoring when a UE monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs where the CORESETs are configured with the same TCI-states with ‘QCL-TypeD’. Consequences if not approved:; Incomplete specifications for PDCCH monitoring for overlapped CORESETs. Clauses affected:; 10.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

10.1	UE procedure for determining physical downlink control channel assignment

*** Unchanged text is omitted ***

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs where none of the CORESETs has TCI-states with 'QCL-TypeD' or all of the CORESETs have the same TCI-states with ‘QCL-TypeD’,

the UE is required to monitor PDCCH candidates in overlapping PDCCH monitoring occasions for search space sets associated with different CORESETs.

*** Unchanged text is omitted ***