3GPP TSG RAN WG1 Meeting #96 R1-1902235 Athens, Greece, February 25th – March 1st, 2019 CR-Form-v11.2. [DRAFT] CHANGE REQUEST. 38.213; CR; rev; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on cross-carrier scheduling. Source to WG:; Samsung. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-02-25. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; 1) Correction to a missing description - If cross-carrier scheduling is configured, a UE does not expect to monitor PDCCH of a scheduled cell for “USS” and the UE can still monitor Type3 CSS if configured. - However, current specification describes that a UE does not monitor PDCCH for the scheduled cell when cross-carrier scheduling is configured, regardless of the SS types. 2) Cross-carrier release for DL SPS and UL grant Type 2 - DCI format 0_1/1_1 can be used for DL SPS and UL grant Type 2 activation, so that cross-carrier activation is supproted by using the CIF field in current specification. - DCI foramt 0_0/1_0 is used for DL SPS and UL grant Type 2 release, however, it is unclear now how to signal DL SPS and UL grant Type 2 release when cross-carrier scheduling is configured since DCI format 0_0/1_0 does not include CIF field.. Summary of change:; 1) Correction to a missing description - Add a missing description 2) Cross-carrier release for DL SPS and UL grant Type 2 - If cross-carrier scheduling is configured, 3 MSB bits of the ‘Time domain resource assignment’ field in DCI format 0_0/1_0 are used for CIF.. Consequences if not approved:; Incomplete specifications for cross-carrier scheduling 1) Correction to a missing description - If CR is not approved, the UE cannot monitor Type3 CSS in SCell when cross-carrier scheduling is configured. - If the UE is implemented according to the change but the NW is not, the NW may transmit DCI via Type3 CSS in SCell, but the UE may not monitor Type3 CSS in SCell. As a consequence, performance is degraded. - If the NW is implemented according to the change but the UE is not, the NW may not transmit DCI via SCell, but the UE may keep monitoring Type3 CSS in SCell. As a consequence, the UE power consumption is increased. 2) Cross-carrier release for DL SPS and UL grant Type 2 - If CR is not approved, the UE cannot monitor DCI format 0_0/1_0 for DL SPS and UL grant Type 2 release in SCell when cross-carrier scheduling is configured and Type3 CSS is not configured. - If the UE is implemented according to the change but the NW is not, the NW cannot transmit DCI format for DL SPS and UL grant Type 2 release via the SCell, and the UE may understand the command of DCI format for DL SPS and UL grant Type 2 release in different way. As a consequence, the system will be broken. - If the NW is implemented according to the change but the UE is not, the NW may transmit DCI format for DL SPS and UL grant Type 2 for the SCell via the scheduling cell, but the UE may apply the command of the detected DCI format to the scheduling cell. As a consequence, the system will be broken.. Clauses affected:; 10.1, 10.2. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

10.1  UE procedure for determining physical downlink control channel assignment

A UE does not expect to monitor PDCCH candidates in a USS on an active DL BWP of a secondary cell if the UE is configured to monitor PDCCH candidates with carrier indicator field corresponding to that secondary cell in another serving cell. For the active DL BWP of a serving cell on which the UE monitors PDCCH candidates, the UE monitors PDCCH candidates at least for the same serving cell.

10.2	PDCCH validation for DL SPS and UL grant Type 2

*** Unchanged text is omitted ***

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If the UE is configured to monitor PDCCH candidates with DCI formats that include a carrier indicator field, the UE assumes that the 3 MSB bits of the time domain resource assignment field correspond to the carrier indicator field. If validation is not achieved, the UE discards all the information in the DCI format.

*** Unchanged text is omitted ***