3GPP TSG-RAN WG1 #96	R1-1902363

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	6.1.4

Source:	Qualcomm Incorporated, Ericsson

Title:	Details for Draft CR R1-1902363 of RSS EPRE

Document for:	Discussion and Decision

# 1	Introduction

- Current specification states that the RSS EPRE is set by parameter powerBoost (in RSS-Config) directly. However, RSS uses single antenna port while CRS may use one or multiple antenna ports. In addition, CRS may be power boosted, resulting in different power ρA and ρB for PDSCH REs. But RSS ERPE should be same for all RSS REs. Considering impact of CRS port number as well as ρA and ρB, the RSS ERPE relative to CRE ERPE should be defined.

# 2	Discussion

- Figure 1 illustrated the power in case of 1, 2 or 4 CRS ports, where ρA and ρB relative to CRS power per port is the power for PDSCH. The ratio of ρA and ρB is cell-specific, indicated by PB as specified in Table 5.2-1 of [1].

- Figure 1 Power for different CRS port number

- In case of single CRS port (p=1), assuming same power for symbol with or without CRS RE, we have 5ρB+PCRS=6ρA. If assuming PCRS=1, the default value relative to CRS power is ρAd=1/(6-5ρ) with ρ=ρB/ρA.

- In case of two CRS ports (p=2), CRS port 0 and port1 are FDMed. Assuming same power for symbol with or without CRS REs, we have 4ρB+PCRS=6ρA. If assuming PCRS=1, the default value relative to CRS power is ρAd=1/(6-4ρ) with ρ=ρB/ρA.

- In case of four CRS ports (p=4), CRS port2 and port 3 are FDMed, but TDMed with the symbol with CRS port 0 and port1. Assuming same power for symbol with or without CRS REs, we have 4ρB+PCRS=6ρA. If assuming PCRS=1, the default value relative to CRS power is same as 2-port case.

- Different from PDSCH, the RSS power should be constant per RE. To keep it within max transmit power per symbol, we take the smaller power for RSS REs. Therefore, accumulating the power on each antenna port as that of a virtual antenna port, the RSS EPRE relative to CRS EPRE is 10log10(p×minρAd, ρBd), where ρAd and ρBd for different PB are given by Table 5.2-1a and p is the number of CRS ports. Note that ρAd and ρBd  allow for uniform power allocation across all RBs. In case of ρAd=ρBd, the RSS EPRE relative to CRS EPRE is 0dB for 1- or 2-port CRS (similar with the agreed NB-IoT WUS EPRE relative to NRS EPRE), and 3dB for 4-port CRS, respectively.

If B/A=2/5 for 1-port CRS, the RSS EPRE without power boosting would be as low as -10dB lower than the CRS EPRE. It means we have to blank additional 18 PRBs to power boost 2-PRB RSS to keep RSS EPRE is same as CRS EPRE, which is unrealistic. While, when B/A=5/4 for 4-port CRS, the RSS EPRE with 4.8dB power boosting (on 2PRB by blanking the remaining 4 RBs) would be +10.8dB higher than CRS EPRE. For such very low/high values, we would prefer to define the RSS EPRE relative to CRS EPRE in RAN1 spec based on existing higher-layer parameter PB, instead of changing the RRC signaling of ce-RSS-PowerBoost-Config.

- If there is additional power boosting signaled by higher layer parameter ce-RSS-PowerBoost-Config, e.g., by blanking {0,2,4,6} additional RBs to power boost {0dB, 3dB, 4.8dB, 6dB} on 2PRB RSS, the RSS EPRE relative to CRS EPRE is ce-RSS-PowerBoost-Config+10log10(p×minρAd, ρBd).

Table 5.2-1: The cell-specific ratio  for 1, 2, or 4 cell specific antenna ports

|  | One Antenna Port | Two and Four Antenna Ports |
| --- | --- | --- |
| 0 | 1 | 5/4 |
| 1 | 4/5 | 1 |
| 2 | 3/5 | 3/4 |
| 3 | 2/5 | 1/2 |

Table 5.2-1a: ρAd and ρBd for 1, 2, or 4 cell specific antenna ports assuming the same transmitted power for symbols with or without CRS

| PB | One Antenna Port |  | Two and Four Antenna Ports |  |
| --- | --- | --- | --- | --- |
|  | ρAd | ρBd | ρAd | ρBd |
| 0 | 1 | 1 | 1 | 5/4 |
| 1 | 1/2 | 2/5 | 1/2 | 1/2 |
| 2 | 1/3 | 1/5 | 1/3 | 1/4 |
| 3 | 1/4 | 1/10 | 1/4 | 1/8 |

- Proposal:

- The RSS EPRE relative to CRE EPRE is defined as ce-RSS-PowerBoost-Config+10log10p×min⁡(ρAd,ρBd), where ρAd and ρBd are the default values for ρA and ρB relative to CRS EPRE assuming the same transmitted power for symbols with or without CRS, and p is the number of CRS ports.

# 3	Conclusion

- Proposal:

- The RSS EPRE relative to CRE EPRE is defined as ce-RSS-PowerBoost-Config +10log10p×min⁡(ρAd,ρBd), where ρAd and ρBd are the default values for ρA and ρB relative to CRS EPRE assuming the same transmitted power for symbols with or without CRS, and p is the number of CRS ports.

# Reference

[1] 3GPP TS36.213.