3GPP TSG-RAN WG1 Meeting #96	R1-1902389

Athens, Greece 25 February – 01 March 2019

Agenda item:	7.2.4.1.4

Title:	Discussion on UE autonomous RA in NR-V2X

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

For UE autonomous resource allocation mode, generally referred as Mode 2 in NR sidelink, operation of sub-modes 2(a)/(c)/(d) were discussed in RAN1-AH#1901 meeting. Some top-level progress was achieved in terms of sensing and resource (re)selection under Mode 2. And at some level, a basic working principal of how sub-mode 2(d) would be supported in NR-V2X Rel-16 is also taking form. A full list of agreements reached during the meeting [1] is listed below for convenience.

Agreements: - Mode-2 supports the sensing and resource (re)-selection procedures according to the previously agreed definitions. - FFS resource granularity for sensing & resource (re)-selection, e.g., PRB(s), slots, resource patterns (when applicable), etc. - FFS detailed conditions when these procedures can apply Agreements: - For the purpose of performance evaluation for Mode-2(c), the following Mode-2(c) transmission pattern selection is used when a UE is configured with a pool of patterns: - Sensing based pattern selection (e.g. UE selects unused pattern based on sensing results) - Additional information to assist pattern selection is not precluded, e.g., by using UE geographical location information Agreements: - Sub-channel based resource allocation is supported for PSSCH - FFS details for sub-channels - FFS other use cases for sub-channel (e.g., measurement, interaction with PSCCH, etc.) - Agreements: - SCI decoding applied during sensing procedure provides at least information on sidelink resources indicated by the UE transmitting the SCI - Agreements: - At least for the purpose of evaluation, in Mode-2(d), at least for group operation, a member UE transmits on resources configured by another UE (S-UE) within the same group - High layer signaling is assumed between S-UE and a member UE

In this contribution, we focus on sub-modes 2(a)/(d) operations only and discuss about related aspects, FFS items identified during RAN1-AH#1901 meeting and possible enhancement from the LTE-V2X mode 4 operation. In addition, we also discuss about the need to support mixed-mode operation in NR-V2X.

2. Discussion

2.1 Sub-mode 2(a) UE autonomously (re)select sidelink resource for transmission

Sensing + SPS for periodic traffic

Under sub-mode 2(a) for UE autonomous (re)selection of SL resource for transmission, the same basic working principle as LTE-V2X mode 4, where a UE first perform sensing before resource selection and reservation in a SPS manner, would seem still appropriate at least for periodic traffic transmissions in NR-V2X when the traffic pattern is predictable and the reservation can be easily understood by other sensing UEs to avoid/minimize Tx collisions. However, in the LTE-V2X mode 4 resource selection procedure, it adopted a hard resource exclusion criterion where a SL resource must be excluded from a set of candidate resources for selection if it has been already reserved and the corresponding PSSCH-RSRP measurement result is higher than a threshold. As a consequence, some candidate SL resources or a set of resources could not be selected by a UE due to only a single or just a few resources down the line (in the future) that have already been reserved by others, and thus limiting number of resource candidates for selection and resulting in poor SL resource utilization even though CBR could be measured at a reasonable level.

To improve from this, some enhancement or modifications to the existing sensing + SPS resource selection procedure should be considered, such as SL resource pre-emption. In the SL resource pre-emption, it allows a UE with high priority packets to select SL resources that have already been reserved by other UEs with lower priority packets and “pre-empt these resources” by sending its SL resource reservation in advance so that lower priority packet UEs have sufficient time to perform resource reselection for the pre-empted resources. This type of new behaviour is expected to be useful for medium to heavy congested resource pools, but some restrictions (e.g. no consistent pre-emption/take-over) should be imposed so to minimize negative impact to the original resource reserving UEs with lower priority packets.

Observation 1: Same basic working principle in LTE-V2X mode 4, where a UE first perform sensing before resource selection and reservation in a SPS manner, would still be appropriate for at least periodic traffic transmissions in NR-V2X sub-mode 2(a).

Proposal 1: Enhancement to sub-mode 2(a) to incorporate resource pre-emption by reservation as part of sensing and resource selection procedure would help to secure SL resources for transmitting higher priority messages and to improve utilization of SL resources.

For periodic traffic, in addition, according to the normative requirements specified in [2], the minimum set of reservation periods that should be supported in NR-V2X are {10ms, 20ms, 33ms, 100ms, 500ms}. These, however, do not have to correspond to a set of packet latency requirement, which are normally needed for setting the T2 value {3ms, 5ms, 10ms, 20ms, 25ms, 50ms, 100ms, 500ms}. For future prove or for compatibility to LTE-V2X, some additional reservation periods can be considered. Based on this, the longest/max reservation interval is 500ms in NR-V2X, then the sensing period/window should be also 500ms.

Proposal 2: The minimum set of reservation periods that should be at least supported in NR-V2X are {10ms, 20ms, 33ms, 100ms, 500ms}. FFS: if additional reservation periods are needed for the purposes of future provision and/or compatibility to LTE-V2X.

- For other aspects of sub-mode 2(a) operation, such as information that need to be extracted from SCI decoding should include at least the same set of information as in LTE mode 4. That is, reservation periodicity for periodic traffic, and time/frequency location of SL resources for initial transmission and re-transmissions – if supported. In terms of sidelink measurements, this could also follow the same as in LTE-V2X, i.e. PSSCH-RSRP. On the timescale for aperiodic traffic, the sensing timescale could be much smaller. This is further explained in the next section of this paper. For starter, 50ms can be an option for short term sensing.

Proposal 3: The purpose of SCI decoding during sensing procedure is at least to extract the following information on SL resources that is explicitly indicated by the Tx-UE in PSCCH:

- Time/frequency location(s) and number of PSSCH sub-channels for initial and re-transmission(s)

- Reservation periodicity (for periodic traffic); this could be set to zero for one-shot transmission (aperiodic traffic)

- For NR-V2X, it is already agreed to support SFCI reporting from the Rx-UE to the Tx-UE as part of NR sidelink operation. Therefore, relevant SFCI that should be extracted during the sensing procedure and subsequently used in resource selection could include ACK/NACK feedback (CBG level if supported), CQI/MCS level, and possibly power control indications for deciding whether to increase/decrease Tx power or changing MCS level/size of time/frequency resources. In addition, the sensing procedure could also include assistance and/or scheduling information from another UE (e.g. group header), as these information influence resource selection outcome for the target UE.

- For SL resource (re)selection, also same as LTE-V2X mode 4, the timescale and conditions should include at least packet latency period, end of resource reservation period, change of message TB size, change of MCS selection, new/additional data TB arrival, and CBR level. In addition, as discussed earlier on SL pre-emption operation to further protect / enhance reliability of high priority packet delivery, if a reserved SL resource(s) is pre-empted by another UE, the affected UE should re-select a different SL resource(s) if the pre-emption is detected for another higher priority transmission. If considering the operation of SL resources being scheduled by another UE, other conditions for triggering resource (re)selection should include also resource feedback/suggestion from the Tx UE and addition/removal of one or more group member UEs.

- Observation 2: One sidelink measurement that would be useful for Mode 2 operation and should continue to be supported for NR-V2X is PSSCH-RSRP.

- Observation 3: Relevant SFCI information that should be part of sensing procedure and also be taken into account during SL resource (re)selection would include ACK/NACK feedback (CBG level if supported), CQI/MCS level, and possibly power control indications.

- Observation 4: The timescale and conditions for SL resource (re)selection should include at least packet latency period, end of resource reservation period, change of message TB size, change of MCS selection, new/additional data TB arrival, and CBR level.

- Proposal 4: When a reserved SL resource(s) is pre-empted by another UE, the affected UE should re-select a different SL resource(s) if the pre-emption is detected for another higher priority transmission.

- Proposal 5: For the operation of SL resources being scheduled by another UE, other conditions for triggering resource (re)selection should include resource suggestion feedback from the Tx UE to the scheduling UE and the addition/removal of one or more group member UEs.

Since sub-channel based RA is supported for PSSCH, it should be used as a basic unit for SL sensing, resource (re)selection and measurements.

- Time duration of a PSSCH sub-channel should be exactly one slot. Slot aggregation can be supported and used when it is necessary (e.g. large packets, limited SL resources in the frequency domain, latency tolerate transmissions).

- Frequency size (x Hz) of a PSSCH sub-channel can be left FFS for now as it depends on the minimum packet size that should be supported. In addition, this could be SCS dependent as well.

- Proposal 6: Since sub-channel based RA is supported for PSSCH, it should be used as a basic unit for SL sensing, resource (re)selection and measurements. In addition, Time duration of a PSSCH sub-channel should be exactly one slot. FFS on the frequency size.

- As for PSFCH resource selection / re-selection, it depends on whether there is a fixed relationship between PSCCH/PSSCH and PSFCH resources. If a fixed relationship is defined, e.g. PSFCH is always after PSSCH in a self-contained TTI, PSFCH resource is determined based on the (re)selection of PSSCH resource. If PSFCH resource is to be decided by the Rx UE, then the selection of resource should be based on sensing operation by the Rx UE and may be also confined by a latency requirement which should be indicated to the Rx UE from the Tx. If PSFCH resource is not dependent or associated with PSSCH and it is to be decided by the Tx UE, then the selection of resource should be based on sensing operation by the Tx UE. If the Rx UE is located in another cell, information relating to transmission resource pool from the Rx UE should be provided to the Tx-UE.

- Observation 5:

- If there is a fixed relationship between PSCCH/PSSCH and PSFCH resources (e.g. in a self-contained slot/TTI), PSFCH resource would be determined based on the (re)selection of PSSCH resource.

- If PSFCH resource is decided by the Rx UE, then the selection of resource should be based on sensing operation by the Rx UE and confined within a latency requirement which is to be indicated to the Rx UE from the Tx UE.

- If PSFCH resource is decided by the Tx UE, then the selection of resource should be based on Tx UE sensing. If he Rx UE is located in another cell, information relating to transmission resource pool from the Rx UE should be provided to the Tx.

One-shot transmission for aperiodic traffic

For aperiodic traffic, In LTE-V2X mode 4, it is already supported for UEs to perform one-shot transmission with sensing. In a way, it is almost the same mechanism as sensing + SPS. The only difference in this case the transmission is only for one-shot and resource reservation field in the SCI is set to zero. Although the difference is small, but overall implication to the packet reception and Tx collision is quite different. That is, in the case of SPS, the reservation of periodic SL resources can be detected and known to other UEs performing sensing. But for the one-shot transmission, it is not possible for others to know in advance another UE’s intention to transmit. Therefore, the risk of Tx collision will be higher than the periodic reservation (SPS) case.

To resolve this, pre-emption type of resource booking should be considered for aperiodic transmissions in NR-V2X mode 2(a). For each UE, up on selection of a target PSSCH resource(s) to be used for message transmission, the Tx-UE sends in advance a pre-emption message (e.g. carried in PSCCH) to indicate and book the target PSSCH resource(s). In order to ensure the target PSSCH resource booking is received by others, the pre-emption message can be transmitted twice in two PSCCH resources that are separated in time and frequency, in case some UEs could not receive the message in the first transmission due to half-duplex issue or Tx collision with other pre-emptions. In addition, we also see pre-emption type of resource booking can be quite beneficial for high priority packet transmissions where scarcity in number of unused/empty resources often occurs with a short latency requirement.

Since the traffic is one-shot / aperiodic in nature and does not have a fixed periodicity (no resource reservation needed), most likely the UE would transmit the message upon its arrival as soon as there is an available SL resource. Therefore, the gap between pre-emption messages and the selected target SL resource(s) should not be far apart and within the latency limit. As such, the sensing period for resource pre-emption would not need to be too long either to save UE power consumption and memory buffer. In the following, we evaluated this “short term sensing” for PSSCH resource pre-emption for both periodic and aperiodic types of traffic and also evaluated various pre-emption schemes as described in the following.

- Short term sensing (STS) baseline scheme: It’s same working principle as LTE-sidelink mode 4 sensing and resource selection with a SA reading/sensing period prior to packet generation/arrival from the upper layers and a resource selection window after the packet generation/arrival. In this baseline scheme, the SA reading/sensing period is set at 50ms and the resource selection window is also set at 50ms for performance evaluation. Based on SA reading/sensing results, first a set of available PSSCH resources are determined within the resource selection window and a PSSCH resource is randomly selected among all of the available resources for data transmission. Then a PSCCH resource is also randomly selected for the pre-emption indication (SCI) within the resource selection window but before the selected PSSCH resource.

- Short term sensing (STS) within Xms: Same as the above STS-baseline scheme, where a PSSCH resource is randomly selected among the set of available / unused resources within the selection window. But after the random selection of the PSSCH resource, the associated PSCCH resource is randomly selected and the pre-emption indication (SCI) is transmitted as soon as possible, within Xms. In the following simulation evaluation, X is set to 5ms.

- Short term sensing with two PSCCH transmissions: Same as the above STS-baseline scheme, a PSSCH resource is randomly selected among the set of available / unused resources within the selection window. But the associated PSCCH / pre-emption indication is repeated and transmitted twice before the PSSCH to improve detection performance, instead of once in the baseline scheme.

- Short term sensing (STS) continuous sensing: Same as the above STS-baseline scheme, a PSCCH and a PSSCH resource are randomly selected among the set of available / unused resources within the selection window. Furthermore, UE continue to perform SA reading and sensing before the associated PSCCH / pre-emption SCI is transmitted. If potential Tx collision is detected (e.g. a pre-emption SCI is received from another UE indicating the same or overlapping the selected PSSCH resource), PSSCH resource is re-selected among the remaining available resources.

Figure 1: PRR results for SL resource pre-emption with short term sensing for periodic traffic (highway)

Figure 2: PRR results for SL resource pre-emption with short term sensing for aperiodic traffic (highway)

Main results observations:

- From the results shown in Figure 1 and Figure 2, it is seen the baseline scheme of short term sensing (STS-Baseline) can already provide enhanced performance over schemes that are based on LTE SA decoding (no pre-emption) and random resource selection for aperiodic traffic and a comparable performance to the LTE SA decoding scheme for periodic traffic. It is therefore, suitable for use in both periodic and aperiodic broadcast types of traffic, at least in high vehicle speed.

- In Figure 2 for aperiodic traffic type, various further enhanced STS schemes for PSSCH pre-emption are additionally evaluated. As shown, they all provided improved performance over the STS-baseline scheme. Numerically, when compared with the existing LTE SA decoding and random selection based schemes, they provided the following PRR performance gain at 500 meters range as

  - The STS-baseline scheme provided 6.4% PRR improvement

  - STS-scheme with two pre-emption repeatedly transmitted provided 12.3% improvement

  - STS-scheme with pre-emption SCI transmitted within 5ms provided 18.6% improvement

  - STS-scheme with continuous sensing provided 22.1% improvement

Observation 6: From the evaluation results of SL pre-emption with short term sensing based PSSCH resource selection scheme, the scheme can provide a large performance gain in PRR over the existing LTE SA decoding and random selection at least for aperiodic traffic type, and a comparable performance to the LTE SA decoding scheme for the periodic traffic.

Observation 7: By continuous sensing of other SCI messages before pre-emption SCI transmission, PRR results showed the performance can be further improved by up to 22% at least with aperiodic traffic.

Proposal 7: It is proposed to support SL resource pre-emption with short term sensing based resource selection scheme and further evaluate its performance in other scenarios e.g. Urban Option A and 30GHz frequency range.

2.2 Sub-mode 2(d): One UE schedules SL resources for other UEs

##### For sub-mode 2(d), the most applicable use case or scenario is unicast and groupcast type of communication/transmission, where a group header UE (scheduling UE) would coordinate SL resources for remaining group members. One motivation to introduce this type of resource assignment mechanism is to avoid Tx collisions at least among the member UEs within the same group. In addition, the group header/scheduling UE is able to also avoid or minimize half-duplex problem, where more than one group member UEs are transmitting at the same time and not being able to receive one another’s messages. However, if the group header is to provide dynamic scheduling and assigning SL resources for all group member UEs individually to transmit their messages, this will incur a very significant signalling overhead of exchanging SL scheduling requests and buffer status reports from group member UEs and responding with SL scheduling assignments from the group header. This, by itself, would have the same Tx collision and half-duplex challenges that we were trying to solve in the first place for unicast and groupcast communications. Therefore, it is observed the degree of assistance from the group header should be at the coordination level and quasi-statically to avoid any heavy signalling overhead of group member UEs sending their SRs/BSRs and group header responding with SL grants.

In order for the group header to schedule SL transmissions among all group member UEs (including itself), one way is to coordinate TDM’ed SL resources or Tx timing to each member UE to improve hear-ability (half duplex issue) and avoid Tx collision among them. It is then up to the individual member UE to decide when and which of the allocated Tx timing slots to use for transmitting own messages. One way to allocate/pre-assign Tx opportunities and evenly distribute them among all UEs is for each UE to take turn in SL transmission. Alternatively, the group header can configure via RRC signalling a slot bitmap to each UE (e.g. based on BSR / traffic needs), indicating Tx opportunities for which the UE can use to perform SL transmissions. And such indication signaling of TDM resources or rules can be delivered via PSSCH from the group header to all group member UEs. If a UE is involved in multiple unicast / groupcast sessions at the same time and getting SL resource coordination, the UE could potentially receive conflicting/colliding Tx timings from different scheduling UEs or network scheduled UL transmissions or SL broadcast. In another scenario, the conflicting Tx timing could be due to limited UE capability. In any of these cases, the UE should be allowed to provide feedback suggestions of its preferred Tx opportunities/timings to scheduling UE(s) for consideration and subsequently the S-UE re-assign/allocate a new set of resources to resolve any scheduling/coordination conflict (e.g. assigned with different Tx opportunity/timing for different unicast/groupcast sessions).

Observation 8: In sub-mode 2 (d), it is observed the degree of scheduling from a group header UE to group member UEs within a unicast or a groupcast session should be at the coordination level quasi-statically to avoid heavy signalling overhead of group member UEs sending SRs/BSRs and the group header responding with SL grants.

Proposal 8: For a group header UE to schedule SL resources to group member UEs, the group header should coordinate TDM’ed Tx timings within the group so to avoid Tx collisions and half-duplex problem. And the Tx timing slots of TDM’ed resources can be represented by a slot bitmap and indicated from the group header to group member UEs via RRC signalling in PSSCH.

Proposal 9: If a group member UE receives conflicting / colliding Tx opportunities/timings from different scheduling UEs, the member UE should be allowed to feedback and report to the scheduling UE(s) suggesting a preferred set of Tx opportunities / timings.

- The procedure(s) to become or to serve as a scheduling UE is fundamentally a higher layer decision. At the PHY layer, it does not make much of a difference which UE within a group is the scheduling UE in sub-mode 2(d) operation. At most, the scheduling UE may need to have additional processing or hardware capabilities than group member UEs, e.g. number of Tx/Rx processing chains or scheduling functionality, but these would have no impact to the procedure of how sub-mode 2(d) would operate in L1. In certain operating scenario, it makes sense that the scheduling UE within a unicast/groupcast session should be configured by gNB when all group member UEs are in-coverage, in some scenarios it is perfectly reasonable for the application layer to appoint a scheduling UE in vehicle platooning, and in another scenario it is also necessary for the receiver UE to schedule transmissions of the Tx UE during the session when the Rx UE is in another cell or the Rx UE involves in more than one sub-mode 2(d) sessions and the feedback is necessary to avoid conflicts in scheduling decision. However, the procedure to determine or appoint the scheduling UE within a group has little or no impact to sub-mode 2(d) operation in L1 and as such it should be decided by higher layers and indicated the decision to L1.

- Observation 9: The decision or procedure(s) to become or to serve as a scheduling UE involves depending on operating scenario gNB, application layer, and groupcast session setup. As such should be a higher layers decision and inform to the radio layer (L1) if necessary.

Proposal 10: The decision to become/serve as a scheduling UE should be made by upper layers and discussed in other working group(s). The radio layer can be informed of the decision made by upper layers if necessary.

Functionalities of initialization of sub-mode 2(d) operation are somewhat unclear from L1’s perspective. For much of an extend, before a scheduling UE can give the first scheduling grant or coordination to a member UE, the initialization stage involves at least the procedure of setting up a groupcast session, determining group member UEs and the scheduling UE, assigning groupcast/unicast session, source and destination IDs, and determining a set of sidelink resources a scheduling UE can use for scheduling of other UEs (arguable if this would also be considered as part of scheduling procedure). Once again, all of these functionalities are upper layers decisions and the outcomes would be indicated to L1. However, during the setup / initialization stage of sub-mode 2(d), all of the necessary communication / exchange of information over SL would still need to be carried out in sub-mode 2(a) at least for UEs in RRC idle or out-of-coverage. One would then reasonably consider if sub-mode 2(d) can be a standalone sub-mode and ask if it could be a sub-feature of sub-mode 2(a). In our opinion, if supported, mode 2(d) would not be a standalone operating mode. It should be considered as a sub-functionality of mode 2(a) with somewhat modified/simplified sensing and resource (re)selection procedures. Full mode 2(a) operation should be performed in the following cases:

  - Initialisation stage (setting up) of a groupcast / unicast session

  - After termination of a groupcast / unicast session

  - RLF of a groupcast / unicast session or losing connection to the group header (e.g. header disappeared)

Observation 10: Much of functionalities of sub-mode 2(d) initialization are upper layers procedures and their outcomes or decisions should be indicated to L1.

Observation 11: During the setup / initialization stage of sub-mode 2(d), after termination of a groupcast/unicast session and in the event of RLF or loss of connection to the group header, sub-mode 2(a) would be needed to carried out all the necessary exchange of information over SL.

Proposal 11: Sub-mode 2(d) is not be a standalone operating mode. It should be considered as a sub-functionality of sub-mode 2(a) with somewhat modified/simplified sensing and resource (re)selection procedures.

Proposal 12: Initialization of sub-mode 2(d) operation should be decided by upper layers and discussed in other working group(s).

2.2 Mixed operating modes

For NR-V2X operation, it is envisioned that a NR-V2X capable UE may involve simultaneously in different application and services. As such, it is likely for a UE to transmit broadcast messages and at the same time be involved in unicast and/or groupcast session(s) concurrently. And therefore, it is reasonable to assume that a UE should at least be operating in both sub-mode 2(a) for broadcast transmissions and sub-mode 2(d) for unicast/groupcast where its transmissions and resource selection are coordinated by another UE.

If extending from the above thinking, then the following “mixed mode” operations of a network scheduling mode + a UE autonomous selection mode seems logical as well:

- LTE mode 3 + NR sub-mode 2(a)

- LTE mode 4 + NR mode 1

- NR mode 1 + NR sub-mode 2(d)

In fact, for the first two mixed modes (LTE mode 3 + NR sub-mode 2(a)) and (LTE mode 4 + NR mode 1), they should be supported by default. For NR mode 1 + NR sub-mode 2(d), it would be beneficial to support this as well for the same reasons of supporting the first two mixed modes operation. Additionally, it would be very difficult to resolve all technical challenges at least in Rel-16 to support unicast and groupcast communications in NR mode 1 when some UEs are in-coverage and some OOC, group member UEs belong to different cells, and possibly different PLMNs with different sync timings.

Proposal 13: For a NR sidelink capable UE, it is likely need to support and operate in different advanced V2X applications and broadcast/unicast/groupcast SL communication types at the same time. And therefore, mixed modes of operation between a network scheduling mode and UE autonomous selection mode seems to be necessary and should be supported in Rel-16.

3. Conclusion

In this contribution, we discussed about related aspects for some of the potential sub-modes within mode 2 that were identified in the last meeting. In summary, we have the following observations and proposals:

Observation 1: Same basic working principle in LTE-V2X mode 4, where a UE first perform sensing before resource selection and reservation in a SPS manner, would still be appropriate for at least periodic traffic transmissions in NR-V2X sub-mode 2(a).

Proposal 1: Enhancement to sub-mode 2(a) to incorporate resource pre-emption by reservation as part of sensing and resource selection procedure would help to secure SL resources for transmitting higher priority messages and to improve utilization of SL resources.

Proposal 2: The minimum set of reservation periods that should be at least supported in NR-V2X are {10ms, 20ms, 33ms, 100ms, 500ms}. FFS: if additional reservation periods are needed for the purposes of future provision and/or compatibility to LTE-V2X.

Proposal 3: The purpose of SCI decoding during sensing procedure is at least to extract the following information on SL resources that is explicitly indicated by the Tx-UE in PSCCH:

- Time/frequency location(s) and number of PSSCH sub-channels for initial and re-transmission(s)

- Reservation periodicity (for periodic traffic); this could be set to zero for one-shot transmission (aperiodic traffic)

- Observation 2: One sidelink measurement that would be useful for Mode 2 operation and should continue to be supported for NR-V2X is PSSCH-RSRP.

- Observation 3: Relevant SFCI information that should be part of sensing procedure and also be taken into account during SL resource (re)selection would include ACK/NACK feedback (CBG level if supported), CQI/MCS level, and possibly power control indications.

- Observation 4: The timescale and conditions for SL resource (re)selection should include at least packet latency period, end of resource reservation period, change of message TB size, change of MCS selection, new/additional data TB arrival, and CBR level.

- Proposal 4: When a reserved SL resource(s) is pre-empted by another UE, the affected UE should re-select a different SL resource(s) if the pre-emption is detected for another higher priority transmission.

- Proposal 5: For the operation of SL resources being scheduled by another UE, other conditions for triggering resource (re)selection should include resource suggestion feedback from the Tx UE to the scheduling UE and the addition/removal of one or more group member UEs.

- Proposal 6: Since sub-channel based RA is supported for PSSCH, it should be used as a basic unit for SL sensing, resource (re)selection and measurements. In addition, Time duration of a PSSCH sub-channel should be exactly one slot. FFS on the frequency size.

- Observation 5:

- If there is a fixed relationship between PSCCH/PSSCH and PSFCH resources (e.g. in a self-contained slot/TTI), PSFCH resource would be determined based on the (re)selection of PSSCH resource.

- If PSFCH resource is decided by the Rx UE, then the selection of resource should be based on sensing operation by the Rx UE and confined within a latency requirement which is to be indicated to the Rx UE from the Tx UE.

- If PSFCH resource is decided by the Tx UE, then the selection of resource should be based on Tx UE sensing. If he Rx UE is located in another cell, information relating to transmission resource pool from the Rx UE should be provided to the Tx.

Observation 6: From the evaluation results of SL pre-emption with short term sensing based PSSCH resource selection scheme, the scheme can provide a large performance gain in PRR over the existing LTE SA decoding and random selection at least for aperiodic traffic type, and a comparable performance to the LTE SA decoding scheme for the periodic traffic.

Observation 7: By continuous sensing of other SCI messages before pre-emption SCI transmission, PRR results showed the performance can be further improved by up to 22% at least with aperiodic traffic.

Proposal 7: It is proposed to support SL resource pre-emption with short term sensing based resource selection scheme and further evaluate its performance in other scenarios e.g. Urban Option A and 30GHz frequency range.

Observation 8: In sub-mode 2 (d), it is observed the degree of scheduling from a group header UE to group member UEs within a unicast or a groupcast session should be at the coordination level quasi-statically to avoid heavy signalling overhead of group member UEs sending SRs/BSRs and the group header responding with SL grants.

Proposal 8: For a group header UE to schedule SL resources to group member UEs, the group header should coordinate TDM’ed Tx timings within the group so to avoid Tx collisions and half-duplex problem. And the Tx timing slots of TDM’ed resources can be represented by a slot bitmap and indicated from the group header to group member UEs via RRC signalling in PSSCH.

Proposal 9: If a group member UE receives conflicting / colliding Tx opportunities/timings from different scheduling UEs, the member UE should be allowed to feedback and report to the scheduling UE(s) suggesting a preferred set of Tx opportunities / timings.

- Observation 9: The decision or procedure(s) to become or to serve as a scheduling UE involves depending on operating scenario gNB, application layer, and groupcast session setup. As such should be a higher layers decision and inform to the radio layer (L1) if necessary.

Proposal 10: The decision to become/serve as a scheduling UE should be made by upper layers and discussed in other working group(s). The radio layer can be informed of the decision made by upper layers if necessary.

Observation 10: Much of functionalities of sub-mode 2(d) initialization are upper layers procedures and their outcomes or decisions should be indicated to L1.

Observation 11: During the setup / initialization stage of sub-mode 2(d), after termination of a groupcast/unicast session and in the event of RLF or loss of connection to the group header, sub-mode 2(a) would be needed to carried out all the necessary exchange of information over SL.

Proposal 11: Sub-mode 2(d) is not be a standalone operating mode. It should be considered as a sub-functionality of sub-mode 2(a) with somewhat modified/simplified sensing and resource (re)selection procedures.

Proposal 12: Initialization of sub-mode 2(d) operation should be decided by upper layers and discussed in other working group(s).

Proposal 13: For a NR sidelink capable UE, it is likely need to support and operate in different advanced V2X applications and broadcast/unicast/groupcast SL communication types at the same time. And therefore, mixed modes of operation between a network scheduling mode and UE autonomous selection mode seems to be necessary and should be supported in Rel-16.

4. References

[1]	RAN1-AH#1901 Chairman’s notes (final version)

[2]	3GPP TS22.186, “Enhancement of 3GPP support for V2X scenarios”, V15.3.0, June 2018