3GPP TSG RAN WG1 Meeting #96	R1-1902491

February 25th – March 1st, 2019

Athens, Greece

Agenda item:	7.2.5.3

Source: 	Intel Corporation

Title: 	Remaining details of RIM-RS configuration

Document for:	Discussion and decision

# Introduction

In RAN1 AH1901 meeting, the following agreements regarding RIM-RS resource configuration were made [1]:

- OAM configures RIM-RS configurations and provide necessary additional configuration information for one or more of functionalities of RIM-RS.

  - RIM-RS configuration refers to the parameter set that includes sequence, time-frequency resource, periodicity, etc.

- FFS: detailed signalling for RIM-RS configuration for repetition and near/far functionality indication,

  - FFS: How to support repetition by configuration, as well as if explicit indication of near/far functionality is needed

- Specification defines unique RIM-RS indices for indication of RIM-RS resources (in time-domain, frequency-domain, and sequence).

- Specification provides configurable mapping rule between the RIM-RS indices and Set ID;

- A gNB can be configured with multiple set IDs.

  - For each set ID, multiple RIM-RS configurations can be configured.

- From the same transmitting gNB perspective, RS-1 and RS-2 differentiation is done in time domain only.

- For different gNB in a network, RS-1 and RS-2 differentiation is done in time domain only.

- For indication of either “enough mitigation” or “not enough mitigation”, two basic RIM-RS resources are used

  - The two resources use the same time/frequency resource but different sequences.

- For repetition/near-far-functionality, multiple basic RIM-RS resources for a gNB transmission can be configured only in consecutive TDD DL/UL switching period, with one basic RIM-RS resource per TDD DL/UL switching period

  - The maximum number of consecutive TDD DL/UL switching periods for repetition/near-far-functionality is 8 (the number can be either 2, 4, or 8) with near-far functionality and with repetition

    - The first half of consecutive TDD DL/UL switching periods is for repetition.

    - The second half of consecutive TDD DL/UL switching periods is for repetition.

    - Between the first and the second half of consecutive TDD DL/UL switching periods is for near-far-functionality.

  - The maximum number of consecutive TDD DL/UL switching periods for repetition is 4 (the number can be either 1, 2, or 4) without near-far functionality and with repetition only.

  - The maximum number of consecutive TDD DL/UL switching periods is 2 with near-far functionality only.

- For each basic RIM-RS resource, the OFDM symbol position within the TDD DL/UL switching period can be individually configured.

- RIM-RS sequence is mapped in the frequency with respect to a common reference point for all gNBs. The reference point is configured by OAM.

- The RIM RS bandwidth is defined as the number of PRBs..

- The initialization seed for RIM-RS is configured by OAM and the subsequent seed will be changing over time.

- Maximum 22 bit gNB set ID is supported. Actual number of gNB set ID is configurable.

- The number of RIM RS sequence (CDM) in each DL/UL switching points is configurable and common for all the DL/UL switching points transmitting RIM RS-1.

- The number of RIM RS sequence (CDM) in each DL/UL switching points is configurable and common for all the DL/UL switching points transmitting RIM RS-2.

- The maximum number of total RIM RS sequence within 10ms is 32 regardless how many DL/UL switching points are configured in the 10ms.

In this contribution, we discuss the remaining details of the RIM-RS resource configuration regarding unique indexing of RIM-RS resources, initialization seed configuration with change over time and mapping rules for set IDs, RS-1/RS-2 and mitigation sufficiency.

# RIM-RS configuration overview

For RIM-RS configuration, each gNB is configured with a set of global parameters and multiple basic RIM-RS resource configurations. We provide an overview in Table 1 of all parameters for RIM-RS configuration and we discuss the details in the subsequent sections.

Table 1. Overview of RIM-RS configuration parameters.

| Global parameters | - RIM-RS transmission period Pt. - Frequency pool: list of NF frequency locations with respect to a configured reference frequency point. - Sequence pool: list of NS sequences identified by their (base) initialization seeds. - Reference frequency point. - RIM-RS subcarrier spacing. - The number T' of TDD DL/UL periods assigned to RS-1 where T'≤Pt. - If T'=Pt is configured, it means that there is no RS-2. - For RS-1: - Indication σ of whether to differentiate “enough mitigation” and “not enough mitigation”: OFF(σ=0)/ON(σ =1). - Indication φ1 of whether near-far functionality is enabled: OFF(ϕ1=0)/ON(ϕ1=1). - The number of consecutive TDD DL/UL periods for repetition/near-far: R1. - The number of RIM-RS sequences in one TDD DL/UL period: S1 where S1 ≤ NS. - For RS-2: - Indication of whether near-far functionality is enabled: OFF(ϕ2=0)/ON(ϕ2=1). - The number of consecutive TDD DL/UL periods for repetition/near-far: R2. - The number of RIM-RS sequences in one TDD DL/UL period: S2 where S2 ≤ NS. - Note: Rm∈{1,2,4,8} for m=1,2. |
| --- | --- |
| For each basic RIM-RS resource | - RIM-RS resource configuration index IRIM-RS, which uniquely identifies - Time index it to indicate which TDD DL/UL period within the RIM-RS transmission period to be used. - Frequency index if to indicate which frequency location from the “frequency pool” is to be used. - Sequence index is to indicate which sequence from the “sequence pool” is to be used. - Symbol offset within the TDD DL/UL period. |

We remark that the global RIM-RS transmission periodicity can be calculated as

Pt=⌊2σN1sets/(NFS1)⌋⋅R1+⌊2N2sets/(NFS2)⌋⋅R2

where N1sets and N2sets The total number of victim set IDs (for RS-1) and the total number of aggressor set IDs (for RS-2), respectively, in the entire network. The value of σ is 0 if the indicator of whether to differentiate “enough mitigation” and “not enough mitigation” for RS-1 is OFF and σ is 1, otherwise. However, the gNBs shall be configured directly the RIM-RS transmission periodicity with Pt and it is up to the operator to make the calculation above.

# Unique RIM-RS resource indices

It was agreed that the specification defines unique RIM-RS indices for indication of RIM-RS resources (in time-domain, frequency-domain, and sequence). We discuss next how to define such unique indices. With the global variables Pt, NF and NS, a basic RIM-RS resource can be uniquely identified by the time-frequency-sequence tuple (it, if, is) where ∈{0,…, Pt-1},  if∈{0,…, NF} and  is∈{0,…, NS}. The time-frequency-sequence tuple can be consolidated into one number by defining a RIM-RS resource configuration index IRIM-RS as follows:

IRIM-RS=if⋅Pt⋅NS+is⋅Pt+it

which can uniquely identify a basic RIM-RS resource. To support a set ID up to 22 bits in addition to RS-1/RS-2 indication (1 bit) and “not enough mitigation”/ “enough mitigation” indication (1-bit), a total of 24 bits is sufficient for the RIM-RS resource configuration index IRIM-RS. The time-frequency-sequence tuple (it, if, is) can be recovered from the RIM-RS resource configuration index IRIM-RS using the following expressions:

if=IRIM-RSNSPtmod NF,

it=IRIM-RSPtmod NS,

it=IRIM-RS mod Pt.

Define a RIM-RS resource configuration index IRIM-RS of size up to 24 bits to uniquely identify a basic RIM-RS resource in time-domain, frequency-domain and sequence.

# Mapping rules

The RIM-RS conveys three pieces of information: 1) whether RS is transmitted by a victim or an aggressor, i.e., RS-1/RS-2 indication, 2) “Not enough mitigation”/ “Enough mitigation” for RS transmitted by a victims and 3) set ID. The aforementioned information is encoded in the time index, frequency index and sequence index of a RIM-RS resource. To provide all gNBs with a common understanding, the specification should include mapping rules describing how the information is encoded in the time-frequency-sequence indices. Namely, three mapping rules should be specified:

- Mapping rule from RIM-RS resource time-frequency-sequence tuple to the RS-1/RS-2 indication.

- Mapping rule from RIM-RS resource time-frequency-sequence tuple to “Not enough mitigation” / “Enough mitigation” for victims.

- Mapping rule from RIM-RS resource time-frequency-sequence tuple to the set ID.

We argue that the first two mapping rules (for RS-1/RS-2 and “Not enough mitigation”/ “enough mitigation”) are more important than the third mapping rule (for set ID) because a gNB may behave differently based on whether a detected RIM-RS is RS-1 from victim or RS-2 from aggressor and whether a victim is signalling that mitigation is sufficient or not. In contrast, gNB does not necessarily need to know which set ID is associated with a detected RIM-RS, especially that there is no direct communication (on Xn interface) between gNBs even in Framework 2. Instead, the backhaul signalling between gNBs is done via the core network according to the WID [2]. We discuss next the three mapping rules.

## Mapping rule for RS-1/RS-2 indication:

It was agreed in RAN1 AH1901 that, from the same transmitting gNB perspective, RS-1 and RS-2 differentiation is done in time domain only. In order to minimize the average detection delay of victims (RS-1), all RS-1 from all gNBs should be transmitted first before any transmission RS-2 from any gNB. This comes at the expense at increasing the average detection delay of aggressor RS, which is not an issue because the aggressor RS is used merely to convey that the atmospheric duct still exists. A by-product of such arrangement is that RS-1/RS-2 indication can be conveyed by a simple rule: time index it<T' indicates RS-1, whereas time index it≥T' indicates RS-2 where T' is configurable. A reasonable value for T' is T'=⌊2σN1sets/(NFS1)⌋⋅R1 where the value of σ is 0 if the indicator of whether to differentiate “enough mitigation” and “not enough mitigation” for RS-1 is OFF and σ is 1, otherwise.

RS-1 and RS-2 are differentiated in time domain such that RIM-RS with time index it<T' indicates RS-1, whereas RIM-RS with time index it≥T' indicates RS-2 where T' is configurable.

## Mapping rule for mitigation sufficiency indication

It was agreed in the previous meeting that, a gNB differentiates RIM-RS for “enough mitigation” from “not enough mitigation” only via different sequences. There are several mapping rules that can accomplish such goal. A simple mapping rule is that the least significant bit (LSB) of the sequence index is indicates mitigation sufficiency: (is mod 2) = 0 indicates “Not enough mitigation” whereas (is mod 2) = 1 indicates “Enough mitigation”, if the indication for “enough mitigation” or “not enough mitigation” is ON. The simplicity of this rules comes from its independence of any other RIM-RS configuration parameters.

The least significant bit (LSB) of the sequence index is indicates mitigation sufficiency: (is mod 2) = 0 indicates “Not enough mitigation” whereas (is mod 2) = 1 indicates “Enough mitigation”, if the indication for “enough mitigation” or “not enough mitigation” is ON.

## Repetition/near-far-functionality

It was agreed in the previous meeting that multiple basic RIM-RS resources for a gNB transmission can be configured in consecutive TDD DL/UL period for repetition/near-far-functionality such that there is one basic RIM-RS resource per TDD DL/UL switching period. We argue that the number of consecutive TDD DL/UL periods should be the same for all gNBs for each RS type, but can be different for different types. Specifically, the number R1 of consecutive TDD DL/UL periods for repetitions/near-far of RS-1 shall be the same for all gNBs in the network. The number R2 of consecutive TDD DL/UL periods for repetitions/near-far of RS-1 shall be the same for all gNBs in the network. R1 and R2 may be different. The justification for having the same R1 for all gNBs is that it enables the gNBs to easily identify which TDD DL/UL periods are “near” and which ones are “far” for RS-1. For example, if the mapping rule for RS-1/RS-2 indication described above is adopted, then the “near”/“far” TDD periods can be identified as described in Table 2. Similarly, having the same R2 for all gNBs enables the gNBs to easily identify which TDD DL/UL periods are “near” and which ones are “far” for RS-2, also show inTable 2. In contrast, if the number of consecutive TDD DL/UL is allowed to be entirely independent for each, a bitmap indicating “near”/“far” needs to be explicitly configured at each gNB, which seems to be unnecessary overhead.

Table 2. Near-far indication.

| Condition | Near-far |
| --- | --- |
| If near-far functionality is enabled for RS-1 (ϕ1=1) and it/T' = 0 | ⌊2it/R1⌋=0 indicates “near” ⌊2it/R1⌋=1 indicates “far”. |
| If near-far functionality is enabled for RS-2 (ϕ2=1) and it/T' = 1 | ⌊2(it-T')/R2⌋=0 indicates “near” ⌊2(it-T')/R2⌋=1 indicates “far”. |

The reasoning for allowing R1 and R2 to be different is that the grouping of victims may differ from the grouping of aggressors, and therefore, the group size (i.e., the number of gNBs per set ID) for victims can be different than the group size of aggressors leading to different detection probability of victim RS-1 and aggressor RS-2. Consequently, different number of repetitions may be needed for RS-1 and RS-2.

The number R1 of consecutive TDD DL/UL periods for repetitions/near-far of RS-1 shall be the same for all gNBs in the network. The number R2 of consecutive TDD DL/UL periods for repetitions/near-far of RS-1 shall be the same for all gNBs in the network. R1 and R2 may be different.

Configuring repetitions/near-far for RIM-RS can be done by simply assigning consecutive RIM-RS resource configuration index values to multiple basic RIM-RS configurations. Among these multiple configurations, the ones for the “far” range can be configured with symbol offset within the TDD DL/UL period different than the symbol offset for the “near” range.

## Mapping rule for set ID

The mapping rule for set ID depends on whether a RIM-RS is transmitted by a victim (RS-1) or an aggressor (RS-2). It also depends on whether differentiating “Enough mitigation” and “Not enough” is enabled or not. We provide in Table 3 a mapping rule for set ID in addition to the mapping rules for RS-1/RS-2 and mitigation sufficiency indication. We remark that the mapping rule for set ID requires knowledge of T', NF, S1, R1, S2 and R2 but the mapping rule for RS-1/RS-1 only requires knowledge of T' whereas the mapping rule for mitigation sufficiency indication does not require knowledge of any of those parameters.

Table 3. Mapping rules for set ID, RS-1/RS-2 and mitigation sufficiency.

|  | Differentiate “Enough mitigation” and “Not enough”? | Set ID | RS-1/RS-2 | Mitigation sufficiency |
| --- | --- | --- | --- | --- |
| If it/T' = 0 | If OFF | if⋅NF ⋅S1+itR1⋅S1+ is | RS-1 (transmitted by victim) | N/A |
|  | If ON | if⋅NF ⋅S12+itR1⋅S12+ is' where is'=⌊is/2⌋, if LSB of the sequence index is indicates the mitigation sufficiency (see Note 1). | RS-1 (transmitted by victim) | LSB of is: “Not enough” if (is mod 2) = 0. “Enough” if (is mod 2) = 1. |
| If it/T' = 1 | N/A | if⋅NF ⋅S2+it-T'R2⋅S2+ is | RS-2 (transmitted by aggressor) | N/A |

Note 1: Alternatively, if the mitigation sufficiency is indicated by which half of the sequences is belongs to, i.e., “Not enough” if 2is/S1 = 0 and “Enough” if  2is/S1 = 1, then is'=is mod (S1/2).

The mapping rule for set ID shall be as described in Table 3.

## Initialization seeds for sequences

In the previous meeting, it was agreed that the initialization seed for RIM-RS is configured by OAM and the subsequent seed will be changing over time. We discuss next aspects regarding initialization seeds. The OAM shall configure each gNB with NS base initialization seeds: cinit,0, cinit,1, … , cinit,NS-1. The evolution of the actual initialization seeds depends on the base initialization seeds and time as follows:

cinit,k=cinit,k+fit,ntmod 231

where it is the time index within a RIM-RS transmission period and nt is the counter of the RIM-RS transmission period. For each basic RIM-RS resource, the sequence index is indicates that the initialization seed cinit,is is be used to generate the sequence for RIM-RS transmission. We remark that the base initialization seeds should be the same for all gNBs in the network, in order for a gNB to know the pool of sequences it should attempt to detect at any given time. We discuss next the design of fit,nt. If “no hopping” is configured, fit,nt = 0. If “hopping” is configured, fit,nt≠0 such that the following agreements [2,3] are satisfied:

- The maximum number of sequences that one gNB needs to detect in one DL-UL period for interference identification is 8 [RAN1#95].

- The maximum number of total RIM-RS sequences within 10 ms is 32 regardless how many DL/UL switching points are configured in the 10 ms [RAN1 AH1901].

To satisfy the first agreement, the number NS of sequence per TDD DL/UL period can be restricted:  NS≤8. To satisfy the second agreement, the time-dependent part fit,nt of the initialization seeds shall be in the form

fit,nt=f1it =itγ×η

where η is a constant, γ is the seed update period given by

γ=maxNSTTDD×10ms32,1

and TTDD is the TDD DL/UL period. For example, NS=8 and TTDD=2 ms, then γ=2 and

f1it,nt=it2×η.

The constant η determines how much of the bit space is allocated to the time dimension. For example, η=215 allocates 16 bits to the time dimension and 15 bits to the base initialization seed. The function fit,nt=f1it allows seed change over time with granularity of TDD DL/UL period. Another option is to change the seeds over time with granularity of RIM-RS transmission period, which can achieved by using

fit,nt=f2nt=ntγ×η

where η and γ are as described earlier. For example, suppose that the operator adopts Framework 1where one TDD DL/UL period is assigned for RS-1 of all gNBs and another TDD DL/UL period for RS-2 of all gNBs. In this case, if NS=8, TTDD=2 ms and Pt=4 ms, then γ=2.

All gNBs are configured with NS initialization seeds cinit,k=cinit,k+t/γ×ηmod 231 for  k = 0, …, NS-1 where t can be the time index within a RIM-RS transmission period or the counter of the RIM-RS transmission period, η is a constant determines how much of the bit space is allocated to the time dimension and  γ is the seed update period given by γ=maxNS⋅10ms/(32⋅TTDD),1.

# Conclusion

Our views on the remaining details of RIM-RS resource configuration are summarized by the following proposals:

- Define a RIM-RS resource configuration index IRIM-RS of size up to 24 bits to uniquely identify a basic RIM-RS resource in time-domain, frequency-domain and sequence.

RS-1 and RS-2 are differentiated in time domain such that RIM-RS with time index it<T' indicates RS-1, whereas RIM-RS with time index it≥T' indicates RS-2 where T' is configurable.

The least significant bit (LSB) of the sequence index is indicates mitigation sufficiency: (is mod 2) = 0 indicates “Not enough mitigation” whereas (is mod 2) = 1 indicates “Enough mitigation”, if the indication for “enough mitigation” or “not enough mitigation” is ON.

The number R1 of consecutive TDD DL/UL periods for repetitions/near-far of RS-1 shall be the same for all gNBs in the network. The number R2 of consecutive TDD DL/UL periods for repetitions/near-far of RS-1 shall be the same for all gNBs in the network. R1 and R2 may be different.

The mapping rule for set ID shall be as described in Table 3.

All gNBs are configured with NS initialization seeds cinit,k=cinit,k+t/γ×ηmod 231 for  k = 0, …, NS-1 where t can be the time index within a RIM-RS transmission period or the counter of the RIM-RS transmission period, η is a constant determines how much of the bit space is allocated to the time dimension and  γ is the seed update period given by γ=maxNS⋅10ms/(32⋅TTDD),1.

# References

[1] Chairman’s Note RAN1 AH1901, 2019.

[2] RP-182864, “Revised WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”, LG Electronics, RAN#82, Sorrento, Italy, December, 2018.

[3] Chairman’s Note RAN1#95, 2018.

[4] 3GPP TR 38.866 V1.0.0: “Study on remote interference management for NR”.