3GPP TSG RAN WG1 #96                                                                    R1-1902501

Athens, Greece, 25th February – 1st March, 2019

Source: 	Intel Corporation

Title:	Type II CSI feedback compression

Agenda item:	7.2.8.1

Document for:	Discussion and Decision

# Introduction

At the 3GPPRAN#81 meeting work item to specify enhancements for NR MIMO was approved. Objectives of the work item include the following enhancements to CSI for MU-MIMO [1].

- Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead

- Perform study and, if needed, specify extension of Type II CSI feedback to rank > 2

At the RAN1#95 [2] it was agreed to support DFT-based compression for Type II CSI as formulated in Alt 1.1 [3].

- Precoders for a layer is given by size-P×N3 matrix W=W1W2WfH - P=2N1N2= #SD dimensions - N3= #FD dimensions - FFS value and unit of N3 - Precoder normalization: the precoding matrix for given rank and unit of N3 is normalized to norm 1/sqrt(rank) - Spatial domain (SD) compression - L spatial domain basis vectors (mapped to the two polarizations, so 2L in total) selected - Compression in spatial domain using W1=v0v1…vL-100v0v1…vL-1, where vii=0L-1 are N1N2×1 orthogonal DFT vectors (same as Rel. 15 Type II) - Frequency-domain (FD) compression - Compression via Wf=Wf0,…,Wf2L-1 where Wf(i)=fki,0 fki,1…fki,Mi-1, where fki,mm=0Mi-1 are Mi size-N3×1 orthogonal DFT vectors for SD-component i=0,…,2L-1 - Number of FD-components {Mi} or i=02L-1Mi is configurable, FFS value range - FFS: choose one of the following alternatives - Alt1. common basis vectors: Wf=fk0 fk1…fkM-1, i.e. Mi=M ∀i and ki,mm=0Mi-1 are identical (i.e., ki,m=km, i=0,…,2L-1) - Alt2. independent basis vectors: Wf=Wf0,…,Wf2L-1, where Wf(i)=fki,0 fki,1…fki,Mi-1, i.e. Mi frequency-domain components (per SD-component) are selected - Note: kmm=0M-1 or ki,mm=0Mi-1,i=0,…,2L-1 are all selected from the index set {0,1,…,N3-1} from the same orthogonal basis group - FFS: If oversampled DFT basis or DCT basis is used instead of orthogonal DFT basis - FFS: Same or different FD-basis selection across layers - Linear combination coefficients (for a layer) - FFS if W2 is composed of K=2LM or K=i=02L-1Mi linear combination coefficients - FFS if only a subset K0<K of coefficients are reported (coefficients not reported are zero). - FFS if layer compression is applied so that i=02L-l-1Mi transformed coefficients are used to construct W2 for layer l (where the transformed coefficients are the reported quantity) - FFS quantization/encoding/reporting structure - Note: The terminology “SD-compression” and “FD-compression” are for discussion purposes only and are not intended to be captured in the specification

At the last RAN1 meeting [4] significant progress was achieved on support of DFT-based compression for Type II CSI. However, there are several unresolved issues including constraints for the smaller size of FD compression unit (R = 2), basis/coefficient subset selection for the first and second layers, range of supported codebook parameters, support of segmentation or padding for DFT size N3 > 13.

In this contribution we provide system level evaluation results and analysis on the open issues for Type II CSI with DFT-based FD compression.

# Discussion

## 2.1. Constraints for the smaller size of FD compression unit (R = 2)

At the last RAN1 meeting the following agreement was achieved on supported size of FD compression unit.

Agreement On FD compression unit, agree on Alt1 (PMI subband size = CQI subband size) as the default, along with Alt2.2 (PMI subband size = CQI subband size / R) as secondary - The value of R is fixed to 2 - FFS: Whether secondary implies a separate UE capability or restricted use cases - Include issues such as limitation on the number of FD compression units, CPU occupation, latency constraint and/or BW constraint - FFS: Whether FD compression unit is higher-layer configured or reported by the UE

As it is stated in the above agreement, smaller size of FD compression unit (R = 2) is supported as secondary value, where secondary implies a separate UE capability or restricted use cases. The main reasons why UE capability or restricted use case is needed for the case of smaller size of FD compression unit are complexity of PMI calculation and implementation complexity.

PMI search algorithm with R = 2 has multiple differences comparing to the case with R = 1. For example, smaller size of FD compression unit require modification of channel estimation on CSI-RS and CQI calculation algorithm since two different precoding matrixes should be used for calculation of CQI for a subband. Moreover, memory size for storage of some PMI components is increased for R = 2 comparing to R = 1 (e.g. memory for channel measurements and frequency-domain coefficients). Thus, implementation of PMI search algorithm with R = 2 require additional effort and design considerations comparing to the case with R = 1. Considering the above, separate UE capability is required for the smaller size of FD compression unit (R = 2).

Increased complexity of PMI calculation for the smaller size of FD compression unit (R = 2) translates to higher required computational power at the UE or higher required CSI calculation time. Since increase of total UE computation power is not desirable due to UE cost and power consumption constraints, additional limitations are needed in order to facilitate implementation of PMI search with R = 2. Total computation power at the UE is limited by the number of CSI processing units (CPUs), while the amount of computational power allocated to a CSI report corresponds to the number of CSI procession units (CPUs) occupied by the CSI report. So, in order to consider increased complexity for PMI calculation with R = 2, number of CPUs allocated to CSI report with R = 2 should be increased to 2.

Proposal 1:

- Define separate UE capability to indicate if CSI reporting with smaller size of FD compression unit (R = 2) is supported by the UE

- CSI report with smaller size of FD compression unit (R = 2) occupies 2 CSI processing units

## 2.2. Range of supported codebook parameters

At the last RAN1 meeting [4] it was agreed to support common selection for M-size subset of FD-components for all the 2L beams/polarizations with further downselection of K0 < 2LM coefficients. The following agreement was made on the values of codebook parameter M.

Agreement Two values of M are supported. In RAN1#96, down select between the following alternatives (p∈14,12): - Alt1. M=p×N3R - Alt2. M=p×N3 - FFS: support for p=1/8 and/or p=3/4 in addition to 1/4 and 1/2

There are two alternatives identified for the calculation of M based on DFT size (N3). The number of bits required for PMI reporting depends on value of parameter M. Thus, Alt2 assumes higher overhead comparing to Alt1 for R = 2 and for a given value of parameter p. In our view Alt1 should be supported in order to guarantee similar overhead for R = 1 and R = 2.

At the last RAN1 meeting it was further agreed that K0 is calculated from the value of M using equation, where two values from the set {1/8, 1/4, 1/2, 3/4} are supported for parameter β. In order to downselect values of parameters p and β system level evaluation were carried out for Dense Urban scenario with 16 Tx antennas at the gNB. Evaluation results are presented in figure 1 and figure 2 for DFT-based Type II CSI compression with L = 4, R = 1, p = {1/8, 1/4, 1/2, 3/4} and β = {1/8, 1/4, 1/2, 3/4}, evaluation results for Rel. 15 Type I and Type II are presented for reference. The detailed evaluation assumptions can be found in the Appendix.

Figure 1. Average packet throughput for different values of p and β

Figure 2. Cell-edge packet throughput for different values of p and β

As it can be observed based on the above evaluation results, performance is slightly improved for p > 1/2 and β  > 1/2; significant performance degradation is observed for p < 1/4 and β  < 1/4. Considering the above evaluation results, we propose to support p = {1/4, 1/2} and β = {1/4, 1/2}. Since the performance is similar for (p, β) = (1/4, 1/2) and (1/2, 1/4), one combination can be downselected to further reduce number of codebook configurations.

Observation 1:

- Performance is slightly improved for p > 1/2 and β  > 1/2

- Significant performance degradation is observed for p < 1/4 and β  < 1/4

Proposal 2:

- Support p = {1/4, 1/2} and β = {1/4, 1/2}

  - FFS: downselection of one combination for (p, β) from {(1/4, 1/2), (1/2, 1/4)}

## 2.3. Segmentation or padding for N3 > 13

At the last RAN1 meeting the following agreement was achieved for supported values of DFT size (N3).

Agreement Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R≤19, Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R>19, downselect among the following alternatives in RAN1#96 - Alt1: N3N3 is smallest multiple of 2, 3, or 5 which is ≥NSB×R - Alt2: N3N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

In order to compare performance of padding and segmentation system level evaluations were carried out for Dense Urban scenario with 16 Tx antennas at the gNB for 20 MHz and 50 MHz bandwidth with 30 kHz subcarrier spacing. Evaluation results are presented in figures 3, 4, 5, 6 for DFT-based Type II CSI compression with L = 4, R = 2, p = {1/4, 1/2} and β = {1/4, 1/2}, evaluation results for Rel. 15 Type I and Type II are presented for reference. The detailed evaluation assumptions can be found in the Appendix. Values of CSI configuration parameters considered for evaluations are presented in table 1.

Table 1. Values of CSI configuration parameters

|  | 20 MHz bandwidth |  |  | 50 MHz bandwidth |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | No padding | Padding | Segmentation | No padding | Padding | Segmentation |
| Number of subbands | 13 | 13 | 13 | 17 | 17 | 17 |
| N3 (for one segment) | 26 | 27 | 15 | 34 | 36 | 18 |
| M (for one segment) | {4, 4, 7, 7} | {4, 4, 7, 7} | {2, 2, 4, 4} | {5, 5, 9, 9} | {5, 5, 9, 9} | {3, 3, 5, 5} |
| K0 (for one segment) | {8, 16, 14, 28} | {8, 16, 14, 28} | {4, 8, 8, 16} | {10, 20, 18, 36} | {10, 20, 18, 36} | {6, 12, 10, 20} |

Figure 3. Average packet throughput for padding and segmentation with 20 MHz band

Figure 4. Cell-edge packet throughput for padding and segmentation with 20 MHz band

Figure 5. Average packet throughput for padding and segmentation with 50 MHz band

Figure 6. Average packet throughput for padding and segmentation with 50 MHz band

As it can be seen from the above evaluation results, padding or segmentation of FD coefficients provide similar performance comparing to the case without padding (with DFT size equal to the number of FD compression units). Type II CSI DFT-based compression with segmentation does not provide performance gains over other cases while it has slightly higher overhead.

Observation 2:

- Padding or segmentation of FD coefficients provide similar performance comparing to the case without padding (with DFT size equal to the number of FD compression units)

- Type II CSI DFT-based compression with segmentation does not provide performance gains over other cases while it has slightly higher overhead

Proposal 3:

- Support padding of FD coefficients to smallest multiple of 2, 3, or 5 which is ≥ NSB×R for N3 > 13

## 2.4. Number of SD vectors

At the last RAN1 meeting [4] the following agreement was made.

Agreement On the values of L, support L={2,4} - Decide whether to support L=3 and/or L=6 in the future meetings considering the performance-overhead trade-off for different RI values and/or different number of antenna ports

For Rel. 15 Type II CSI the maximum value of SD vectors in linear combination is L = 4 since Rel. 15 Type II CSI reporting with larger values of L provides increased overhead. For Rel. 16 Type II CSI with DFT-based FD compression it is possible to support larger values of L with considerable overhead comparing to Rel. 15 Type II CSI. However, support of larger number of SD vectors in linear combination leads to increased UE complexity due to increased size of matrix with FD coefficients. Considering the above, introduction of larger value of L should be justified by evaluation results.

In order to compare performance of Type II CSI with DFT-based FD compression with different number of SD vectors L system level evaluations were carried out for Dense Urban scenario with 16 Tx antennas at the gNB. Evaluation results are presented in figures 7, 8 for DFT-based Type II CSI compression with L = 4 and L = 6, R = 1, p = {1/8, 1/4, 1/2} and β = {1/4, 1/2}, evaluation results for Rel. 15 Type I and Type II are presented for reference. The detailed evaluation assumptions can be found in the Appendix.

Figure 7. Average packet throughput for different number of SD vectors L

Figure 8. Cell-edge packet throughput for different number of SD vectors L

As it can be observed from the above results, Type II CSI with DFT-based FD compression with L = 6 provides similar performance/overhead tradeoff comparing to the case with L = 4. Considering increased PMI search complexity for the case with L = 6 we propose to support L = {2, 3, 4} as for Rel. 15 Type II CSI.

Observation 3:

- Type II CSI with DFT-based FD compression with L = 6 provides similar performance/overhead tradeoff comparing to L = 4

Proposal 4:

- Support L = {2,3,4} for Rel. 16 Type II CSI with DFT-based FD compression as for Rel. 15 Type II CSI

## 2.5. Basis subset selection across layers and polarization

At the last RAN1 meeting [4] it was agreed to support common selection for M-size subset of FD-components for all the 2L beams with further downselection of K0 < 2LM coefficients for one layer. For two layers it is possible to do common or separate selection for M-size subset of FD-components and for downselection of K0 coefficients. Moreover, it is possible to further reduce overhead by doing common selection of K0 coefficients across different polarizations.

In order to compare different alternatives on basis subset selection across layers and polarizations system level evaluations were carried out for Dense Urban scenario with 16 Tx antennas at the gNB. Evaluation results are presented in figures 9, 10 for DFT-based Type II CSI compression with L = 4, R = 1, p = {1/4, 1/2} and β = {1/4, 1/2}, evaluation results for Rel. 15 Type I and Type II are presented for reference. The detailed evaluation assumptions can be found in the Appendix. Considered alternatives on basis subset selection across layers and polarizations are presented in table 2.

Table 2. Basis subset selection across layers and polarizations

|  | M-size subset selection across layer | Selection of K0 coefficients across layers | Selection of K0 coefficients across polarizations |
| --- | --- | --- | --- |
| Alt1 | Common | Common | Common |
| Alt2 | Common | Common | Separate |
| Alt3 | Common | Separate | Common |
| Alt4 | Common | Separate | Separate |
| Alt5 | Separate | Separate | Common |
| Alt6 | Separate | Separate | Separate |

Figure 9. Average packet throughput for different alternatives of basis subset selection across layers and polarization

Figure 10. Cell-edge packet throughput for different alternatives of basis subset selection across layers and polarization

As it can be observed from the above evaluation results the performance of different alternatives of basis subset selection across layers and polarization is similar while common subset selection for M-size subset of FD-components across layers and common selection of K0 coefficients across layer and polarizations provide smaller overhead.

Observation 4:

- The performance of different alternatives of basis subset selection across layers and polarization is similar

- Common subset selection for M-size subset of FD-components across layers and common selection of K0 coefficients across layer and polarizations provide smaller overhead

Proposal 5:

- Support common subset selection for M-size subset of FD-components across layers and common selection of K0 coefficients across layer and polarizations

## 2.6. Quantization

There are three different alternatives identified for the quantization of amplitude of coefficients. The first alternative assumes direct quantization of amplitude relative to the strongest coefficient in W2 matrix without additional operations. Second alternative is differential quantization of amplitude across SD vectors (quantization of amplitude relative to the the strongest coefficient in W2 matrix and relative to the strongest coefficient for the SD vector), the strongest coefficient for the SD vector is reported separately. Third alternative of amplitude quantization, also referred as ABC approach, assumes that amplitudes are derived from the product of three matrixes ABC, where A and B are diagonal matrixes. Third alternative is described in [5] in more details.

In order to compare different alternatives on amplitude quantization system level evaluations were carried out for Dense Urban scenario with 16 Tx antennas at the gNB. Evaluation results are presented in figures 11 and 12 for DFT-based Type II CSI compression with L = 4, p = {1/4, 1/2} and β = {1/4, 1/2}, evaluation results for Rel. 15 Type I and Type II are presented for reference. The detailed evaluation assumptions can be found in the Appendix. The following alternatives were considered for amplitude quantization.

- Alt 1: Direct quantization of amplitude and phase, 3 bits for amplitude and 3 bits for phase

- Alt 2a: Differential quantization of amplitude and direct quantization of phase, 4 bits for amplitude of the strongest coefficient for each SD vector, 3 bits for differential amplitude and 3 bits for phase

- Alt 2b: Differential quantization of amplitude and direct quantization of phase, 3 bits for amplitude of the strongest coefficient for each SD vector, 2 bits for differential amplitude and 3 bits for phase

Figure 11. Average packet throughput for different alternatives of amplitude quantization

Figure 12. Cell-edge packet throughput for different alternatives of amplitude quantization

As it can be observed from the above evaluation results, Alt 1 provides similar performance comparing to Alt 2a and Alt 2b. Since Alt 1 corresponds to the simplest quantization scheme we propose to support Alt 1 for amplitude quantization.

Observation 5:

- Direct quantization of amplitude provides similar performance comparing to differential quantization of amplitude

Proposal 6:

- Support direct quantization of amplitude

Another issue related to quantization is alphabet for quantization of phase. For Rel. 15 Type II CSI alphabet size for quantization of phase of phase is configurable: 2 bits (QPSK) and 3 bits (8-PSK). Since the overhead of Type II CSI with DFT-based FD compression is reduced comparing to Rel. 15 Type II CSI, there is an opportunity to increase number of bits for quantization of phase in order to improve performance.

In order to evaluate performance of phase quantization with different alphabet size system level simulations were carried out for Dense Urban scenario with 16 Tx antennas at the gNB. Evaluation results are presented in figures 13 and 14 for DFT-based Type II CSI compression with L = 4, R = 1, p = {1/4, 1/2} and β = {1/4, 1/2}, evaluation results for Rel. 15 Type I and Type II are presented for reference. The detailed evaluation assumptions can be found in the Appendix.

Figure 13. Average packet throughput for different number of bits for phase quantization

Figure 14. Cell-edge packet throughput for different number of bits for phase quantization

As it can be observed from the above evaluation results, QPSK phase quantization provides worse performance-overhead trade-off comparing to 8-PSK. 16-PSK phase quantization doesn’t provide significant performance gains over 8-PSK quantization while has higher overhead.

Observation 6:

- QPSK phase quantization provides worse performance-overhead trade-off comparing with 8-PSK

- 16-PSK phase quantization doesn’t provide significant performance gains over 8-PSK quantization while has higher overhead

Proposal 7:

- Support 8-PSK phase quantization

## 2.7. Reporting of coefficients associated with the strongest beam/polarization

In Rel. 15 Type II CSI coefficients corresponding to the strongest beam/polarization are not reported; it is a priory assumed that coefficients corresponding to the strongest beam/polarization are equal to one. Similar approach can be used for the agreed Type II CSI feedback compression.

As it is pointed out in [6], uncertainty of unit-phase factor for a SVD vector should be considered at the UE side in order to avoid phase discontinuity in frequency domain. Phase correction can be done by multiplying SVD vector by the complex conjugate of the entry corresponding to the strongest beam/polarization with the amplitude normalized to one. If such phase correction is applied, phase of all FD coefficients associated with the strongest beam/polarization is constant and equal to zero, thus it can be apriory assumed that FD coefficients associated with the strongest beam/polarization are real values. According to evaluation results provided in [7] it is valid to assume that FD coefficients associated with the strongest beam/polarization are constant real values equal to 1.

Proposal 8:

- Study reporting of coefficients associated with the strongest beam/polarization considering assumption of constant phase and/or amplitude

# Conclusion

In this contribution enhancements to CSI for MU-MIMO are discussed including overhead reduction for Type II CSI and the support of higher ranks for Type II CSI. The following proposals and observations were made.

Proposal 1:

- Define separate UE capability to indicate if CSI reporting with smaller size of FD compression unit (R = 2) is supported by the UE

- CSI report with smaller size of FD compression unit (R = 2) occupies 2 CSI processing units

Observation 1:

- Performance is slightly improved for p > 1/2 and β  > 1/2

- Significant performance degradation is observed for p < 1/4 and β  < 1/4

Proposal 2:

- Support p = {1/4, 1/2} and β = {1/4, 1/2}

  - FFS: downselection of one combination for (p, β) from {(1/4, 1/2), (1/2, 1/4)}

Observation 2:

- Padding or segmentation of FD coefficients provide similar performance comparing to the case without padding (with DFT size equal to the number of FD compression units)

- Type II CSI DFT-based compression with segmentation does not provide performance gains over other cases while it has slightly higher overhead

Proposal 3:

- Support padding of FD coefficients to smallest multiple of 2, 3, or 5 which is ≥ NSB×R for N3 > 13

Observation 3:

- Type II CSI with DFT-based FD compression with L = 6 provides similar performance/overhead tradeoff comparing to L = 4

Proposal 4:

- Support L = {2,3,4} for Rel. 16 Type II CSI with DFT-based FD compression as for Rel. 15 Type II CSI

Observation 4:

- The performance of different alternatives of basis subset selection across layers and polarization is similar

- Common subset selection for M-size subset of FD-components across layers and common selection of K0 coefficients across layer and polarizations provide smaller overhead

Proposal 5:

- Support common subset selection for M-size subset of FD-components across layers and common selection of K0 coefficients across layer and polarizations

Observation 5:

- Direct quantization of amplitude provides similar performance comparing to differential quantization of amplitude

Proposal 6:

- Support direct quantization of amplitude

Observation 6:

- QPSK phase quantization provides worse performance-overhead trade-off comparing with 8-PSK

- 16-PSK phase quantization doesn’t provide significant performance gains over 8-PSK quantization while has higher overhead

Proposal 7:

- Support 8-PSK phase quantization

Proposal 8:

- Study reporting of coefficients associated with the strongest beam/polarization considering assumption of constant phase and/or amplitude

# References

- RP-181453 WI Proposal on NR MIMO Enhancements, Samsung

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #95, Spokane, USA, November 12th – 16th, 2018

- R1-1813002 Summary of CSI enhancement for MU-MIMO support, Samsung

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting Ad-Hoc Meeting 1901, Taipei, Taiwan, 21st – 25th January, 2019

- R1-1900360 Enhancements on Type-II CSI reporting, Fraunhofer IIS, Fraunhofer HHI

- R1-1813663 Phase Randomization and Correction for CSI quantization in frequency domain, Huawei, HiSilicon

- R1-1900501 Type II CSI feedback compression, Intel Corporation

# Appendix A

Table. Evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban (Macro only) |
| Layout | Hexagonal Grid with 2 tiers |
| ISD | 200 m |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 10 MHz with 15 kHz subcarrier spacing, 52 PRB |
| Tx power | 41 dBm |
| UE distribution | Uniform 20% outdoor (30 km/h), 80% indoor (3 km/h) |
| UE antenna configuration | 2 Rx X-pol, slant 0/90 degrees |
| BS antenna configuration | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Traffic model | FTP 1 with 0.5 Mbytes packet size, high traffic load with ~70% resource utilization |
| TRP association | RSRP based Handover margin = 0 dB |
| Transmission mode | MU-MIMO with 8 BS layers maximum; Rank adaptation with max rank 2 |
| Scheduling | Proportional Fair |
| OLLA | 10% BLER target |
| MU-MIMO precoding | MMSE |
| Elevation beamforming | One vertical beam per TXRU electrically down-tilted to 100 degrees |
| UE receiver | MMSE-IRC |
| UE noise figure | 9 dB |
| Max number of HARQ transmissions | 4 |
| Coefficients quantization | Rel. 15 Type II CSI: - QPSK/8-PSK + WB/WB+SB amplitude Type Ii CSI with DFT-based FD compression: - Direct quantization of phase (3 bits) and amplitude (3 bits) |