3GPP TSG RAN WG1 #96	R1-1902579

Athens, Greece, 25th February – 1st March, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	On Uu-based sidelink resource allocation/configuration

Agenda item:	7.2.4.3

Document for:	Discussion and Decision

# Introduction

At RAN#80, a study item “Study on NR V2X” (FS_NR_V2X) was approved ‎[1].

One of the objectives deals with enhancements of LTE Uu and NR Uu to control NR sidelink:

3: Uu-based sidelink resource allocation/configuration (LTE V2X Mode3-like and Mode4-like) [RAN1, RAN2]:

- Identify necessary enhancements of LTE Uu and NR Uu to control NR sidelink from the cellular network

- Identify necessary enhancements of NR Uu to control LTE sidelink from the cellular network

The following agreements have been reached at RAN1#94 Error! Reference source not found.:

Agreements:

- NR Uu can assign NR sidelink resources for the following:

  - Shared licensed carrier between Uu and NR sidelink

  - Dedicated NR sidelink carrier

Agreements:

- Study at least the following NR sidelink resource allocation techniques:

  - Dynamic resource allocation

  - Activation/deactivation based

    - E.g., semi-persistent scheduling allocation or NR grant free type-2

  - RRC (pre-)configured

    - E.g., configured NR grant type-1, UE autonomous selection of resource(s) from resources configured by RRC

- RAN1 will study the level of network control, e.g., whether the UE may select other parameters (e.g., MCS) and/or the exact transmission resources, and whether the selection is autonomous or not

The following agreements have been reached at RAN1#94bis [4]

Agreements:

Continue studying NR sidelink resource allocation techniques by NR Uu for mode-1:

- Dynamic resource allocation

- Semi-persistent scheduling allocation or NR grant type-2 (activation/de-activation by physical layer signaling)

- Grant free transmission i.e., configured NR grant type-1

Agreements:

- Study further which resources to use for SL transmission and other network-control sidelink issues (e.g., power control) in the case of shared carrier

The following agreements have been reached at RAN1#95 [5]:

Agreements:

The following NR sidelink resource allocation techniques by NR Uu for mode-1 are supported:

- Dynamic resource allocation

- Configured grant.

  - FFS whether type-1 and/or type-2

Agreements:

- In continuing evaluating NR Uu scheduling of LTE sidelink mode-3, consider at least:

  - What will be required on the UE side to support such feature

  - DCI design (e.g., whether DCI 5A can be reused)

  - Deployment scenarios where it is beneficial

The following agreements have been reached at RAN1#AH_1901 [6]:

Agreements:

- When NR Uu schedules NR SL mode 1, both type 1 and type 2 configured grants are supported for NR SL

Agreements:

- LTE Uu to schedule NR sidelink mode 1 is supported:

  - The support is done based on type 1 configured grant with configuration restricted to time/frequency resources & periodicity, with the condition that no additional function/procedure is to be introduced for LTE Uu

  - Both DCI based scheduling and type 2 configured grant scheduling are not supported for scheduling NR sidelink mode 1

- Send an LS to RAN2 – Phillipe (HW), R1-1901410, which is approved with the final LS in R1-1901445

# Discussion

## UE assistance to gNB

To assist the eNB in its scheduling decisions, LTE-based V2X supports reporting of geo-information by the UE to the eNB; RAN1#84 reached the following conclusion and agreement:

Conclusion:

- Geo-information (e.g. vehicle location) signaled in the radio layers can be used by eNB, e.g., for sidelink resource allocation.

  - This does not preclude reporting any other information to eNB.

  - Note: Using geo-information at the transmitter UE for UE autonomous resource selection is already agreed.

Agreements:

- Mechanisms to report UE geographical information to the eNB are supported.

  - FFS the protocol and exact content of the report

  - FFS whether the report is carried as L1 control information (in which case it is FFS which physical channel(s) carry such information) or L2/3 control information (e.g, MAC or RRC signaling).

The same considerations apply to NR-based V2X and further enhancements compared to the LTE solution are possible; hence we propose:

Proposal 1: Support reporting of geo-information (such as vehicle location and speed/direction) by the UE to the gNB.

Note: A similar agreement was reached for V2X over Uu in RAN1#94bis.

On the topic of UE assistance to gNB’s scheduling decisions, another piece of information that can help improve efficiency of resource utilization is the “target geographical area” of the UE’s intended sidelink transmission; if two UEs target their transmissions to disjoint geographical areas then the gNB can schedule these transmissions on the same time/frequency resources.

Proposal 2: Support reporting of target geographical area by the UE to the gNB.

## Resource pool sharing between modes

When LTE V2X was designed in Rel-14, radio resource pool sharing between UEs using mode 3 and UEs using mode 4 was not considered. In the Rel-15 LTE_eV2X WI an objective was included to support such resource pool sharing; having to deal with legacy Rel-14 UEs made realization of this objective more difficult. Hence we propose:

Proposal 3: Support resource pool sharing between mode 1 and mode 2 from the beginning.

# Conclusions

In this contribution we discussed Uu-based sidelink resource allocation/configuration and make the following proposals:

Proposal 1: Support reporting of geo-information (such as vehicle location and speed/direction) by the UE to the gNB.

Proposal 2: Support reporting of target geographical area by the UE to the gNB.

Proposal 3: Support resource pool sharing between mode 1 and mode 2 from the beginning.

# References

- RP-182491, “Revised SID: Study on NR V2X”

- TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR

- RAN1#94 chairman notes

- RAN1#94bis chairman notes

- RAN1#95 chairman notes

- RAN1 AdHoc 1901 chairman notes