3GPP TSG RAN WG1 #96											  R1-1902599

Athens, Greece, 25th February – 1st March, 2019

Agenda Item: 	7.2.4.1.5

Source:	InterDigital Inc.

Title:	On Waveform for NR V2X Sidelink

Document for:	Discussion and Decision

# Introduction

In the WG1 #95 meeting, the following were agreed on for the physical layer structure [1].

Agreement:

- At least CP-OFDM is supported.

- Continue study on whether to support DFT-S-OFDM including the potential issues and the following potential benefit:

  - Synchronization coverage enhancement

  - PSCCH coverage enhancement, e.g., with Option 2 of PSCCH/PSSCH multiplexing with the restriction that PSCCH and PSSCH use adjacent frequency resources

  - Feedback channel coverage enhancement

- A single waveform is used in all the sidelink channels in a carrier.

  - Note: A sequence based channel can be supported in any waveform.

  - (Pre-)configuration will be used to determine the used waveform if the specification supports multiple waveforms.

It is well known that DFT-s-OFDM has lower PAPR compared to CP-OFDM, resulting in smaller back-off and less non-linear distortion when a power amplifier (PA) is used. In this contribution, we discuss on the benefit of supporting DFT-s-OFDM waveform and evaluate the performance of CP-OFDM and DFT-s-OFDM waveforms with a polynomial PA model [2] incorporated into the simulation.

# Evaluation Results

The BLER performance of CP-OFDM and DFT-s-OFDM are compared using the simulation assumptions given in the Appendix. Specifically, the BLER is evaluated when there is no PA and when the target output power of the PA is set to one of 25 dBm, 26 dBm, or 27 dBm. Two different allocation sizes of 5 and 15 RBs are used, and the relative speed is set to 30 km/h or 280 km/h.

Figure 2-1 and Figure 2-2 illustrate the BLER for 5 and 15 RBs with 30km/h UE speed. In addition, Table 1 provides the gain of DFT-s-OFDM over CP-OFDM at 10-1 BLER level. The results show that CP-OFDM outperforms DFT-s-OFDM without PA. But with PA, the gain of DFT-s-OFDM increases with increasing output power, reaching to 3.5 dB when the target output power is set to 27 dBm. We can also see that the gain is slightly less with a larger allocation size because CP-OFDM provides higher coding gain in a more selective channel.

Figure 2-1 BLER comparison with 30 km/h and 5 RBs

Figure 2-2 BLER comparison with 30 km/h and 15 RBs

Table 1 Gain (dB) of DFT-s-OFDM over CP-OFDM at 10-1 BLER

| Allocation size | Target output power |  |  |  |
| --- | --- | --- | --- | --- |
|  | No PA | 25 dBm | 26 dBm | 27 dBm |
| 5 RB | -0.9 | 0.7 | 2.1 | 3.5 |
| 15 RB | -1.4 | 0.34 | 1.8 | 3.5 |

Figure 2-3 and Figure 2-4 illustrate the BLER for 5 and 15 RBs with UE speed 280km/h while Table 2 provides the gain of DFT-s-OFDM over CP-OFDM at 10-1 BLER level. The results show that CP-OFDM outperforms DFT-s-OFDM without PA. As the output power is increased, however, the gain of DFT-s-OFDM increases, reaching to about 3 dB when the target output power is set to 27 dBm. It is noticeable that the gain of DFT-s-OFDM is reduced slightly with the increased Doppler.

Figure 2-3 BLER comparison with 280 km/h and 5 RBs

Figure 2-4 BLER comparison 280 km/h and 15 RBs

Table 2 Gain (dB) of DFT-s-OFDM over CP-OFDM at 10-1 BLER (280 km/h)

| Allocation size | Target output power |  |  |  |
| --- | --- | --- | --- | --- |
|  | No PA | 25 dBm | 26 dBm | 27 dBm |
| 5 RB | -1.5 | 0.1 | 1.4 | 2.9 |
| 15 RB | -1.6 | -0.6 | 1.4 | 3.3 |

Observation -1: The results presented above show that the performance of DFT-s-OFDM can offer much better tolerance to nonlinear PA and, therefore, it can provide significantly larger coverage by using higher Tx power than CP-OFDM due to smaller PAPR.

# Discussion

In NR Uu, both DFT-s-OFDM and CP-OFDM are supported to meet the various throughput and coverage requirements. SA identified 25 use cases for NR V2X which includes different latency, reliability and range requirements [3]. The performance of DFT-s-OFDM and CP-OFDM has been evaluated in 3GPP since LTE Rel-8 and RAN1 concluded to support DFT-s-OFDM for uplink for both LTE and NR at least for uplink coverage extension purposes. Given that NR V2X use cases also have various range requirements, supporting DFT-s-OFDM for NR V2X seems obvious in terms of the performance benefit. For the NR V2X use cases requiring high throughput performance, CP-OFDM could be used to enable efficient MIMO operation when the range is short enough.

Observation-2: in some NR V2X scenarios which requires a wide range requirement and high reliability, DFT-s-OFDM provides benefits

Supporting two waveforms may increase implementation complexity. However, based on what NR V2X use case is mainly used a NR V2X UE, the benefit of supporting additional DFT-s-OFDM waveform could largely compensate the increased implementation complexity. Note that some of NR V2X use cases require high reliability even in high mobility case and DFT-s-OFDM waveform easily provides additional few dB coverage gain over CP-OFDM. Based on a target use case, a NR V2X UE may support a subset of identified use cases. Although unified design simplifies specification, still optional support of DFT-s-OFDM waveform for better coverage seems to be beneficial. In addition, to support Mode-1, a NR V2X UE should support DFT-s-OFDM waveform anyhow for uplink. Therefore, the additional complexity is mainly for the receiver design to support DFT-s-OFDM. Furthermore, if a V2X UE implemented both NR based sidelink and LTE based sidelink, additional support of DFT-s-OFDM waveform for NR based sidelink can be easily implemented.

Observation-3: DFT-s-OFDM waveform can be supported optionally based on the V2X use cases for a UE

# Conclusion

In this contribution, we provided an evaluation of CP-OFDM and DFT-s-OFDM waveforms using link level BLER results that incorporated the impact of a nonlinear PA. Also, the benefit of supporting DFT-s-OFDM was discussed. Based on the results and discussions, we propose the following:

Proposal 1: DFT-s-OFDM is supported in NR V2X sidelink as an optional feature.

# References

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #95 Meeting, Spoken, USA, Nov. 2018

- R1-165006 “On the evaluation of PA model,” 3GPP TSG RAN WG1 #85 Nanjing, China, 23th – 27th May 2016

- TS 22.186, “5G; Service requirements for enhanced V2X scenarios”

# Appendix

Table 3 Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Bandwidth | 5 RB, 15 RB |
| Channel | CDL 100A |
| MCS | QPSK, ½ code rate |
| Waveform | CP-OFDM, DFT-s-OFDM |
| Subcarrier Spacing | 30 kHz |
| Slot format | DMRS are mapped to every other subcarrier |
| CP length | Normal CP |
| Frequency synchronization error | Not modeled |
| Time synchronization error | Not modeled |
| UE relative speed | 30 km/h, 280 km/h |
| Channel estimation | Realistic |
| UE receiver algorithm | MMSE |
| PA MODEL | Polynomial [2], target output power set to 25, 26, 27 dBm |
| Number of Tx/Rx antennas | 1 Tx, 2Rx |